IEEE 802.11

Wireless Access Method and Physical Specification

Title:	FH PHY Proposed Revisions to Section 10 of D1.1

Date:	May 26, 1995

Author:	Dean Kawaguchi	Ed Geiger

Symbol Technologies, Inc.	Apple Computer, Inc.

2145 Hamilton Ave.	One Infinite Loop  M/S 301-4J

San Jose, CA.  95125	Cupertino, CA.  95014

Voice:  (408)369-2629	Voice: (408)974-4907

FAX:  (408)446-4630	FAX: (408)974-9793

email:  DeanK@psd.symbol.com	email: edg@apple.com

This submission documents the FH PHY subgroup’s proposed changes to Section 10 of the D1.1 draft.  The changes include both editorial changes as implemented by the listed authors above and technical changes as voted and approved in the FH PHY subgroup sections at the May 1995 plenery.  Some portions have not been addressed, either editorially or technically, and some of the technical changes voted in the May meeting have not been implemented due to time constraints.  All effort has been made to implement the intent of the technical change decisions made in the sub-group meeting in the cases that lacked specific change text at the time of the meeting.  As stated before, the intent of this interim draft is not to get it perfect, but to get it closer to the final version.  If there a few points which have not been addressed to your satisfaction, you are invited to bring your comments to the next meeting.  However, if in your opinion, the section is an improvement over the previous version, the authors recommend that you vote affirmatively so that progress may be made in the next meeting.

# 1

# 2

# 3

# 4

# 5

# 6

# 7

# 8

# 9

# µ10	2.4 GHz ISM Band Frequency Hopping Spread Spectrum Physical Layer Specification

## 11 Introduction

### 12 Introduction to FHSS Physical Layer

This section describes the physical layer services provided to the 802.11 Wireless LAN MAC for the 2.4 GHz Frequency Hopping Spread Spectrum (FHSS) system.  The FHSS physical layer consists of the following two protocol functions:

a)	A physical layer convergence function which adapts the capabilities of the physical medium dependent system into the Physical Layer service.  This function is supported by the Physical Layer Convergence Procedure (PLCP) which defines a method of mapping the 802.11 MAC layer Protocol Data Units (MPDU) into a framing format suitable for sending and receiving user data and management information between two or more stations using the associated physical medium dependent system.

b)	A Physical Medium Dependent (PMD) system whose function defines the characteristics of, and method of transmitting and receiving data via a wireless media between two or more stations.

### 13 FHSS Physical Layer Functions

The 2.4 GHz Frequency Hopping Spread Spectrum Physical Layer architecture is shown in Figure 10-1.  The  FHSS physical layer contains three functional entities:  the physical medium dependent function, the physical layer convergence function, and the physical layer management function.  Each of these functions is described in detail in the following subsections.

The FHSS Physical (PHY) Layer service is provided to the Media Access Control entity at the station through a PHY Service Access Point (SAP) as shown in Figure 10-1 called the PHY_SAP.  A set of primitives will also be defined to describe the interface between the physical layer convergence protocol sublayer and the physical medium dependent sublayer called the PMD_SAP.

#### 14 Physical Layer Convergence Procedure Sublayer

In order to allow the 802.11 MAC to operate with minimum dependence on the PMD sublayer, a physical layer convergence sublayer is defined.  This function simplifies a physical layer service interface to the 802.11 MAC services.

#### 15 Physical Layer Management Entity (LME)

The Physical LME performs management of the local Physical Layer Functions in conjunction with the MAC Management entity.

#### 16Physical Medium Dependent Sublayer

The physical medium dependent sublayer provides a transmission interface used to send or receive data between two or more stations.

µ §

Figure 10-1: Protocol Reference Model

### 17 Service Specification Method and Notation

The models represented by state diagrams in the following sections are intended as the primary specifications of the functions provided.  It is important to distinguish, however, between a model and a real implementation.  The models are optimized for simplicity and clarity of presentation, while any realistic implementation may place heavier  emphasis on efficiency and suitability to a particular implementation technology.

The service of a layer or sublayer is the set of capabilities that it offers to a user in the next higher layer (or sublayer).  Abstract services are specified here by describing the service primitives and parameters that characterize each service.  This definition of service is independent of any particular implementation.

## 18 FHSS PHY Specific Service Parameter Lists

### 19 Introduction

The architecture of the 802.11 MAC is intended to be physical layer independent.  Some physical layer implementations require medium management state machines running in the media access sublayer in order to meet certain PMD requirements.  These physical layer dependent MAC state machines reside in a sublayer define as the MAC Layer Management Entity (LME).  The MAC LME in certain PMD implementations may need to interact with the Physical LME as part of the normal PHY_SAP primitives.  These interactions are defined by the Physical Layer Management Entity parameter list currently defined in the PHY Service Primitives as TXVECTOR and RXVECTOR.  The list of these parameters and the values they may represent are defined in the specific physical layer specifications for each PMD.  The following section addresses the PLME_TXVECTOR and RXVECTOR  for the FHSS PHY.

All of the values included in the TXVECTOR or RXVECTOR  described in this section are considered mandatory unless otherwise specified.

### 20 TXVECTOR Parameters

The following parameters are defined as part of the TXVECTOR parameter list in the PHY_DATA.request service primitive.

| Parameter | Associate Primitive | Value |
| --- | --- | --- |
| LENGTH | PHY_DATA.request(TXVECTOR) | 0-2047 |
| PLCP_BITRATE | PHY_DATA.request(TXVECTOR) | BASIC, HIGHSPEED |

Table 10-1:  TXVECTOR Parameters

#### 21 TXVECTOR  LENGTH

The LENGTH parameter has the value of 0 to 2047.  This parameter is used to indicate the number of octets in the MPDU that the MAC is currently requesting the PHY to transmit.  This value is used by the PHY to determine the number of octet transfers which will occur between the MAC and the PHY after receiving a request to start a transmission.

#### 22  TXVECTOR  PLCP_BITRATE

The PLCP_BITRATE parameter is an optional parameter.  Its value describes the bit rate the PLCP should use to transmit the PLCP_PDU.  Its value can be BASIC or HIGHSPEED.  The BASIC rate is defined as the BSS_Basic_Rate in the FHSS PHY MIB.  The HIGHSPEED rate is defined by the Current_HighS_Rate in the MIB.

### 23 PHY_DATA.indicate RXVECTOR

The following parameters are defined as part of the RXVECTOR parameter list in the PHY_DATA.indicate service primitive.

| Parameter | Associate Primitive | Value |
| --- | --- | --- |
| LENGTH | PHY_DATA.indicate (RXVECTOR) | 0-2047 |
| ANTSEL | PHY_DATA.indicate (RXVECTOR) | ANT1, ...., ANTn |
| RSSI | PHY_DATA.indicate (RXVECTOR) | 0 - RSSI Max0-15 |

Table 10-2:  RXVECTOR Parameters

#### 24 RXVECTOR  LENGTH

The LENGTH parameter has the value of 0 to 2047.  This parameter is used to indicate the value contained in the LENGTH field the PLCP has received in the PLCP Header.  The MAC and PLCP will use this number to determine the number of octet transfers that will occur between the two sublayers during the transfer of the received MPDU.

#### 25 RXVECTOR  ANTSEL

The ANTSEL parameter is an optional parameter.  Its value describes the antenna used by the PLCP to receive the current MPDU.  The antenna value is defined as ANT1, ...., ANTn where n is the number of antennae supported by the PMD implementation.

#### 26 RXVECTOR RSSI

The Receive Signal Strength Indicator (RSSI) parameter has  a value of 0 through RSSI Max15.  This parameter is  an indication by the PHY sublayer the value of the energy observed on the antenna used to receive the current MPDU.  RSSI shall  be measured  between the beginning of the start frame delimiter and the end of the PLCP Header error check. RSSI shall  be measured  between the beginning of the start frame delimiter and the end of the PLCP Header error check.  RSSI is intended to be used in a relative manner. Absolute accuracy of the RSSI reading is not specified. Since RSSI is only used in a relative manner by MAC sublayer, this parameter is define to have 16 values that range from 0 through 15.  The value zero is the weakest signal strength while 15 is the strongest signal strength.

## 27 FHSS Physical Layer Convergence Procedure Sublayer

### 28 Introduction

This section provides a convergence procedure to map MAC PDUs into a frame format designed for FHSS radio transceivers.  The procedures for transmission, carrier sense, and reception are defined for single and multiple antenna diversity radios.

#### 29 State Diagram Notation

The operation of the procedures can be described by state diagrams.  Each diagram represents the domain of a function and consists of a group of connected, mutually exclusive states.  Only one state of a function is active at any given time.  Each state that the function can assume is represented by a rectangle as shown in Figure 10-2.  These are divided into two parts by a horizontal line.  In the upper part the state is identified by a name in capital letters.  The lower part contains the name of any  signal that is generated by the function.  Actions are described by short phrases and enclosed in brackets.

µ §

Figure 10- 2:  State Diagram Notation Example

All permissible transitions between the states of a function are represented graphically by arrows between them.  A transition that is global in nature (for example, an exit condition from all states to the IDLE or RESET state) is indicated by an open arrow.  Labels on transitions are qualifiers that must be fulfilled before the transition will be taken.  The label UCT designates an unconditional transition.  Qualifiers described by short phrases are enclosed in parentheses.

State transitions and sending and receiving of messages occur instantaneously.  When a state is entered and the condition to leave that state is not immediately fulfilled, the state executes continuously, sending the messages and executing the actions contained in the state in a continuous manner.

Some devices described in this standard are allowed to have two or more ports.  State diagrams capable of describing the operation of devices with an unspecified number of ports, require qualifier notation that allows testing for conditions at multiple ports.  The notation used is a term that includes a description in parentheses of which ports must meet the term for the qualifier to be satisfied (e.g., ANY and ALL).  It is also necessary to provide for term-assignment statements that assign a name to a port that satisfies a qualifier.  The following convention is used to describe a term-assignment statement that is associated with a transition:

(1)	The character ":" (colon) is a delimiter used to denote that a term assignment statement follows.

(2)	The character "<" (left arrow) denotes assignment of the value following the arrow to the term preceding the arrow.

The state diagrams contain the authoritative statement of the functions they depict; when apparent conflicts between descriptive text and state diagrams arise, the state diagrams are to take precedence.  This does not override, however, any explicit description in the text that has no parallel in the state diagrams.

The models presented by state diagrams are intended as the primary specifications of the functions to be provided.  It is important to distinguish, however, between a model and a real implementation.  The models are optimized for simplicity and clarity of presentation, while any realistic implementation may place heavier emphasis on efficiency and suitability to a particular implementation technology.  It is the functional behavior of any unit that must match the standard, not its internal structure.  The internal details of the model are useful only to the extent that they specify the external behavior clearly and precisely.

### 30  Physical Layer Convergence Procedure Frame Format

The PLCP Frame Format provides for the asynchronous transfer of MAC Layer MPDU from any transmitting station to all receiving stations within the wireless LAN.  The PLCP frame format illustrated in Figure 10-3 consists of three parts: a PLCP Preamble, a PLCP Header, and a PLCP_PDU.  The PLCP Preamble provides a period of time for several receiver functions.  These functions include antenna diversity, clock and data recovery and field delineation of the PLCP Header and the PLCP_PDU.  The PLCP Header is used to specify the length of the MPDU field and support any PLCP management information.  The PLCP_PDU contains the MPDU data modified by the PLCP_PDU data whitener.

Figure 10-3:  PLCP Frame Format

#### 31  PLCP Preamble

The PLCP preamble contains two separate sub-fields; the sync field and the start frame delimiter (SFD), to allow the PHY circuitry to reach steady state demodulation and synchronization of bit clock and frame start.

##### 32  Sync

The preamble sync field is an 80-bit field containing an alternating zero-one pattern, transmitted starting with zero and ending with one, to be used by the PHY sub-layer to detect a signal to receive, select an antenna if diversity is utilized, and to reach steady-state frequency offset correction and synchronization with the received packet timing.

##### 33 Start Frame Delimiter

The start frame delimiter (SFD) consists of the 16-bit binary pattern 0000 1100 1011 1101 (transmitted left-most bit first).  The first bit of the start frame delimiter follows the last bit of the sync pattern.  The start frame delimiter defines the frame timing.

#### 34  PLCP Header

The PLCP Header field contains three separate sub-fields: an 11-bit PLCP_PDU Length Word (PLW), a 5-bit PLCP Signaling Field (PSF) and a 16-bit PLCP Header Error Check (HEC) field.

##### 35  PLCP_PDU Length Word

The PLCP_PDU Length Word (PLW) is passed down from the MAC as a parameter within the PHY_DATA.request primitive in the transmitting station.  The PLW represents the number of octets contained in the MPDU packet.  Its valid states are 000h - 7FFh, representing counts of zero to 2047 octets.  The PLW is transmitted LSB first and MSB last.  The PLW is used by the receiving station to determine the last bit in the packet.

##### 36 PLCP Signaling Field

The 5-bit PLCP Signaling Field (PSF) is defined in Table 10-3.  The PSF is transmitted LSB (bit 0) first and MSB (bit 4) last.

| Bit | Parameter Name | Parameter Values | Description |
| --- | --- | --- | --- |
| 0 | Reserved | Default = 0 | Reserved for length expansion |
| 1 | Reserved | Default = 0 | Reserved |
| 2 | Reserved | Default = 0 | Reserved |
| 3 | Reserved | Default = 0 | Reserved |
| 4 | PDU_RATE | 0=1Mb, 1=2Mb, | This field indicates the bit rate of the PLCP_PDU |

Table 10-3:  PLCP Signaling Field Bit Descriptions

##### 37  Header Error Check Field

The Header Error Check (HEC) field is a 16-bit CCITT CRC-16 error detection field.  The HEC uses the CCITT CRC-16 generator polynomial G(x) as follows:

G(x) = x16+x12+x5+1

The HEC shall be the ones complement of the sum (modulo 2) of the following:

1) The remainder of xk • ( x15 + x14 + ... + x2 + x1 +1) divided (modulo 2) by G(x), where k is the number of bits in the PSF and PLW fields of the PLCP Header;

2) The remainder after multiplication by x16 and then division (modulo 2) by G(x) of the content (treated as a polynomial) of the PSF and PLW fields.

The HEC shall be transmitted with the coefficient of the highest term first.

As a typical implementation, at the transmitter, the initial remainder of the division is preset to all ones and is then modified by division of the PSF and PLW fields by the generator polynomial, G(x).  The ones complement of this remainder is inserted in the HEC field with the most significant bit transmitted first.

At the receiver, the initial remainder of the division is again preset to all ones.  The division of the received PSF, PLW, and HEC fields by the generator polynomial, G(x), results, in the absence of transmission errors, in a unique nonzero value, which is the following polynomial R(x):

R(x) = x12 + x11 + x10 + x8 + x3 + x2 + x1 + 1

#### 38  PLCP_PDU Data Whitener

The PLCP_PDU data whitener uses a length-127 frame synchronous scrambler followed by a 32/33 Bias Suppression Encoding to randomize the data and to minimize the data DC bias and maximum run lengths.  Data octets are placed in the transmit serial bit stream LSB first and MSB last.  The frame synchronous scrambler uses the generator polynomial S(x) as follows:

S(x) = x7+x4+1

and is illustrated in Figure 10-4. The 127-bit sequence generated repeatedly  by the scramber is (leftmost used first) 00001110 11110010 11001001 00000010 00100110 00101110 10110110 00001100 11010100 11100111 10110100 00101010 11111010 01010001 10111000 1111111.    The same scrambler is used to scramble transmit data and descramble receive data.  The data whitening starts with the first bit of the PLCP_PDU which follows the last bit of the PLCP Header.  The specific bias suppression encoding and decoding method used is defined in Figures 10-7a and 10-11a.  The format of the packet after data whitening is as shown in Figure 10-5.

µ §µ §

Figure 10-4:  Frame Synchronous Scrambler/Descrambler.

µ §

Figure 10-5:  PLCP_PDU Data Whitener Format.

### 39 PLCP State Machines

The PLCP consists of three state machines as illustrated in the overview  diagram of Figure 10-6: the transmit (TX), carrier sense/clear channel assessment (CS/CCA), and receive (RX) state machines.  The three PLCP state machines are defined in the subsections below; figure 10-6 is not a state diagram itself.  Execution of the PLCP state machines normally transfers from the FH PLME state machine and begins at the CS/CCA state machine.  The PLCP exits back to the FH PLME state machine upon interrupt to service a PLME service request, e.g., PLME_SETCHNL, PLME_PHY_RESET, etc.

Figure 10-6:  PLCP Top Level State Diagram.

#### 40 PLCP Transmit Procedure

The PLCP transmit procedure is invoked by the CS/CCA procedure immediately upon receiving a PHY_DATA.request(Start_of_Data, TXVECTOR) from the MAC layer.  The CSMA/CA protocol is performed by the MAC with the PHY PLCP in the CS/CCA procedure prior to executing the transmit procedure.

##### 41Transmit State Machine

The PLCP transmit state machine illustrated in Figure 10-7 includes functions that must be performed prior to, during, and after MPDU data transmission.  Upon entering the transmit procedure in response to a PHY_DATA.request (Start_of_Data , TXVECTOR) from the MAC, the PLCP shall switch the PHY PMD circuitry from receive to transmit state; ramp on the transmit power amplifier in the manner prescribed in Section 10.6 (PMD specification); and transmit the preamble sync pattern and start frame delimiter.  The PLCP shall generate the PLCP header as defined in Section 10.3.2.2 (PLCP Header) in sufficient time to send the bits at their designated bit slot time.  The PLCP shall add the PLCP header to the start of the PLCP_PDU data.

Prior to transmitting the first MPDU data bit, the PLCP shall send a PHY_DATA.confirm message to the MAC indicating that the PLCP is ready to receive an MPDU data octet.  The MAC will respond by sending a PHY_DATA.request(Data, DATA).  This sequence of PHY_DATA. confirm and PHY_DATA.request(Data, DATA)shall be executed whenever the PLCP is ready for more data and until the last data octet is passed to the PLCP.  During transmission of the PLCP_PDU data, each bit of the MPDU passed down from the MAC shall be processed by the data whitener algorithm defined in Figure 10-7a and described in section 10.3.2.3 (PLCP_PDU Data Whitener).  The MPDU data bytes are processed and transmitted LSB first and MSB last.

After the last MPDU octet is passed to the PLCP, the MAC will respond to the following PHY_DATA. confirm with a PHY_DATA.request(End_of_Data, NULL).  After the last bit of the PLCP_PDU data has completed propagation through the radio and been transmitted onto the air, the PLCP shall complete the transmit procedure by sending a PHY_DATA.confirm to the MAC layer, ramp off the power amplifier in the manner prescribed in the Section 10.6 (PMD), and switch the PHY PMD circuitry from transmit to receive state.  The execution shall then return to the CS/CCA procedure.

Figure 10-7:  Transmit State Machine

Data Whitener Encoding Algorithm:

/*	If MSB of stuff symbol = 1 = next block is inverted; 0 = not inverted */

/*	Accumulate PLCP Header; begin stuffing on first bit of the PLCP_PDU */

/********* Calculate number of 32-symbol BSE blocks required to send MPDU;

no padding is necessary for number of symbols not multiple of 32 *********/

Input parameter: number_of_MPDU_octets, rate;		/* rate is 1 or 2*/

number_of_symbols= (number_of_MPDU_octets *8) /rate;

number_of_blocks_in_packet = truncate{(number_of_symbols + 31) / 32)};

/********* Accumulate the bias in the header to use in calculating the inversion state of the first 				block of PLCP_PDU data *********/

Read in header {b(1),...,b(32)};				/* b(1) is first bit in */

header_bias = Sum{weight(b(1)),...,weight(b(32))};

/* calculate bias in header; weights are defined in Table 10-4 */

Transmit {b(1),...,b(32)};					/* no stuffing on header */

accum=header_bias;					/* initialize accum */

Initialize scrambler to all ones;

/********* Whiten the  PLCP_PDU data with scrambler and BSE encoder *********/

For n = 1 to number_of_blocks_in_packet

{

b(0) = 0 for 1 Mbps; b(0)=00 for 2 Mbps;		/* b(0) is the stuff symbol */

N = min(32, number_of_symbols);			/* N= block size in symbols */

Read in next symbol block {b(1),...,b(N)};		/* b(n) = {0,1} or {0,1,2,3};

1 - 8 octets, use PHY_DATA.req(Data, DATA), PHY_DATA.confirm for each octet*/

Scramble {b(1),...,b(N)};				/* see section 10.3.2.3 */

bias_next_block = Sum{weight(b(0)),...,weight(b(N))};	/* calculate bias with b(0)=0 */

/***** if accum and bias of next block has the same sign, then invert block; 					if accum=0 or bias_next_block=0, don't invert *****/

If {[accum * bias_next_block > 0] then

{

Invert {b(0),...,b(N)}; 		/* Invert deviation, or, negate MSB of symbol */

bias_next_block = - bias_next_block;

}

accum = accum + bias_next_block;

transmit {b(0),...,b(N)};				/* b(0) is first symbol out */

number_of_symbols = number_of_symbols - N

}

Figure 10-7a:  Data Whitener Encoding Procedure

The weights assigned to each value of the symbols are defined in Table 10-4 for the 1 Mbps (2GFSK) and 2 Mbps (4GFSK) symbols.

| 2GFSK | 4GFSK | weight |
| --- | --- | --- |
|  | 10 | 3 |
| 1 |  | 2 |
|  | 11 | 1 |
| center | center | 0 |
|  | 01 | -1 |
| 0 |  | -2 |
|  | 00 | -3 |

Table 10-4:  PLCP Signaling Field Bit Descriptions

##### 42 Transmit State Timing

The transmit timing illustrated in Figure 10-8 is defined from the instant that the PHY_DATA.request(Start_of_Data , TXVECTOR) is received from the MAC layer.  The PLCP shall switch the PMD circuitry from receive to transmit,  turn on and settle the transmitter, and begin transmitting the first bit of the preamble at the antenna within a maximum of 20 µs of receiving the PHY_DATA.request(Start_of_Data , TXVECTOR).  The PLCP preamble shall be transmitted at 1 Mbps and be completed in 96 µs.  The PLCP header shall be transmitted at 1 Mbps and be completed in 32 µs.  The variable length PLCP_PDU shall be transmitted at the selected data rate.  After the last bit of the PLCP_PDU data has completed propagation through the radio and been transmitted onto the air, the PLCP shall send the last PHY_DATA.confirm to the MAC layer.  The PLCP shall turn off the power amplifier and be less than the specified off-mode transmit power within the time specified in section 10.6.  At the end of the power amplifier ramp down period, the PLCP shall switch the PMD circuitry from transmit to receive.  All of the PHY_DATA. confirm and PHY_DATA.request(Data, DATA) exchanges prior to the final PHY_DATA.confirm  shall occur prior to the points at which data octets are needed but do not have any additional time significance.

Figure 10-8:  Transmit State Timing

#### 43  Carrier Sense/Clear Channel Assessment Procedure

The PLCP carrier sense/clear channel assessment (CS/CCA) procedure is executed while the receiver is turned on and the station is not currently receiving or transmitting a packet.  The CS/CCA procedure is used for two purposes:  to detect the start of a network signal that can be received (CS) and to determine whether the channel is clear prior to transmitting a packet (CCA).

##### 44 Carrier Sense/Clear Channel Assessment State Machine

The carrier sense/clear channel assessment (CS/CCA) state machine is shown in Figure 10-9.  The PLCP shall perform a CS/CCA assessment on a minimum of one antenna within a contention backoff slot time of 50 µs.   The PLCP shall be capable of detecting within the slot time an FH PHY conformant signal which is received at the selected antenna up to 20 µs after the start of the slot time with the detection performance specified in section 10.6.15.32.3.  Section 10.6.15.32.3 specifies detection performance with zero-one sync patterns and with random data patterns.  If a start of a transmission is asynchronous with the BSS and arrives after the start of the slot but at least 16 usec prior to the end of the slot, the PLCP shall indicate a busy channel prior to the end of the slot time with the detection performance specified in section 10.6.15.32.3.  The CCA indication immediately prior to transmission shall be performed on an antenna with essentially the same free space gain and gain pattern as the antenna to be used for transmission.  The method of determining CS/CCA is unspecified except for the detection performance of a conformant method as specified in section 10.6.15.32.3.

If a PHY_DATA.request (Start_of_Data, TXVECTOR) is received, the CS/CCA procedure shall exit to the transmit procedure.   If a PHY_DATA.request(End_of_Activity, NULL)  is received, the PLCP shall reset all relevant CS/CCA assessment timers to the state appropriate for the end of a complete received frame.  This service primitive is generated by the MAC at the end of a NAV period.

If a CS/CCA assessment returns a channel idle result, the PHY shall send a PHY_DATA.indicate (End_of_Activity, NULL) to the MAC.

If a CS/CCA assessment returns a channel busy result, the PHY shall send a PHY_DATA.indicate(Start_of_Activity, NULL) to the MAC.  Upon a channel busy assessment, the PLCP shall stop any antenna switching prior to the earliest possible arrival time of the start frame delimiter (SFD) and detect a valid SFD if received.  If a valid SFD is detected, the CS/CCA procedure shall exit to the receive procedure.  The PLCP shall dwell and search for the SFD for a minimum period longer than the latest arrival time of the SFD.  Indication of a busy channel does not necessarily lead to the successful reception of a frame.

Upon exiting the CS/CCA procedure to receive a frame, the last indication of CS/CCA was BUSY.  The indication remains BUSY when returning from the receive procedure until the first CS/CCA assessment is performed and determines that the channel is IDLE.

The countdown timer may be a non-zero value when returning from the receive procedure if a signal in the process of being received was lost prior to the end as positively indicated in the length field of a valid PLCP header.  The countdown timer shall be used to force the CS/CCA indication to remain in the BUSY state until the predicted end of the frame regardless of actual CS/CCA indications.  However, if the CS/CCA procedure indicates the start of a new frame within the countdown timer period, it is possible to transition to the receive procedure prior to the end of the countdown timer period.  When a non-zero countdown timer reaches zero, the PLCP shall reset all relevant CS/CCA assessment timers to the state appropriate for the end of a complete received frame. The initial value of the countdown timer upon entry of the CS/CCA procedure from power up or PHY reset is unspecified.

µ §

Figure 10-9:  CS/CCA State Machine.

##### 45  Carrier Sense/Clear Channel Assessment State Timing

Timing for priority (PIFS, DIFS), contention backoff (slot times), and CS/CCA assessment windows are defined relative to the end the last bit of the last packet on the air.  The PLCP shall perform a CS/CCA assessment on a minimum of one antenna within a slot time including the PIFS and DIFS windows.  The appropriate CS/CCA indication shall be generated prior to the end of each 50 µs slot time with the performance specified in section 10.6 (PMD).

If a station has not successfully received the previous packet, the perceived packet end time and slot boundary times will have a higher uncertainty for that station.

µ §

Figure 10-10:  CS/CCA State Timing

#### 46 PLCP Receive Procedure

The PLCP receive procedure is invoked by the PLCP carrier sense/clear channel assessment (CS/CCA) procedure upon detecting a portion of the preamble sync pattern followed by a complete start frame delimiter.

##### 47 Receive State Machine

The PLCP receive procedure shown in Figure 10-11 includes functions that must be performed while receiving the PLCP header and the PLCP_PDU data.  The PLCP receive procedure begins upon detection of a complete start frame delimiter in the CS/CCA procedure.  The PLCP shall read in the 32-bit PLCP header and perform an error check using the header error check field.  Immediately upon detection of a valid PLCP header, the PLCP shall send a PHY_DATA.indicate (Start_of_Data, RXVECTOR) message to the MAC layer.  The PLCP shall set a PLCP_PDU byte/bit counter to indicate the last bit of the packet, receive the PLCP_PDU data bits and perform the data whitening decoding procedure shown in Figure 10-11a on each PLCP_PDU bit.  The PLCP shall pass correctly received data octets to the MAC with a PHY_DATA.indicate(Data, DATA).  After the last PLCP_PDU bit is received and the last octet is passed up to the MAC, the PLCP shall send a PHY_DATA.indicate(End_of_Data, RXERROR= no_error) to the MAC layer.  Upon error-free completion of a packet reception, the PLCP shall exit the receive procedure and return to the PLCP CS/CCA procedure with TIME_REMAINING=0.

If the PLCP header contains any bit errors, the PLCP shall immediately set the byte/bit counter to zero, complete the receive procedure with a PHY_DATA.indicate(End_of_Data, RXERROR=header violation) to the MAC, and return to the CS/CCA procedure with TIME_REMAINING=0.  If any format error was detected during the reception of the packet, the PLCP shall immediately complete the receive procedure with a PHY_DATA.indicate(End_of_DATA, RXERROR=format violation) to the MAC, and return to the CS/CCA procedure with TIME_REMAINING=byte/bit count remaining.

Figure 10-11:  Receive State Machine

Data Whitener Decoding Algorithm:

/*	If stuff bit = 1 = next block is inverted; 0 = not inverted */

/*	Stuffing begins on first bit of PLCP Header following the start frame delimiter */

/*	Accumulate from start of PLCP Header to verify format */

/*	Algorithm begins after verifying validity of header with HEC */

/********* Calculate bias in header for format error checking *********/

Read in header {b(1),...,b(32)};				/* b(1) is first bit in */

accum = 2 [Sum{b(1),...,b(32)}] - (32);			/* calculate bias in header */

Get number_of_MPDU_octets, rate from header;		/* rate is 1 or 2 */

number_of_symbols = (number_of_MPDU_octets*8)/rate

number_of_blocks_in_packet = truncate{(number_of_symbols + 31) / 32};

Initialize scrambler to all ones;

/********* De-whiten the PLCP_PDU data with BSE decoder and de-scrambler *********/

For n = 1 to number_of_blocks_in_packet

{

N = min(4, # of bytes remaining) * 8;		/* N= block size in bits */

Read in next block {b(0),...,b(N)};			/* b(n) =  {0,1} or {0,1,2,3} */

bias_next_block = Sum{weight(b(0)),...,weight(b(N))};

/* calculate bias with b(0); weights are defined in Figure 10-7b */

accum = accum + bias_next_block;

If (((rate=1 & (accum < -64 or accum > +64)) or ((rate=2 ) & (accum <-96 or accum>+96)))

send format error indication to MAC;

If {[b(0)=1] then Invert {b(1),...,b(N)};		/* if invert bit=true */

Descramble {b(1),...,b(N)};			/* see section 10.3.2.3 */

Send {b(1),...,b(N)} to MAC

/* 1 - 8 octets; use PHY_DATA.ind(Data, DATA) for each octet. */

}

Figure 10-11a:  Data Whitener Decoding Procedure

##### 48 Receive State Timing

The receive state timing shown in Figure 10-12 is defined to begin upon detection of a complete start frame delimiter in the carrier sense procedure.  The PLCP shall send a PHY_DATA.indicate (Start_of_Data, RXVECTOR) to the MAC layer after receiving the last bit of a valid PLCP header.  The PLCP shall begin receiving the variable length PLCP_PDU immediately after the end of the last bit of the PLCP header.  The PLCP shall send a PHY_DATA.indicate(End_of_Data, RXERROR) after receiving the last PLCP_PDU data bit.  There is no timing significance of the PHY_DATA.indicate (Data, DATA) indications other than the requirement that they occur after the time-significant PHY_DATA.indicate(Start_of_Data, RXVECTOR) and before the PHY_DATA.indicate (End_of_Data, RXERROR).

If there was an error in the PLCP header, the PLCP shall terminate the receive procedure within 8 µs of the end of the PLCP header.  If any other error was detected during the reception of the packet, the PLCP shall terminate the receive procedure within 8 µs of detecting the error.

Figure 10-12:  Receive Timing

## 49 PLME_SAP Layer Management.

### 50  Introduction

This section describes the services provided by the FHSS PLME to the upper layer management entities.  These primitives are abstract representations of the services and are not intended to restrict implementations.  All of the service primitives described in this section are considered mandatory unless otherwise specified.

### 51 PLME_SAP Sublayer Management Primitives

The following messages may be sent between the PHY sublayer entities and intralayer or higher Layer Management Entities (LME).

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PLME_PLCPRESET | X |  | X |  |
| PLME_POWER | X |  | X |  |
| PLME_SETCHNL | X |  | X |  |

Table 10-54: PLME_SAP Sublayer Management Primitives

#### 52 PLME_SAP Management Service Primitive Parameters

The following table shows the parameters used by one or more of the PLME_SAP Sublayer Management Primitives.

| Parameter | Associate Primitive | Value |
| --- | --- | --- |
| SET | PLME_SETCHNL | 0,1,2,3 |
| PATTERN | PLME_SETCHNL | 0, 2-23, 24-45,47-68 |
| INDEX | PLME_SETCHNL | 1-79 |
| STATE | PLME_POWER | ON, OFF |

Table 10-65:  PHY_SAP Service Primitive Parameters

#### 53  PLME_SAP Detailed Service Specifications

The following section describes the services provided by each PLME_SAP Service Primitive.

##### 54  PLME_PLCPRESET.request

Function.  This primitive is a request by the LME to reset the PHY sublayer  PLCP state machine.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_PLCPRESET.request

There are no parameters associated with this primitive.

When Generated.  This primitive can be generated at anytime to reset the PLCP state machine in the PHY sublayer.

Effect of Receipt.   Receipt of this primitive by the PHY sublayer will cause the PHY entity to reset the PLCP state machine to its idle state.

##### 55 PLME_PLCPRESET.confirm

Function.  This primitive is a confirmation by the PHY sublayer to the local LME that the PLCP state machine was successfully reset.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_PLCPRESET.confirm

There are no parameters associated with this primitive.

When Generated.  This primitive will be generated as a response to a PLME_PLCPRESET.request primitive once the PLCP has successfully completed the reset.

Effect of Receipt.  The effect of receipt of this primitive by the LME is unspecified.

##### 56 PLME_POWER.request

Function.  This primitive is a request by the LME to power the PHY sublayer and PMD state machine on or off.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_POWER.request(STATE)

The STATE parameter can have the value of ON or OFF.  When the value is ON power will be applied to the PLCP and PMD electronics.

When Generated.  This primitive can be generated at anytime to control the state of the PLCP and PMD power.

Effect of Receipt.   Receipt of this primitive by the PHY sublayer will cause the PHY LME entity to modify the state of the PLCP and PMD power.

##### 57PLME_POWER.confirm

Function.  This primitive is a confirmation by the PHY sublayer to the local LME that the PLCP and the PMD successfully powered up.  It will not be sent to confirm a request to remove power.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_POWER.confirm

There are no parameters associated with this primitive.

When Generated.  This primitive will be generated as a response to a PLME_POWER.request(ON) primitive once the PLCP and PMD has successfully completed power restart.

Effect of Receipt.  The effect of receipt of this primitive by the LME is unspecified.

##### 58 PLME_SETCHNL.request

Function.  This primitive is a request by the LME to the PHY sublayer to change frequency.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_SETCHNL.request (SET,PATTERN,INDEX)

The SET parameter can be one of four values: 0, 1, 2, and 3.  This parameter selects the sets of hopping patterns as shown in the annex.  SET equals 0 is a special set used by the MAC to discover what LANs currently exist prior to associating with a given LAN.  The PATTERN parameter specifies one of 23 hop patterns within a given hop SET.  The hop patterns are listed in the annex.  PATTERN equal 0 is a special pattern used by the MAC to discover what LANs currently exist prior to associating with a given LAN.  The INDEX parameter can be one of 80 values ranging from 1 to 80.  The INDEX value is used to select a frequency within a given SET and PATTERN as shown in the annex.  If the SET and PATTERN parameters are both zero, then the INDEX value is equal to the frequency or channel desired.

When Generated.  This primitive can be generated at anytime to hop to a new channel or frequency.

Effect of Receipt.  Receipt of this primitive by the PHY sublayer will cause the PHY entity to start the channel change or hop state machine.

##### 59 PLME_SETCHNL.confirm

Function.  This primitive is a confirmation by the PHY sublayer to the local LME that the frequency change or hop was successfully completed.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PLME_SETCHNL.confirm

There are no parameters associated with this primitive.

When Generated.  This primitive will be generated as a response to a PLME_SETCHNL.request primitive once the PLME has successfully completed the frequency change.

Effect of Receipt.  The effect of receipt of this primitive by the LME is unspecified.

### 60FH PHY Specific MAC Layer Management State Machines

#### 61   Introduction

This section identifies the specific MAC Management state machines required for operating the FHSS PHY.  The relationship between the MAC Management and FH PLME state machines are also described.

### 62  FH PHY Layer Management Entity State Machines

#### 63   Introduction

This section describes the FH PLME PHY Layer Management state machines to turn the PMD on/off, reset the PLCP state machine, and change the frequency hop channel.

#### 64   PLME State Machine

The PLME state machine in Figure 10-13 begins with a PLME_POWER.request(State=ON) request which turns on the PMD circuitry.  The MAC then sends a PLME_SETCHNL.request(set, pattern, index) to tune the PMD to the selected channel.  The PLME then transfers execution to the PLCP state machine as defined in section 10.3.3.  Upon receiving a PLME request from a higher level LME, the PLCP shall return execution to the PLME state machine and process the request.  A PLME_PLCPRESET.request returns the PLCP state machine to its entry state.  A PLME_SETCHNL.request(set, pattern, index) will cause the PLCP to terminate a receive or CS/CCA process and change frequency before returning to the PLCP state machine.  A PLME_POWER.request(state=OFF) request will cause the PLCP to terminate a receive or CS/CCA process, power down the PMD circuitry, and return the PLME state machine to the idle state.  The MAC should not send a PLME_SETCHNL or PLME_POWER while the PLCP is in the transmit state.

µ §

Figure 10-13: PLME State Machine

## 65 FHSS Physical Medium Dependent Sublayer Services

### 66 Scope and Field of Application

This section describes the PMD services provided to the PLCP for the FHSS Physical Layer.  Also defined in this section are the functional, electrical and RF characteristics required for interoperability of implementations conforming to this specification.  The relationship of this specification to the entire FHSS Physical Layer is shown in Figure 10-14.

µ §

Figure 10-14:  PMD Layer Reference Model

### 67 Overview of Services

In general, The FHSS Physical Medium Dependent Sublayer accepts Physical Layer Convergence Procedure sublayer-service primitives and provides the actual means by which the signals required by these primitives are imposed onto the medium.   In the FHSS Physical Medium Dependent Sublayer at the receiver the process is reversed.  The combined function of the transmitting and receiving FHSS PMD Sublayers results in a data stream, timing information, and receive parameter information being delivered to the receiving Physical Layer Convergence Procedure Sublayer.

### 68  Overview of Interactions

The primitives associated with the 802.11 PLCP sublayer to the FHSS PMD sublayer falls into two basic categories:

(1)  Service primitives that support PLCP peer-to-peer interactions

(2)  Service primitives that have local significance and support sublayer-to-sublayer interactions.

### 69  Basic Service and Options

All of the service primitives described in this section are considered mandatory unless otherwise specified.

#### 70  PMD_SAP Peer-to-Peer Service Primitives

The following table indicates the primitives for peer-to-peer interactions.

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_DATA | X | X |  |  |

Table 10-76:  PMD_SAP Peer-to-Peer Service Primitives

#### 71  PMD_SAP Sublayer-to-Sublayer Service Primitives

The following table indicates the primitives for sublayer-to-sublayer interactions.

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_TXRX | X |  |  |  |
| PMD_PARAMP | X |  |  |  |
| PMD_ANTSEL | X |  |  |  |
| PMD_TXPWRLVL | X |  |  |  |
| PMD_FREQ | X |  |  |  |
| PMD_RSSI |  | X |  |  |
| PMD_PWRMGNT | X |  |  |  |
| PMD_SYNLOCK |  | X |  |  |

Table 10-87:  PMD_SAP Sublayer-to-Sublayer Service Primitives

#### 72PMD_SAP Service Primitives Parameters

The following table shows the parameters used by one or more of the PMD_SAP Service Primitives.

| Parameter | Associate Primitive | Value |
| --- | --- | --- |
| TXD_UNIT | PMD_DATA.request | 1 Mbps: 0, 1 2 Mbps: 0, 1, 2, 3 |
| RXD_UNIT | PMD_DATA.indicate | 1 Mbps: 0, 1 2 Mbps: 0, 1, 2, 3 |
| RF_STATE | PMD_TXRX.request | TRANSMIT, RECEIVE |
| RAMP_STATE | PMD_PARAMP.request | ON, OFF |
| ANTENNA_ STATE | PMD_ANTSEL.request | 1,2 |
| TXPWR_LEVEL | PMD_TXPWRLVL.request | LEVEL1, LEVEL2, LEVEL3, LEVEL 4 |
| CHNL_ID | PMD_FREQ.request | 2 through 80 inclusive |
| STRENGTH | PMD_RSSI.indicate | 0 - RSSI MaxTBD |
| MODE | PMD_PWRMGNT.request | ON, OFF |
| STATUS | PMD_SYNLOCK.indicate | LOCKED, UNLOCKED |

Table 10-98:  List of Parameters for PMD Primitives.

### 73PMD_SAP Detailed Service Specification

The following section describes the services provided by each PMD primitive.

#### 74 PMD_DATA.request

Function.  This primitive defines the transfer of data from the PLCP sublayer to the PMD entity.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_DATA.request  (TXD_UNIT)

The TXD_UNIT parameter can take on one of two values: ONE, or ZERO.  This parameter represents a single data bit.  The effect of this parameter is that the PMD will properly modulate the medium to represent ONEs or ZEROs as defined in the FHSS PMD Modulation Specifications for a given data rate.

When Generated.  This primitive is generated by the PLCP sublayer to request the transmission of a single data bit on the Physical Medium Dependent sublayer.  The bit clock is assumed to be resident or part of the PLCP and this primitive is issued at every clock edge once the PLCP has begun transmitting data.

Effect of Receipt.  The receipt of this primitive will cause the PMD entity to encode and transmit a single data bit.

#### 75 PMD_DATA.indicate

Function.  This primitive defines the transfer of data from the PMD entity to the PLCP sublayer.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_DATA.indicate  (RXD_UNIT)

The RXD_UNIT parameter can take on one of two values: ONE, or ZERO  This parameter represents the current state of the media as determined by the FHSS PMD Modulation Specifications for a given data rate.

When Generated.  The PMD_DATA.indicate is generate to all receiving PLCP entities in the network after a PMD_DATA.request is issued.

Effect of Receipt.  The effect of receipt of this primitive by the PLCP is unspecified.

#### 76PMD_TXRX.request

Function.  This primitive is used to place the PMD entity into the transmit or receive function.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_TXRX.request  (RF_STATE)

The RF_STATE parameter can take on one of two values: TRANSMIT or RECEIVE.  When the value of the primitive is TRANSMIT, the RF state of the radio is transmit.  If the value of the primitive is RECEIVE, the RF state of the radio is receive.

When Generated.  This primitive is generated whenever the mode of the radio needs to be set or when changing from transmit to receive or receive to transmit.

Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the mode of the radio to be in either transmit or receive.

#### 77PMD_PARAMP.request

Function.  This primitive defines the start of the ramp up or ramp down of the radio transmitter's Power Amplifier.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_PARAMP.request  (RAMP_STATE)

The RAMP_STATE parameter can take on one of two values: ON or OFF.  When the value of the primitive is ON, the state of the transmit power amplifier is "on".  If the value of the primitive is OFF, the state of the transmit power amplifier is "off".

When Generated.  This primitive is issued only during transmit and to establish the initial state.  It is generated by the PLCP at the start of the transmit function to turn the transmitter's power amplifier "on".  A power amplifier ramp up period follows the change of state from "off" to "on".  After the PLCP has transferred all require data to the PMD entity, this primitive again will be issued by the PLCP to place the transmit power amplifier back into the "off" state.  A power amplifier ramp down period follows the change of state from "on" to "off".

Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the transmit power amplifier to be on or off.

#### 78PMD_ANTSEL.request

Function.  This primitive is used to select which antenna the PMD entity will use to transmit or receive data.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_ANTSEL.request  (ANTENNA_STATE)

The ANTENNA_STATE parameter can take on one of two values: ONE or TWO.  When the value of the primitive is a ONE, the PMD will switch to antenna 1 for receive or transmit.  If the value of the primitive is TWO, the PMD entity will switch to antenna 2 for receive or transmit.

When Generated.  This primitive is generate at various times by the PLCP entity to select an antenna.  During receive, this primitive can be used to do antenna diversity.  During transmit, this primitive can be use to select a transmit antenna.  This primitive will also be used during Clear Channel Assessment.

Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the radio to select the antenna specified.

#### 79 PMD_TXPWRLVL.request

Function.  This primitive defines the power level the PMD entity will use to transmit data.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_TXPWRLVL.request  (TXPWR_LEVEL)

The TXPWR_LEVEL parameter can be one of the following values listed in Table 10-109 below.

| TXPWR_LEVEL | Level Description |
| --- | --- |
| LEVEL1 | Defined as Tx_Pwr_Lvl_1 in MIB |
| LEVEL2 | Defined as Tx_Pwr_Lvl_2 in MIB |
| LEVEL3 | Defined as Tx_Pwr_Lvl_3 in MIB |
| LEVEL4 | Defined as Tx_Pwr_Lvl_4 in MIB |

Table 10-109:  Transmit Power Levels

When Generated.  This primitive is generated as part of the transmit sequence.

Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the transmit power level to be modify.

#### 80PMD_FREQ.request

Function.  This primitive defines the frequency the PMD entity will use to receive   or transmit data.  Since changing the radio frequency is not an immediate function, this primitive serves also as an indication of the start of this process.  The completion of this process is dictated by other PMD specifications.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_FREQ.request  (CHNL_ID)

The CHNL_ID parameter can be one of the following values list in Table 10-131 or 10-14.

When Generated.  This primitive is generated by the PLCP whenever a change to a new frequency is required.

Effect of Receipt.  The receipt of this primitive by the PMD entity will cause the radio to change to a new frequency defined by the value of the CHNL_ID.

#### 81 PMD_RSSI.indicate

Function.  This primitive transfers a receiver signal strength indication of the physical medium from the PMD sublayer to the PLCP sublayer.  This value will be used by the PLCP to performing any diversity or clear channel assessment functions required by the PLCP or other sublayers.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_RSSI.indicate  (STRENGTH)

The STRENGTH parameter can be a value from 0-15.   This parameter is an indication by the PMD sublayer of the value of the energy observed on the selected antenna.  This value is used to generate the RSSI term in the PHY_DATA.ind(RXVECTOR) primitive and might also be used by any diversity function.  Since RSSI is only used in a relative manner by MAC sublayer, this parameter is define to have 16 values that range from 0 through RSSI Max15.  The value zero is the weakest signal strength while RSSI_Max15 is the strongest signal strength.

When Generated.  This primitive is generated continually by the PMD entity to transfer a receive signal strength indication to the PLCP.

Effect of Receipt.  The effect of receipt of this primitive by the PLCP is unspecified.

#### 82 PMD_PWRMGNT.request

Function.  This primitive is used by the higher layers entities to manage or control the power consumption of the PMD when not in use.  This allows higher layer entities to put the radio into a sleep or standby mode when not expecting to receive or send any data.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_PWRMGNT.request  (MODE)

The MODE parameter can be one of two values: ON or OFF.  When the value of the parameter is ON, the PMD entity will enter into a fully functional mode which allows it to send or receive data.  When the value of the parameter is OFF, the PMD entity will place itself in a standby or low power mode.  In the low power mode, the PMD entity is not expected to be able to perform any request by the PLCP nor is it expected to indicate any change in PMD state or status.

When Generated.  This primitive is delivered by the PLCP but actually is generated by a high layer management entity

Effect of Receipt.  Upon receipt of this primitive, the PMD entity will enter a fully functional or low power consumption state depending on the value of the primitive's parameter.

#### 83 PMD_SYNLOCK.indicate

Function.  This primitive is a indication by the PMD entity to the PLCP that the radio synthesizer is locked to the frequency specified by the PMD_FREQ primitive.

Semantics of the Service Primitive.  The primitive shall provide the following parameters:

PMD_SYNLOCK.indicate  (STATUS)

The STATUS parameter can be one of two values: LOCKED or UNLOCKED.  When the value of the parameter is LOCKED, the radio's synthesizer will be on the frequency specified by the PMD_FREQ primitive.  When the value of the parameter is UNLOCKED, the radio's synthesizer frequency will not match the frequency specified in the PMD_FREQ primitive.

When Generated.  The primitive will be issued whenever the PMD entity is required to change channels or hop.  The UNLOCKED value will always appear whenever the PMD_FREQ primitive is issued and the CHNL_ID value of that primitive doesn't match the current frequency of the synthesizer.  The LOCKED value will be issued whenever the frequency of the synthesizer and the CHNL_ID value become matched.  In the case which the radio's synthesizer is already at the frequency specified by PMD_FREQ primitive, the LOCKED indication will immediately be returned to the PLCP or high layer entity.

Effect of Receipt.  The effect of receipt of this primitive by the PLCP is unspecified.

## 84 FHSS Physical Medium Dependent Sublayer 1.0M Bit

### 85   1M Bit PMD Operating Specifications General

In general, the PMD accepts Convergence Layer-service primitives and provides the actual means by which the signals required by these primitives are imposed onto the medium.   In the Physical Medium Dependent sublayer at the receiver the process is reversed.  The combined function of the transmitting and receiving Physical Medium Dependent sublayers results in a data stream, timing information, and receive parameter information being delivered to the receiving Convergence Sublayer.

### 86   Regulatory Requirements

Wireless LANs implemented in accordance with this standard are subject to equipment certification and operating requirements established by Regional and National regulatory administrations. The PMD specification establishes minimum technical requirements for interoperability, based upon established regulations for  Europe, Japan, and the USA at the time of the draft. These regulations are subject to revision, or may be superseded.  Requirements that are subject to local geographic regulations are annotated within the PMD specification. Regulatory requirements that do not affect interoperability are not addressed within this standard. Implementers are referred to the  following regulatory sources for further information. Operation in countries within Europe, or other regions outside  Japan or North America,  may be subject to additional or alternative National regulations.

The documents listed below specify the current regulatory requirements for various  geographic  areas at the time the standard was developed. They are provided for information only, and are subject to change or revision at any time.

Europe:

Approval Standards: European Technical Standards Institute

Documents: ETS 300-328, ETS 300-339

Approval Authority: National Type Approval Authorities

Japan:

Approval Standards:  Research and Development Center for Radio Communications (RCR)

Documents: RCR STD-33

Approval Authority: Ministry of  Telecommunications (MKK)

North America:

Approval Standards:	Department of Communications (DOC), Canada

Documents: GL36

Federal Communications Commission (FCC), USA

Documents: CFR47, Part 15,  Sections 15.205, 15.209, 15.247.

Approval Authority: DOC (Canada), FCC (USA)

### 87 Operating Frequency Range

A conformant PMD implementation shall be able to select the carrier frequency (Fc) from the full geographic-specific set of available set of carrier frequencies.  Table 10-11.10. summarizes these frequencies for a number of geographic locations:

| Lower Limit | Upper Limit | Regulatory Range | Geography |
| --- | --- | --- | --- |
| 2.402 GHz | 2.402 GHz | 2.400-2.4835 GHz | USA* |
| 2.402 GHz | 2.480 GHz | 2.400-2.4835 GHz | Europe* |
| 2.473 GHz | 2.495 GHz | 2.471-2.497 GHz | Japan* |

Table 10-110:   Operating Frequency Range.

* The frequency ranges in this table are subject to the geographic specific Regulatory Authorities

### 88 Number of Operating Channels

The number of transmit and receive frequency channels used for operating the PMD entity is 79 for the US and Europe and 23 channels for Japan. This is more fully defined in Tables 10-13.12 and 10-14.13 of Section 10.6.54.

| Minimum* | Hopping Set | Geography |
| --- | --- | --- |
| 75 | 79 | USA* |
| 20 | 79 | Europe* |
| 10 | 23 | Japan* |

Table 10-121:   Number of Operating Channels

* The number of required hopping channels are subject to the geographic specific Regulatory Authorities

### 89 Operating Channel Center Frequency

The channel center frequency is defined in sequential 1.0 MHz steps beginning with the first channel, channel 2.402 GHz for the U.S.A. and Europe, as listed in Table 10-132.  The channel centers for Japan, starting at 2.473 GHz with 1 MHz increments, is listed in Table 10-143.

| Channel # | Value | Channel # | Value | Channel # | Value |
| --- | --- | --- | --- | --- | --- |
| 2 | 2.402 | 28 | 2.428 | 54 | 2.454 |
| 3 | 2.403 | 29 | 2.429 | 55 | 2.455 |
| 4 | 2.404 | 30 | 2.430 | 56 | 2.456 |
| 5 | 2.405 | 31 | 2.431 | 57 | 2.457 |
| 6 | 2.406 | 32 | 2.432 | 58 | 2.458 |
| 7 | 2.407 | 33 | 2.433 | 59 | 2.459 |
| 8 | 2.408 | 34 | 2.434 | 60 | 2.460 |
| 9 | 2.409 | 35 | 2.435 | 61 | 2.461 |
| 10 | 2.410 | 36 | 2.436 | 62 | 2.462 |
| 11 | 2.411 | 37 | 2.437 | 63 | 2.463 |
| 12 | 2.412 | 38 | 2.438 | 64 | 2.464 |
| 13 | 2.413 | 39 | 2.439 | 65 | 2.465 |
| 14 | 2.414 | 40 | 2.440 | 66 | 2.466 |
| 15 | 2.415 | 41 | 2.441 | 67 | 2.467 |
| 16 | 2.416 | 42 | 2.442 | 68 | 2.468 |
| 17 | 2.417 | 43 | 2.443 | 69 | 2.469 |
| 18 | 2.418 | 44 | 2.444 | 70 | 2.470 |
| 19 | 2.419 | 45 | 2.445 | 71 | 2.471 |
| 20 | 2.420 | 46 | 2.446 | 72 | 2.472 |
| 21 | 2.421 | 47 | 2.447 | 73 | 2.473 |
| 22 | 2.422 | 48 | 2.448 | 74 | 2.474 |
| 23 | 2.423 | 49 | 2.449 | 75 | 2.475 |
| 24 | 2.424 | 50 | 2.450 | 76 | 2.476 |
| 25 | 2.425 | 51 | 2.451 | 77 | 2.477 |
| 26 | 2.426 | 52 | 2.452 | 78 | 2.478 |
| 27 | 2.427 | 53 | 2.453 | 79 | 2.479 |
|  |  |  |  | 80 | 2.480 |

Table 10-132:  USA and European Requirements

(Values specified in GHz)

| Channel # | Value | Channel # | Value | Channel # | Value |
| --- | --- | --- | --- | --- | --- |
| 73 | 2.473 | 81 | 2.481 | 89 | 2.489 |
| 74 | 2.474 | 82 | 2.482 | 90 | 2.490 |
| 75 | 2.475 | 83 | 2.483 | 91 | 2.491 |
| 76 | 2.476 | 84 | 2.484 | 92 | 2.492 |
| 77 | 2.477 | 85 | 2.485 | 93 | 2.493 |
| 78 | 2.478 | 86 | 2.486 | 94 | 2.494 |
| 79 | 2.479 | 87 | 2.487 | 95 | 2.495 |
| 80 | 2.480 | 88 | 2.488 | - | - |

Table 10-143:  Japan Requirements

(Values specified in GHz)

### 90 Occupied Channel Bandwidth

Occupied channel bandwidth shall meet all applicable local geographic regulations for 1 MHz channel spacing.  The rate at which the PMD entity will hop at is governed by the MAC. The hop rate, on the other hand, is a managed object with a maximum dwell time subject to local geographic regulations.Occupied channel bandwidth shall meet all applicable requirements issued by regulatory organizations for the geography of operation.

### 91 Minimum Hop Rate

The PMD entity will hop at a rate governed by the MAC. Since the MAC must have the ability to maximize the use of each hop interval, the MAC must tell the PMD when to hop, thus defining the system hop rate.

The minimum hop rate will be governed by the regulatory authorities. The minimum hop rate is specified by the number of channels visited divided by the total time spent on each of these channels. For the U.S.A., Part 15.247 of the Rules of the FCC states that a PMD must visit at least 75 channels in a 30 second period:

µ §

### 92  Hop Sequences

The hopping sequence of an individual PMD entity is used to co-locate multiple PMD entities in similar networks in the same geographic area and to enhance the overall efficiency and throughput capacity of each individual network. A frequency hopping pattern, Fx , consists of a permutation of all frequency channels defined in Tables 10-132 and 10-143.  For a given pattern number, x, the hopping sequence can be written as:

Fx = {fx (1), fx (2),...fx (p)}

where,

fx (i) = channel number (as defined in 10.6.4) for ith frequency in xth hopping pattern

p = number of frequency channels in hopping pattern (79 for US/Europe, 23 for Japan)

Given the hopping pattern number, x, and the index for the next frequency, i, the channel number shall be defined to be:

fx (i)  =  [(i - 1) * x] mod (79) + 2          in US and Europe

=  [(i - 1) * x] mod (23) + 73        in Japan.

Each of the Fx contains each of the p frequency channels equally often. Given a pattern length of p and the criterion of minimal adjacent channel interference, the number of usable hopping sequences in a geographically co-located area that can be derived is:

µ §

where k = the number of adjacent channel interferers on each side of the channel frequency and F is the minimum distance in frequency between contiguous hops. For the 802.11 compliant FHSS PMD in the USA and Europe, there are three sets of hopping sequences with 22 patterns per sequence that meet the criterion of one adjacent channel interferer on each side of the desired channel.  The three sets of hopping sequences of 22 patterns each are listed Tables A, B, and C in the AnnexSection 10.8.  Similarly, there are three sets in Japan, except each set has four patterns as listed in Table D in the AnnexSection 10.8.  The channel numbers listed under each pattern refer to the actual frequency values listed in Tables 10-131 and 10-142.

### 93   Unwanted Emissions

Conformant PMD implementations of this FHSS standard shall limit the emissions that fall outside of the operating frequency range, defined in Table 10-110 of Section 10.6.32, to the geographically applicable limitsregulations. For the U.S.A., the Rules of the FCC parts 15.247, 15.205 and 15.209 are the applicable regulations that govern these emissions.  For Japan, RCR STD-33 and for Europe, ETS 300 328, are the applicable regulations.

### 94  Modulation

The minimum set of requirements for a PMD to be compliant with the 802.11 FHSS PHY shall be as follows.

The PMD shall be capable of operating using 2 level Gaussian Frequency Shift Key (GFSK) modulation with a nominal bandwidth bit-period (BT)=0.5.   The PMD shall accept symbols from the set {{1},{0}}from the PLCP.  The symbol {1} shall be encoded with a peak deviation of (+f), giving a peak transmit frequency of  (Fc+f), which is greater than the carrier center frequency (Fc).  The symbol {0} shall be encoded with a peak frequency deviation of (-f), giving a peak transmit frequency of (Fc-f).

The peak frequency deviation, as shown in  Figure10-156 below shall be greater than 110 kHz relative to the nominal center frequency Fc.  Fc is the average center frequency of the last 8 bits of the preamble prior to the unique word.  The deviation shall be measured mid symbol.  Maximum deviation is not specified, but modulation is subject to the occupied bandwidth limits of 10.6.5.

The zero crossing error shall be less than +/- 1/8 of a symbol period.  The zero crossing error is the time difference between the ideal symbol periods and measured crossings of Fc.  This is illustrated in Figure 10-156 below.

µ §

Figure 10-156:  Transmit Modulation Mask

### 95  Channel Data Rate

A compliant 802.11 FHSS PMD shall be capable of transmitting and receiving at a nominal data rate of 1.0 Mbps.

### 96  Channel Switching/Settling Time

The time to change from one operating channel frequency, as specified in section 10.6.32, is defined as 224 usec.  A conformant PMD is said to meet this switching time specification when the desired final operating channel center frequency has settled to within +/- 60 kHz of the operating channel center frequency as outlined in section 10.6.32.

#### 97  (Not Used)

### 98 Receive to Transmit Switch Time

The maximum time for a conformant PMD to switch the radio from the receive state to the transmit state and place the start of the first bit on the air shall be 19 usec.  At the end of this 19 usec, the rf carrier shall be within 1 dB of its final transmit power level, and within the described modulation specifications.

#### 99 (Not Used)

### 100  PMD Transmit Specifications

The following section describes the transmit functions and parameters associated with the Physical Medium Dependent sublayer. In general, these are specified by primitives from the PLCP and the Transmit PMD entity provides the actual means by which the signals required by the PLCP primitives are imposed onto the medium.

#### 101  Nominal Frame Transmit Power

The nominal transmit power of a frame is defined as the power averaged between the start of the first symbol in the PLCP header to the end of the last symbol in the PLCP header.  When in the transmit state, the transmit power shall be within 2dB of the nominal transmit power from the start of the first symbol in the PLPC header to the last symbol at the end of the frame.

#### 102  Transmit Power Levels

Unless governed by more stringent local geographic regulations, the radiated emissions from compliant devices shall meet ANSI C95.1-1991 (latest revision) limits for controlled or uncontrolled environments, in accordance with their intended usage.  In addition, all conformant PMD implementations shall support at least one power level with a minimum Equivalent Isotropically Radiated Power (EIRP) of 10 mW.

#### 103 Transmit Power Level Control

If a conformant PMD implementation has the ability to transmit in a manner that results in the EIRP of the transmit signal exceeding the level of 100 mW, at least one level of transmit power control shall be implemented.  This transmit power control shall be such that the level of the emission is reduced to a level at or below 100 mW under the influence of said power control.

#### 104  Transmit Spectrum Shape

Transmitter shall pass a spectrum mask test.  The duty cycle between Tx and Rx is nominally 50% and the transmit frame length is nominally 400 usec.  The adjacent channel power, which is the sum of the power measured in a 1 MHz band, shall be either less than -70 dBm or a function of channel offset N from the assigned transmitter channel M.

Channel

N=M+/-2	-20dBm or -40dBc, whichever is the lower power.

N>=M+/-3	-40dBm or -60dBc, whichever is the lowest power.

The levels given in dBc are measured relative to the transmitter power measured in a 1MHz channel centered on the transmitter center frequency.  The adjacent channel power and the transmitter power for this section of the specification shall be measured with a resolution bandwidth of 100 kHz, with a peak detector and the measurement device set to maximum hold.

Within the frequency band of 2.4 GHz to 2.4835 GHz, two failures are permitted providing they are less than -50dBc.

#### 105  Transmit Center Frequency Tolerance

The PMD shall have a transmit center frequency accuracy, as measured from Fc, of ± 60 Khz.

#### 106  Transmitter Ramp Periods

The transmitter shall go from off to within 2 dB of the nominal transmit power in 8 usec or less.  The transmitter shall go from within 2dB of the nominal transmit power, to off (less than -50dBm) in 8 usec. or less.

### 107  PMD Receiver Specifications

The following section describes the receive functions and parameters associated with the Physical Medium Dependent sublayer. In general, these are specified by primitives from the PLCP and the Receive PMD entity provides the actual means by which the signals required by the PLCP primitives are recovered from the medium. The PMD sublayer monitors signals on the medium and will return symbols from the set {{1},{0}} to the PLCP Sublayer.  The signal leakage when receiving shall not exceed -45 dBm peak in the operating frequency range.

#### 108 Input Dynamic Range

The PMD shall be capable of recovering a conformant PMD signal from the medium, as described in related sections, whose level at the output of the antenna is between -80 dBm (defined as minimum sensitivity) and -10 dBm (defined as maximum allowable input level).  A conformant PMD shall maintain a PERBER of less than or equal to 10-25 over this dynamic range.

#### 109  Receive Center Frequency Acceptance Range

An 802.11 FHSS compliant PMD shall meet all specifications with an input signal having a center frequency range of ± 60kHz from nominal.

#### 110  Clear Channel Assessment Power Threshold

The PHY shall, in the presence of any 802.11 compliant FH PMD signal above -85 dBm,  signal busy with a 90% probability in detection of the preamble within the CCA assessment window.  The PHY shall, in the presence of any 802.11 compliant FH PMD signal above -65 dBm, signal busy with a 70% probability for detection of random data within the CCA assessment window.  This specification applies to a PMD operating with a nominal EIRP of  <100mW.  A compliant PMD operating at a nominal output power greater than 100mW shall use the following equation to define the CCA threshold.  Pt represents Transmit Power.

CCA Threshold (Preamble) =-85dBm -(5*log10(Pt/100mW))dBm

CCA Threshold (Random Data) = CCA Threshold (Preamble) + 20 dB

##### 111 Ramp Up Period

The purpose of a conformant PMD Ramp Up Time Period is to control the rate of change of the amplitude of the transmit signal during its transition to the desired steady state transmit output level.  The ramp-up period is defined as a window consisting of 8 usec and is governed by the mask of Figure 10-18.

The following states are defined by the mask of diagram Figure 10-18.  The transmitter is considered "off" (i.e. less than -50 dBm EIRP) at the start of the first 1 usec period and is less than 0 dBm at the end of the first 1 usec period.  The output level at the end of the seventh 1 usec period is within 3 dB of the desired steady state transmit power level and within 1 dB at the end of the eighth 1 usec period.

µ §

Figure 10-18:  Ramp Up

##### 112 Ramp Down Period

The purpose of a conformant PMD Ramp Down Time Period is to control the rate of change of the amplitude of the transmit signal during its transition to the desired steady state transmit output level.  The ramp-down period is defined as a window consisting of an 8 usec period and is governed by the mask of Figure 10-19.

The following states are defined by the mask of Figure 10-19.  The transmitter is considered "off" (i.e. less than -50 dBm EIRP) at the start of the first 1 usec period and is less than 0 dBm at the end of the first 1 usec.  The output level at the end of the seventh 1 usec period is within 3 dB of the desired steady state transmit power level and within 1 dB at the end of the eighth 1 usec period.

µ §

Figure 10-19:  Ramp Down

#### 113  Minimum Receiver Sensitivity

Minimum sensitivity is defined as the minimum signal level required to produce a PERBER of 10-25.  The PMD shall have the minimum sensitivity be less than or equal to -80 dBm across the operating frequency range specified in 10.6.2.

#### 114 Intermodulation

Intermodulation protection (IMp) is defined as the ratio of the desired signal strength toto measured sensitivity of the minimum amplitude of one of the two equal level interfering signals at 4 and 8 MHz removed from center frequency, both on the same side of center frequency, that cause the PERBER of the receiver to be increased to 10-25, when the desired signal is -77 dBm3 dB above sensitivity.  Each interfering signal is modulated with the FH PMD modulation uncorrelated in time to each other or the desired signal.  ThePMD shall have the IMp for the interfering signal at 4 and 8 MHz be greater than or equal to 30 dB.

#### 115  Desensitization

Desensitization (Dp) is defined as the ratio to measured sensitivity of the minimum amplitude of an interfering signal that causes the PERBER at the output of the receiver to be increased to 10-25 when the desired signal is -77 dBm.  The interfering signal shall be modulated with the FHSS PMD modulation uncorrelated in time to the desired signal.  The minimum Dp shall be as given in Table 10-154.  The sprectral purity of the interferer shall be sufficient to ensure that the measurement is limited by the receiver performance.

| Interferer Frequency | DP Minimum |
| --- | --- |
| M=N+/-2 | 30dB |
| M=N+/-3 or more | 40dB |

Table 10-154:  1M Bit Desensitization

*M is the interferer frequency and N is the desired channel frequency

### 116  Operating Temperature Range

Two temperature ranges for full operation compliance to the FH PHY are specified.  Type 1 is defined as 0° to 40° C is designated for office environments.  Type 2 is defined as -30° to 70° C and is designated for industrial environments.

## 117 FHSS Physical Medium Dependent Sublayer 2.0M Bit

### 118 Introduction

The following section defines the requirements for implementing the FHSS PHY 2.0M bit PMD.  Since many of the requirements of this PMD are the same as the 1.0M bit PMD, this section only identifies the specifications where the two PMDs differ.  When the specifications are the same, these specifications will be contained in section 10.6.

### 119 Regulatory Requirements

See section 10.6.2

### 120 Operating Frequency Range

See Section 10.6.32

### 121 Number of Operating Channels

See Section 10.6.43

### 122 Operating Channel Center Frequency

See Section 10.6.54

### 123 Occupied Channel Bandwidth

See Section 10.6.65

### 124  Minimum Hop Rate

See Section 10.6.76

### 125  Hop Sequences

See Section 10.6.87

### 126  Unwanted Emissions

See Section 10.6.98

### 127 4 Level GFSK Modulation

For a FHSS 2MB/sec PMD, the modulation scheme shall be 4 level Gaussian Frequency Shift Keying (4GFSK), with a nominal symbol-period bandwidth product (BT) = 0.5.  The four level deviation factor, defined as the frequency separation of adjacent symbols divided by symbol rate, h4, shall be related to the deviation factor of the 2GFSK modulation, h2, by the following equation:

h4/h2 = 0.45+/-0.01

An incoming bit stream at 2 Mb/sec will be converted to 2 bit words or symbols, with a rate of Fclk=1M symbol/sec.  The first received bit will be encoded as the left most bit of the symbol in the table below.   The bits will be encoded into symbols as shown in Ttable 10-16.15 below:

1 Mbit/sec, 2-GFSK

| Symbol | Carrier Deviation |
| --- | --- |
| 1 | 1/2 * h2*Fclk |
| 0 | -1/2 * h2*Fclk |

2 Mbit/sec, 4-GFSK

| Symbol | Carrier Deviation |
| --- | --- |
| 10 | 3/2 * h4*Fclk |
| 11 | 1/2 * h4*Fclk |
| 01 | -1/2 * h4*Fclk |
| 00 | -3/2 * h4*Fclk |

tc "Table 10.15.  Symbol Encoding vs. Carrier Deviation"\l§Table 10-16.15.  Symbol Encoding into Carrier Deviation

*Note: The frequency deviations shown in the Ttable 10-16.15 are achieved by symbols being surrounded by identical symbols; in actual data stream the instantaneous deviation will vary due to Gaussian pulse shaping.

The modulation error shall be less than +/-15kHz at the mid symbol time for 4-GFSK, from the frequency deviations specified above, for a symbol surrounded by identical symbols, and less than +/-25 KHz for any symbol. The deviation is relative to the nominal center frequency of the RF carrier.  For definition purposes, the nominal center frequency is the mid frequency between symbols 11 and 01.  The nominal center frequency shall not vary greater than +/-10kHz/msec, from the start to end of the 4GFSK data word.   H4, measured as a difference between the outmost frequencies, divided by 3, divided by 1 MHz, should have a minimum value of 0.140. The ratio h4/h2 will be 0.45+/-0.01. The peak to peak deviation h2 of the 2-GFSK is measured in the middle of  0000 and 1111 patterns encountered in the  unique word in the PHY header.  Symbols and terms used within this section are illustrated in the figure 10-1620 below:

µ §

tc "Figure 10-20.  4Level GFSK Transmit Modulation"\l§Figure 10-1620.  4 Level GFSK Transmit Modulation

#### 128 Frame Structure for HS FHSS PHY

The HS FHSS PHY frame consists of PLCP preamble, PLCP header and PLCP_PDU. The PLCP preamble and PLCP header format are identical to 1 Mbit PHY, as described in 10.3.2. The PLCP_PDU is transmitted in 2GFSK, 4GFSK or higher format, according to the rate chosen. The rate is indicated in a 2 bit field in a PLCP header, having value of 1 or 2 bits/symbol (or Mbit/sec).

The PLCP_PDU is transmitted as 4 level symbols, with the amount determined by number_of_symbols = (number_of_MPDU_bytes*8)/rate .

The input bits are scrambled according to method in 10.3.2.3.

The scrambled bit stream is divided into groups of rate (1 or 2) consecutive bits. The bits are mapped into symbols according to Ttable 10-16.15.

A Bias suppression algorithm is applied to the resulting symbol stream. The bias suppression algorithm is defined in 10.3.2.3, figs. 10-5,10-7a. A polarity control symbol is inserted prior to each block of 32 symbols (or less for the last block). The polarity control signals are 4GFSK symbols 10 or 00. The algorithm is equivalent to the case of 2GFSK, with the polarity symbol 2GFSK '1' replaced with 4GFSK symbol 10, and the 2GFSK polarity symbol '0' replaced with a 4GFSK symbol '00'.

### 129 Channel Data Rate

The FHSS 2M bit per sec PMD will be capable of transmitting and receiving at a nominal data rate of 2.0 Mbps and 1 Mbps.

### 130 Channel Switching/Settling Time

See Section 10.6.121.

#### 131 Transmit to Receive Switch Time

See Section 10.6.12

### 132 Receive to Transmit Switch Time

See Section 10.6.13

#### 133 Operating Channel Availability

See Section 10.6.14

### 134  PMD Transmit Specifications

See Section 10.6.145

#### 135 Nominal Frame Transmit Power

See Section 10.6.14.1

#### 136 Transmit Power Levels

See Section 10.6.14.216

#### 137  Transmit Power Level Control

See Section 10.6.14.317

#### 138 Transmit Spectrum Shape

See Section 10.6.14.418

#### 139 Transmit Center Frequency Tolerance

See Section 10.6.14.519

#### 140 Transmitter Ramp Periods

See Secion 10.6.14.6

### 141 PMD Receiver Specifications

See Section 10.6.1520

#### 142  Input Dynamic Range

The PMD implementation shall be capable of recovering a conformant PMD signal from the medium, as described in related sections, whose level is between -75 dBm (defined as minimum sensitivity) and -10 dBm (defined as maximum allowable input level).

#### 143 Receive Center Frequency Acceptance Range

See Sections 10.6.15.222

#### 144 Clear Channel Assessment Power Threshhold

See Sections 10.6.15.323

##### 145 Ramp Up Period

See Sections 10.6.24

##### 146 Ramp Down Period

See Sections 10.6.25

#### 147 Minimum Receiver Sensitivity

Sensitivity is defined as the minimum signal level required to produce a PERBER of 10-25.  ThePMD shall have the minimum signal level be less than or equal to -75 dBm across the operating frequency range specified in 10.6.2.

#### 148 Intermodulation

Intermodulation protection (IMp) is defined as the ratio to -77 dBm of the minimum amplitude of one of the two equal level interfering signals at 4 and 8 MHz removed from center frequency, both on the same side of center frequency, that cause the PERBER of the receiver to be increased to 10-25, when the desired signal is -72 dBm.  Each interfering signal is modulated with the FH 1Mb/sec PMD modulation uncorrolated in time to each other or the desired signal.  The FHSS 2Mb/sec PMD shall have the IMp for the interfering signal at 4 and 8 MHz be greater than or equal to 30 dB.

#### 149  Desensitization

Desensitization (Dp) is defined as the ratio to measured sensitivity of the minimum amplitude of an interfering signal that causes the PERBER of the receiver to be increased to 10-25 when the desired signal is -72 dB (3 dB above sensitivity specified in section 10.7.26).  The interfering signal shall be modulated with the FHSS PMD modulation uncorrolated in time to the desired signal.  Dp shall be greater than or equal to the valuesfigures given in Table 10-176 below:

| Interferer Frequency | DP Minimum |
| --- | --- |
| M=N+/-2 | 24dB |
| M=N+/-3 or more | 35dB |

Table 10-176:  2M Bit Desensitization

*M is the interferer frequency and N is the desired channel frequency

### 150  Operating Temperature Range

See Section 10.6.16

## 151 Hopping Sequences

Table A  Hopping Sequence Set 1

Pattern

| Index | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 3 | 16 | 22 | 28 | 34 | 40 | 46 | 52 | 58 | 64 | 70 | 76 |
| 4 | 23 | 32 | 41 | 50 | 59 | 68 | 77 | 7 | 16 | 25 | 34 |
| 5 | 30 | 42 | 54 | 66 | 78 | 11 | 23 | 35 | 47 | 59 | 71 |
| 6 | 37 | 52 | 67 | 3 | 18 | 33 | 48 | 63 | 78 | 14 | 29 |
| 7 | 44 | 62 | 80 | 19 | 37 | 55 | 73 | 12 | 30 | 48 | 66 |
| 8 | 51 | 72 | 14 | 35 | 56 | 77 | 19 | 40 | 61 | 3 | 24 |
| 9 | 58 | 3 | 27 | 51 | 75 | 20 | 44 | 68 | 13 | 37 | 61 |
| 10 | 65 | 13 | 40 | 67 | 15 | 42 | 69 | 17 | 44 | 71 | 19 |
| 11 | 72 | 23 | 53 | 4 | 34 | 64 | 15 | 45 | 75 | 26 | 56 |
| 12 | 79 | 33 | 66 | 20 | 53 | 7 | 40 | 73 | 27 | 60 | 14 |
| 13 | 7 | 43 | 79 | 36 | 72 | 29 | 65 | 22 | 58 | 15 | 51 |
| 14 | 14 | 53 | 13 | 52 | 12 | 51 | 11 | 50 | 10 | 49 | 9 |
| 15 | 21 | 63 | 26 | 68 | 31 | 73 | 36 | 78 | 41 | 4 | 46 |
| 16 | 28 | 73 | 39 | 5 | 50 | 16 | 61 | 27 | 72 | 38 | 4 |
| 17 | 35 | 4 | 52 | 21 | 69 | 38 | 7 | 55 | 24 | 72 | 41 |
| 18 | 42 | 14 | 65 | 37 | 9 | 60 | 32 | 4 | 55 | 27 | 78 |
| 19 | 49 | 24 | 78 | 53 | 28 | 3 | 57 | 32 | 7 | 61 | 36 |
| 20 | 56 | 34 | 12 | 69 | 47 | 25 | 3 | 60 | 38 | 16 | 73 |
| 21 | 63 | 44 | 25 | 6 | 66 | 47 | 28 | 9 | 69 | 50 | 31 |
| 22 | 70 | 54 | 38 | 22 | 6 | 69 | 53 | 37 | 21 | 5 | 68 |
| 23 | 77 | 64 | 51 | 38 | 25 | 12 | 78 | 65 | 52 | 39 | 26 |
| 24 | 5 | 74 | 64 | 54 | 44 | 34 | 24 | 14 | 4 | 73 | 63 |
| 25 | 12 | 5 | 77 | 70 | 63 | 56 | 49 | 42 | 35 | 28 | 21 |
| 26 | 19 | 15 | 11 | 7 | 3 | 78 | 74 | 70 | 66 | 62 | 58 |
| 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| 28 | 33 | 35 | 37 | 39 | 41 | 43 | 45 | 47 | 49 | 51 | 53 |
| 29 | 40 | 45 | 50 | 55 | 60 | 65 | 70 | 75 | 80 | 6 | 11 |
| 30 | 47 | 55 | 63 | 71 | 79 | 8 | 16 | 24 | 32 | 40 | 48 |
| 31 | 54 | 65 | 76 | 8 | 19 | 30 | 41 | 52 | 63 | 74 | 6 |
| 32 | 61 | 75 | 10 | 24 | 38 | 52 | 66 | 80 | 15 | 29 | 43 |
| 33 | 68 | 6 | 23 | 40 | 57 | 74 | 12 | 29 | 46 | 63 | 80 |
| 34 | 75 | 16 | 36 | 56 | 76 | 17 | 37 | 57 | 77 | 18 | 38 |
| 35 | 3 | 26 | 49 | 72 | 16 | 39 | 62 | 6 | 29 | 52 | 75 |
| 36 | 10 | 36 | 62 | 9 | 35 | 61 | 8 | 34 | 60 | 7 | 33 |
| 37 | 17 | 46 | 75 | 25 | 54 | 4 | 33 | 62 | 12 | 41 | 70 |
| 38 | 24 | 56 | 9 | 41 | 73 | 26 | 58 | 11 | 43 | 75 | 28 |
| 39 | 31 | 66 | 22 | 57 | 13 | 48 | 4 | 39 | 74 | 30 | 65 |
| 40 | 38 | 76 | 35 | 73 | 32 | 70 | 29 | 67 | 26 | 64 | 23 |

Table A  Hopping Sequence Set 1

Pattern

| Index | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 45 | 7 | 48 | 10 | 51 | 13 | 54 | 16 | 57 | 19 | 60 |
| 42 | 52 | 17 | 61 | 26 | 70 | 35 | 79 | 44 | 9 | 53 | 18 |
| 43 | 59 | 27 | 74 | 42 | 10 | 57 | 25 | 72 | 40 | 8 | 55 |
| 44 | 66 | 37 | 8 | 58 | 29 | 79 | 50 | 21 | 71 | 42 | 13 |
| 45 | 73 | 47 | 21 | 74 | 48 | 22 | 75 | 49 | 23 | 76 | 50 |
| 46 | 80 | 57 | 34 | 11 | 67 | 44 | 21 | 77 | 54 | 31 | 8 |
| 47 | 8 | 67 | 47 | 27 | 7 | 66 | 46 | 26 | 6 | 65 | 45 |
| 48 | 15 | 77 | 60 | 43 | 26 | 9 | 71 | 54 | 37 | 20 | 3 |
| 49 | 22 | 8 | 73 | 59 | 45 | 31 | 17 | 3 | 68 | 54 | 40 |
| 50 | 29 | 18 | 7 | 75 | 64 | 53 | 42 | 31 | 20 | 9 | 77 |
| 51 | 36 | 28 | 20 | 12 | 4 | 75 | 67 | 59 | 51 | 43 | 35 |
| 52 | 43 | 38 | 33 | 28 | 23 | 18 | 13 | 8 | 3 | 77 | 72 |
| 53 | 50 | 48 | 46 | 44 | 42 | 40 | 38 | 36 | 34 | 32 | 30 |
| 54 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 |
| 55 | 64 | 68 | 72 | 76 | 80 | 5 | 9 | 13 | 17 | 21 | 25 |
| 56 | 71 | 78 | 6 | 13 | 20 | 27 | 34 | 41 | 48 | 55 | 62 |
| 57 | 78 | 9 | 19 | 29 | 39 | 49 | 59 | 69 | 79 | 10 | 20 |
| 58 | 6 | 19 | 32 | 45 | 58 | 71 | 5 | 18 | 31 | 44 | 57 |
| 59 | 13 | 29 | 45 | 61 | 77 | 14 | 30 | 46 | 62 | 78 | 15 |
| 60 | 20 | 39 | 58 | 77 | 17 | 36 | 55 | 74 | 14 | 33 | 52 |
| 61 | 27 | 49 | 71 | 14 | 36 | 58 | 80 | 23 | 45 | 67 | 10 |
| 62 | 34 | 59 | 5 | 30 | 55 | 80 | 26 | 51 | 76 | 22 | 47 |
| 63 | 41 | 69 | 18 | 46 | 74 | 23 | 51 | 79 | 28 | 56 | 5 |
| 64 | 48 | 79 | 31 | 62 | 14 | 45 | 76 | 28 | 59 | 11 | 42 |
| 65 | 55 | 10 | 44 | 78 | 33 | 67 | 22 | 56 | 11 | 45 | 79 |
| 66 | 62 | 20 | 57 | 15 | 52 | 10 | 47 | 5 | 42 | 79 | 37 |
| 67 | 69 | 30 | 70 | 31 | 71 | 32 | 72 | 33 | 73 | 34 | 74 |
| 68 | 76 | 40 | 4 | 47 | 11 | 54 | 18 | 61 | 25 | 68 | 32 |
| 69 | 4 | 50 | 17 | 63 | 30 | 76 | 43 | 10 | 56 | 23 | 69 |
| 70 | 11 | 60 | 30 | 79 | 49 | 19 | 68 | 38 | 8 | 57 | 27 |
| 71 | 18 | 70 | 43 | 16 | 68 | 41 | 14 | 66 | 39 | 12 | 64 |
| 72 | 25 | 80 | 56 | 32 | 8 | 63 | 39 | 15 | 70 | 46 | 22 |
| 73 | 32 | 11 | 69 | 48 | 27 | 6 | 64 | 43 | 22 | 80 | 59 |
| 74 | 39 | 21 | 3 | 64 | 46 | 28 | 10 | 71 | 53 | 35 | 17 |
| 75 | 46 | 31 | 16 | 80 | 65 | 50 | 35 | 20 | 5 | 69 | 54 |
| 76 | 53 | 41 | 29 | 17 | 5 | 72 | 60 | 48 | 36 | 24 | 12 |
| 77 | 60 | 51 | 42 | 33 | 24 | 15 | 6 | 76 | 67 | 58 | 49 |
| 78 | 67 | 61 | 55 | 49 | 43 | 37 | 31 | 25 | 19 | 13 | 7 |
| 79 | 74 | 71 | 68 | 65 | 62 | 59 | 56 | 53 | 50 | 47 | 44 |

Table A  Hopping Sequence Set 1 continued

Pattern

| Index | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| 3 | 3 | 9 | 15 | 21 | 27 | 33 | 39 | 45 | 51 | 57 | 63 |
| 4 | 43 | 52 | 61 | 70 | 79 | 9 | 18 | 27 | 36 | 45 | 54 |
| 5 | 4 | 16 | 28 | 40 | 52 | 64 | 76 | 9 | 21 | 33 | 45 |
| 6 | 44 | 59 | 74 | 10 | 25 | 40 | 55 | 70 | 6 | 21 | 36 |
| 7 | 5 | 23 | 41 | 59 | 77 | 16 | 34 | 52 | 70 | 9 | 27 |
| 8 | 45 | 66 | 8 | 29 | 50 | 71 | 13 | 34 | 55 | 76 | 18 |
| 9 | 6 | 30 | 54 | 78 | 23 | 47 | 71 | 16 | 40 | 64 | 9 |
| 10 | 46 | 73 | 21 | 48 | 75 | 23 | 50 | 77 | 25 | 52 | 79 |
| 11 | 7 | 37 | 67 | 18 | 48 | 78 | 29 | 59 | 10 | 40 | 70 |
| 12 | 47 | 80 | 34 | 67 | 21 | 54 | 8 | 41 | 74 | 28 | 61 |
| 13 | 8 | 44 | 80 | 37 | 73 | 30 | 66 | 23 | 59 | 16 | 52 |
| 14 | 48 | 8 | 47 | 7 | 46 | 6 | 45 | 5 | 44 | 4 | 43 |
| 15 | 9 | 51 | 14 | 56 | 19 | 61 | 24 | 66 | 29 | 71 | 34 |
| 16 | 49 | 15 | 60 | 26 | 71 | 37 | 3 | 48 | 14 | 59 | 25 |
| 17 | 10 | 58 | 27 | 75 | 44 | 13 | 61 | 30 | 78 | 47 | 16 |
| 18 | 50 | 22 | 73 | 45 | 17 | 68 | 40 | 12 | 63 | 35 | 7 |
| 19 | 11 | 65 | 40 | 15 | 69 | 44 | 19 | 73 | 48 | 23 | 77 |
| 20 | 51 | 29 | 7 | 64 | 42 | 20 | 77 | 55 | 33 | 11 | 68 |
| 21 | 12 | 72 | 53 | 34 | 15 | 75 | 56 | 37 | 18 | 78 | 59 |
| 22 | 52 | 36 | 20 | 4 | 67 | 51 | 35 | 19 | 3 | 66 | 50 |
| 23 | 13 | 79 | 66 | 53 | 40 | 27 | 14 | 80 | 67 | 54 | 41 |
| 24 | 53 | 43 | 33 | 23 | 13 | 3 | 72 | 62 | 52 | 42 | 32 |
| 25 | 14 | 7 | 79 | 72 | 65 | 58 | 51 | 44 | 37 | 30 | 23 |
| 26 | 54 | 50 | 46 | 42 | 38 | 34 | 30 | 26 | 22 | 18 | 14 |
| 27 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 |
| 28 | 55 | 57 | 59 | 61 | 63 | 65 | 67 | 69 | 71 | 73 | 75 |
| 29 | 16 | 21 | 26 | 31 | 36 | 41 | 46 | 51 | 56 | 61 | 66 |
| 30 | 56 | 64 | 72 | 80 | 9 | 17 | 25 | 33 | 41 | 49 | 57 |
| 31 | 17 | 28 | 39 | 50 | 61 | 72 | 4 | 15 | 26 | 37 | 48 |
| 32 | 57 | 71 | 6 | 20 | 34 | 48 | 62 | 76 | 11 | 25 | 39 |
| 33 | 18 | 35 | 52 | 69 | 7 | 24 | 41 | 58 | 75 | 13 | 30 |
| 34 | 58 | 78 | 19 | 39 | 59 | 79 | 20 | 40 | 60 | 80 | 21 |
| 35 | 19 | 42 | 65 | 9 | 32 | 55 | 78 | 22 | 45 | 68 | 12 |
| 36 | 59 | 6 | 32 | 58 | 5 | 31 | 57 | 4 | 30 | 56 | 3 |
| 37 | 20 | 49 | 78 | 28 | 57 | 7 | 36 | 65 | 15 | 44 | 73 |
| 38 | 60 | 13 | 45 | 77 | 30 | 62 | 15 | 47 | 79 | 32 | 64 |
| 39 | 21 | 56 | 12 | 47 | 3 | 38 | 73 | 29 | 64 | 20 | 55 |
| 40 | 61 | 20 | 58 | 17 | 55 | 14 | 52 | 11 | 49 | 8 | 46 |

Table A  Hopping Sequence Set 1 continued

Pattern

| Index | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 22 | 63 | 25 | 66 | 28 | 69 | 31 | 72 | 34 | 75 | 37 |
| 42 | 62 | 27 | 71 | 36 | 80 | 45 | 10 | 54 | 19 | 63 | 28 |
| 43 | 23 | 70 | 38 | 6 | 53 | 21 | 68 | 36 | 4 | 51 | 19 |
| 44 | 63 | 34 | 5 | 55 | 26 | 76 | 47 | 18 | 68 | 39 | 10 |
| 45 | 24 | 77 | 51 | 25 | 78 | 52 | 26 | 79 | 53 | 27 | 80 |
| 46 | 64 | 41 | 18 | 74 | 51 | 28 | 5 | 61 | 38 | 15 | 71 |
| 47 | 25 | 5 | 64 | 44 | 24 | 4 | 63 | 43 | 23 | 3 | 62 |
| 48 | 65 | 48 | 31 | 14 | 76 | 59 | 42 | 25 | 8 | 70 | 53 |
| 49 | 26 | 12 | 77 | 63 | 49 | 35 | 21 | 7 | 72 | 58 | 44 |
| 50 | 66 | 55 | 44 | 33 | 22 | 11 | 79 | 68 | 57 | 46 | 35 |
| 51 | 27 | 19 | 11 | 3 | 74 | 66 | 58 | 50 | 42 | 34 | 26 |
| 52 | 67 | 62 | 57 | 52 | 47 | 42 | 37 | 32 | 27 | 22 | 17 |
| 53 | 28 | 26 | 24 | 22 | 20 | 18 | 16 | 14 | 12 | 10 | 8 |
| 54 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 |
| 55 | 29 | 33 | 37 | 41 | 45 | 49 | 53 | 57 | 61 | 65 | 69 |
| 56 | 69 | 76 | 4 | 11 | 18 | 25 | 32 | 39 | 46 | 53 | 60 |
| 57 | 30 | 40 | 50 | 60 | 70 | 80 | 11 | 21 | 31 | 41 | 51 |
| 58 | 70 | 4 | 17 | 30 | 43 | 56 | 69 | 3 | 16 | 29 | 42 |
| 59 | 31 | 47 | 63 | 79 | 16 | 32 | 48 | 64 | 80 | 17 | 33 |
| 60 | 71 | 11 | 30 | 49 | 68 | 8 | 27 | 46 | 65 | 5 | 24 |
| 61 | 32 | 54 | 76 | 19 | 41 | 63 | 6 | 28 | 50 | 72 | 15 |
| 62 | 72 | 18 | 43 | 68 | 14 | 39 | 64 | 10 | 35 | 60 | 6 |
| 63 | 33 | 61 | 10 | 38 | 66 | 15 | 43 | 71 | 20 | 48 | 76 |
| 64 | 73 | 25 | 56 | 8 | 39 | 70 | 22 | 53 | 5 | 36 | 67 |
| 65 | 34 | 68 | 23 | 57 | 12 | 46 | 80 | 35 | 69 | 24 | 58 |
| 66 | 74 | 32 | 69 | 27 | 64 | 22 | 59 | 17 | 54 | 12 | 49 |
| 67 | 35 | 75 | 36 | 76 | 37 | 77 | 38 | 78 | 39 | 79 | 40 |
| 68 | 75 | 39 | 3 | 46 | 10 | 53 | 17 | 60 | 24 | 67 | 31 |
| 69 | 36 | 3 | 49 | 16 | 62 | 29 | 75 | 42 | 9 | 55 | 22 |
| 70 | 76 | 46 | 16 | 65 | 35 | 5 | 54 | 24 | 73 | 43 | 13 |
| 71 | 37 | 10 | 62 | 35 | 8 | 60 | 33 | 6 | 58 | 31 | 4 |
| 72 | 77 | 53 | 29 | 5 | 60 | 36 | 12 | 67 | 43 | 19 | 74 |
| 73 | 38 | 17 | 75 | 54 | 33 | 12 | 70 | 49 | 28 | 7 | 65 |
| 74 | 78 | 60 | 42 | 24 | 6 | 67 | 49 | 31 | 13 | 74 | 56 |
| 75 | 39 | 24 | 9 | 73 | 58 | 43 | 28 | 13 | 77 | 62 | 47 |
| 76 | 79 | 67 | 55 | 43 | 31 | 19 | 7 | 74 | 62 | 50 | 38 |
| 77 | 40 | 31 | 22 | 13 | 4 | 74 | 65 | 56 | 47 | 38 | 29 |
| 78 | 80 | 74 | 68 | 62 | 56 | 50 | 44 | 38 | 32 | 26 | 20 |
| 79 | 41 | 38 | 35 | 32 | 29 | 26 | 23 | 20 | 17 | 14 | 11 |

Table B  Hopping Sequence Set 2

Pattern

| Index | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 3 | 18 | 24 | 30 | 36 | 42 | 48 | 54 | 60 | 66 | 72 | 78 |
| 4 | 26 | 35 | 44 | 53 | 62 | 71 | 80 | 10 | 19 | 28 | 37 |
| 5 | 34 | 46 | 58 | 70 | 3 | 15 | 27 | 39 | 51 | 63 | 75 |
| 6 | 42 | 57 | 72 | 8 | 23 | 38 | 53 | 68 | 4 | 19 | 34 |
| 7 | 50 | 68 | 7 | 25 | 43 | 61 | 79 | 18 | 36 | 54 | 72 |
| 8 | 58 | 79 | 21 | 42 | 63 | 5 | 26 | 47 | 68 | 10 | 31 |
| 9 | 66 | 11 | 35 | 59 | 4 | 28 | 52 | 76 | 21 | 45 | 69 |
| 10 | 74 | 22 | 49 | 76 | 24 | 51 | 78 | 26 | 53 | 80 | 28 |
| 11 | 3 | 33 | 63 | 14 | 44 | 74 | 25 | 55 | 6 | 36 | 66 |
| 12 | 11 | 44 | 77 | 31 | 64 | 18 | 51 | 5 | 38 | 71 | 25 |
| 13 | 19 | 55 | 12 | 48 | 5 | 41 | 77 | 34 | 70 | 27 | 63 |
| 14 | 27 | 66 | 26 | 65 | 25 | 64 | 24 | 63 | 23 | 62 | 22 |
| 15 | 35 | 77 | 40 | 3 | 45 | 8 | 50 | 13 | 55 | 18 | 60 |
| 16 | 43 | 9 | 54 | 20 | 65 | 31 | 76 | 42 | 8 | 53 | 19 |
| 17 | 51 | 20 | 68 | 37 | 6 | 54 | 23 | 71 | 40 | 9 | 57 |
| 18 | 59 | 31 | 3 | 54 | 26 | 77 | 49 | 21 | 72 | 44 | 16 |
| 19 | 67 | 42 | 17 | 71 | 46 | 21 | 75 | 50 | 25 | 79 | 54 |
| 20 | 75 | 53 | 31 | 9 | 66 | 44 | 22 | 79 | 57 | 35 | 13 |
| 21 | 4 | 64 | 45 | 26 | 7 | 67 | 48 | 29 | 10 | 70 | 51 |
| 22 | 12 | 75 | 59 | 43 | 27 | 11 | 74 | 58 | 42 | 26 | 10 |
| 23 | 20 | 7 | 73 | 60 | 47 | 34 | 21 | 8 | 74 | 61 | 48 |
| 24 | 28 | 18 | 8 | 77 | 67 | 57 | 47 | 37 | 27 | 17 | 7 |
| 25 | 36 | 29 | 22 | 15 | 8 | 80 | 73 | 66 | 59 | 52 | 45 |
| 26 | 44 | 40 | 36 | 32 | 28 | 24 | 20 | 16 | 12 | 8 | 4 |
| 27 | 52 | 51 | 50 | 49 | 48 | 47 | 46 | 45 | 44 | 43 | 42 |
| 28 | 60 | 62 | 64 | 66 | 68 | 70 | 72 | 74 | 76 | 78 | 80 |
| 29 | 68 | 73 | 78 | 4 | 9 | 14 | 19 | 24 | 29 | 34 | 39 |
| 30 | 76 | 5 | 13 | 21 | 29 | 37 | 45 | 53 | 61 | 69 | 77 |
| 31 | 5 | 16 | 27 | 38 | 49 | 60 | 71 | 3 | 14 | 25 | 36 |
| 32 | 13 | 27 | 41 | 55 | 69 | 4 | 18 | 32 | 46 | 60 | 74 |
| 33 | 21 | 38 | 55 | 72 | 10 | 27 | 44 | 61 | 78 | 16 | 33 |
| 34 | 29 | 49 | 69 | 10 | 30 | 50 | 70 | 11 | 31 | 51 | 71 |
| 35 | 37 | 60 | 4 | 27 | 50 | 73 | 17 | 40 | 63 | 7 | 30 |
| 36 | 45 | 71 | 18 | 44 | 70 | 17 | 43 | 69 | 16 | 42 | 68 |
| 37 | 53 | 3 | 32 | 61 | 11 | 40 | 69 | 19 | 48 | 77 | 27 |
| 38 | 61 | 14 | 46 | 78 | 31 | 63 | 16 | 48 | 80 | 33 | 65 |
| 39 | 69 | 25 | 60 | 16 | 51 | 7 | 42 | 77 | 33 | 68 | 24 |
| 40 | 77 | 36 | 74 | 33 | 71 | 30 | 68 | 27 | 65 | 24 | 62 |

Table B  Hopping Sequence Set 2

Pattern

| Index | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 6 | 47 | 9 | 50 | 71 | 75 | 79 | 4 | 8 | 12 | 16 |
| 42 | 14 | 58 | 23 | 67 | 12 | 19 | 26 | 33 | 40 | 47 | 54 |
| 43 | 22 | 69 | 37 | 5 | 32 | 42 | 52 | 62 | 72 | 3 | 13 |
| 44 | 30 | 80 | 51 | 22 | 52 | 65 | 78 | 12 | 25 | 38 | 51 |
| 45 | 38 | 12 | 65 | 39 | 72 | 9 | 25 | 41 | 57 | 73 | 10 |
| 46 | 46 | 23 | 79 | 56 | 13 | 32 | 51 | 70 | 10 | 29 | 48 |
| 47 | 54 | 34 | 14 | 73 | 33 | 55 | 77 | 20 | 42 | 64 | 7 |
| 48 | 62 | 45 | 28 | 11 | 53 | 78 | 24 | 49 | 74 | 20 | 45 |
| 49 | 70 | 56 | 42 | 28 | 73 | 22 | 50 | 78 | 27 | 55 | 4 |
| 50 | 78 | 67 | 56 | 45 | 14 | 45 | 76 | 28 | 59 | 11 | 42 |
| 51 | 7 | 78 | 70 | 62 | 34 | 68 | 23 | 57 | 12 | 46 | 80 |
| 52 | 15 | 10 | 5 | 79 | 54 | 12 | 49 | 7 | 44 | 2 | 39 |
| 53 | 23 | 21 | 19 | 17 | 74 | 35 | 75 | 36 | 76 | 37 | 77 |
| 54 | 31 | 32 | 33 | 34 | 15 | 58 | 22 | 65 | 29 | 72 | 36 |
| 55 | 39 | 43 | 47 | 51 | 35 | 2 | 48 | 15 | 61 | 28 | 74 |
| 56 | 47 | 54 | 61 | 68 | 55 | 25 | 74 | 44 | 14 | 63 | 33 |
| 57 | 55 | 65 | 75 | 6 | 75 | 48 | 21 | 73 | 46 | 19 | 71 |
| 58 | 63 | 76 | 10 | 23 | 16 | 71 | 47 | 23 | 78 | 54 | 30 |
| 59 | 71 | 8 | 24 | 40 | 36 | 15 | 73 | 52 | 31 | 10 | 68 |
| 60 | 79 | 19 | 38 | 57 | 56 | 38 | 20 | 2 | 63 | 45 | 27 |
| 61 | 8 | 30 | 52 | 74 | 76 | 61 | 46 | 31 | 16 | 80 | 65 |
| 62 | 16 | 41 | 66 | 12 | 17 | 5 | 72 | 60 | 48 | 36 | 24 |
| 63 | 24 | 52 | 80 | 29 | 37 | 28 | 19 | 10 | 80 | 71 | 62 |
| 64 | 32 | 63 | 15 | 46 | 57 | 51 | 45 | 39 | 33 | 27 | 21 |
| 65 | 40 | 74 | 29 | 63 | 77 | 74 | 71 | 68 | 65 | 62 | 59 |
| 66 | 48 | 6 | 43 | 80 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| 67 | 56 | 17 | 57 | 18 | 38 | 41 | 44 | 47 | 50 | 53 | 56 |
| 68 | 64 | 28 | 71 | 35 | 58 | 64 | 70 | 76 | 3 | 9 | 15 |
| 69 | 72 | 39 | 6 | 52 | 78 | 8 | 17 | 26 | 35 | 44 | 53 |
| 70 | 80 | 50 | 20 | 69 | 19 | 31 | 43 | 55 | 67 | 79 | 12 |
| 71 | 9 | 61 | 34 | 7 | 39 | 54 | 69 | 5 | 20 | 35 | 50 |
| 72 | 17 | 72 | 48 | 24 | 59 | 77 | 16 | 34 | 52 | 70 | 9 |
| 73 | 25 | 4 | 62 | 41 | 79 | 21 | 42 | 63 | 5 | 26 | 47 |
| 74 | 33 | 15 | 76 | 58 | 20 | 44 | 68 | 13 | 37 | 61 | 6 |
| 75 | 41 | 26 | 11 | 75 | 40 | 67 | 15 | 42 | 69 | 17 | 44 |
| 76 | 49 | 37 | 25 | 13 | 60 | 11 | 41 | 71 | 22 | 52 | 3 |
| 77 | 57 | 48 | 39 | 30 | 80 | 34 | 67 | 21 | 54 | 8 | 41 |
| 78 | 65 | 59 | 53 | 47 | 21 | 57 | 14 | 50 | 7 | 43 | 79 |
| 79 | 73 | 70 | 67 | 64 | 41 | 80 | 40 | 79 | 39 | 78 | 38 |

Table B  Hopping Sequence Set 2 continued

Pattern

| Index | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 |
| 3 | 5 | 11 | 17 | 23 | 29 | 35 | 41 | 47 | 53 | 59 | 65 |
| 4 | 46 | 55 | 64 | 73 | 3 | 12 | 21 | 30 | 39 | 48 | 57 |
| 5 | 8 | 20 | 32 | 44 | 56 | 68 | 80 | 13 | 25 | 37 | 49 |
| 6 | 49 | 64 | 79 | 15 | 30 | 45 | 60 | 75 | 11 | 26 | 41 |
| 7 | 11 | 29 | 47 | 65 | 4 | 22 | 40 | 58 | 76 | 15 | 33 |
| 8 | 52 | 73 | 15 | 36 | 57 | 78 | 20 | 41 | 62 | 4 | 25 |
| 9 | 14 | 38 | 62 | 7 | 31 | 55 | 79 | 24 | 48 | 72 | 17 |
| 10 | 55 | 3 | 30 | 57 | 5 | 32 | 59 | 7 | 34 | 61 | 9 |
| 11 | 17 | 47 | 77 | 28 | 58 | 9 | 39 | 69 | 20 | 50 | 80 |
| 12 | 58 | 12 | 45 | 78 | 32 | 65 | 19 | 52 | 6 | 39 | 72 |
| 13 | 20 | 56 | 13 | 49 | 6 | 42 | 78 | 35 | 71 | 28 | 64 |
| 14 | 61 | 21 | 60 | 20 | 59 | 19 | 58 | 18 | 57 | 17 | 56 |
| 15 | 23 | 65 | 28 | 70 | 33 | 75 | 38 | 80 | 43 | 6 | 48 |
| 16 | 64 | 30 | 75 | 41 | 7 | 52 | 18 | 63 | 29 | 74 | 40 |
| 17 | 26 | 74 | 43 | 12 | 60 | 29 | 77 | 46 | 15 | 63 | 32 |
| 18 | 67 | 39 | 11 | 62 | 34 | 6 | 57 | 29 | 80 | 52 | 24 |
| 19 | 29 | 4 | 58 | 33 | 8 | 62 | 37 | 12 | 66 | 41 | 16 |
| 20 | 70 | 48 | 26 | 4 | 61 | 39 | 17 | 74 | 52 | 30 | 8 |
| 21 | 32 | 13 | 73 | 54 | 35 | 16 | 76 | 57 | 38 | 19 | 79 |
| 22 | 73 | 57 | 41 | 25 | 9 | 72 | 56 | 40 | 24 | 8 | 71 |
| 23 | 35 | 22 | 9 | 75 | 62 | 49 | 36 | 23 | 10 | 76 | 63 |
| 24 | 76 | 66 | 56 | 46 | 36 | 26 | 16 | 6 | 75 | 65 | 55 |
| 25 | 38 | 31 | 24 | 17 | 10 | 3 | 75 | 68 | 61 | 54 | 47 |
| 26 | 79 | 75 | 71 | 67 | 63 | 59 | 55 | 51 | 47 | 43 | 39 |
| 27 | 41 | 40 | 39 | 38 | 37 | 36 | 35 | 34 | 33 | 32 | 31 |
| 28 | 3 | 5 | 7 | 9 | 11 | 13 | 15 | 17 | 19 | 21 | 23 |
| 29 | 44 | 49 | 54 | 59 | 64 | 69 | 74 | 79 | 5 | 10 | 15 |
| 30 | 6 | 14 | 22 | 30 | 38 | 46 | 54 | 62 | 70 | 78 | 7 |
| 31 | 47 | 58 | 69 | 80 | 12 | 23 | 34 | 45 | 56 | 67 | 78 |
| 32 | 9 | 23 | 37 | 51 | 65 | 79 | 14 | 28 | 42 | 56 | 70 |
| 33 | 50 | 67 | 5 | 22 | 39 | 56 | 73 | 11 | 28 | 45 | 62 |
| 34 | 12 | 32 | 52 | 72 | 13 | 33 | 53 | 73 | 14 | 34 | 54 |
| 35 | 53 | 76 | 20 | 43 | 66 | 10 | 33 | 56 | 79 | 23 | 46 |
| 36 | 15 | 41 | 67 | 14 | 40 | 66 | 13 | 39 | 65 | 12 | 38 |
| 37 | 56 | 6 | 35 | 64 | 14 | 43 | 72 | 22 | 51 | 80 | 30 |
| 38 | 18 | 50 | 3 | 35 | 67 | 20 | 52 | 5 | 37 | 69 | 22 |
| 39 | 59 | 15 | 50 | 6 | 41 | 76 | 32 | 67 | 23 | 58 | 14 |
| 40 | 21 | 59 | 18 | 56 | 15 | 53 | 12 | 50 | 9 | 47 | 6 |

Table B  Hopping Sequence Set 2 continued

Pattern

| Index | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 62 | 24 | 65 | 27 | 68 | 30 | 71 | 33 | 74 | 36 | 77 |
| 42 | 24 | 68 | 33 | 77 | 42 | 7 | 51 | 16 | 60 | 25 | 69 |
| 43 | 65 | 33 | 80 | 48 | 16 | 63 | 31 | 78 | 46 | 14 | 61 |
| 44 | 27 | 77 | 48 | 19 | 69 | 40 | 11 | 61 | 32 | 3 | 53 |
| 45 | 68 | 42 | 16 | 69 | 43 | 17 | 70 | 44 | 18 | 71 | 45 |
| 46 | 30 | 7 | 63 | 40 | 17 | 73 | 50 | 27 | 4 | 60 | 37 |
| 47 | 71 | 51 | 31 | 11 | 70 | 50 | 30 | 10 | 69 | 49 | 29 |
| 48 | 33 | 16 | 78 | 61 | 44 | 27 | 10 | 72 | 55 | 38 | 21 |
| 49 | 74 | 60 | 46 | 32 | 18 | 4 | 69 | 55 | 41 | 27 | 13 |
| 50 | 36 | 25 | 14 | 3 | 71 | 60 | 49 | 38 | 27 | 16 | 5 |
| 51 | 77 | 69 | 61 | 53 | 45 | 37 | 29 | 21 | 13 | 5 | 76 |
| 52 | 39 | 34 | 29 | 24 | 19 | 14 | 9 | 4 | 78 | 73 | 68 |
| 53 | 80 | 78 | 76 | 74 | 72 | 70 | 68 | 66 | 64 | 62 | 60 |
| 54 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 |
| 55 | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 |
| 56 | 45 | 52 | 59 | 66 | 73 | 80 | 8 | 15 | 22 | 29 | 36 |
| 57 | 7 | 17 | 27 | 37 | 47 | 57 | 67 | 77 | 8 | 18 | 28 |
| 58 | 48 | 61 | 74 | 8 | 21 | 34 | 47 | 60 | 73 | 7 | 20 |
| 59 | 10 | 26 | 42 | 58 | 74 | 11 | 27 | 43 | 59 | 75 | 12 |
| 60 | 51 | 70 | 10 | 29 | 48 | 67 | 7 | 26 | 45 | 64 | 4 |
| 61 | 13 | 35 | 57 | 79 | 22 | 44 | 66 | 9 | 31 | 53 | 75 |
| 62 | 54 | 79 | 25 | 50 | 75 | 21 | 46 | 71 | 17 | 42 | 67 |
| 63 | 16 | 44 | 72 | 21 | 49 | 77 | 26 | 54 | 3 | 31 | 59 |
| 64 | 57 | 9 | 40 | 71 | 23 | 54 | 6 | 37 | 68 | 20 | 51 |
| 65 | 19 | 53 | 8 | 42 | 76 | 31 | 65 | 20 | 54 | 9 | 43 |
| 66 | 60 | 18 | 55 | 13 | 50 | 8 | 45 | 3 | 40 | 77 | 35 |
| 67 | 22 | 62 | 23 | 63 | 24 | 64 | 25 | 65 | 26 | 66 | 27 |
| 68 | 63 | 27 | 70 | 34 | 77 | 41 | 5 | 48 | 12 | 55 | 19 |
| 69 | 25 | 71 | 38 | 5 | 51 | 18 | 64 | 31 | 77 | 44 | 11 |
| 70 | 66 | 36 | 6 | 55 | 25 | 74 | 44 | 14 | 63 | 33 | 3 |
| 71 | 28 | 80 | 53 | 26 | 78 | 51 | 24 | 76 | 49 | 22 | 74 |
| 72 | 69 | 45 | 21 | 76 | 52 | 28 | 4 | 59 | 35 | 11 | 66 |
| 73 | 31 | 10 | 68 | 47 | 26 | 5 | 63 | 42 | 21 | 79 | 58 |
| 74 | 72 | 54 | 36 | 18 | 79 | 61 | 43 | 25 | 7 | 68 | 50 |
| 75 | 34 | 19 | 4 | 68 | 53 | 38 | 23 | 8 | 72 | 57 | 42 |
| 76 | 75 | 63 | 51 | 39 | 27 | 15 | 3 | 70 | 58 | 46 | 34 |
| 77 | 37 | 28 | 19 | 10 | 80 | 71 | 62 | 53 | 44 | 35 | 26 |
| 78 | 78 | 72 | 66 | 60 | 54 | 48 | 42 | 36 | 30 | 24 | 18 |
| 79 | 40 | 37 | 34 | 31 | 28 | 25 | 22 | 19 | 16 | 13 | 10 |

Table C  Hopping Sequence Set 3

Pattern

| Index | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 |
| 3 | 20 | 26 | 32 | 38 | 44 | 50 | 56 | 62 | 68 | 74 | 80 |
| 4 | 29 | 38 | 47 | 56 | 65 | 74 | 4 | 13 | 22 | 31 | 40 |
| 5 | 38 | 50 | 62 | 74 | 7 | 19 | 31 | 43 | 55 | 67 | 79 |
| 6 | 47 | 62 | 77 | 13 | 28 | 43 | 58 | 73 | 9 | 24 | 39 |
| 7 | 56 | 74 | 13 | 31 | 49 | 67 | 6 | 24 | 42 | 60 | 78 |
| 8 | 65 | 7 | 28 | 49 | 70 | 12 | 33 | 54 | 75 | 17 | 38 |
| 9 | 74 | 19 | 43 | 67 | 12 | 36 | 60 | 5 | 29 | 53 | 77 |
| 10 | 4 | 31 | 58 | 6 | 33 | 60 | 8 | 35 | 62 | 10 | 37 |
| 11 | 13 | 43 | 73 | 24 | 54 | 5 | 35 | 65 | 16 | 46 | 76 |
| 12 | 22 | 55 | 9 | 42 | 75 | 29 | 62 | 16 | 49 | 3 | 36 |
| 13 | 31 | 67 | 24 | 60 | 17 | 53 | 10 | 46 | 3 | 39 | 75 |
| 14 | 40 | 79 | 39 | 78 | 38 | 77 | 37 | 76 | 36 | 75 | 35 |
| 15 | 49 | 12 | 54 | 17 | 59 | 22 | 64 | 27 | 69 | 32 | 74 |
| 16 | 58 | 24 | 69 | 35 | 80 | 46 | 12 | 57 | 23 | 68 | 34 |
| 17 | 67 | 36 | 5 | 53 | 22 | 70 | 39 | 8 | 56 | 25 | 73 |
| 18 | 76 | 48 | 20 | 71 | 43 | 15 | 66 | 38 | 10 | 61 | 33 |
| 19 | 6 | 60 | 35 | 10 | 64 | 39 | 14 | 68 | 43 | 18 | 72 |
| 20 | 15 | 72 | 50 | 28 | 6 | 63 | 41 | 19 | 76 | 54 | 32 |
| 21 | 24 | 5 | 65 | 46 | 27 | 8 | 68 | 49 | 30 | 11 | 71 |
| 22 | 33 | 17 | 80 | 64 | 48 | 32 | 16 | 79 | 63 | 47 | 31 |
| 23 | 42 | 29 | 16 | 3 | 69 | 56 | 43 | 30 | 17 | 4 | 70 |
| 24 | 51 | 41 | 31 | 21 | 11 | 80 | 70 | 60 | 50 | 40 | 30 |
| 25 | 60 | 53 | 46 | 39 | 32 | 25 | 18 | 11 | 4 | 76 | 69 |
| 26 | 69 | 65 | 61 | 57 | 53 | 49 | 45 | 41 | 37 | 33 | 29 |
| 27 | 78 | 77 | 76 | 75 | 74 | 73 | 72 | 71 | 70 | 69 | 68 |
| 28 | 8 | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 |
| 29 | 17 | 22 | 27 | 32 | 37 | 42 | 47 | 52 | 57 | 62 | 67 |
| 30 | 26 | 34 | 42 | 50 | 58 | 66 | 74 | 3 | 11 | 19 | 27 |
| 31 | 35 | 46 | 57 | 68 | 79 | 11 | 22 | 33 | 44 | 55 | 66 |
| 32 | 44 | 58 | 72 | 7 | 21 | 35 | 49 | 63 | 77 | 12 | 26 |
| 33 | 53 | 70 | 8 | 25 | 42 | 59 | 76 | 14 | 31 | 48 | 65 |
| 34 | 62 | 3 | 23 | 43 | 63 | 4 | 24 | 44 | 64 | 5 | 25 |
| 35 | 71 | 15 | 38 | 61 | 5 | 28 | 51 | 74 | 18 | 41 | 64 |
| 36 | 80 | 27 | 53 | 79 | 26 | 52 | 78 | 25 | 51 | 77 | 24 |
| 37 | 10 | 39 | 68 | 18 | 47 | 76 | 26 | 55 | 5 | 34 | 63 |
| 38 | 19 | 51 | 4 | 36 | 68 | 21 | 53 | 6 | 38 | 70 | 23 |
| 39 | 28 | 63 | 19 | 54 | 10 | 45 | 80 | 36 | 71 | 27 | 62 |
| 40 | 37 | 75 | 34 | 72 | 31 | 69 | 28 | 66 | 25 | 63 | 22 |

Table C  Hopping Sequence Set 3

Pattern

| Index | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 46 | 8 | 49 | 11 | 52 | 14 | 55 | 17 | 58 | 20 | 61 |
| 42 | 55 | 20 | 64 | 29 | 73 | 38 | 3 | 47 | 12 | 56 | 21 |
| 43 | 64 | 32 | 79 | 47 | 15 | 62 | 30 | 77 | 45 | 13 | 60 |
| 44 | 73 | 44 | 15 | 65 | 36 | 7 | 57 | 28 | 78 | 49 | 20 |
| 45 | 3 | 56 | 30 | 4 | 57 | 31 | 5 | 58 | 32 | 6 | 59 |
| 46 | 12 | 68 | 45 | 22 | 78 | 55 | 32 | 9 | 65 | 42 | 19 |
| 47 | 21 | 80 | 60 | 40 | 20 | 79 | 59 | 39 | 19 | 78 | 58 |
| 48 | 30 | 13 | 75 | 58 | 41 | 24 | 7 | 69 | 52 | 35 | 18 |
| 49 | 39 | 25 | 11 | 76 | 62 | 48 | 34 | 20 | 6 | 71 | 57 |
| 50 | 48 | 37 | 26 | 15 | 4 | 72 | 61 | 50 | 39 | 28 | 17 |
| 51 | 57 | 49 | 41 | 33 | 25 | 17 | 9 | 80 | 72 | 64 | 56 |
| 52 | 66 | 61 | 56 | 51 | 46 | 41 | 36 | 31 | 26 | 21 | 16 |
| 53 | 75 | 73 | 71 | 69 | 67 | 65 | 63 | 61 | 59 | 57 | 55 |
| 54 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 55 | 14 | 18 | 22 | 26 | 30 | 34 | 38 | 42 | 46 | 50 | 54 |
| 56 | 23 | 30 | 37 | 44 | 51 | 58 | 65 | 72 | 79 | 7 | 14 |
| 57 | 32 | 42 | 52 | 62 | 72 | 3 | 13 | 23 | 33 | 43 | 53 |
| 58 | 41 | 54 | 67 | 80 | 14 | 27 | 40 | 53 | 66 | 79 | 13 |
| 59 | 50 | 66 | 3 | 19 | 35 | 51 | 67 | 4 | 20 | 36 | 52 |
| 60 | 59 | 78 | 18 | 37 | 56 | 75 | 15 | 34 | 53 | 72 | 12 |
| 61 | 68 | 11 | 33 | 55 | 77 | 20 | 42 | 64 | 7 | 29 | 51 |
| 62 | 77 | 23 | 48 | 73 | 19 | 44 | 69 | 15 | 40 | 65 | 11 |
| 63 | 7 | 35 | 63 | 12 | 40 | 68 | 17 | 45 | 73 | 22 | 50 |
| 64 | 16 | 47 | 78 | 30 | 61 | 13 | 44 | 75 | 27 | 58 | 10 |
| 65 | 25 | 59 | 14 | 48 | 3 | 37 | 71 | 26 | 60 | 15 | 49 |
| 66 | 34 | 71 | 29 | 66 | 24 | 61 | 19 | 56 | 14 | 51 | 9 |
| 67 | 43 | 4 | 44 | 5 | 45 | 6 | 46 | 7 | 47 | 8 | 48 |
| 68 | 52 | 16 | 59 | 23 | 66 | 30 | 73 | 37 | 80 | 44 | 8 |
| 69 | 61 | 28 | 74 | 41 | 8 | 54 | 21 | 67 | 34 | 80 | 47 |
| 70 | 70 | 40 | 10 | 59 | 29 | 78 | 48 | 18 | 67 | 37 | 7 |
| 71 | 79 | 52 | 25 | 77 | 50 | 23 | 75 | 48 | 21 | 73 | 46 |
| 72 | 9 | 64 | 40 | 16 | 71 | 47 | 23 | 78 | 54 | 30 | 6 |
| 73 | 18 | 76 | 55 | 34 | 13 | 71 | 50 | 29 | 8 | 66 | 45 |
| 74 | 27 | 9 | 70 | 52 | 34 | 16 | 77 | 59 | 41 | 23 | 5 |
| 75 | 36 | 21 | 6 | 70 | 55 | 40 | 25 | 10 | 74 | 59 | 44 |
| 76 | 45 | 33 | 21 | 9 | 76 | 64 | 52 | 40 | 28 | 16 | 4 |
| 77 | 54 | 45 | 36 | 27 | 18 | 9 | 79 | 70 | 61 | 52 | 43 |
| 78 | 63 | 57 | 51 | 45 | 39 | 33 | 27 | 21 | 15 | 9 | 3 |
| 79 | 72 | 69 | 66 | 63 | 60 | 57 | 54 | 51 | 48 | 45 | 42 |

Table C  Hopping Sequence Set 3 continued

Pattern

| Index | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| 2 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 |
| 3 | 7 | 13 | 19 | 25 | 31 | 37 | 43 | 49 | 55 | 61 | 67 |
| 4 | 49 | 58 | 67 | 76 | 6 | 15 | 24 | 33 | 42 | 51 | 60 |
| 5 | 12 | 24 | 36 | 48 | 60 | 72 | 5 | 17 | 29 | 41 | 53 |
| 6 | 54 | 69 | 5 | 20 | 35 | 50 | 65 | 80 | 16 | 31 | 46 |
| 7 | 17 | 35 | 53 | 71 | 10 | 28 | 46 | 64 | 3 | 21 | 39 |
| 8 | 59 | 80 | 22 | 43 | 64 | 6 | 27 | 48 | 69 | 11 | 32 |
| 9 | 22 | 46 | 70 | 15 | 39 | 63 | 8 | 32 | 56 | 80 | 25 |
| 10 | 64 | 12 | 39 | 66 | 14 | 41 | 68 | 16 | 43 | 70 | 18 |
| 11 | 27 | 57 | 8 | 38 | 68 | 19 | 49 | 79 | 30 | 60 | 11 |
| 12 | 69 | 23 | 56 | 10 | 43 | 76 | 30 | 63 | 17 | 50 | 4 |
| 13 | 32 | 68 | 25 | 61 | 18 | 54 | 11 | 47 | 4 | 40 | 76 |
| 14 | 74 | 34 | 73 | 33 | 72 | 32 | 71 | 31 | 70 | 30 | 69 |
| 15 | 37 | 79 | 42 | 5 | 47 | 10 | 52 | 15 | 57 | 20 | 62 |
| 16 | 79 | 45 | 11 | 56 | 22 | 67 | 33 | 78 | 44 | 10 | 55 |
| 17 | 42 | 11 | 59 | 28 | 76 | 45 | 14 | 62 | 31 | 79 | 48 |
| 18 | 5 | 56 | 28 | 79 | 51 | 23 | 74 | 46 | 18 | 69 | 41 |
| 19 | 47 | 22 | 76 | 51 | 26 | 80 | 55 | 30 | 5 | 59 | 34 |
| 20 | 10 | 67 | 45 | 23 | 80 | 58 | 36 | 14 | 71 | 49 | 27 |
| 21 | 52 | 33 | 14 | 74 | 55 | 36 | 17 | 77 | 58 | 39 | 20 |
| 22 | 15 | 78 | 62 | 46 | 30 | 14 | 77 | 61 | 45 | 29 | 13 |
| 23 | 57 | 44 | 31 | 18 | 5 | 71 | 58 | 45 | 32 | 19 | 6 |
| 24 | 20 | 10 | 79 | 69 | 59 | 49 | 39 | 29 | 19 | 9 | 78 |
| 25 | 62 | 55 | 48 | 41 | 34 | 27 | 20 | 13 | 6 | 78 | 71 |
| 26 | 25 | 21 | 17 | 13 | 9 | 5 | 80 | 76 | 72 | 68 | 64 |
| 27 | 67 | 66 | 65 | 64 | 63 | 62 | 61 | 60 | 59 | 58 | 57 |
| 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 | 48 | 50 |
| 29 | 72 | 77 | 3 | 8 | 13 | 18 | 23 | 28 | 33 | 38 | 43 |
| 30 | 35 | 43 | 51 | 59 | 67 | 75 | 4 | 12 | 20 | 28 | 36 |
| 31 | 77 | 9 | 20 | 31 | 42 | 53 | 64 | 75 | 7 | 18 | 29 |
| 32 | 40 | 54 | 68 | 3 | 17 | 31 | 45 | 59 | 73 | 8 | 22 |
| 33 | 3 | 20 | 37 | 54 | 71 | 9 | 26 | 43 | 60 | 77 | 15 |
| 34 | 45 | 65 | 6 | 26 | 46 | 66 | 7 | 27 | 47 | 67 | 8 |
| 35 | 8 | 31 | 54 | 77 | 21 | 44 | 67 | 11 | 34 | 57 | 80 |
| 36 | 50 | 76 | 23 | 49 | 75 | 22 | 48 | 74 | 21 | 47 | 73 |
| 37 | 13 | 42 | 71 | 21 | 50 | 79 | 29 | 58 | 8 | 37 | 66 |
| 38 | 55 | 8 | 40 | 72 | 25 | 57 | 10 | 42 | 74 | 27 | 59 |
| 39 | 18 | 53 | 9 | 44 | 79 | 35 | 70 | 26 | 61 | 17 | 52 |
| 40 | 60 | 19 | 57 | 16 | 54 | 13 | 51 | 10 | 48 | 7 | 45 |

Table C  Hopping Sequence Set 3 continued

Pattern

| Index | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 23 | 64 | 26 | 67 | 29 | 70 | 32 | 73 | 35 | 76 | 38 |
| 42 | 65 | 30 | 74 | 39 | 4 | 48 | 13 | 57 | 22 | 66 | 31 |
| 43 | 28 | 75 | 43 | 11 | 58 | 26 | 73 | 41 | 9 | 56 | 24 |
| 44 | 70 | 41 | 12 | 62 | 33 | 4 | 54 | 25 | 75 | 46 | 17 |
| 45 | 33 | 7 | 60 | 34 | 8 | 61 | 35 | 9 | 62 | 36 | 10 |
| 46 | 75 | 52 | 29 | 6 | 62 | 39 | 16 | 72 | 49 | 26 | 3 |
| 47 | 38 | 18 | 77 | 57 | 37 | 17 | 76 | 56 | 36 | 16 | 75 |
| 48 | 80 | 63 | 46 | 29 | 12 | 74 | 57 | 40 | 23 | 6 | 68 |
| 49 | 43 | 29 | 15 | 80 | 66 | 52 | 38 | 24 | 10 | 75 | 61 |
| 50 | 6 | 74 | 63 | 52 | 41 | 30 | 19 | 8 | 76 | 65 | 54 |
| 51 | 48 | 40 | 32 | 24 | 16 | 8 | 79 | 71 | 63 | 55 | 47 |
| 52 | 11 | 6 | 80 | 75 | 70 | 65 | 60 | 55 | 50 | 45 | 40 |
| 53 | 53 | 51 | 49 | 47 | 45 | 43 | 41 | 39 | 37 | 35 | 33 |
| 54 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| 55 | 58 | 62 | 66 | 70 | 74 | 78 | 3 | 7 | 11 | 15 | 19 |
| 56 | 21 | 28 | 35 | 42 | 49 | 56 | 63 | 70 | 77 | 5 | 12 |
| 57 | 63 | 73 | 4 | 14 | 24 | 34 | 44 | 54 | 64 | 74 | 5 |
| 58 | 26 | 39 | 52 | 65 | 78 | 12 | 25 | 38 | 51 | 64 | 77 |
| 59 | 68 | 5 | 21 | 37 | 53 | 69 | 6 | 22 | 38 | 54 | 70 |
| 60 | 31 | 50 | 69 | 9 | 28 | 47 | 66 | 6 | 25 | 44 | 63 |
| 61 | 73 | 16 | 38 | 60 | 3 | 25 | 47 | 69 | 12 | 34 | 56 |
| 62 | 36 | 61 | 7 | 32 | 57 | 3 | 28 | 53 | 78 | 24 | 49 |
| 63 | 78 | 27 | 55 | 4 | 32 | 60 | 9 | 37 | 65 | 14 | 42 |
| 64 | 41 | 72 | 24 | 55 | 7 | 38 | 69 | 21 | 52 | 4 | 35 |
| 65 | 4 | 38 | 72 | 27 | 61 | 16 | 50 | 5 | 39 | 73 | 28 |
| 66 | 46 | 4 | 41 | 78 | 36 | 73 | 31 | 68 | 26 | 63 | 21 |
| 67 | 9 | 49 | 10 | 50 | 11 | 51 | 12 | 52 | 13 | 53 | 14 |
| 68 | 51 | 15 | 58 | 22 | 65 | 29 | 72 | 36 | 79 | 43 | 7 |
| 69 | 14 | 60 | 27 | 73 | 40 | 7 | 53 | 20 | 66 | 33 | 79 |
| 70 | 56 | 26 | 75 | 45 | 15 | 64 | 34 | 4 | 53 | 23 | 72 |
| 71 | 19 | 71 | 44 | 17 | 69 | 42 | 15 | 67 | 40 | 13 | 65 |
| 72 | 61 | 37 | 13 | 68 | 44 | 20 | 75 | 51 | 27 | 3 | 58 |
| 73 | 24 | 3 | 61 | 40 | 19 | 77 | 56 | 35 | 14 | 72 | 51 |
| 74 | 66 | 48 | 30 | 12 | 73 | 55 | 37 | 19 | 80 | 62 | 44 |
| 75 | 29 | 14 | 78 | 63 | 48 | 33 | 18 | 3 | 67 | 52 | 37 |
| 76 | 71 | 59 | 47 | 35 | 23 | 11 | 78 | 66 | 54 | 42 | 30 |
| 77 | 34 | 25 | 16 | 7 | 77 | 68 | 59 | 50 | 41 | 32 | 23 |
| 78 | 76 | 70 | 64 | 58 | 52 | 46 | 40 | 34 | 28 | 22 | 16 |
| 79 | 39 | 36 | 33 | 30 | 27 | 24 | 21 | 18 | 15 | 12 | 9 |

Table D  Hopping Sequence for Japan

Pattern

|  | Set 1 |  |  |  | Set 2 |  |  |  | Set 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | 6 | 9 | 12 | 15 | 7 | 10 | 13 | 16 | 8 | 11 | 14 | 17 |
| 1 | 73 | 73 | 73 | 73 | 73 | 73 | 73 | 73 | 73 | 73 | 73 | 73 |
| 2 | 79 | 82 | 85 | 88 | 80 | 83 | 86 | 89 | 81 | 84 | 87 | 90 |
| 3 | 85 | 91 | 74 | 80 | 87 | 93 | 76 | 82 | 89 | 95 | 78 | 84 |
| 4 | 91 | 77 | 86 | 95 | 94 | 80 | 89 | 75 | 74 | 83 | 92 | 78 |
| 5 | 74 | 86 | 75 | 87 | 78 | 90 | 79 | 91 | 82 | 94 | 83 | 95 |
| 6 | 80 | 95 | 87 | 79 | 85 | 77 | 92 | 84 | 90 | 82 | 74 | 89 |
| 7 | 86 | 81 | 76 | 94 | 92 | 87 | 82 | 77 | 75 | 93 | 88 | 83 |
| 8 | 92 | 90 | 88 | 86 | 76 | 74 | 95 | 93 | 83 | 81 | 79 | 77 |
| 9 | 75 | 76 | 77 | 78 | 83 | 84 | 85 | 86 | 91 | 92 | 93 | 94 |
| 10 | 81 | 85 | 89 | 93 | 90 | 94 | 75 | 79 | 76 | 80 | 84 | 88 |
| 11 | 87 | 94 | 78 | 85 | 74 | 81 | 88 | 95 | 84 | 91 | 75 | 82 |
| 12 | 93 | 80 | 90 | 77 | 81 | 91 | 78 | 88 | 92 | 79 | 89 | 76 |
| 13 | 76 | 89 | 79 | 92 | 88 | 78 | 91 | 81 | 77 | 90 | 80 | 93 |
| 14 | 82 | 75 | 91 | 84 | 95 | 88 | 81 | 74 | 85 | 78 | 94 | 87 |
| 15 | 88 | 84 | 80 | 76 | 79 | 75 | 94 | 90 | 93 | 89 | 85 | 81 |
| 16 | 94 | 93 | 92 | 91 | 86 | 85 | 84 | 83 | 78 | 77 | 76 | 75 |
| 17 | 77 | 79 | 81 | 83 | 93 | 95 | 74 | 76 | 86 | 88 | 90 | 92 |
| 18 | 83 | 88 | 93 | 75 | 77 | 82 | 87 | 92 | 94 | 76 | 81 | 86 |
| 19 | 89 | 74 | 82 | 90 | 84 | 92 | 77 | 85 | 79 | 87 | 95 | 80 |
| 20 | 95 | 83 | 94 | 82 | 91 | 79 | 90 | 78 | 87 | 75 | 86 | 74 |
| 21 | 78 | 92 | 83 | 74 | 75 | 89 | 80 | 94 | 95 | 86 | 77 | 91 |
| 22 | 84 | 78 | 95 | 89 | 82 | 76 | 93 | 87 | 80 | 74 | 91 | 85 |
| 23 | 90 | 87 | 84 | 81 | 89 | 86 | 83 | 80 | 88 | 85 | 82 | 79 |

## 152FHSS PHY Management Information Base

### 153Introduction

The following is the Management Information Base for the Frequency Hopping Spread Spectrum PHY.

### 154 FHSS PHY Management Information Base Definitions

#### 155 FHSS PHY Attributes

##### 156 agPhyOperation_grp

aPHY_Type

aNbr_Geo_Supported

aGeo_USA

aGeo_Japan

aGeo_Europe

aCCA_Method

aCCA_MaxP_Det_Time

aCCA_Asmnt_Time

aCCA_Decay_Time

aRxTx_Switch_Time

aTxRx_Switch_Time

aTxRamp_On_Time

aTxRamp_Off_Time

aTx_RF_Delay

aRx_RF_Delay

aTx_PLCP_Delay

aRx_Clk_Rcvy_Delay

aTxRx_Turnaround_Time

aRxTx_Turnaround_Time

aMAC_Ack_Delay

aTx_SIFS

aRx_SIFS

aSlot_Time

aMPDU_Maximum_Length

aMPDU_Current_Max_Length

##### 157 agPhyRate_grp

aNbr_Supported_Rates

aRate_1MHz

aRate_2MHz

aBSS_Basic_rate

##### 158  agPhyAntenna_grp

aNbr_Supported_Antenna

aAntenna_One_Type

aAntenna_Two_Type

##### 159  agPhyPower_grp

aNbr_Supported_Power_Levels

aTx_Pwr_Lvl_1

aTx_Pwr_Lvl_2

aTx_Pwr_Lvl_3

aTx_Pwr_Lvl_4

aCurrent_Tx_PwrLvl

##### 160  agPhyHopping_grp

aCurrent_Channel_ID

aCurrent_Country_Code

aHop_Time

aMax_Dwell_Time

aCurrent_Dwell_Time

aSynthesizer_Locked

#### 161  FHSS PHY Object Class

PHY MANAGED OBJECT CLASS

DERIVED FROM "ISO/IEC 10165-2":top;

CHARACTERIZED BY

pPHY_base		PACKAGE

BEHAVIOR

bPHY_base BEHAVIOR

DEFINED AS "The PHY object class provides the necessary support for the timing information, rate support, antenna definition, and power level information which may vary from PHY to PHY and from STA to STA to be communicated to upper layers."

ATTRIBUTES

aPHY_Type					GET,

aNbr_Geo_Supported				GET,

aGeo_USA					GET,

aGeo_Japan					GET,

aGeo_Europe					GET,

aCCA_Method					GET,

aCCA_MaxP_Det_Time				GET,

aCCA_Asmnt_Time				GET,

aCCA_Decay_Time				GET,

aRxTx_Switch_Time				GET,

aTxRx_Switch_Time				GET,

aTxRamp_On_Time				GET,

aTxRamp_Off_Time				GET,

aTx_RF_Delay					GET,

aRx_RF_Delay					GET,

aTx_PLCP_Delay				GET,

aRx_Clk_Rcvy_Delay				GET,

aTxRx_Turnaround_Time				GET,

aRxTx_Turnaround_Time				GET,

aMAC_Ack_Delay				GET-REPLACE,

aTx_SIFS					GET,

aRx_SIFS					GET,

aSlot_Time					GET,

aMPDU_Maximum				GET,

aMPDU_Current_Max				GET-REPLACE,

aNbr_Supported_Rates				GET,

aRate_1MHz					GET,

aRate_2MHz					GET,

aCurrent_Bit_rate				GET-REPLACE

aNbr_Supported_Antenna				GET,

aAntenna_One_Type				GET,

aAntenna_Two_Type				GET,

aNbr_Supported_Power_Levels			GET,

aTx_Pwr_Lvl_1					GET,

aTx_Pwr_Lvl_2					GET,

aTx_Pwr_Lvl_3					GET,

aTx_Pwr_Lvl_4					GET,

aCurrent_Tx_PwrLvl				GET-REPLACE,

aCurrent_Channel_ID				GET,

aCurrent_Country_Code				GET-REPLACE,

aHop_Time					GET,

aMax_Dwell_Time				GET,

aCurrent_Dwell_Time				GET-REPLACE

aSynthesizer_Locked				GET,

ATTRIBUTE GROUPS

agPhyOperation_grp,

agPhyRate_grp,

agPhyAntenna_grp,

agPhyPower_grp,

agPhyHopping_grp

ACTIONS

acPHY_init,

acPHY_reset;

NOTIFICATIONS

none

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) };

#### 162  FHSS PHY Attribute Group Templates

##### 163  agPhyOperation_grp

PhyRate_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aPHY_Type

aNbr_Geo_Supported

aGeo_USA

aGeo_Japan

aGeo_Europe

aCCA_Method

aCCA_MaxP_Det_Time

aCCA_Asmnt_Time

aCCA_Decay_Time

aRxTx_Switch_Time

aTxRx_Switch_Time

aTxRamp_On_Time

aTxRamp_Off_Time

aTx_RF_Delay

aRx_RF_Delay

aTx_PLCP_Delay

aRx_Clk_Rcvy_Delay

aTxRx_Turnaround_Time

aRxTx_Turnaround_Time

aMAC_Ack_Delay

aTx_SIFS

aRx_SIFS

aSlot_Time

aMPDU_Maximum

aMPDU_Current_Max

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(1) PhyRate_grp(0) };

##### 164  agPhyRate_grp

PhyRate_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aNbr_Supported_Rates

aRate_1MHz

aRate_2MHz

aCurrent_Bit_rate

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(1) PhyRate_grp(1) };

##### 165  agPhyAntenna_grp

PhyRate_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aNbr_Supported_Antenna

aAntenna_One_Type

aAntenna_Two_Type

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(1) PhyRate_grp(2) };

##### 166  agPhyPower_grp

PhyRate_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aNbr_Supported_Power_Levels

aTx_Pwr_Lvl_1

aTx_Pwr_Lvl_2

aTx_Pwr_Lvl_3

aTx_Pwr_Lvl_4

aCurrent_Tx_PwrLvl

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(1) PhyRate_grp(3) };

##### 167  agPhyHopping_grp

PhyRate_grp ATTRIBUTE GROUP

GROUP ELEMENTS

aCurrent_Channel_ID

aCurrent_Country_Code

aHop_Time

aMax_Dwell_Time

aCurrent_Dwell_Time

aSynthesizer_Locked

REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(xxxx) phy(1) PhyRate_grp(4) };

#### 168 FHSS PHY Attribute Templates

##### 169  aPHY_Type

PHY_Type ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This identifies the PHY Type supported by the attached PLCP and PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) PHY_Type(1) };

##### 170  aNbr_Cntry_Supported

Nbr_Cntry_Supported ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"There are different operational requirements dependent on the geographical location.  This attribute indicates the number of geographical locations supported by this implementation.";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Nbr_Cntry_Supported(2) };

##### 171  aGeo_USA

Geo_USA  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Boolean;

BEHAVIOR DEFINED AS

"An indication whether or not the  PHY implementation supports the operational requirements of the USA ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Geo_USA(3) };

##### 172  aGeo_Japan

Geo_Japan  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Boolean;

BEHAVIOR DEFINED AS

"An indication whether or not the current PHY implementation supports the operational requirements of Japan ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Geo_Japan(4) };

##### 173  aGeo_Europe

Geo_Europe  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Boolean;

BEHAVIOR DEFINED AS

"An indication whether or not the current PHY implementation supports the operational requirements of Europe";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Geo_Europe(5) };

##### 174  aCCA_Method

CCA_Method  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This integer specifices whether the current PHY implementation supports Clear Channel Assessment using Carrier Sense only or Carrier Sense and Compliant Data Detect";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) CCA_Method(6) };

##### 175  aCCA_MaxP_Det_Time

CCA_MaxP_Det_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The maximum time in nanoseconds for the CCA mechanism to determine that the state of the media has changed from clear to busy when evaluating a valid preamble";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) CCA_MaxP_Det_Time(7) };

##### 176 aCCA_Asmnt_Time

CCA_Asmnt_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the CCA mechanism must assess the media within every slot to determine whether the media is clear or busy";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) CCA_Asmnt_Time(8) };

##### 177  aCCA_Decay_Time

CCA_Decay_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The maximum time in nanoseconds the CCA mechanism can take to signal that the channel has become clear after the end of a data transaction";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) CCA_Decay_Time(9) };

##### 178  aRxTx_Switch_Time

RxTx_Switch_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PMD takes to switch the radio from Receive to Transmit";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) RxTx_Switch_Time(10) };

##### 179  aTxRx_Switch_Time

TxRx_Switch_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PMD takes to switch the radio from Transmit to Receive";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) TxRx_Switch_Time(11) };

##### 180  aTxRamp_On_Time

TxRamp_On_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PMD takes to turn the Transmit Power Amplifer (PA) on";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) TxRamp_On_Time(12) };

##### 181  aTxRamp_Off_Time

TxRamp_Off_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PMD takes to turn the Transmit PA off";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) TxRamp_Off_Time(13) };

##### 182  aTx_RF_Delay

Tx_RF_Delay  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PMD uses to transfer a bit through the Transmit path of the PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_RF_Delay(14) };

##### 183  aRx_RF_Delay

Rx_RF_Delay  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PMD uses to deliver a bit from the antenna to the clock recovery circuitry in the Receive path of the PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_RF_Delay(15) };

##### 184  aTx_PLCP_Delay

Tx_PLCP_Delay  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PLCP uses to deliver a bit from the MAC interface to the tranmit data path of the PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_PLCP_Delay(16) };

##### 185  aRx_Clk_Rcvy_Delay

Rx_Clk_Rcvy_Delay  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PHY uses to recover clock and data and perform the required PLCP processing in the receive path prior to delivering data to the MAC interface";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Rx_Clk_Rcvy_Delay(17) };

##### 186  aTxRx_Turnaround_Time

TxRx_Turnaround_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PHY requires to change from transmitting the last bit out of the MAC to being perpared to receive a bit at the antenna";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) TxRx_Turnaround_Time(18) };

##### 187  aRxTx_Turnaround_Time

RxTx_Turnaround_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the PHY requires from to changing from receiving data to transmittimg the first bit out on the air";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) RxTx_Turnaround_Time(19) };

##### 188  aMAC_Ack_Delay

MAC_Ack_Delay  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the MAC requires to process an acknowledge.  This value is set by the MAC and passed to the PHY via this attribute.  This delay is measured from the last bit received by the MAC to the time the MAC indicates to the PHY to start the RxTx turnaround process";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) MAC_Ack_Delay (20) };

##### 189  aTx_SIFS

Tx_SIFS  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The maximum time in nanoseconds the PHY will experience in waiting for an acknowledge to appear on its antenna if it is the station transmitting the data requiring the acknowledge ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_SIFS (21) };

##### 190 aRx_SIFS

Rx_SIFS  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The maximum time in nanoseconds the PHY will experience in waiting for an acknowledge to appear on its antenna if it was not the station transmitting the data requiring the acknowledge ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Rx_SIFS (22) };

##### 191  aSlot_time

Slot_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds the MAC will use for defining the PIFS and DIFS periods ";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Slot_Time (23) };

##### 192  aMPDU_Maximum_Length

MPDU_Maximum Length  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The maximum number of bytes in the MPDU that can be load into the PLCP_PDU";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) MPDU_Maximum_Length (24) };

##### 193  aMPDU_Current_Max_Length

MPDU_Current_Max_Length  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The maximum number of bytes in the MPDU that can be load into the PLCP_PDU as currently defined by the MAC";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) MPDU_Current_Max_Length (25) };

##### 194 aNbr_Supported_Rates

Nbr_Supported_Rates  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The number of bit rates supported by the PLCP and PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Nbr_Supported_Rates (26) };

##### 195  aRate_1MHz

Rate_1MHz  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Boolean;

BEHAVIOR DEFINED AS

"This attribute defines if the PMD supports the 2 Level GFSK modulation scheme and the PLCP supports the 1Mbit data rate";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Rate_1MHz (27) };

##### 196  aRate_2MHz

Rate_2MHz  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Boolean;

BEHAVIOR DEFINED AS

"This attribute defines if the PMD supports the 4 Level GFSK modulation scheme and the PLCP supports the 2Mbit data rate";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Rate_1MHz (28) };

##### 197  aCurrent_Bit_Rate

Current_Bit_Rate  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the current modulation scheme and bit rate the PMD and PLCP are operating under";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Current_Bit_Rate (29) };

##### 198  aNbr_Supported_Antenna

Nbr_Supported_Antenna  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This is the number of antennae supported by the PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Nbr_Supported_Antenna (30) };

##### 199  aAntenna_One_Type

Antenna_One_Type  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the antenna type for antenna number one";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Antenna_One_Type (31) };

##### 200  aAntenna_Two_Type

Antenna_Two_Type  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the antenna type for antenna number two";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Antenna_Two_Type (32) };

##### 201 aNbr_Supported_Power_Levels

Nbr_Supported_Power_Levels  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the number of power levels supported by the PMD";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Nbr_Supported_Power_Levels (33) };

##### 202 aTx_Pwr_Lvl_1

Tx_Pwr_Lvl_1  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL1";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_Pwr_Lvl_1 (34) };

##### 203 aTx_Pwr_Lvl_2

Tx_Pwr_Lvl_2  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL2";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_Pwr_Lvl_2 (35) };

##### 204  aTx_Pwr_Lvl_3

Tx_Pwr_Lvl_3  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL3";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_Pwr_Lvl_3 (36) };

##### 205 aTx_Pwr_Lvl_4

Tx_Pwr_Lvl_4  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL4";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Tx_Pwr_Lvl_4 (37) };

##### 206 aCurrent_Tx_PwrLvl

Current_Tx_PwrLvl  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the transmit output power for LEVEL4";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Current_Tx_PwrLvl (38) };

##### 207  aCurrent_Channel_ID

Current_Channel_ID  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the current channel number of the frequency loaded in the RF synthesizer";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Current_Channel_ID (39) };

##### 208  aCurrent_Country_Code

Current_Country_Code  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"This defines the current country implementation the PMD is supportting";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Current_Country_Code (40) };

##### 209  aHop_Time

Hop_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The time in nanoseconds for the PMD to change from channel 2 to 80";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Hop_Time (41) };

##### 210  aMax_Dwell_Time

Max_Dwell_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The maximum time in nanoseconds that the radio can operate on a single channel";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Max_Dwell_Time (42) };

##### 211  aCurrent_Dwell_Time

Max_Dwell_Time  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Integer;

BEHAVIOR DEFINED AS

"The current maximum time in nanoseconds that the radio can operate on a single channel set by the MAC";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Current_Dwell_Time (43) };

##### 212  aSynthesizer_Locked

Synthesizer_Locked  ATTRIBUTE

WITH APPROPRIATE SYNTAX

Boolean;

BEHAVIOR DEFINED AS

"This is an indication that the PMD's synthesizer is locked to the current channel speciifed in the Current_Channel_ID";

REGISTERED AS

{ iso(1) member-body(2) us(840) ieee802dot11(xxxx) PHY(1) attribute(7) Synthesizer_Locked (44) };

### 213  FH PHY Managed Objects

The following section defines the managed objects for the FHSS MIB.  Table 10-6 lists these managed objects and the default values.  Preceeding the table is a description of each managed object.

Table 10-17:  FHSS PHY Managed Objects

| Managed Object | Default Value | Operational Semantics | Operational Behavior |
| --- | --- | --- | --- |
| PHY_Type | FHSS | Static | Identical for all PHYs |
| Nbr_Geo_Supported | Implementation Dep. | Static | Identical for all PHYs |
| Geo_USA | FF=Suprt, 00=none | Static | Identical for all PHYs |
| Geo_Japan | FF=Suprt, 00=none | Static | Identical for all PHYs |
| Geo_Europe | FF=Suprt, 00=none | Static | Identical for all PHYs |
| CCA_Method | 00=CS only, FF=CS & Data Det. | Static | Identical for all PHYs |
| CCA_MaxP_Det_Time | 16 usec. | Static | Identical for all PHYs |
| CCA_Asmnt_Time | 30 usec. | Static | Identical for all PHYs |
| CCA_Decay_Time | Implementaion Dep. | Static | Identical for all PHYs |
| RxTx_Switch_Time | 10 usec. | Static | Implementation Dep. |
| TxRx_Switch_Time | 10 usec. | Static | Implementation Dep. |
| TxRamp_On_Time | 8 usec. | Static | Implementation Dep. |
| TxRamp_Off_Time | 8 usec. | Static | Implementation Dep. |
| Tx_RF_Delay | 1 usec. | Static | Implementation Dep. |
| Rx_RF_Delay | 4 usec. | Static | Implementation Dep. |
| Tx_PLCP_Delay | 1 usec | Static | Implementation Dep. |
| Rx_Clk_Rcvy_Delay | 2 usec | Static | Implementation Dep. |
| TxRx_Turnaround_Time | Implementaion Dep. | Static | Implementation Dep. |
| RxTx_Turnaround_Time | 20 usec | Static | Identical for all PHYs |
| MAC_Ack_Delay | 0 | Static | Set by LME |
| Tx_SIFS | Implementaion Dep. | Static | Identical for all PHYs |
| Rx_SIFS | Implementaion Dep. | Static | Identical for all PHYs |
| Slot_Time | 50 usec | Static | Identical for all PHYs |
| MPDU_Maximum_Length | 400 bytes | Static | Identical for all PHYs |
| MPDU_Current_Max_Length | 0 | Dynamic | Set by LME |
| Nbr_Supported_Rates | Implementaion Dep. | Static | Implementation Dep. |
| Rate_1MHz | FF=Suprt, 00=none | Static | Implementation Dep. |
| Rate_2MHz | FF=Suprt, 00=none | Static | Implementation Dep. |
| Current_Bit_Rate |  | Dynamic | Set by LME |
| Nbr_Supported_Antennae | Implementaion Dep. | Static | Implementation Dep. |
| Antenna_One_Type | Implementaion Dep. | Static | Implementation Dep. |
| Antenna_Two_Type | Implementaion Dep. | Static | Implementation Dep. |

FHSS PHY Managed Objects Continued

| Managed Object | Default Value | Operational Semantics | Operational Behavior |
| --- | --- | --- | --- |
| Nbr_Supported_Power_Levels | Implementaion Dep. | Static | Implementation Dep. |
| Tx_Pwr_Lvl_1 | Implementaion Dep. | Static | Implementation Dep. |
| Tx_Pwr_Lvl_2 | Implementaion Dep. | Static | Implementation Dep. |
| Tx_Pwr_Lvl_3 | Implementaion Dep. | Static | Implementation Dep. |
| Tx_Pwr_Lvl_4 | Implementaion Dep. | Static | Implementation Dep. |
| Current_Tx_PwrLvl | 0 | Dynamic | Set by LME |
| Current_Channel_ID | 0 | Dynamic | Set by LME |
| Current_Country_Code | 0 | Dynamic | Set by LME |
| Hop_Time | 224 usec. | Static | Identical for all PHYs |
| Max_Dwell_Time | 400 msec | Static | Identical for all PHYs |
| Current_Dwell_Time | 20 msec | Dynamic | Set by LME |
| Synthesizer_Locked | 0 | Dynamic | Set by PLME |

#### 214  FH PHY Managed Objects Definitions

##### 215 PHY_Type

The PHY_Type is Frequency Hopping Spread Spectrum.  The LME uses this oject to determine what PLCP and PMD is providing services to the MAC.  It also is used by the MAC to determine what MAC Layer Management State machines must be invoke to support the PHY.

##### 216  Nbr_Geo_Supported.

Operational requirements for FHSS PHY are define by agencies representing certain geographical regions.  The FHSS PHY has defined operating specifications for three specific regions.  These regions are USA, Japan and Europe.  This object defines the number of regions supported by any FHSS implementation.  This number therefore can be 1, 2, or 3.

##### 217  Geo_USA.

This is a boolean indication of whether or not the implementation supports the operational requirements for the USA.  A 00h indicates that the USA requirements are not supported while a FFh indicates that the implementation does support operation in the USA.

##### 218  Geo_Japan.

This is a boolean indication of whether or not the implementation supports the operational requirements for Japan.  A 00h indicates that the Japanesse requirements are not supported while a FFh indicates that the implementation does support operation in Japan.

##### 219  Geo_Europe.

This is a boolean indication of whether or not the implementation supports the operational requirements for Europe.  A 00h indicates that the European requirements are not supported while a FFh indicates that the implementation does support operation in Europe.

##### 220  CCA_Method.

add Text

##### 221  CCA_MaxP_Det_Time.

add Text

##### 222  CCA_Asmnt_Time.

add Text

##### 223  CCA_Decay_Time.

add Text

##### 224  RxTx_Switch_Time.

add Text

##### 225  TxRx_Switch_Time.

add Text

##### 226  TxRamp_On_Time.

add Text

##### 227  TxRamp_Off_Time.

add Text

##### 228  Tx_RF_Delay.

add Text

##### 229  Rx_RF_Delay.

add Text

##### 230  Tx_PLCP_Delay.

add Text

##### 231  Rx_Clk_Rcvy_Delay.

add Text

##### 232  TxRx_Turnaround_Time.

add Text

##### 233  RxTx_Turnaround_Time.

add Text

##### 234  MAC_Ack_Delay.

add Text

##### 235  TX_SIFS.

add Text

##### 236  RX_SIFS.

add Text

##### 237  Slot_Time.

add Text

##### 238  MPDU_MAximum_Length.

add Text

##### 239  MPDU_Current_Max_Length.

This is the current maximum MPDU length the MAC has determined to use for the present state of the media.  The PHY can use this value to timeout the CCA state machine if the channel is be busy for a longer period of time than what would normally take to receive an MPDU of this length.

##### 240 Nbr_Supported_Rates.

add Text

##### 241 Rate 1MHz.

add Text

##### 242 Rate 2MHz.

add Text

##### 243 Current_Bit_Rate.

add Text

##### 244  Nbr_Supported_Antenna.

add Text

##### 245  Antenna_One_Type.

add Text

##### 246  Antenna_Two_Type.

add Text

##### 247  Nbr_Supported_Power_Levels.

add Text

##### 248  Tx_Pwr_Lvl_1.

add Text

##### 249  Tx_Pwr_Lvl_2.

add Text

##### 250  Tx_Pwr_Lvl_3.

add Text

##### 251 Tx_Pwr_Lvl_4.

add Text

##### 252 Current_Tx_PwrLvl.

add Text

##### 253 Current_Channel_ID.

add Text

##### 254 Current_Country_Code.

add Text

##### 255 Hop_Time.

add Text

##### 256  Max_Dwell_Time.

add Text

##### 257  Current_Dwell_Time.

add Text

##### 258  Synthesizer_Locked.

add Text

§