IEEE 802.11

Wireless Access Method and Physical Layer Specifications

Title:	Direct Sequence PHY PICS Proforma

Presented by:	Mike Trompower

Aironet Wireless Communications, Inc.

367 Ghent Road, Suite 300

Akron, Ohio 44333

Tel:	(216) 665-7920

Fax:	(216) 665-7301

Email:	mtrom@aironet.com

Abstract:	This paper provides the necessary text aimed at providing conformance to the Direct Sequence PHY specification.  This text should be included in a complete section which also contains the necessary instruction and vendor identification information.  The information in this text does not provide any detail necessary for compliance to the MAC layer.

Action:	Adopt the text  in this paper for inclusion in the relevant portions of P802.11/D2.x pertaining to conformance issues.

Introduction

Using the IEEE 802.12 PICS Proforma as a guide, the following text and tables were created using section 12 of P802.11 draft 2.

Annex A.  DSSS PHY PICS Proforma

A.1  Introduction

The supplier of a direct sequence physical layer implementation which is claimed to conform to this Standard shall complete the following Protocol Implementation Conformance Statement (PICS) proforma.

A.2  Abbreviations and Special Symbols

A.2.1  Status Symbols

M	mandatory

O	optional

O.n	optional, but support of at least one of the group of options labeled by the same numeral n is required

A.2.2  General Abbreviations

n/a	not applicable

PICS	Protocol Implementation Conformace Statement

A.3  Instructions for Completing the PICS Proforma

A.3.1  General Structure of the PICS Proforma

{as detailed in doc IEEE P802.11-95/194}

A.3.2  Additional Information

{as detailed in doc IEEE P802.11-95/194}

A.3.3  Exception Information

{as detailed in doc IEEE P802.11-95/194}

A.3.4  Conditional Status

A.3.4.1

A.3.4.2

{as detailed in doc IEEE P802.11-95/194}

A.3.5  Copyright Release for PICS Proformas

{as detailed in doc IEEE P802.11-95/194}

A.4  PICS Proforma -- P802.11:  Identification

A.4.1  Implementation Identification

A.4.2  Physical Layer Support Summary

{as detailed in doc IEEE P802.11-95/194}

A.5  DSSS PHY Capabilities

| Item | PHY Feature | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | PLCP Sublayer Procedures Preamble prepend on TX Preamble frame format Preamble integrity check generation TX Rate change capability Supported Data Rates 1 Mb/s 2 Mb/s Data whitener scrambler scrambler initialization Preamble process on RX Preamble integrity check verify RX Rate change capability Data whitener descrambler | 12.2 12.2.1 12.2.2, 12.2.3 12.2.3, 12.2.3.6 12.2.3.3, 12.2.5 12.1, 12.2.3.3 12.1, 12.2.3.3 12.1, 12.2.3.3 12.2.4 12.2.4 12.2.1 12.2.3, 12.2.3.6 12.2.3.3, 12.2.5 12.2.4 | M M M M M M M M M M M M M | YES _ YES _ YES _ YES _ YES _ YES _ YES _ YES _ YES _ YES _ YES _ YES _ YES _ |
|  | Use specific 11 chip barker code | 12.4.6.3 | M | YES _ |
|  | Chipping continue on power Down | 12.2.6 | O | YES _ NO _ |
|  | Operating Channel Capability North America (FCC) channel 1 channel 2 channel 3 channel 4 channel 5 channel 6 channel 7 channel 8 channel 9 channel 10 channel 11 Europe (ETSI) channel 3 channel 4 channel 5 channel 6 channel 7 channel 8 channel 9 channel 10 channel 11 Japan (RCR) | 12.2.6, 12.4.6.2 | O.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1.1 O.1 O.1.2 O.1.2 O.1.2 O.1.2 O.1.2 O.1.2 O.1.2 O.1.2 O.1.2 O.1 | YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ YES _ NO _ |
|  | Available Receive Parameters RSSI SQ Rate | 12.2.7 | O O O | YES _ NO _ YES _ NO _ YES _ NO _ |
|  | CCA functionality Energy Only (RSSI above threshold) Identification of 802.11 DS signal by correlation both methods | 12.4.8.4 | O.2 O.2 O.2 | YES _ NO _ YES _ NO _ YES _ NO _ |
|  | PLME Functions PLME_RESET.request PLME)RESET.confirm | 12.3 12.3.3.1 12.3.3.2 | O O | YES _ NO _ YES _ NO _ |
|  | Number of transmit antennas | 12.4.5.5, 12.4.5.6 | O | YES _ NO _ |
|  | Number of receive antennas | 12.4.5.5, 12.4.5.6 | O | YES _ NO _ |
|  | antenna selection (diversity) capability | 12.4.5.7 | O | YES _ NO _ |
|  | antenna port(s) availability if available (50 ohm impedance) | 12.4.6.9 | O M | YES _ NO _ YES _ |
|  | transmit power level support if greater than 100mW capability | 12.4.5.8 12.4.7.3 | O M | YES _ NO _ YES _ |
|  | radio type (temperature range) Type 1 Type 2 | 12.4.6.10 | O.3 O.3 | YES _ NO _ YES _ NO _ |
|  | Spurious Emissions conformance | 12.4.6.5 | M | YES _ |
|  | TX - RX turnaround time | 12.4.6.6 | M | YES _ |
|  | RX - TX turnaround time | 12.4.6.7 | M | YES _ |
|  | minimum transmit power level | 12.4.7.2 | M | YES _ |
|  | transmit spectral mask conformance | 12.4.7.4 | M | YES _ |
|  | transmitted center frequency tolerance | 12.4.7.5 | M | YES _ |
|  | chip clock frequency tolerance | 12.4.7.6 | M | YES _ |
|  | transmit power on ramp | 12.4.7.7 | M | YES _ |
|  | transmit power down ramp | 12.4.7.7 | M | YES _ |
|  | RF carrier suppresssion | 12.4.7.8 | M | YES _ |
|  | transmit modulation accuracy | 12.4.7.9 | M | YES _ |
|  | receiver minimum input level sensitivity | 12.4.8.1 | M | YES _ |
|  | receiver maximum input level | 12.4.8.2 | M | YES _ |
|  | receiver adjacent channel rejection | 12.4.8.3 | M | YES _ |