Proposal for Frame Processing Simplification by applying WEP to MPDU payload rather than MSDU payload.

David Bagby

Advanced Micro Devices

7 July 1995

This paper is very short. It simply proposes that WEP be applied to the MPDU body instead of the MSDU body.

This has the advantage of allowing a simpler interaction between Fragmentation and WEP (by applying WEP after fragmentation rather than before). With this change, it will not be necessary to receive an entire MSDU before decrypting the contents, resulting in implementation simplification.

Essentially, this change would logically move WEP processing from the top of the MAC layer to the bottom. Since WEP is invisible outside of the MAC layer, this change has no noticeable impact external to the MAC.

The included printed pages from D1.2 are the actual text alterations that would be required to implement this change to D1.2 text.

Moved:

That WEP processing be changed from an MSDU basis to and MPDU basis by adopting the changes shown to D1.2, said changes to be reflected in the next draft standard revision.

(Section 5)  1	Authentication and Privacy

Note to Editors: This section was inserted into D1.2 after section 4. It was supposed to be inserted immediately after section 3. It is written to follow section 2 and 3 as it contains the description of the logical information needed for the authentication management messages and thus should precede the details of fame formats in section 4. Please move this section as part of the next draft revision (at which time this editor’s note should be deleted from the draft). - Dave Bagby

## 2 Authentication Services

802.11 defines two subtypes of  authentication service; “Open System” and “Shared Key”. The subtype invoked is indicated  in the body of authentication management frames. Thus authentication frames are self identifying with respect to authentication algorithm.

### 3 Open System Authentication

Open System authentication is the simplest of the available authentication algorithms. Essentially it is a null authentication algorithm. Anyone who requests authentication with this algorithm becomes authenticated.

Open system authentication involves a two step authentication transaction sequence. The first step in the sequence is Identity assertion and request for authentication. The second frame in the sequence is a (usually) successful authentication result.

#### 4 Open System Authentication (First frame)

Message type:

Management

Message sub-type:

Authentication

Information Items:

Authentication Algorithm Identification = “open system”

Station Identity Assertion (in SA field of header)

Authentication transaction sequence number = 1

Authentication algorithm dependent information (none)

Direction of message:

From authentication initiating  STA

This frame shall be sent with WEP OFF.

#### 5 Open System Authentication (second and final frame).

Message type:

Management

Message sub-type:

Authentication

Information Items:

Authentication Algorithm Identification = “open system”

Authentication transaction sequence number = 2

Authentication algorithm dependent information. (none)

The result of the requested authentication. This is a fixed length item with values "successful" and "unsuccessful".

Direction of message:

From authenticating station to initiating station.

This frame shall be sent with WEP OFF.

### 6 Shared Key Authentication

Shared Key authentication supports authentication of Stations as either a member of those who know a shared secret key or a member of those who do not.  802.11 shared key authentication accomplishes this without  the need to transmit the secret key in the clear; requiring the use of the WEP privacy mechanism.

The Shared Key authentication scheme is only available if the WEP option is implemented.  Additionally, the Shared Key authentication facility shall be implemented if WEP is implemented.

The required secret, shared key is presumed to have been delivered to participating stations via a secure channel which is independent of 802.11.  This shared key is stored in a write-only MIB variable via the MAC management path.

#### 7 Shared Key Authentication (First frame)

Message type:

Management

Message sub-type:

Authentication

Information Items:

Station Identity Assertion (in SA field of header)

Authentication Algorithm Identification = “shared key”

Authentication transaction sequence number = 1

Authentication algorithm dependent information (none)

Direction of message:

From authentication initiating  STA

This frame shall be sent with WEP OFF.

#### 8 Shared Key Authentication (Second frame)

Before sending the second frame in the shared key authentication sequence,  the sender shall use WEP to generate a string of octets which shall be used as the authentication challenge text.

Message type:

Management

Message sub-type:

Authentication

Information Items:

Authentication Algorithm Identification = “shared key”

Authentication transaction sequence number = 2

Authentication algorithm dependent information = Challenge text.

This field shall be of fixed length of 128 octets. The field shall be filled with octets generated by the WEP mechanism using a random initial seed.

Direction of message:

To authentication initiating  STA

This frame shall be sent with WEP OFF.

#### 9 Shared Key Authentication (Third frame)

The station which received the second frame in the sequence shall copy the challenge text from the second frame into the third frame. The third frame shall be transmitted after encryption by WEP using the shared secret  key.

Message type:

Management

Message sub-type:

Authentication

Information Items:

Authentication Algorithm Identification = “shared key”

Authentication transaction sequence number = 3

Authentication algorithm dependent information = challenge text from sequence two frame.

Direction of message:

From authentication initiating  STA

This frame shall be sent with WEP ON.

#### 10 Shared Key Authentication (Fourth and final frame).

The stations which receives the third frame of the authentication sequence shall attempt to decrypt it using what it believes to be the shared WEP key.  It shall then compare the challenge text recovered to that sent in frame 2 of the sequence. If they are the same then the two stations must have the same shared key. This is presumed to be sufficient authentication for the two  stations involved in the authentication sequence. Upon successful completion of the authentication sequence, both stations shall change their state with respect to each other to “Authenticated

Message type:

Management

Message sub-type:

Authentication

Information Items:

Authentication Algorithm Identification = “shared key”

Authentication transaction sequence number = 4

Authentication algorithm dependent information. = the authentication result.

The result of the requested authentication.

This is a fixed length item with values "successful" and "unsuccessful".

Direction of message:

To authentication initiating station.

This frame shall be sent with WEP OFF.

## 11 The Wired Equivalent Privacy (WEP) Algorithm

### 12	Introduction

Eavesdropping is a familiar problem to users of other types of wireless technology.  P802.11 specifies a wired LAN equivalent data confidentiality algorithm. Wired equivalent privacy is defined as protecting authorized users of a wireless LAN from casual eavesdropping. This service is intended to provide  privacy functionality for the Wireless LAN equivalent to that provided by the physical security attributes inherent to a wired media.

Data confidentiality depends on an external key management service to authenticate users and distribute data enciphering/deciphering keys. P802.11 specifically recommends against running an 802.11 with privacy but without authentication. While this combination is possible, it leaves the system open to significant security threats.

### 13	Properties of the WEP Algorithm

The WEP algorithm has the following properties:

Reasonably Strong:

The security afforded by the algorithm relies on the difficulty of discovering the secret key through a brute-force attack.  This in turn is related to the length of the secret key and the frequency of changing keys.  WEP allows for the changing of the key (k) and frequent changing the Initialization Vector (IV).

Self Synchronizing:

WEP encrypts each message independentlyis . This property is critical for a data-link level encryption algorithm, where "best effort" delivery is assumed and packet loss rates can be high.

Efficient:

The WEP algorithm is efficient and can be implemented in either hardware or software.

Export:

Every effort has been made to design the WEP system operation so as to maximize the chances of approval for export from the U.S. (of products containing a WEP implementation via the Commerce Department). However, due to the legal and political climate toward cryptography at the time of publication, no guarantee can be made that any specific 802.11 implementations that use WEP will be exportable from the United States.

Optionality:

The implementation and use of WEP is an 802.11 option.

### 14	WEP Theory of Operation

The process of disguising (binary) data in order to hide its information content is called encryption.  Data that is not enciphered is called plaintext (denoted by P) and data that is enciphered is called ciphertext (denoted by C).  The process of turning ciphertext back into plaintext is called decryption.  A cryptographic algorithm, or cipher, is a mathematical function used for enciphering or deciphering data.  Modern cryptographic algorithms use a key sequence (denoted by k) to modify their output.  The encryption function E operates on P to produce C:

Ek(P) = C

In the reverse process, the decryption function D operates on C to produce P:

Dk(C) = P

As illustrated in Figure 5-1, note that if the same key is used for encryption and decryption then

Dk(Ek(P)) = P

Figure 5-1:  A Confidential Data Channel

The WEP algorithm is a form of stream cipher in which a block of plaintext is bitwise XOR'd with a pseudo random key sequence of equal length.  The key sequence is generated by the WEP algorithm.

Figure 5-2:  WEP Encipherment Block Diagram

Referring to Figure 5-2 and following from left to right, encipherment begins with a secret key that has been distributed to cooperating stations by an external key management service.  WEP is a symmetric algorithm in which the same key is used for encipherment and decipherment.

The secret key is concatenated with an initialization vector (IV) and the resulting seed is input to a pseudo random number generator (PRNG).  The PRNG outputs a key sequence k of pseudo-random bits equal in length to the largest possible MPDU.  Two processes are applied to the plaintext MPDU.  To protect against unauthorized data modification, an integrity algorithm operates on P to produce an integrity check value (ICV).  Encipherment is then accomplished by mathematically combining the key sequence with P.  The output of the process is a message containing the resulting ciphertext, the IV, and the ICV.

The WEP PRNG is the critical component of this process, since it transforms a relatively short secret key into an arbitrarily long key sequence.  This greatly simplifies the task of key distribution as only the secret key needs to be communicated between stations. The secret key remains constant while the IV changes periodically.  Each new IV results in a new seed and key sequence, thus there is a one-to-one correspondence between the IV and k. The strength of the WEP algorithm increases as  the IV change frequency increases. The IV may be changed as frequently as every MPDU and, since it travels with the message, the receiver will always be able to decipher any message.  The IV shall be transmitted in the clear (since it does not provide an attacker with any information about the secret key).

The WEP algorithm is applied after any fragmentation of the MSDU, i.e. WEP is applied to an MPDU payload.  The {IV, encrypted MPDU, ICV} triplet forms the actual data to be sent in the data frame.

For WEP protected frames, the first four octets of the frame contain the IV field for the MSDU. The IV field shall contain two sub-fields: :A 3-octet field that contains the initialization vector itself, followed by a 1-octet alignment field that shall be all zero. The WEP IV is 24 bits. The 64-bit PRNG seed is formed using the secret key as the most significant 40 bits and the initialization vector as the least significant 24 bits. The IV field is followed by the encrypterd MPDU body, which is followed by the ICV.  The WEP ICV is 32 bits and shall be transmitted in the clear. The WEP Integrity Check algorithm is CRC-32.

As stated previously, WEP combines k with P using bitwise XOR.

Figure 5-3:  WEP Decipherment Block Diagram

Referring to Figure 5-3 and following from left to right, decipherment begins with the arrival of a message.  The IV of the incoming message is used to generate the key sequence necessary to decipher the incoming message. Combining the ciphertext with the proper key sequence yields the original plaintext.  Correct decipherment is verified by performing the integrity algorithm on the recovered plaintext and comparing the computed ICV' to the ICV transmitted with the message. If ICV’ is not equal to ICV, the received MSDU  (of which the unsuccesfully decrypted MPDU is a part) is not passed to LLC andan error indication is sent to MAC management.

### 15	WEP Algorithm Specification

WEP uses the RC4  PRNG algorithm from RSA Data Security, Inc..

Details of the RC4 algorithm are available from RSA. Please contact RSA for algorithm details and the uniform RC4 licensee terms which RSA offers to anyone wishing to use RC4 for the purpose of implementing the 802.11 WEP option.

### 16	 WEP MPDU Expansion:

Figure 5-4 shows the encrypted MPDU as constructed by the WEP.

The WEP ICV = 32 bits.  The expanded MPDU  shall include the IV field immediately preceding the encrypted MPDU.

The  WEP mechanism is invisible to entities outside the 802.11 MAC.

Figure 5-4: Construction of expanded WEP MSDU

## 17 Security services related MIB variables

The 802.11 security mechanisms are controlled via the MAC management path and related MIB variables. This section gives an overview of the security related MIB variables and how they are used. For details of the MIB variable definitions, refer to section 8.X.

### 18 Authentication related MIB variables

The type of authentication invoked when authentication is attempted is controlled by the MIB variable Authentication_Type. This variable may have the following values:

1 = Open System

2 = Shared Key

All other values are reserved.

### 19 Privacy related MIB variables.

The default value for all shared, secret WEP keys shall be Null. This indicates an invalid WEP key. An

All MIB variables which store WEP keys shall be write-only variables. The use of write-only MIB variables prevents entities from learning secret WEP keys via MAC management facilities.

attempt to use WEP with a Null key shall result in an error condition.

To support shared key configurations, the MIB contains a variable called “Default_WEP_Key”. The default value for this variable is Null. If not null, this variable contains the default key to be used with WEP.

An additional variable called “WEP_Default” is a Boolean. If set True then on transmit, Data frames shall be encrypted using Default_WEP_Key and on receive they shall be decrypted using Default_WEP_Key. The MIB shall not allow WEP_Default to be set to TRUE if Default_WEP_Key is Null. The default value of WEP_Default is False.

802.11 does not require that the same WEP key be used for all stations. The MIB supports the ability to have a separate WEP key for each station with which a Station directly communicates. This is supported by a MIB variable which is a two dimensional array called “WEP_Key_Mapping”. The array is indexed by MAC address and contains two fields for each entry: “WEP_ON” and the corresponding WEP_Key. The MIB shall not allow WEP_ON to be set to TRUE if the corresponding WEP_key entry is Null. The default value for all WEP_ON fields is False. This variable is always indexed by either RA to TA addresses (since WEP is applied only to the wireless link).

The values in this array variable take precedence over the WEP_Default and Default_WEP_Key variables.

The minimal length of WEP_Key_Mapping shall be 10. This value represents a minimum capability that may be assumed for any station which implements the WEP option.

The maximum length of WEP_Key_Mapping shall be implementation dependent and the actual length of the array can be inquired from the read only MIB variable “WEP_Key_Mapping_Length”.

The interactions between these variables is described below:

Transmit case:

if WEP_Key_Mapping(RA, WEP_On) = True then use WEP_KEY_Mapping(RA, WEP_Key) for encryption,

if WEP_Default = True then Use WEP_Default for encryption,

otherwise do no encrypt the frame.

Receive case:

if WEP_Key_Mapping(TA, WEP_On) = True then use WEP_KEY_Mapping(TA, WEP_Key) for decryption,

if WEP_Default = True then Use WEP_Default for decryption,

otherwise do no attempt to decrypt the frame.

Bruce Schneier,  "Applied Cryptography, Protocols, Algorithms and Source Code in C", John Wiley & Sons, Inc. 1994