IEEE 802.11

Wireless Access Method and Physical Specification

Title:	Proposed Starting Point for Section 8 Edit

Date:	July 10, 1995

Author:	Dean Kawaguchi

Symbol Technologies, Inc.

2145 Hamilton Ave.

San Jose, CA.  95125

Voice:  (408)369-2629

FAX:  (408)446-4630

email:  DeanK@psd.symbol.com

This submission is a proposed starting point for editing Section 8.  The most significant change was clearing up the inconsistencies of the PHY_DATA primitive classes, data types, and acceptable combinations.  The new text in this submission reflects the vote which accepted the classes as defined by 94/241, which was a joint submission of the FH, DS, and IR PHY editors.  In addition, a number of editorial changes has been made to clarify the text without changing the intent of the previous text.  A number of technical comments made on this section has not been addressed in this submission.  These will hopefully be addressed in the July 95 meeting.

# 1

# 2

# 3

# 4

# 5

# 6

# 7

# µ8	Physical Service Specification

(editor's note: new text - September 1993)

This section specifies the services and interactions provided by the Physical (PHY) layer to the Medium Access Control (MAC) layer.

a)	by Physical layer (PhL) to Medium Access Control (MAC) - MAC being a sublayer of the Data Link Layer (DLL);

b)	by PhL to MAC;

c)	Other (TBD).

## 9	Detailed Service Specifications

(editor's note: new text - September 1993)

All primitives are specified in an exemplary form only.  Each service defines the primitive and the required information that shall be passed between the MAC and PHY entities.

### 10	Ph-PHY_DATA. request

(editor's note: Issue 12.1 - September 1993))

This primitive defines the transfer of data from the MAC entity to the PHYPh entity.

Ph-PHY_DATA. request  (Class, Data)

Class. This parameter specifies the Ph interface control information component of the Physical Service Ph Interface Data Unit (PSDUPhIDU). The possible values are:

a)	START-OF-ACTIVITY - request transmission of PhPDU (i.e. preamble and Ph headers) prior to Ph user data transmission.

b)	DATA - request the transfer of the associated single octet 'Data' parameter.

c)	END-OF-DATA-AND-ACTIVITY - request:

d)	transmission of  the PhPDU terminating the Ph user data transfer immediately following the last Ph data user transfer.

e)	cessation of active transmission.

Data.   This parameter supplies additional information required to execute the specific primitive.  In the case of a Ph-PHY_DATA.request with class START-OF-DATAACTIVITY, it provides specific values for the interface control parameters associated with a specific PHYh-Layer type.  In the case of a Ph-PHY_DATA.request with class DATA, it provides the specific value of the user data to be transmitted.

Acceptable Combinations

a)	class=Start_of_Data, data=TXVECTOR

Starts the transmit state machine.  TXVECTOR contains the packet length and 	other packet control information which are PHY dependent.

b)	class=Data, data=DATA

One octet of MPDU data

c)	class=End_of_Data, data=NULL

Indicates end of data to transmit.

d)	class=End_of_Activity, data=NULL

Resets the PLCP CS/CCA assessment timers to the state appropriate for the end of a received packet.  This is generated by the MAC at the end of a NAV timer.  This request is used by some PHY implementations which may synchronize antenna diversity switching with the contention backoff timing.  The use of this request in the PHY is optional.

### 11	Ph-PHY_DATA. indication

(editor's note: Issue 12.1 - September 1993))

This primitive defines the transfer of data from the PHYh entity to the MAC entity.

Ph-PHY_DATA. indication  (Class, Data)

Class. This parameter specifies the PHYPh interface control information component of the Physical ServicePh Interface Data Unit (PSDUPhIDU). The possible values are:

a)	START-OF-ACTIVITY - indicates  reception of an apparent transmission from one or more peer Ph entities.

b)	DATA - indicates that the associated 'Data' parameter was received as part of a continuous correctly structured  reception.

c)	END-OF-DATA - indicates that the continuous correctly structured reception of Ph user data is concluded with correct reception of PhPDU implying end of data.

d)	END-OF-ACTIVITY - indicates that the ongoing reception (of an apparent transmission from one or more peer Phs) is concluded, with no further evidence of Ph transmission.

e)	END-OF-DATA-AND-ACTIVITY - indicates the simultaneous occurrence of the end of Ph user data and activity.

Data.  This parameter supplies additional information required to execute the specific primitive.  In the case of a Ph-PHY_DATA.indication with class START-OF-ACTIVITY, START-OF-DATA, END-OF-DATA, or END-OF ACTIVITY or END-OF-DATA-AND-ACTIVITY, it provides specific values for the interface control parameters associated with a specific PHYh-Layer type.  In the case of a Ph-PHY_DATA.indication with class DATA, it provides the specific value of the user data to be transmitted.

Acceptable Combinations

a)	class=Start_of_Activity, data=NULL

CS/CCA channel busy

b)	class=Start_of_Data, data=RXVECTOR

Start of the MPDU portion of a packet following a valid SFD and PLCP Header.  RXVECTOR contains the packet length and other packet information including RSSI.  RXVECTOR is PHY dependent.

c)	class=Data, data=DATA

d)	class=End_of_Data, data=RXERROR

End of a received packet.  RXERROR contains the condition of the received 	packet with states including no_error, header_violation, format_violation, carrier_lost.

e)	class=End_of_Activity, data=NULL

CS/CCA channel idle

### 12	Ph-PHY_DATA. confirm

(editor's note: Issue 12.1 - September 1993))

This primitive is passed from the PHYPh entity to the MAC entity to convey the results from an associated previous service request. The Ph-PHY_DATA. confirm provides the critical timing feedback to inhibit the MAC from starting a new transmission before the previous one is completed.  The final Ph-PHY_DATA. confirmation should not be issued until the PHYPh entity has completed the current transmission.  After all PHY_DATA.request(Data, DATA), it indicates the PLCP is ready to receive another byte.  After the PHY_DATA.request(End_of_Data, NULL), it indicates that transmission is complete.

Ph-PHY_DATA. confirm  (Status)

Status. This parameter specifies either success or the locally detected reason for inferring failure.

Acceptable Combinations

### 13	Others

## 14	Overview of Physical Layer Services

### 15	General Description of Service Provided

(editor's note: Issue 12.1 - September 1993)

The interface specification proposed provides the following basic services:

a)	Transfer Physical ServiceLayer Interface Data Units (PSDUPhIDUs) between the Media Access Control (MAC) layer Data Link Layer (DLL) and the Physical (PHY) Layer (PhL) in a manner consistent with ISO 7498 [3].

b)	This specification is intended to insure interoperability between conformant stations of the same Physical Layer type

c)	The intention is to support a variety of different PHYh's, using a common medium independent interface. The current defined PHYh types are: Direct Sequence Spread Spectrum (DSSS) in the 2.54 GHz ISM Band, Frequency Hopping Spread Spectrum (FHSS) in the 2.54 GHz ISM Band and baseband IR.

d)	In addition to PShIDU's, information regarding the characteristics of the receive signal and current state of PHYh Control Parameter Vector are passed across the PHY-MACPh-DLL interface on a frame by frame basis.  There is also the capability for the adjustment of transmission parameters by the MACData Link Layer on a frame by frame basis.  This is in addition to conventional station management information on a per request basis.

### 16	Overview of Interactions

(editor's note: Issue 12.1 - September 1993))

The transmission of normal data between Physical (PHYh)   and Media Access Control Data Link (MACDL) layer entities takes place via the Physical Service Access Point (PHY-PhSAP).

The PHYh entity determines the timing of all transmissions. When the MAC entity  is ready to transmit a Physical ServiceMAC Protocol Data Unit (PSDUMPDU), it shall pass the PSDUMPDU with the concatenated FCS to the PHYh entity using a sequence of Ph-PHY_DATA. request primitives. This sequence of   requests consist of a single Ph-PHY_DATA. request specifying START-OF-DATAACTIVITY, followed by n consecutive Ph-PHY_DATA. requests specifying Data ( where n  defines the packet size (note 1)), and concluded by a single Ph-PHY_DATA. request specifying END-OF-DATA-AND-ACTIVITY.  The data parameter of the Ph-PHY_DATA.request primitive is used to convey specific values of interface control information parameters when the class of the Ph-PHY_DATA.request primitive is START-OF-DATAACTIVITY.

(editor note: the minimum and maximum packet sizes are TBD - September 1993))

The PHYh entity signals the process completion of each Ph-PHY_DATA. request primitive and its readiness to accept a new Ph-PHY_DATA. request with a Ph-PHY_DATA.confirm primitive.  A Ph-PHY_DATA. request should not be issued by the MAC entity until a Ph-PHY_DATA. confirm corresponding to the previous request has been received from the PHYh entity.

The PHYh entity reports, using the data PHY-SAP (PhSAP), a received PSDUMPDU with a sequence of Ph-PHY_DATA. indication primitives  which shall consist of: a single PHY_DATA.indication specifying START-OF-ACTIVITY indicating that the channel is busy, followed by a PHY_DATA.indication specifying START-OF-DATA and indicating a valid PLCP header has been received, followed by n consecutive PHY_DATA.indications specifying DATA, followed by a single PHY_DATA.indication specifying END-OF-DATA and indicating the end of the received frame.  The receive sequence is conditionally concluded by a single PHY_DATA.indication specifying END-OF-ACTIVITY if the clear channel assessment following the received packet indicates that the channel is clear.

Under some channel conditions, the PHY_DATA.indication specifying START-OF-ACTIVITY and indicating that the channel is busy may occur without being followed by the reception of an MSDU.  The CCA channel busy indication will remain set until the PHY_DATA.indication specifying END-OF-ACTIVITY is issued by the Physical Layer.

a)	a single Ph-DATA indication specifying START-OF-ACTIVITY, followed by consecutive Ph-DATA indications specifying DATA, followed by a single Ph-DATA indication specifying END-OF-DATA, and concluded by a single Ph-DATA indication specifying END-OF-ACTIVITY; or,

b)	a single Ph-DATA indication specifying START-OF-ACTIVITY, followed by consecutive Ph-DATA indications specifying DATA, followed by a single Ph-DATA indication specifying END-OF-DATA-AND-ACTIVITY; or,

c)	a single Ph-DATA indication specifying START-OF-ACTIVITY which may be followed by one or more consecutive Ph-DATA indications specifying DATA, and concluded by a single Ph-DATA indication specifying END-OF-ACTIVITY (note: this last sequence is indicative of an incomplete or incorrect reception).

The PHYh entity may also reports a set of PHYh specific parameters using the signal parameter vector (i.e. signal quality, channel used, received signal strength etc.). This reporting is synchronous with the reporting of the data on a frame by frame basis and is implemented through the use of the data parameter of the Ph-HY_DATA.indication primitive when the class is anything other than DATA.  In addition, when requested by the Station Management entity, information on the managed objects will be reported by the PHYh entity  through the Layer Management Entity Service Access Point (LME-SAP).

### 17	Basic Services and Options

(editor's note: Issue 12.1 - September 1993))

The PHYPhS shall support the transfer of Physical ServiceMAC Protocol Data Units (PSDUMPDU).  The PHYPhS shall support a minimum of onesingle channel.  Support of additional channels is optional.  If more than one channel is implemented, the MAC will be informed about the number of channels and the channel in use by indication passed across the Layer Management EntityPh Service Access Point (LME-SAP).  The MAC will be able to change channels using the LME-SAPPhSAP.

The PHYPhS shall support a minimum of onesingle level of transmit power.  Support of additional levels is optional.  If more than one level is implemented, the MAC will be informed about the number of levels and the level in use by indication passed across the LME-SAPPhSAP.  The MAC will be able to change transmit power levels using the LME-SAPPhSAP.

PHYh entities shall report the received signal strength as a relative indicationto one threshold level.  Support of additional thresholds is optional.  If more than one threshold is implemented, the MAC will be informed about the number of thresholds and the value of the threshold, by using the PhSAP.  The MAC will be able to change channels using the PhSAP.

PHYh entities shall implement a jabber control function.

(Note: the need for an indication of a jabber control condition to the MAC is for further study (TBD - September 1993).

§