IEEE P802.11

Wireless Access Method and Physical Layer Specifications

Replacement text for Hopping Sequences subsection

Naftali Chayat

BreezeCom Ltd. (formerly LANNAIR)

Atidim Technological Park

Devorah Haneviah St. - Neve Sharet

Tel Aviv 61131 - ISRAEL

Voice: +972-3-6458423

FAX:  +972-3-6459191

The new text incorporates the decisions of 7 Nov. 1995 to use same hopping patterns for North America/Europe as in 246, with number of sequences increased to 26 per set, and not to modify the sequences for Japan.

### x.x.x	Hop Sequences

The hopping sequence of an individual PMD entity is used to create a pseudorandom hopping pattern utilizing uniformly the designated frequency band. Sets of hopping sequences are used to co-locate multiple PMD entities in similar networks in the same geographic area and to enhance the overall efficiency and throughput capacity of each individual network.

A frequency hopping pattern, Fx , consists of a permutation of all frequency channels defined in Tables 10-11 and 10-12.  For a given pattern number, x, the hopping sequence can be written as:

Fx = {fx (1), fx (2),...fx (p)}

where,

fx (i) = channel number (as defined in 10.6.4) for ith frequency in xth hopping pattern

p = number of frequency channels in hopping pattern (79 for Nortern America/Europe, 23 for Japan)

Given the hopping pattern number, x, and the index for the next frequency, i (in the range 1 .. p) , the channel number shall be defined to be:

fx (i)	=  [b(i) + x] mod (79) + 2	in Nortern America and Europe, with b(i) defined by table 11-xx

=  [(i - 1) * x] mod (23) + 73	in Japan.

| i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) | i | b(i) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 11 | 76 | 21 | 18 | 31 | 34 | 41 | 14 | 51 | 20 | 61 | 48 | 71 | 55 |
| 2 | 23 | 12 | 29 | 22 | 11 | 32 | 66 | 42 | 57 | 52 | 73 | 62 | 15 | 72 | 35 |
| 3 | 62 | 13 | 59 | 23 | 36 | 33 | 7 | 43 | 41 | 53 | 64 | 63 | 5 | 73 | 53 |
| 4 | 8 | 14 | 22 | 24 | 72 | 34 | 68 | 44 | 74 | 54 | 39 | 64 | 17 | 74 | 24 |
| 5 | 43 | 15 | 52 | 25 | 54 | 35 | 75 | 45 | 32 | 55 | 13 | 65 | 6 | 75 | 44 |
| 6 | 16 | 16 | 63 | 26 | 69 | 36 | 4 | 46 | 70 | 56 | 33 | 66 | 67 | 76 | 51 |
| 7 | 71 | 17 | 26 | 27 | 21 | 37 | 60 | 47 | 9 | 57 | 65 | 67 | 49 | 77 | 38 |
| 8 | 47 | 18 | 77 | 28 | 3 | 38 | 27 | 48 | 58 | 58 | 50 | 68 | 40 | 78 | 30 |
| 9 | 19 | 19 | 31 | 29 | 37 | 39 | 12 | 49 | 78 | 59 | 56 | 69 | 1 | 79 | 46 |
| 10 | 61 | 20 | 2 | 30 | 10 | 40 | 25 | 50 | 45 | 60 | 42 | 70 | 28 |  |  |

Table 11-xx: Base hopping sequence b(i) for Northern America - Europe

The sequences are designed to ensure some minimum distance in frequency between contiguous hops. The minimum hop size is 6 Mhz for North America/Europe and 5 Mhz for Japan.

The hopping pattern numbers x are divided into three sets. The sets are designed to avoid prolonged collision periods between different hopping sequences in a set. Hopping sequence sets contain 26 sequences for North America/Europe and 4 sequences per set for Japan:

For North America/Europe:

x =  {0,3,6,9,12,15,18,21,24,27,30,33,36,39,42,45,48,51,54,57,60,63,66,69,72,75}	Set 1

x =  {1,4,7,10,13,16,19,22,25,28,31,34,37,40,43,46,49,52,55,58,61,64,67,70,73,76}	Set 2

x =  {2,5,8,11,14,17,20,23,26,29,32,35,38,41,44,47,50,53,56,59,62,65,68,72,74,77}	Set 3

For Japan:

x =  {6,9,12,15}		Set 1

x =  {7,10,13,16}		Set 2

x =  {8,11,14,17}		Set 3

The three sets of hopping sequences for North America/Europe, of 26 patterns each, are listed Tables A, B, and C in the Annex.  Similarly, the three sets for Japan, of four patterns each, are listed in Table D in the Annex.  The channel numbers listed under each pattern refer to the actual frequency values listed in Tables 10-11 and 10-12.

Table A: Hopping Sequence Set 1

| index | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| 2 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 |
| 3 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 |
| 4 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 5 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 |
| 6 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 |
| 7 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 8 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 |
| 9 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 |
| 10 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 |
| 11 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 |
| 12 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 |
| 13 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 |
| 14 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 |
| 15 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 |
| 16 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 |
| 17 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 |
| 18 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 19 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 |
| 20 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 21 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 |
| 22 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 |
| 23 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 |
| 24 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 |
| 25 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 |
| 26 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 |
| 27 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 |
| 28 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 |
| 29 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| 30 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 31 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| 32 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 |
| 33 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 34 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |
| 35 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 |
| 36 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 37 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 |
| 38 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 |
| 39 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 |

| index | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 |
| 41 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 |
| 42 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 |
| 43 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 |
| 44 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 45 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| 46 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 |
| 47 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 48 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 |
| 49 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| 50 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 |
| 51 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 |
| 52 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 |
| 53 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 |
| 54 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| 55 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 |
| 56 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| 57 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| 58 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 |
| 59 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 |
| 60 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 |
| 61 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 |
| 62 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 |
| 63 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| 64 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 |
| 65 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 |
| 67 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 |
| 68 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 |
| 69 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 70 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 |
| 71 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 |
| 72 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 |
| 73 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 |
| 74 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 |
| 75 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 |
| 76 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 |
| 77 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| 78 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 |
| 79 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 |

Table A: Hopping Sequence Set 1- continued

| index | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| 2 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 |
| 3 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 |
| 4 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 |
| 5 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 |
| 6 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 |
| 7 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 |
| 8 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 9 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 |
| 10 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 |
| 11 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 |
| 12 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |
| 13 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 |
| 14 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 |
| 15 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 |
| 16 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 |
| 17 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| 18 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| 19 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 |
| 20 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 |
| 21 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 |
| 22 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 |
| 23 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 |
| 24 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| 25 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 |
| 26 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 |
| 27 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 |
| 28 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 |
| 29 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 |
| 30 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 |
| 31 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 |
| 32 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 |
| 33 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 |
| 34 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 |
| 35 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 |
| 36 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 |
| 37 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 |
| 38 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 |
| 39 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 |

| index | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 |
| 41 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 |
| 42 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 |
| 43 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 44 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| 45 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 46 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 |
| 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 |
| 48 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 |
| 49 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| 50 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| 51 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 |
| 52 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| 53 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 |
| 54 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| 55 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 |
| 56 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 |
| 57 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 |
| 58 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 59 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 |
| 60 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 61 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 62 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 |
| 63 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 |
| 64 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 |
| 65 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 |
| 66 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 |
| 67 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 68 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| 69 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 |
| 70 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 |
| 71 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 |
| 72 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 73 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 |
| 74 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 |
| 75 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 76 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 |
| 77 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 78 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 |
| 79 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |

Table B: Hopping Sequence Set 2

| index | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 2 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 |
| 3 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 |
| 4 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 5 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 |
| 6 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 |
| 7 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 |
| 8 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 |
| 9 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 |
| 10 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 |
| 11 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 12 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 |
| 13 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 |
| 14 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 |
| 15 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 |
| 16 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 |
| 17 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 |
| 18 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| 19 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| 20 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 |
| 21 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 |
| 22 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 |
| 23 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| 24 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 |
| 25 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 |
| 26 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 |
| 27 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 |
| 28 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 29 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| 30 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 |
| 31 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 |
| 32 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 |
| 33 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 34 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 |
| 35 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 |
| 36 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| 37 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 |
| 38 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 |
| 39 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 |

| index | 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 |
| 41 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 |
| 42 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 |
| 43 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 |
| 44 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 |
| 45 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| 46 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 47 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 48 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 |
| 49 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| 50 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 |
| 51 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 |
| 52 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 53 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| 54 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 |
| 55 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 |
| 56 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| 57 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 |
| 58 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 |
| 59 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 |
| 60 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 |
| 61 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 |
| 62 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 |
| 63 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 64 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 |
| 65 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 66 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |
| 67 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 |
| 68 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 |
| 69 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 70 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 |
| 71 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 |
| 72 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 |
| 73 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 |
| 74 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 |
| 75 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 |
| 76 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 |
| 77 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| 78 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 |
| 79 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 |

Table B: Hopping Sequence Set 2 - continued

| index | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 |
| 2 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 |
| 3 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 |
| 4 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 |
| 5 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 6 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 |
| 7 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| 8 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 9 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 |
| 10 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 |
| 11 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| 12 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 |
| 13 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 |
| 14 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 |
| 15 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 |
| 16 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 |
| 17 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 |
| 18 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| 19 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 20 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 |
| 21 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 |
| 22 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 |
| 23 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 |
| 24 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| 25 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 |
| 26 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 |
| 27 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 |
| 28 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 |
| 29 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 30 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 |
| 31 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 32 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 |
| 33 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 |
| 34 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 |
| 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 |
| 36 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 |
| 37 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 |
| 38 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 |
| 39 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 |

| index | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| 41 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 |
| 42 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 |
| 43 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 44 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 |
| 45 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 |
| 46 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 |
| 47 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 |
| 48 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 |
| 49 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| 50 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 51 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 |
| 52 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| 53 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 |
| 54 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| 55 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 |
| 56 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 |
| 57 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 |
| 58 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 |
| 59 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 |
| 60 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 |
| 61 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 62 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 |
| 63 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 |
| 64 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 |
| 65 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 |
| 66 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 |
| 67 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 68 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 69 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 |
| 70 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |
| 71 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 |
| 72 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 |
| 73 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 |
| 74 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 |
| 75 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| 76 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 |
| 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| 78 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 |
| 79 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |

Table C: Hopping Sequence Set 3

| index | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 2 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 |
| 3 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 |
| 4 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 5 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 |
| 6 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 |
| 7 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 |
| 8 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 |
| 9 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 |
| 10 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 |
| 11 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| 12 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 |
| 13 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 |
| 14 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 |
| 15 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 |
| 16 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| 17 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 |
| 18 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| 19 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| 20 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 21 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 |
| 22 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 |
| 23 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| 24 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 25 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 |
| 26 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 27 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 |
| 28 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| 29 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| 30 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 |
| 31 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 |
| 32 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |
| 33 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 34 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 |
| 35 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 36 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 37 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 |
| 38 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 |
| 39 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 |

| index | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 |
| 41 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 |
| 42 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 |
| 43 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 |
| 44 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 |
| 45 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| 46 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 |
| 47 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 |
| 48 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 |
| 49 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 50 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 |
| 51 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 |
| 52 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 |
| 53 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 |
| 54 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 |
| 55 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 |
| 56 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 |
| 57 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 |
| 58 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 |
| 59 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 |
| 60 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 |
| 61 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 |
| 62 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 |
| 63 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 64 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 |
| 65 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |
| 66 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 |
| 67 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 |
| 68 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 |
| 69 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 |
| 70 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 |
| 71 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 |
| 72 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| 73 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 |
| 74 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 |
| 75 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 |
| 76 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 |
| 77 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 |
| 78 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| 79 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 |

Table C: Hopping Sequence Set 3 - continued

| index | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 |
| 2 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 |
| 3 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 |
| 4 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 |
| 5 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 |
| 6 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 |
| 7 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 |
| 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 |
| 9 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 |
| 10 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 |
| 11 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 |
| 12 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 |
| 13 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 |
| 14 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 |
| 15 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 |
| 16 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 |
| 17 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 |
| 18 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| 19 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 |
| 20 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 |
| 21 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 |
| 22 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 |
| 23 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 |
| 24 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 |
| 25 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 |
| 26 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 |
| 27 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 |
| 28 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 |
| 29 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 |
| 30 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 |
| 31 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 |
| 32 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 |
| 33 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 |
| 34 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 |
| 35 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 |
| 36 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 |
| 37 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 | 60 |
| 38 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 |
| 39 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 |

| index | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 |
| 41 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 |
| 42 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 | 54 | 57 |
| 43 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 |
| 44 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 |
| 45 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 |
| 46 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 |
| 47 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 |
| 48 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 |
| 49 | 42 | 45 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 |
| 50 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 |
| 51 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 |
| 52 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 |
| 53 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 |
| 54 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 |
| 55 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 |
| 56 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 |
| 57 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 |
| 58 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 |
| 59 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 | 56 |
| 60 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 |
| 61 | 12 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 |
| 62 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 |
| 63 | 48 | 51 | 54 | 57 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 |
| 64 | 60 | 63 | 66 | 69 | 72 | 75 | 78 | 2 | 5 | 8 | 11 | 14 | 17 |
| 65 | 49 | 52 | 55 | 58 | 61 | 64 | 67 | 70 | 73 | 76 | 79 | 3 | 6 |
| 66 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 | 58 | 61 | 64 | 67 |
| 67 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 |
| 68 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 |
| 69 | 44 | 47 | 50 | 53 | 56 | 59 | 62 | 65 | 68 | 71 | 74 | 77 | 80 |
| 70 | 71 | 74 | 77 | 80 | 4 | 7 | 10 | 13 | 16 | 19 | 22 | 25 | 28 |
| 71 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 | 49 | 52 | 55 |
| 72 | 78 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 |
| 73 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 | 47 | 50 | 53 |
| 74 | 67 | 70 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 |
| 75 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 | 41 | 44 |
| 76 | 15 | 18 | 21 | 24 | 27 | 30 | 33 | 36 | 39 | 42 | 45 | 48 | 51 |
| 77 | 2 | 5 | 8 | 11 | 14 | 17 | 20 | 23 | 26 | 29 | 32 | 35 | 38 |
| 78 | 73 | 76 | 79 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 79 | 10 | 13 | 16 | 19 | 22 | 25 | 28 | 31 | 34 | 37 | 40 | 43 | 46 |