IEEE P802.11

Wireless Access Method and Physical Layer Specification

Fixed Fragment Size per MSDU

Michael Fischer
Digital Ocean, Inc.
4242Ð3 Medical Drive
San Antonio, TX  78229

# Summary

This submission contains modified text for a portion of section 6.4 of the D2.0 draft to remove the possibility of variation of fragment sizes for hop dwell optimization.  Voters favoring this proposal can cite this document as the source of replacement text for their D2.0 letter ballot comments.

# Modifications to Text in Section 6.4

The MAC may fragment and reassemble MSDUs, directed and multicast/broadcast. The primary reason for fragmenting an MSDU is that it is larger than the PHY is capable of sending in one MPDU. The fragmentation and reassembly mechanisms allows for fragments to be retransmitted.

All stations shall support the simultaneous reception of a minimum of 6 MSDUs.

The payload of a fragment shall be an even number of octets for all fragments except the last. The payload of a fragment shall never be larger than aFragment_Payload (including IV and ICV if WEP is invoked for the MPDU). However, it may be less than aFragment_Payload.

When an MSDU  is to be transmitted, the number of octets in the payload of each  fragment, other than the final fragment, shall be set to an equal value less than or equal to aFragment_Payload.  The number of octets in the payload of the final fragment of an MSDU shall not be greater than the number of octets in the preceding fragments of the MSDU. . Once any fragments of an MSDU have been transmitted, the fragment sizes shall be fixed for the remainder of the MSDU lifetime, including any retransmissions that may be necessary .

Each fragment shall contain a Sequence Control Field, which is comprised of a Sequence Number and Fragment Number. When a station is transmitting an MSDU, the Sequence Number shall remain the same for all fragments of that MSDU.  The fragments shall be sent in order of lowest Fragment Number to highest Fragment Number, where the fragment number value starts at zero, and increases by one for each successive fragment. The Frame Control Field also contains a bit, the Last Fragment bit, that is equal to one to indicates the last (or only) fragment of the MSDU.

The source station will maintain a aTransmit_MSDU_Timer attribute for each MSDU being transmitted. There is also an attribute, aMax_Transmit_MSDU_Lifetime, that specifies the maximum amount of time allowed to transmit a MSDU. The aTransmit_MSDU_Timer starts on the attempt to transmit the first fragment of the MSDU. If aTransmit_MSDU_Timer exceeds aMax_Transmit_MSDU_Lifetime than all remaining fragments are discarded by the source station and no attempt is made to complete transmission of the MSDU.