| Seq. # | Section number | your initials | Cmnt type E, e, T, t |  | Part of NO vote |  | Corrected Text/Comment |  | Rationale |  | Disposition/Rebuttal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8.1.3.3 | RMr | t |  |  | Else select an ad hoc BSSID.... |  | The ESSID ad hoc designation is not defined anywhere, and does not seem appropriate. |  | Concept Accepted, the clause was revised in D2.1. Specific wording was not used. Greg E to create edits to reflect this in sec 8 - to be reviewed in MAC group tues afternoon. |  |
| 2 | 8.4.1.1.1 8.4.2.1.1 8.4.3.1.1 8.4.4.1.14 | RMr |  |  |  | Change aPower_Management_State to aPower_Management_Mode. |  | Changes required to match with 8.2.1.2 (that defines aPower_Management_Mode). |  | accepted - in D2.1 |  |
| 3 | 8.4.1.1.1 | RMr | T |  |  | Add the following: aPower_Mgt_Ability, aCF_Aware |  | 1. Manufactures should have the ability to provide either power saving STAs, or devices for which power saving is not applicable. 2. CF awareness is optional throughout the draft. |  | re 1: declined, group does not see why the mgt entity would want this info or what it would do with it. Re 2: Accepted. Bob O to do edits and bring text back to MAC group for approval. |  |
| 4 | 8.4.2.1.1 | RMr | T |  |  | Add the following: aPower_Mgt_Ability GET, aCF_Aware GET, |  | Refer to my comment on 8.4.1.1.1 |  | see 3 |  |
| 5 | 8.4.3.1.1 | RMr | T |  |  | Add the following: aPower_Mgt_Ability, aCF_Aware, |  | Refer to my comment on 8.4.1.1.1 |  | see 3 |  |
| 6 | 8.4.4.1 | RMr | T |  |  | Add two new subsections: ##### 8.4.4.x. aPower_Mgt_Ability Power_Mgt_Ability ATTRIBUTE WITH APPROPRIATE SYNTAX Boolean; BEHAVIOUR DEFINED AS "True if this station has power saving ability, false otherwise."; REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(10036) SMT(0) attribute(7) power_mgt_ability(x) }; ##### 8.4.4.y. aCF_Aware CF_Aware ATTRIBUTE WITH APPROPRIATE SYNTAX boolean; BEHAVIOUR DEFINED AS "True if this station is contention-free aware, false otherwise."; REGISTERED AS { iso(1) member-body(2) us(840) ieee802dot11(10036) SMT(0) attribute(7) cf_aware(y) }; |  | Refer to my comment on 8.4.1.1.1 |  | see 3 |  |
| 7 | 8.4.4.1 | RMr | E |  |  | From subsection 18 onwards, replace the "###" notations in the REGISTERED AS paragraphs, with the appropriate numbering. |  |  |  | refered to editors, acticipated to be in D2.2 |  |