IEEE 802.11

Wireless Access Method and Physical Layer Specification

Title:		Addition of ASN.1 MIB code to Draft 3.1

Authors:	Howard Hall and Richard Ozer

Windata, Inc.

543 Great Road

Littleton,  MA    01460-1208

U.S.A.

Phone:  508-952-0170

Fax:      508-952-0169

Email:  howardh@wireless.windata.com

oz@wireless.windata.com

Abstract:

This paper provides ASN.1 code that implements the IEEE 802.11 MIB.  The introduction details implementation details and deviations from the OSI specifications.

Action:

Adopt the text additions proposed in this paper for inclusion in P802.11/D4.0.

Introduction:

The Managed Object Class Templates in Chapters 11 (14.4.2) and 13 (13.1) were implemented in the ASN.1 language as a SNMPv2 compliant Management Information Base (MIB).    During this implementation, some deviations from the P802.11D3 templates were necessary.  These deviations are noted in the next section, Implementation Notes.

Implementation Notes:

- Attribute Names - The ASN.1 language does not support underlines or dashes within attribute names.  These have been removed.   The attribute Transmitted_MPDU_Count (11.4.4.2.4) has been renamed to aTransmittedFrameCount.  Naming conventions were standardized, for example, all attributes start with the letter ‘a’.

- Attribute Deletions:

- aDTIMInterval is a duplicate of aDTIMPeriod and has been deleted.

- aTransmitEnableStatus and aPromiscuousStatus have been deleted from the latest specification and thus are not included in this MIB implementation.

- Attribute Numbering - Attributes cannot be numbered as zero (0) in ASN.1, only Groups can.  The numbering of attributes has been modified accordingly.

- The MIB has been implemented following SNMPv2 recommendations.  Attributes defined as integer that are not enumerated are represented as Integer32.  In SNMPv2 the key word INTEGER is reserved for enumerated types only.  In SNMPv2 all tables must have an additional attribute of type RowStatus.  This attribute controls and displays the status of a row - enable, disable, incomplete.

- Multiple Instantiations  - The 802.11 MIB has been designed to support multiple instantiations on a single agent.  In some implementations the agent can contain more than one Station card each with its own unique MIB.  To support this, most groups have been implemented as two dimensional tables, with one index always being a TableIndex.  The value of the TableIndex is linked to an unique StationID (Mac Address) of a Station card residing in the agent in a new MIB table: agStationIDTable.

- agStationIDTable  -  Certain implementations of SNMP cannot handle table indices that are not integers.  To prevent this problem, all Table Indices are defined as Integer32 and a new Table, agStationIDTable, was added at the end of the MIB to link each unique StationID Mac Address to an integer Index.   These indices are used for multiple instantiation of the MIB tables.

- ResourceInfo -  this attribute was not implemented as a two dimensional table, since this information will be unique to each agent, whether or not that agent has multiple Station cards.

- New Tables - Since ASN.1 does not allow Tables within Tables, it was necessary to pull certain attributes out of their specified groups and define them as separate items.  For example, aAuthenticationAlgorithms is defined in section 11.4.1.1.2 as part of the agAuthenticationGrp.  However since it is a table in its own right, it has been pulled out of that group.  Other tabular  attributes that were pulled out of their groups are:

- aDefault_WEP_Key was pulled out of agPrivacyGrp

- aWEP_Key_Mapping was pulled out of agPrivacyGrp

- aGroupAddresses was pulled out of agAddressGrp

- aRegDomainsSuprt was pulled out of agPhyOperationGrp

- aSuprtDataRates was pulled out of agPhyRateGrp

- aSuprtTxAntennas was pulled out of agPhyAntennaGrp

- aSuprtRxAntennas was pulled out of agPhyAntennaGrp

- aDiversitySlctRx was pulled out of agPhyAntennaGrp

- Deleted Groups :

- agAuthentication_grp -  Since the aAuthenticationAlgorithms attribute (Table) had to be pulled out of the agAuthentication_grp, that left only one attribute, aAuthentication_Type.  In order to minimize code size, this attribute was moved to the agStation_Config_grp.   The agAuthentication_grp was not implemented.

- agAddressGrp - Since the aGroupAddress attribute is a table it was moved out of this group, leaving only one attribute aMacAddress.  In order to minimize code size, the aMacAddress attribute was moved to the agOperationGrp and the agAddressGrp was not implemented (again simply to save resources).

- agStatusGrp  -  Since both group attributes aTransmitEnableStatus and aPromiscuousStatus have been deleted from the latest specification there is no longer a need for this group.

- New Groups   -  The following new ASN.1 groups were created:

- agIDGrp - In order to support multiple instantiations within an agent, the SMT attributes, aCurrentBSS_ID, aCurrent_ESS_ID, which are not grouped in the D3 specification, were placed within a new group, agIDGrp.

- agActionGrp -  Instead of having two very small groups, each with its own considerable code and processing overhead, All action items for SMT and MAC were combined into one group to allow for multiple instantiation on an agent.

- agAntennasList -  Instead of having three very small groups, each with its own considerable code and processing overhead, the three tabular antenna lists, aSuprtTxAntennas, aSuprtRxAntennas and aDiversitySlctRx were combined into one new table: agAntennasList.

- Moved Attributes -

- attributes aManufacturerID and aProductID were not in a group in the D3 specification.  In order to allow for multiple instantiation, these attributes were moved to the MAC agOperationGrp.

- aStationID - not grouped in the D3 specification, this attribute was moved to the SMT agStationConfigGrp.

- Notification attributes - these attributes are not implemented in the MIB but are performed by the agent implementation.

Text Insertion Follows:

11.4.7  ASN.1 Definitions

-- ******************************************************************

--  IEEE 802.11 Management Information Base

--

--

--    From Draft Standard IEEE 802.11 p802.11D3

--

--  rwo		1/31/96			Copy prose description from 802.11

--					spec and start turning into ASN.1

--

--   NOTE: all comments starting with * after the dashes

--      are verbatim copies from the 802.11 spec

-- ******************************************************************

IEEE800dot11-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, NOTIFICATION-TYPE,

ObjectName, Integer32, Counter32, snmpModules, Counter64

FROM SNMPv2-SMI

TruthValue, DisplayString, TestAndIncr, TimeStamp

FROM SNMPv2-TC

TEXTUAL-CONVENTION, RowStatus

FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP

FROM SNMPv2-CONF

system, ifIndex, egpNeighAddr

FROM RFC1213-MIB

partyEntry

FROM SNMPv2-PARTY-MIB;

-- *****************************************************

--   MODULE IDENTITY

-- *****************************************************

ieee802dot11 MODULE-IDENTITY

LAST-UPDATED "9605070000Z"

ORGANIZATION "XXXXXXXX"

CONTACT-INFO  "XXXXXXXX"

DESCRIPTION

"The MIB module for IEEE 802.11 entities.

iso(1).member-body(2).us(840).ieee802dot11(10036)"

::= { 1 2 840 10036 }

-- *****************************************************

--   Major sections

-- *****************************************************

-- Station ManagemenT (SMT) Attributes

--* DEFINED AS "The SMT object class provides the necessary support at the

--*  station to manage the processes in the station such that the

--*  station may work cooperatively as a part of an 802.11 network.";

smt OBJECT IDENTIFIER ::= {ieee802dot11 1}

-- * smt GROUPS

--	agStationConfigGrp			smt 1

--	aAuthenticationAlgorithms	              smt 2

--	aKnownAPs OBJECT-TYPE		smt 3

--	aDefaultWEPKey OBJECT-TYPE		smt 4

--	aWEPKeyMapping OBJECT-TYPE	smt 5

--	agPrivacyGrp				smt 6

--            agIDGrp				smt 7

--  MAC Attributes

--* DEFINED AS "The MAC object class provides the necessary support

--* for the access control, generation and verification of frame check

--* sequences, and proper delivery of valid data to upper layers.";

mac OBJECT IDENTIFIER ::= {ieee802dot11 2}

-- * MAC GROUPS

--  reference IEEE P802.1f-1993

-- agOperationGrp	::= {mac 1}

-- agCountersGrp	::= {mac 2}

-- aGroupAddressesGrp    ::= {mac 3}

-- agActionGrp		::= {mac 4}

--  Resource Type ID

res		OBJECT IDENTIFIER ::= {ieee802dot11 3}

resAttribute	OBJECT IDENTIFIER ::= {res 8 }

---  PHY group

phy			OBJECT IDENTIFIER ::= {ieee802dot11 4}

--DEFINED AS "The PHY object class provides the necessary support

--   for all the required PHY operational information which may vary from PHY

--   to PHY and from STA to STA to be communicated to upper layers."

-- * phy GROUPS

--	agPhyOperationGrp	 ::= {phy 1}

--	agPhyRateGrp		 ::= {phy 2}

--	agPhyAntennaGrp	 ::= {phy 3}

--	agPhyTxPwrGrp		 ::= {phy 4}

--	agPhyFHSSGrp		 ::= {phy 5}

--	agPhyDSSSGrp		 ::= {phy 6}

--	agPhyStatusGrp		 ::= {phy 7}

--	agPhyPwrSavingGrp	 ::= {phy 8}

--	aRegDomainsSuprt 	 ::= {phy 9}

--	aSuprtDataRates 		 ::= {phy 10}

--	aSuprtTxAntennas 	 ::= {phy 11}

--	aSuprtRxAntennas 	 ::= {phy 12}

--	aDiversitySlctRx 		 ::= {phy 13}

--  Table to link StationID values to a particular Index for all other MIB tables

agStationIDTable	OBJECT IDENTIFIER ::= {ieee802dot11 5}

-- **********************************************************************

--   Textual conventions from 802 definitions

-- **********************************************************************

-- Power Management, from IEEE P802.11D3 7.1.3.1.7

PowerManagementModetype ::= INTEGER { active (0), powersave (1) }

DefaultWEPKeytype ::= OCTET STRING (SIZE (8))

--  counter types from x.721/DIS 10165-2

--   counters are set to Integer32 instead of Counter so they can be writable

PdusSentCounter ::=			Integer32

PdusReceivedCounter ::=		Integer32

OctetsSentCounter ::=			Integer32

CorruptedPDUsReceivedCounter ::= 	Integer32

-- **********************************************************************

--   MIB attribute OBJECT-TYPE definitions follow

-- **********************************************************************

-- **********************************************************************

--   SMT Station Config Group Table

--* 	agStationConfigGrp  TABLE

-- **********************************************************************

agStationConfigGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagStationConfigGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Station Configuration attributes.  In tablular form to

allow for multiple instances on an agent"

::= {  smt 1 }

agStationConfigGrpEntry OBJECT-TYPE

SYNTAX TypeagStationConfigGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agStationConfigGrp table."

INDEX {aStationTableIndex}

::= {  agStationConfigGrp 1 }

TypeagStationConfigGrpEntry ::=

SEQUENCE {

aStationTableIndex			Integer32,

aStationID				MacAddress,

aActingasAPStatus			INTEGER,

aActingasWirelessAPStatus 		 INTEGER,

aAssociatedState				INTEGER,

aBeaconPeriod				Integer32,

aPowerManagementMode		INTEGER,

aPassiveScanDuration			INTEGER,

aListenInterval				Integer32,

aScanMode				INTEGER,

aScanState				INTEGER,

aCFPRate				Integer32,

aCFPMaxDuration			Integer32,

aDTIMPeriod				Integer32,

aATIMWindow				Integer32,

aMaxMPDUTime			Integer32,

aMediumOccupancyLimit		Integer32,

aCFAware				INTEGER,

aAuthenticationType			INTEGER,

agStationConfigGrpStatus		RowStatus }

--* 	aStationID

aStationID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The purpose of aStationID is to allow a manager to rename a

station for its own purposes.  This attribute provides for

that eventuality while keeping the true MAC address

independent.  Its syntax is MAC address and default value is

the station's MAC address because this is a readily accessible unique value.  "

::= { agStationConfigGrpEntry 1 }

--* 	aActingasAPStatus

aActingasAPStatus OBJECT-TYPE

SYNTAX INTEGER {  true (1),  false (2)    }

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"True if this station is acting as an Access

Point, false otherwise.  The default value of this attribute shall be false."

::= {  agStationConfigGrpEntry 2 }

--*	aActingasWirelessAPStatus

aActingasWirelessAPStatus  OBJECT-TYPE

SYNTAX INTEGER { true (1), false (2)   }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute when true, specifies that the station is acting as a

wireless AP.  The default value of this attribute shall be false."

::= {  agStationConfigGrpEntry 3 }

--* 	aAssociatedState

aAssociatedState OBJECT-TYPE

SYNTAX INTEGER {   true (1),   false (2)	   }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall describe the current associated state of

the station.  The default value of this attribute shall be false."

::= {  agStationConfigGrpEntry 4 }

--* 	aBeaconPeriod

aBeaconPeriod OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The beacon period shall indicate the time, in

kmicroseconds, between the transmission of beacon frames

if the station is acting as an Access Point.  If the

station is not an Access Point but is associated with one,

the beacon period shall indicate the time, in

kmicroseconds, between the expected arrival of beacon

frames.  If the station is not an Access Point and is not

associated with one, the beacon period shall indicate the

time, in kmicroseconds, between the transmission of beacon

frames.  The default value of this attribute shall be 100."

::= {  agStationConfigGrpEntry 5 }

--* 	aPowerManagementMode

aPowerManagementMode  OBJECT-TYPE

SYNTAX INTEGER  { 	active (1), powersave (2) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"An enumerated type that describes the current power

management mode of the station.  The allowable values

for this attribute are ACTIVE and POWER SAVE.  The

default value of this attribute shall be ACTIVE."

::= {  agStationConfigGrpEntry 6 }

--* 	aPassiveScanDuration

aPassiveScanDuration	OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute defines the maximum time, in

kmicroseconds, that a station will remain on a single

channel during a passive scan of that channel.  The

default value of this attribute shall be 50."

::= {  agStationConfigGrpEntry 7 }

--* 	aListenInterval

aListenInterval  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute specifies the number of Beacon intervals which

may pass before the station awakens and listens for the next

beacon.  The default value of this attribute shall be zero."

::= {  agStationConfigGrpEntry 8 }

--* 	aScanMode

aScanMode  OBJECT-TYPE

SYNTAX INTEGER {   active (1),    passive (2)   }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"ScanMode is an enumerated type that can take on the values

ACTIVE or PASSIVE.  The default value of this attribute shall be PASSIVE."

::= {  agStationConfigGrpEntry 9 }

--* 	aScanState

aScanState OBJECT-TYPE

SYNTAX INTEGER {   true (1),     false (2)  }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall indicate when a station is

scanning.  The value of this attribute shall be TRUE

when a station is scanning.  It shall be false

otherwise.  The default value of this attribute shall be false."

::= {  agStationConfigGrpEntry 10 }

--* 	aCFPRate

aCFPRate OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the number of beacon

intervals between the beacons that start contention

free periods.  The default value of this attribute shall be 5."

::= {  agStationConfigGrpEntry 11 }

--* 	aCFPMaxDuration

aCFPMaxDuration OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum amount of time,

in units of 1024 microseconds, between the end of the

beacon frame that starts a contention free period and

the end of the CFÐEnd or CFÐEnd+Ack frame that

ends the contention free period.  The default value of

this attribute shall be twice aMAXMPDUTime."

::= {  agStationConfigGrpEntry 12 }

--* 	aDTIMPeriod

aDTIMPeriod OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the number of beacon intervals

between beacon frames which contain DTIMs.  If this

DTIMInterval is one, there is a DTIM in every beacon frame.

The default value of this attribute shall be 5."

::= {  agStationConfigGrpEntry 13 }

--* 	aATIMWindow

aATIMWindow  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute defines the period of time, in

microseconds, after a target beacon transmission time in

an IBSS during which stations buffering frames for Power

Save mode stations will attempt to notify those stations

by transmitting an ATIM frame. The ATIM window begins at

the Target Beacon Time.  The default value of this

attribute shall be 1000."

::= {  agStationConfigGrpEntry 14 }

--* 	aMaxMPDUTime

aMaxMPDUTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the length of time, in

microseconds, to transmit an MPDU of length

aFragmentationThreshold octets, including all PHY framing

overhead, plus the value of aDIFS, plus the value of aCWmax."

::= {  agStationConfigGrpEntry 15 }

--* 	aMediumOccupancyLimit

aMediumOccupancyLimit OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum amount of time, in

units of 1024 microseconds, that a point coordinator may

control the usage of the wireless medium without

relinquishing control for long enough to allow at least

one instance of DCF access to the medium.  The default

value of this attribute shall be 100.  The maximum value

of this attribute shall be 1000."

::= {  agStationConfigGrpEntry 16 }

--* 	aCFAware

aCFAware OBJECT-TYPE

SYNTAX INTEGER {  true (1),   false (2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"When true, this attribute indicates that the station is

able to respond to a CF-Poll with a data frame within a

SIFS time.  This attribute shall be false if the station

is not able to respond to a CF-Poll with a data frame

within a SIFS time."

::= {  agStationConfigGrpEntry 17 }

--* 	aAuthenticationType

aAuthenticationType OBJECT-TYPE

SYNTAX INTEGER (1..2)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the authentication algorithm

used during the authentication sequence.  The value of this

attribute shall be selected from the set in the

aAuthenticationAlgorithms attribute.  The default value of

this attribute shall be 1."

REFERENCE "IEEE P802.11D3 11.4.4.1.11"

::= {  agStationConfigGrpEntry 18 }

agStationConfigGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the

agStationConfigGrp table."

DEFVAL {active}

::= {  agStationConfigGrpEntry 19 }

aStationTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The index used to access rows in the agStationConfigGrp

table, it is associated with a particular StationID as a MAC

Address.  See agStationTable table above."

::= {  agStationConfigGrpEntry 20 }

-- ****************************************************************

--* 	End of agStationConfigGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	AuthenticationAlgorithms  TABLE

-- ****************************************************************

aAuthenticationAlgorithms OBJECT-TYPE

SYNTAX SEQUENCE OF SmtAuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes shall be a set of all the

authentication algorithms supported by the stations. The

following are the default values and the associated algorithm.

Value = 1: Open System

Value = 2: Shared Key"

REFERENCE "IEEE P802.11D3 11.4.4.1.9"

::= {  smt 2 }

aAuthenticationAlgorithmsEntry OBJECT-TYPE

SYNTAX SmtAuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Auth. Algorithms Table"

INDEX {aStationTableIndex,

aAuthenticationAlgorithm}

::= {  aAuthenticationAlgorithms  1 }

SmtAuthenticationAlgorithmsEntry ::= SEQUENCE {

aAuthenticationAlgorithmsTableIndex	 Integer32,

aAuthenticationAlgorithmsIndex  INTEGER,

aAuthenticationAlgorithm	  INTEGER,

aAuthenticationAlgorithmsStatus RowStatus }

aAuthenticationAlgorithmsTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The index used to access rows in the agStationConfigGrp

table, it is associated with a particular StationID as a MAC

Address.  See agStationTable table above."

::= {  aAuthenticationAlgorithmsEntry 1 }

aAuthenticationAlgorithmsIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the algorithm table."

::= {  aAuthenticationAlgorithmsEntry 2 }

aAuthenticationAlgorithm OBJECT-TYPE

SYNTAX INTEGER {  openSystem (1),   sharedKey (2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall be an

authentication algorithms supported by a station. The

following are the default values and the associated algorithm.

Value = 1: Open System

Value = 2: Shared Key"

::= {  aAuthenticationAlgorithmsEntry 3 }

aAuthenticationAlgorithmsStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the auth. algorithm table."

DEFVAL {active}

::= {  aAuthenticationAlgorithmsEntry 4 }

-- ****************************************************************

--* 	End of AuthenticationAlgorithms  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agIDGrp  TABLE

-- ****************************************************************

agIDGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagIDGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"802.11 SMT group containing attributes containing ID

information, implemented as a tabular object so it can be

multiply instantiated in an agent "

::= {  smt 7 }

agIDGrpEntry OBJECT-TYPE

SYNTAX TypeagIDGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agIDGrp table."

INDEX {agIDGrpTableIndex}

::= {  agIDGrp 1 }

TypeagIDGrpEntry ::= SEQUENCE {

agIDGrpTableIndex	 Integer32,

aCurrentAPMacAddress MacAddress,

aCurrentBSSID		 MacAddress,

aCurrentESSID		 OCTET STRING,

aDesiredSSID		 OCTET STRING,

agIDGrpStatus		 RowStatus	 }

agIDGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agIDGrp table."

::= {  agIDGrpEntry 1 }

--* 	aCurrentAPMacAddress

aCurrentAPMacAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"If aActingasAPStatus is false (i.e. current entity is a

non-AP station), then attribute aCurrentAPMacAddress is

the value of the associated Access Point.  This attribute is not

applicable in a station where aActingasAPStatus is TRUE."

::= {  agIDGrpEntry 2 }

--* 	aCurrentBSSID

aCurrentBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall identify the basic service set

(BSS) with which the station is currently associated.  The

default value of this attribute shall be null."

::= {  agIDGrpEntry 3 }

--* 	aCurrentESSID

aCurrentESSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall identify the extended service set (ESS)

with which the station is associated, if any.  The default

value of this attribute shall be null."

REFERENCE "IEEE P802.11D3 7.3.2.2"

::= {  agIDGrpEntry 4 }

--* 	aDesiredSSID

aDesiredSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall contain the ID of the extended service

set (ESS) of which the station is to become a member. A zero

value indicates that the station may join, or start, and ESS

with any ESSID. A non-zero value indicates that the station

shall only join, or start, a BSS with this ESS ID.  The

default value of this attribute shall be null."

REFERENCE "IEEE P802.11D3 7.3.2.2"

::= {  agIDGrpEntry 5 }

agIDGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agIDGrp table."

DEFVAL {active}

::= {  agIDGrpEntry 6 }

-- ****************************************************************

--* 	End of agIDGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	aKnownAPs  TABLE

-- ****************************************************************

aKnownAPs OBJECT-TYPE

SYNTAX SEQUENCE OF SmtKnownAPsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A Table of identities of the most recently know Access Points."

::= {  smt 3}

aKnownAPsEntry OBJECT-TYPE

SYNTAX SmtKnownAPsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A Known APs Entry."

INDEX {aKnownAPsTableIndex, aKnownAPsIndex}

::= {  aKnownAPs 1 }

SmtKnownAPsEntry ::= SEQUENCE {

aKnownAPsTableIndex  Integer32,

aKnownAPsIndex	Integer32,

aKnownAPsValue	  MacAddress,

aKnownAPsStatus	RowStatus }

aKnownAPsTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the Known AP table."

::= {  aKnownAPsEntry 1 }

aKnownAPsIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the Known AP table."

::= {  aKnownAPsEntry 2 }

aKnownAPsValue OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall be a set of the identities of the most

recently known Access Points. Access Points may be included in

this list even if the station did not associate with them. A

station may delete AP identities from this set using any

algorithm of its choosing.  The set may include fewer AP

identities than the number of APs the station has encountered.

The default value of this attribute shall be null. "

::= {  aKnownAPsEntry 3 }

aKnownAPsStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the known AP table."

DEFVAL {active}

::= {  aKnownAPsEntry 4 }

-- ****************************************************************

--* 	End of KnownAPs  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPrivacyGrp  TABLE

-- ****************************************************************

agPrivacyGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPrivacyGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes concerned with 802.11

Privacy.  Created as a table to allow mulitiple

instantiations on an agent"

::= {  smt 6 }

agPrivacyGrpEntry OBJECT-TYPE

SYNTAX TypeagPrivacyGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPrivacyGrp table."

INDEX {agPrivacyGrpTableIndex}

::= {  agPrivacyGrp 1 }

TypeagPrivacyGrpEntry ::= SEQUENCE {

agPrivacyGrpTableIndex		Integer32,

aPrivacyOptionImplemented	INTEGER,

aPrivacyInvoke			INTEGER,

aWEPDefault			INTEGER,

aExcludeUnencrypted		INTEGER,

aICVerrorCount			Integer32,

agPrivacyGrpStatus		RowStatus }

agPrivacyGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS	read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agPrivacyGrp table."

::= {  agPrivacyGrpEntry 1 }

--* 	aPrivacyOptionImplemented

aPrivacyOptionImplemented OBJECT-TYPE

SYNTAX INTEGER {  true (1),   false (2)}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when true, shall indicate that the 802.11 WEP option

is implemented.  The default value of this attribute shall be false."

::= {  agPrivacyGrpEntry 2}

--* 	aPrivacyInvoke

aPrivacyInvoke OBJECT-TYPE

SYNTAX INTEGER {   true (1),     false (2)	   }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this attribute is true it shall indicate that the 802.11 WEP

mechanism is invoked.  The default value of this attribute shall be false."

::= {  agPrivacyGrpEntry 3}

--* 	aWEPDefault

aWEPDefault  OBJECT-TYPE

SYNTAX INTEGER (0..4)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall indicate the use of the corresponding

element of the DefaultWEPKey array when set to values of

one, two, three or four or that the DefaultWEPKey values are not to

be used when set to zero.  The default value of

this attribute shall be 0."

REFERENCE "IEEE P802.11D3 8.3.2"

::= {  agPrivacyGrpEntry 4}

--* 	aExcludeUnencrypted

aExcludeUnencrypted  OBJECT-TYPE

SYNTAX INTEGER {  true (1),  false (2) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this attribute is true, the station shall discard

received MSDUs that have the WEP Frame Control bit equal to

zero.  When this attribute is false, the station may accept

MSDUs that have the WEP Frame Control bit equal to zero."

::= {  agPrivacyGrpEntry 5}

aICVerrorCount  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS	   current

DESCRIPTION

"This counter shall increment when a frame is received

with the WEP bit set in the frame control field and the

value of the ICV as received in the frame does not match

the ICV as it is currently calculated."

::= {  agPrivacyGrpEntry 6}

agPrivacyGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agPrivacyGrp table."

DEFVAL {active}

::= {  agPrivacyGrpEntry 7}

-- ****************************************************************

--* 	End of agPrivacyGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	DefaultWEPKey  TABLE

-- ****************************************************************

aDefaultWEPKey OBJECT-TYPE

SYNTAX SEQUENCE OF ADefaultWEPKeyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP Key Mapping.

This table shall contain the four default WEP secret key

values corresponding to the four possible KeyID values.  The

default value of this attribute shall be null."

REFERENCE "IEEE 802.11D3 11.4.4.1.13"

::= {  smt 4 }

aDefaultWEPKeyEntry OBJECT-TYPE

SYNTAX ADefaultWEPKeyEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Default WEP Key Table"

INDEX {aDefaultWEPKeyTableIndex, aDefaultWEPKeyIndex}

::= {  aDefaultWEPKey  1 }

ADefaultWEPKeyEntry ::= SEQUENCE {

aDefaultWEPKeyTableIndex	Integer32,

aDefaultWEPKeyIndex		INTEGER,

aDefaultWEPKeyValue	    	DefaultWEPKeytype,

aDefaultWEPKeyStatus		RowStatus}

aDefaultWEPKeyTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the Default WEP Key table."

::= {  aDefaultWEPKeyEntry 1 }

aDefaultWEPKeyIndex OBJECT-TYPE

SYNTAX INTEGER (1..4)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the Default WEP Key table."

::= {  aDefaultWEPKeyEntry 2 }

aDefaultWEPKeyValue OBJECT-TYPE

SYNTAX DefaultWEPKeytype

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A default WEP secret key value OCTET STRING (SIZE(8))."

::= {  aDefaultWEPKeyEntry 3 }

aDefaultWEPKeyStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the Default WEP Key table."

DEFVAL {active}

::= {  aDefaultWEPKeyEntry 4 }

-- ****************************************************************

--* 	End of DefaultWEPKey  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	WEPKeyMapping  TABLE

-- ****************************************************************

aWEPKeyMapping OBJECT-TYPE

SYNTAX SEQUENCE OF SmtWEPKeyMappingEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Conceptual table for WEP Key Mapping.

The MIB supports the ability to have a separate WEP key for

each station to which a Station directly communicates.  This

two dimensional array is indexed by MAC address and contains

two fields for each entry:  WEP_ON and the corresponding WEP

key.  The MIB shall not allow WEP_ON to be set to TRUE if the

corresponding WEP_Key entry is Null.  The default value for

all WEP_ON fields is FALSE."

REFERENCE "IEEE P802.11D3 8.3.2"

::= {  smt 5 }

aWEPKeyMappingEntry OBJECT-TYPE

SYNTAX SmtWEPKeyMappingEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the WEP Key Mapping Table"

INDEX {aWEPKeyMappingTableIndex, aWEPKeyMappingIndex}

::= {  aWEPKeyMapping  1 }

SmtWEPKeyMappingEntry ::= SEQUENCE {

aWEPKeyMappingTableIndex 	Integer32,

aWEPKeyMappingIndex  		Integer32,

aWEPKeyMappingKeyOn 	INTEGER,

aWEPKeyMappingWEPkey	INTEGER,

aWEPKeyMappingStatus 		RowStatus}

aWEPKeyMappingTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the WEP Key Mapping table."

::= {  aWEPKeyMappingEntry 1 }

aWEPKeyMappingIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the WEP Key Mapping table."

::= {  aWEPKeyMappingEntry 2 }

aWEPKeyMappingKeyOn OBJECT-TYPE

SYNTAX INTEGER {   true(1),   false(2) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Boolean as to whether WEP is being used at the

Indexed MacAddress Station."

::= {  aWEPKeyMappingEntry 3}

aWEPKeyMappingWEPkey OBJECT-TYPE

SYNTAX INTEGER (0..4)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Entry in Default WEP Key table - selects which of 4

secret keys should be used."

::= {  aWEPKeyMappingEntry 4}

aWEPKeyMappingStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the

WEP Key Mapping table."

DEFVAL {active}

::= {  aWEPKeyMappingEntry 5}

-- ****************************************************************

--* 	End of WEPKeyMapping  TABLE

-- ****************************************************************

-- *******************************************************************

--* 	MAC Attribute Templates

-- *******************************************************************

-- ****************************************************************

--* 	GroupAddresses  TABLE

-- ****************************************************************

aGroupAddresses OBJECT-TYPE

SYNTAX SEQUENCE OF GroupAddressEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"A conceptual table containing a

set of MacAddresses identifying the multicast addresses

for which this station will receive frames.  The default

value of this attribute shall be null."

::= {  mac 3 }

aGroupAddressesEntry OBJECT-TYPE

SYNTAX GroupAddressEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An Entry (conceptual row) in the Group Addresses Table"

INDEX {aGroupAddressesTableIndex, aGroupAddressesIndex}

::= {  aGroupAddresses  1 }

GroupAddressEntry ::= SEQUENCE {

aGroupAddressesTableIndex    	Integer32,

aGroupAddressesIndex  		Integer32,

aGroupAddress		    	MacAddress,

aGroupAddressesStatus 		RowStatus}

aGroupAddressesTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the Group Addresses table."

::= {  aGroupAddressesEntry 1 }

aGroupAddressesIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the Group Addresses table."

::= {  aGroupAddressesEntry 2 }

aGroupAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"MacAddresses identifying a multicast addresses

from which this station will receive frames."

::= {  aGroupAddressesEntry 3 }

aGroupAddressesStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the

Group Addresses table."

DEFVAL {active}

::= {  aGroupAddressesEntry 4 }

-- ****************************************************************

--* 	End of GroupAddress  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agOperationGrp  TABLE

-- ****************************************************************

agOperationGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagOperationGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains MAC attributes pertaining to the Operation of

the MAC.  This has been implemented as a table in order to

allow for multiple instantiations on an agent"

::= {  mac 1 }

agOperationGrpEntry OBJECT-TYPE

SYNTAX TypeagOperationGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agOperationGrp table."

INDEX {agOperationGrpTableIndex}

::= {  agOperationGrp 1 }

TypeagOperationGrpEntry ::= SEQUENCE {

agOperationGrpTableIndex			Integer32,

aRateFactor 			Integer32,

aHandshakeoverhead  		Integer32,

aRTSThreshold 	 		Integer32,

aCWmax 	 		Integer32,

aCWmin 	 		Integer32,

aCTSTime 	 		Integer32,

aACKTime 	 		Integer32,

aACKTimeout 	 		Integer32,

aShortRetryLimit  		Integer32,

aLongRetryLimit  		Integer32,

aMaxFrameLength 		Integer32,

aFragmentationThreshold 		Integer32,

aProbeDelay 	 		Integer32,

aMinProbeResponseTime 		Integer32,

aMaxProbeResponseTime		Integer32,

--		aDTIMInterval 	 		Integer32,

aMaxTransmitMSDULifetime  	Integer32,

aMaxReceiveMSDULifetime  	Integer32,

aMacAddress			MacAddress,

aManufacturerID 		OctetString,

aProductID			OctetString,

agOperationGrpStatus		RowStatus   }

agOperationGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agOperationGrp table."

::= {  agOperationGrpEntry 1 }

--* 	aRateFactor

aRateFactor OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the current rate (in bytes per

second) at which data is transferred across the medium.

The default value of this attribute shall be 1 000 000."

::= {  agOperationGrpEntry 2 }

--* 	aHandshakeoverhead

aHandshakeoverhead OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is the amount of time, in microseconds,

required to complete an RTS/CTS handshake.  This value, along

with the RateFactor, may be used to determine the desirable

setting of the RTSThreshold to maximize data throughput."

::= {  agOperationGrpEntry 3 }

--* 	aRTSThreshold

aRTSThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the number of bytes in an MPDU,

below which an RTS/CTS handshake will not be performed.

An RTS/CTS handshake shall be performed for all frames

where the length of the MPDU is equal to or larger than

this threshold.  Setting this attribute to be larger than

the maximum MSDU size will have the effect of turning off

the RTS/CTS handshake for frames transmitted by this

station. Setting this attribute to zero will have the

effect of turning on the RTS/CTS handshake for all MPDUs

for frames transmitted by this station.  The default value

of this attribute shall be 2305."

::= {  agOperationGrpEntry 4 }

--* 	aCWmax

aCWmax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum size of the

contention window, in slots.  The default value of this

attribute shall be 255."

::= {  agOperationGrpEntry 5 }

--* 	aCWmin

aCWmin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the minimum size of the contention

window, in slots.  The default value of this attribute shall be 7."

::= {  agOperationGrpEntry 6 }

--* 	aCTSTime

aCTSTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the length of time, in

microseconds, it takes to transmit a CTS frame."

::= {  agOperationGrpEntry 7 }

--* 	aACKTime

aACKTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the length of time, in microseconds,

it takes to transmit an ACK frame."

::= {  agOperationGrpEntry 8 }

--* 	aACKTimeout

aACKTimeout OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute specifies the length of time, in

microseconds, in which an ACK frame will be received in

response to transmission of a frame which requires

acknowledgment, timed from receipt of PHYDATA.confirm at

the MAC. The following equation is used to determine

aACKTimeout: --* aSIFSTime+aACKTime"

::= {  agOperationGrpEntry 9 }

--* 	aShortRetryLimit

aShortRetryLimit OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of

transmission attempts of a frame, the length of which is

less than or equal to aFragmentationThreshold, that will

be made before a failure condition is indicated.  The

default value of this attribute shall be 5."

::= {  agOperationGrpEntry 10 }

--* 	aLongRetryLimit

aLongRetryLimit OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the maximum number of

transmission attempts of a frame, the length of which is

greater than aFragmentationThreshold, that will be made

before a failure condition is indicated.  The default

value of this attribute shall be 7."

::= {  agOperationGrpEntry 11 }

--* 	aMaxFrameLength

aMaxFrameLength OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute specifies the maximum MSDU length that

will be accepted for transmission.  The value of this

attribute shall be 2304 octets."

::= {  agOperationGrpEntry 12 }

--* 	aFragmentationThreshold

aFragmentationThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute specifies the current maximum size, in

octets, of the MPDU that will be delivered to the PHY.  An

MSDU will be broken into fragments if its size exceeds the

value of this attribute after adding MAC headers and

trailers.  The default value for this attribute shall be

equal to the maximum size PSDU of the attached PHY and

shall never exceed the maximum size PSDU of the attached

PHY.  The value of this attribute shall never be less than

256.  The default value of this attribute shall be 2304."

::= {  agOperationGrpEntry 13 }

--* 	aProbeDelay

aProbeDelay OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The ProbeDelay shall be the minimum time, in

microseconds, that a station shall wait on a channel prior

to transmitting a Probe frame during active scanning.  The

default value of this attribute shall be zero."

::= {  agOperationGrpEntry 14 }

--* 	aMinProbeResponseTime

aMinProbeResponseTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MinProbeResponseTime shall be the minimum time, in

kmicroseconds, a station shall wait to detect activity on

a channel while waiting for a Probe Response.  The default

value of this attribute shall be 5."

::= {  agOperationGrpEntry 15 }

--* 	aMaxProbeResponseTime

aMaxProbeResponseTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MaxProbeResponseTime shall be the minimum time, in

kmicroseconds, a station shall wait for a Probe Response

given that activity has been detected on the channel.  The

default value of this attribute shall be 50."

::= {  agOperationGrpEntry 16 }

--* 	aMaxTransmitMSDULifetime

aMaxTransmitMSDULifetime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The MaxTransmitMSDULifetime shall be the elapsed time in

kmicroseconds, after the initial transmission of an MSDU,

that further attempts to transmit the MSDU shall be

terminated.  The default value of this attribute shall be 512."

::= {  agOperationGrpEntry 17 }

--* 	aMaxReceiveMSDULifetime

aMaxReceiveMSDULifetime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

" The MaxReceiveMSDULifetime shall be the elapsed time in

kmicroseconds, after the intital reception of a fragmented

MSDU, that further attempts to reassemble the MSDU shall

be terminated.  The default value shall be 512. "

::= {  agOperationGrpEntry 18 }

--* 	aMacAddress

aMacAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION	"Current MAC Address"

::= {  agOperationGrpEntry 19 }

--* 	aManufacturerID

aManufacturerID OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ManufacturerID shall include, at a minimum, the name

of the manufacturer.  It may include additional

information at the manufacturer's discretion.  The default

value of this attribute shall be null."

::= {  agOperationGrpEntry 20 }

--* 	aProductID

aProductID OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ProductID shall include, at a minimum, an identifier

that is unique to the manufacturer.  It may include

additional information at the manufacturer's discretion.

The default value of this attribute shall be null."

::= {  agOperationGrpEntry 21 }

agOperationGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the

agOperationGrp table."

DEFVAL {active}

::= {  agOperationGrpEntry 22 }

-- ****************************************************************

--* 	End of agOperationGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agCountersGrp  TABLE

-- ****************************************************************

agCountersGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagCountersGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group containing attributes that are MAC counters.

Implemented as a table to allow for multiple instantiations

on an agent."

::= {  mac 2 }

agCountersGrpEntry OBJECT-TYPE

SYNTAX TypeagCountersGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agCountersGrp table."

INDEX {agCountersGrpTableIndex}

::= {  agCountersGrp 1 }

TypeagCountersGrpEntry ::= SEQUENCE {

agCountersGrpTableIndex		Integer32,

aTransmittedFrameCount		Integer32,

aTransmittedMSDUCount  	Integer32,

aOctetsTransmittedCount 		Integer32,

aMulticastTransmittedFrameCount PdusSentCounter,

aBroadcastTransmittedFrameCount PdusSentCounter,

aFailedCount 			Integer32,

aRetryCount 			Integer32,

aMultipleRetryCount 		Integer32,

aRTSSuccessCount 		Integer32,

aRTSFailureCount 		Integer32,

aACKFailureCount 		Integer32,

aReceivedFrameCount 		PdusReceivedCounter,

aOctetsReceivedCount 		OctetsSentCounter,

aMulticastReceivedFrameCount 	PdusReceivedCounter,

aBroadcastReceivedFrameCount 	PdusReceivedCounter,

aErrorCount 			CorruptedPDUsReceivedCounter,

aFCSErrorCount 			Integer32,

aFrameDuplicateCount  		Integer32,

aTotalBackoffTime 	 	Integer32,

agCountersGrpStatus		RowStatus }

agCountersGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agCountersGrp table."

::= {  agCountersGrpEntry 1 }

aTransmittedFrameCount OBJECT-TYPE

SYNTAX PdusSentCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute counts the frames that have been

transmitted successfully."

::= {  agCountersGrpEntry 2 }

--* 	aTransmittedMSDUCount

aTransmittedMSDUCount  OBJECT-TYPE

SYNTAX PdusSentCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute counts the MSDUs that have been

transmitted successfully."

::= {  agCountersGrpEntry 3 }

--*

--* 	aOctetsTransmittedCount

aOctetsTransmittedCount OBJECT-TYPE

SYNTAX OctetsSentCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute counts the Octets that have been

transmitted successfully."

::= {  agCountersGrpEntry 4 }

--* 	aMulticastTransmittedFrameCount

aMulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX PdusSentCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment only when the

multicast/broadcast bit is set in the destination MAC

address and the destination MAC address is not the

broadcast address."

::= {  agCountersGrpEntry 5 }

--* 	aBroadcastTransmittedFrameCount

aBroadcastTransmittedFrameCount OBJECT-TYPE

SYNTAX PdusSentCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment only when the destination

MAC address is the broadcast address."

::= {  agCountersGrpEntry 6 }

--* 	aFailedCount

aFailedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when a frame is not

transmitted due to the number of transmit attempts

exceeding the retrymax value."

::= {  agCountersGrpEntry 7 }

--* 	aRetryCount

aRetryCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when a frame is successfully

transmitted after one or more retransmissions."

::= {  agCountersGrpEntry 8 }

--* 	aMultipleRetryCount

aMultipleRetryCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when a frame is successfully

transmitted after more than one retransmission."

::= {  agCountersGrpEntry 9 }

--* 	aRTSSuccessCount

aRTSSuccessCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received to a RTS."

::= {  agCountersGrpEntry 10 }

--* 	RTSFailureCount

aRTSFailureCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received to a RTS."

::= {  agCountersGrpEntry 11 }

--* 	ACKFailureCount

aACKFailureCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received

to a unicast DATA frame."

::= {  agCountersGrpEntry 12 }

--* 	aReceivedFrameCount

aReceivedFrameCount OBJECT-TYPE

SYNTAX PdusReceivedCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute counts the Frames that have been received successfully."

::= {  agCountersGrpEntry 13 }

--* 	aOctetsReceivedCount

aOctetsReceivedCount OBJECT-TYPE

SYNTAX OctetsSentCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute counts the Octets that have been received successfully."

::= {  agCountersGrpEntry 14 }

--* 	aMulticastReceivedFrameCount

aMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX PdusReceivedCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received

with the multicast/broadcast bit set in the destination

MAC address, the destination MAC address is not the

broadcast address and the destination address is in the

set of GroupAddresses."

::= {  agCountersGrpEntry 15 }

--* 	aBroadcastReceivedFrameCount

aBroadcastReceivedFrameCount OBJECT-TYPE

SYNTAX PdusReceivedCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received

with the destination MAC address equal to the broadcast

address."

::= {  agCountersGrpEntry 16 }

--* 	aErrorCount

aErrorCount OBJECT-TYPE

SYNTAX CorruptedPDUsReceivedCounter

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when an error is detected in a received frame."

::= {  agCountersGrpEntry 17 }

--* 	aFCSErrorCount

aFCSErrorCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This counter shall increment when an FCS error is

detected in a received frame."

::= {  agCountersGrpEntry 18 }

--* 	aFrameDuplicateCount

aFrameDuplicateCount  OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

--  GET-REPLACE

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received which

the Sequence Control field indicates is a duplicate."

::= {  agCountersGrpEntry 19 }

--* 	aTotalBackoffTime

aTotalBackoffTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the length of time, in number of

slots, the MAC has spent in a backoff condition.  The default

value of this attribute shall be zero."

::= {  agCountersGrpEntry 20 }

agCountersGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agCountersGrp table."

DEFVAL {active}

::= {  agCountersGrpEntry 21}

-- ****************************************************************

--* 	End of agCountersGrp  TABLE

-- ****************************************************************

-- ******************************************************************

--* 	Action Templates

-- ******************************************************************

-- ****************************************************************

--* 	agActionGrp  TABLE

-- ****************************************************************

agActionGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagActionGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains action attributes for both SMT and MAC

variables. Implemented as a table to allow for multiple

instantiation on Station ID."

::= {  mac 4 }

agActionGrpEntry OBJECT-TYPE

SYNTAX TypeagActionGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agActionGrp table."

INDEX {agActionGrpTableIndex}

::= {  agActionGrp 1 }

TypeagActionGrpEntry ::= SEQUENCE {

agActionGrpTableIndex		Integer32,

actInitializeSMT		INTEGER,

actInitializeMAC		INTEGER,

actAddGroupAddress 		MacAddress,

actDeleteGroupAddress 		MacAddress,

agActionGrpStatus		RowStatus  }

agActionGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agActionGrp table."

::= {  agActionGrpEntry 1 }

--* 	SMT Action Template

--* 	acInitializeSMT

actInitializeSMT OBJECT-TYPE

SYNTAX INTEGER {  true(1),   false(2) }

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The InitializeSMT action establishes the state of the

station management function and the SMT attributes prior

to beginning operation.  This action shall initialize the

SMT attributes with their default values as described in clause

11.4.4.1 except as may be provided as parameters to this action."

::= {  agActionGrpEntry 2 }

--	actInitializeMAC

actInitializeMAC OBJECT-TYPE

SYNTAX INTEGER {  true(1),   false(2) }

MAX-ACCESS	  read-write

STATUS		  current

DESCRIPTION

"The InitializeMAC action establishes the state of the MAC

entity and the MAC attributes prior to beginning

operation.  This action shall initialize the MAC

attributes with their default values as described in

clause 11.4.4.2 except as may be provided as parameters to

this action.  All MAC counters shall be reset."

::= {  agActionGrpEntry 3 }

--	actAddGroupAddress

actAddGroupAddress OBJECT-TYPE

SYNTAX  MacAddress

MAX-ACCESS		read-write

STATUS		  current

DESCRIPTION

"The AddGroupAddress action shall add the specified group

address to the list of group addresses that will be

accepted by the station."

::= {  agActionGrpEntry 4 }

--* 	actDeleteGroupAddress

actDeleteGroupAddress OBJECT-TYPE

SYNTAX  MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The DeleteGroupAddress action shall remove the specified

group address from the list of group addresses that will

be accepted by the station."

::= {  agActionGrpEntry 5 }

agActionGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the agActionGrp table."

DEFVAL {active}

::= {  agActionGrpEntry 6 }

-- ****************************************************************

--* 	End of agActionGrp  TABLE

-- ****************************************************************

-- **********************************************************************

--* 	Resource Type Attribute Templates

-- *********************************************************************

--* 	aResourceTypeIDName

aResourceTypeIDName OBJECT-TYPE

SYNTAX DisplayString ("RTID")

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Contains the name of the Resource Type ID managed object.

The attribute is read-only and always contains the value RTID.

This attribute value shall not be used as a naming attribute

for any other managed object class."

REFERENCE "IEEE STD 802.1F - 1993 p.24"

::= {  resAttribute 1 }

-- ****************************************************************

--* 	aResourceInfo  TABLE

-- ****************************************************************

aResourceInfo OBJECT-TYPE

SYNTAX SEQUENCE OF TypeaResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"provides a means of indicating, in data readable from a

managed object, information that identifies the source of the

implementation."

REFERENCE "IEEE STD 802.1F - 1993 p.24"

::= {  resAttribute 2 }

aResourceInfoEntry OBJECT-TYPE

SYNTAX TypeaResourceInfoEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the aResourceInfo table."

INDEX {aResourceInfoTableIndex}

::= {  aResourceInfo 1 }

TypeaResourceInfoEntry ::= SEQUENCE {

aResourceInfoTableIndex	Integer32,

manufacturerOUI		OCTETSTRING,

manufacturerName       	 DisplayString,

manufacturerProductName DisplayString,

manufacturerProductVersion	DisplayString,

aResourceInfoStatus	  RowStatus  }

aResourceInfoTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the aResourceInfo table."

::= {  aResourceInfoEntry 1 }

manufacturerOUI OBJECT-TYPE

SYNTAX OCTETSTRING

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"takes the value of an organizationally unique identifier. "

::= {  aResourceInfoEntry 2 }

manufacturerName OBJECT-TYPE

SYNTAX DisplayString

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Printable string used to identify the manufacturer of the

resource.  Max. string length is 128 Octets."

::= {  aResourceInfoEntry 3 }

manufacturerProductName OBJECT-TYPE

SYNTAX DisplayString

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A printable string used to identify the manufacturer's product

name of the resource.  Maximum string length is 128 octets "

::= {  aResourceInfoEntry 4 }

manufacturerProductVersion OBJECT-TYPE

SYNTAX DisplayString

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Printable string used to identify the manufacturer's product

version of the resource.  Max. string length is 128 octets."

::= {  aResourceInfoEntry 5 }

aResourceInfoStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the

aResourceInfo table."

DEFVAL {active}

::= {  aResourceInfoEntry 6 }

-- ****************************************************************

--* 	End of aResourceInfo  TABLE

-- ****************************************************************

-- ****************************************************************

--    PHY Attribute Templates

-- ****************************************************************

-- ****************************************************************

--* 	agPhyOperationGrp  TABLE

-- ****************************************************************

agPhyOperationGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyOperationGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"PHY level attributes concerned with operation.

Implemented as a table indexed on Station ID to allow for

multiple instantiations on an Agent."

::= {  phy 1 }

agPhyOperationGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyOperationGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyOperationGrp table."

INDEX {agPhyOperationGrpTableIndex}

::= {  agPhyOperationGrp 1 }

TypeagPhyOperationGrpEntry ::= SEQUENCE {

agPhyOperationGrpTableIndex  Integer32,

aPHYType Integer32,

aCurrentRegDomain Integer32,

aSlotTime Integer32,

aCCAAsmntTime Integer32,

aRxTxTurnaroundTime Integer32,

aTxPLCPDelay Integer32,

aRxTxSwitchTime Integer32,

aTxRampOnTime Integer32,

aTxRFDelay Integer32,

aSIFSTime Integer32,

aRxRFDelay Integer32,

aRxPLCPDelay Integer32,

aMACPrcTime Integer32,

aTxRampOffTime Integer32,

aPreambleLength Integer32,

aPLCPHeaderLngth Integer32,

aMPDUDurationFactor Integer32,

aAirPropagationTime Integer32,

aTempType Integer32,

agPhyOperationGrpStatus	RowStatus

}

agPhyOperationGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agPhyOperationGrp table."

::= {  agPhyOperationGrpEntry 1 }

aPHYType OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a 8 bit integer value which identifies the PHY Type

supported by the attached PLCP and PMD.

FHSS 2.4GHz = 01 , DSSS 2.4GHz = 02, IR Baseband = 03"

::= {  agPhyOperationGrpEntry 2 }

--    aCurrent_Reg_Domain

aCurrentRegDomain OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This octet defines the current regulatory domain this

implementation of the PMD is supporting.  This octet

corresponds to one of the Reg_Domains list in the

Reg_Domains_Suprt attribute list"

::= {   agPhyOperationGrpEntry 3 }

--  aSlot_Time

aSlotTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time in microseconds the MAC will use for defining the PIFS and

DIFS periods.  The Slot_Time is defined as a function of the following

the equation:

CCA_Asmnt_Time + RxTx_Turnaround_Time + Air_Propagation_Time.

Air_Propagation_Time is defined as 1 usec."

::= {   agPhyOperationGrpEntry 4 }

--  aCCA_Asmnt_Time

aCCAAsmntTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The minimum time in microseconds the CCA mechanism has available to

assess the media within every slot to determine whether the media is clear or busy"

::= { agPhyOperationGrpEntry 5 }

--    aRxTx_Turnaround_Time

aRxTxTurnaroundTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" The maximum time in microseconds the PHY requires to change from

receive to transmitting the start of the first symbol out on the air.

The following equation is used to derive the RxTx_Turnaround_Time:

aTx_PLCP_Delay + aRxTx_Switch_Time + aTxRamp_On_Time + aTx_RF_Delay. "

::= {   agPhyOperationGrpEntry 6 }

--   aTx_PLCP_Delay

aTxPLCPDelay OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The nominal time in microseconds the PLCP uses to deliver a symbol

from the MAC interface to the transmit data path of the PMD"

::= { agPhyOperationGrpEntry 7 }

--   aRxTx_Switch_Time

aRxTxSwitchTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The nominal time in microseconds the PMD takes to switch from

Receive to Transmit"

::= { agPhyOperationGrpEntry 8 }

--   aTxRamp_On_Time

aTxRampOnTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum time in microseconds the PMD takes to turn the Transmitter on"

::= { agPhyOperationGrpEntry 9 }

-- aTx_RF_Delay

aTxRFDelay OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The nominal time in nanoseconds the PMD uses to transfer a symbol

through the Transmit path of the PMD"

::= { agPhyOperationGrpEntry 10 }

--  aSIFS_Time

aSIFSTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time in microseconds the MAC and PHY will require to

receive the last symbol of a frame at the air interface,

process the frame and response with the first symbol on the

air interface of the earliest possible response.  The

following equation is used to determine the SIFS_Time:

aRx_RF_Delay + aRx_PLCP_Delay + aMAC_Prc_Delay +

aRxTx_Turnaround_Time"

::= {   agPhyOperationGrpEntry 11 }

-- aRx_RF_Delay

aRxRFDelay OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The nominal time in nanoseconds the PMD uses to deliver a

symbol from the antenna to the PLCP."

::= { agPhyOperationGrpEntry 12 }

-- aRx_PLCP_Delay

aRxPLCPDelay OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The nominal time in nanoseconds the PLCP uses to deliver a

bit from the PMD receive path to the MAC"

::= {   agPhyOperationGrpEntry 13 }

--  aMAC_Prc_Time

aMACPrcTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The nominal time in microseconds the MAC uses to process

a frame and prepare a response to the frame"

::= {   agPhyOperationGrpEntry 14 }

-- aTxRamp_Off_Time

aTxRampOffTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time in nanoseconds the PMD takes to turn the Transmit PA off"

::= {   agPhyOperationGrpEntry 15 }

-- aPreamble_Length

aPreambleLength OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an indication of the current PHY's Preamble Length in bits. "

::= {   agPhyOperationGrpEntry 16 }

--  aPLCP_Hdr_Lngth

aPLCPHeaderLngth OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an indication of the current PHY's PLCP Header Length in bits. "

::= {   agPhyOperationGrpEntry 17 }

-- aMPDU_Duration_Factor

aMPDUDurationFactor OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The parameter aMPDU_Duration_Factor defines the overhead added by

the PHY to the MPDU as it is transmitted over the air. "

::= {   agPhyOperationGrpEntry 18 }

-- aAir_Propagation_Time

aAirPropagationTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The parameter aAir_Propagation_Time is the time it takes a

transmitted signal to go from the transmitting station to the receiving station."

::= {   agPhyOperationGrpEntry 19 }

--  aTemp_Type

aTempType OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an indication of the current PHY's operating

temperature range capability. "

::= {   agPhyOperationGrpEntry 20 }

agPhyOperationGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agPhyOperationGrp table."

DEFVAL {active}

::= {  agPhyOperationGrpEntry 21 }

-- ****************************************************************

--* 	End of agPhyOperationGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	aRegDomainsSuprt  TABLE

-- ****************************************************************

aRegDomainsSuprt OBJECT-TYPE

SYNTAX SEQUENCE OF TypeaRegDomainsSuprtEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"There are different operational requirements dependent on the

regulatory domain.  This attribute list describes the

regulatory domains the PLCP and PMD support in this

implementation.  Each integer is an 8 bit value as defined below:

FCC = 10, DOC = 20, ETSI = 30, MKK = 40"

::= {   phy 9}

aRegDomainsSuprtEntry OBJECT-TYPE

SYNTAX TypeaRegDomainsSuprtEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the aRegDomainsSuprt table."

INDEX {aRegDomainsSuprtTableIndex, aRegDomainsSuprtIndex}

::= {  aRegDomainsSuprt 1 }

TypeaRegDomainsSuprtEntry ::= SEQUENCE {

aRegDomainsSuprtTableIndex	Integer32,

aRegDomainsSuprtIndex   	Integer32,

aRegDomainsSuprtValue		Integer32,

aRegDomainsSuprtStatus		RowStatus}

aRegDomainsSuprtTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the RegDomainsSuprtEntry table."

::= {  aRegDomainsSuprtEntry 1 }

aRegDomainsSuprtIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the RegDomainsSuprtEntry table."

::= {  aRegDomainsSuprtEntry 2 }

--   aReg_Domains_Suprt_Value

aRegDomainsSuprtValue OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"There are different operational requirements dependent on the

regulatory domain.  This attribute list describes the

regulatory domains the PLCP and PMD support in this

implementation.  Each integer is an 8 bit value as defined below:

FCC = 10, DOC = 20, ETSI = 30, MKK = 40"

::= {  aRegDomainsSuprtEntry 3 }

aRegDomainsSuprtStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the aRegDomainsSuprt table."

DEFVAL {active}

::= {  aRegDomainsSuprtEntry 4 }

-- ****************************************************************

--* 	End of aRegDomainsSuprt  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPhyRateGrp  TABLE

-- ****************************************************************

agPhyRateGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyRateGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of PHY attributes concering PhyRate.  Implemented

as a Table indexed on Station ID to allow for multiple

instances on an Agent."

::= {  phy 2 }

agPhyRateGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyRateGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyRateGrp table."

INDEX {agPhyRateGrpTableIndex}

::= {  agPhyRateGrp 1 }

TypeagPhyRateGrpEntry ::= SEQUENCE {

agPhyRateGrpTableIndex	  Integer32,

aSuprtDataRatesTxValue	  Integer32,

aSuprtDataRatesRxValue	  Integer32,

aMPDUMaxLngth	  Integer32,

aPrefMaxMPDUFrgmntLngth Integer32,

agPhyRateGrpStatus	  RowStatus   }

agPhyRateGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agPhyRateGrp table according to which

Station ID they belong."

::= {  agPhyRateGrpEntry 1 }

aSuprtDataRatesTxValue OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Transmit bit rate supported by the PLCP and PMD.  The folowing

list defines the supported data rates:

1M bps = 01, 2M bps = 02, separate table entries for Transmit and Receive"

::= {  agPhyRateGrpEntry 2 }

aSuprtDataRatesRxValue OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Receive bit rate supported by the PLCP and PMD.  The folowing

list defines the supported data rates:

1M bps = 01, 2M bps = 02, separate table entries for Transmit and Receive"

DEFVAL {NULL}

::= {  agPhyRateGrpEntry 3 }

--  aMPDU_Max_Lngth

aMPDUMaxLngth OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum number of octets in a MPDU that can be load into

the PLCP_PDU when transmitting "

::= {  agPhyRateGrpEntry 4 }

--  aPref_Max_MPDU_Frgmnt_Lngth

aPrefMaxMPDUFrgmntLngth OBJECT-TYPE

SYNTAX Integer32

--   orginally defined as: Null Terminated list of byte integers

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The aPref_Max_MPDU_Frgmnt_Lngth managed object is the

recommended maximum fragment length in octets."

::= {  agPhyRateGrpEntry 5 }

agPhyRateGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the

agPhyRateGrp table."

DEFVAL {active}

::= {  agPhyRateGrpEntry 6 }

-- ****************************************************************

--* 	End of agPhyRateGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agAntennasList  TABLE

-- ****************************************************************

agAntennasList OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagAntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This attribute represent a list of one or more antennas which

can be used as the transmit, Receive or Deiversity_select_Rx

antenna.  Each antenna is define as an integer starting with

antenna 1 to antenna N where N is < or = 255"

::= {   phy 11 }

agAntennasListEntry OBJECT-TYPE

SYNTAX TypeagAntennasListEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agAntennasList table."

INDEX {agAntennasListTableIndex, agAntennasListIndex}

::= {  agAntennasList 1 }

TypeagAntennasListEntry ::= SEQUENCE {

agAntennasListTableIndex 	Integer32,

agAntennasListIndex   		Integer32,

aSuprtTxAntennas		Integer32,

aSuprtRxAntennas		Integer32,

aDiversitySlctRx			Integer32,

agAntennasListStatus		RowStatus  }

agAntennasListTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agAntennasList table."

::= {  agAntennasListEntry 1 }

agAntennasListIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agAntennasList table."

::= {  agAntennasListEntry 2 }

aSuprtTxAntennas OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents an antenna which

can be used as the transmit antenna.  Each antenna is define

as an integer starting with antenna 1 to antenna N where N is < or = 255"

DEFVAL {NULL}

::= {  agAntennasListEntry 3 }

aSuprtRxAntennas OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute represents an antenna which

can be used as the receive antenna.  Each antenna is define

as an integer starting with antenna 1 to antenna N where N is < or = 255"

DEFVAL {NULL}

::= {  agAntennasListEntry 4 }

aDiversitySlctRx OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute represents an antenna which

can be used as receive antenna.  Each antenna is define

as an integer starting with antenna 1 to antenna N where N is < or = 255"

DEFVAL {NULL}

::= {  agAntennasListEntry 5 }

agAntennasListStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the agAntennasList table."

DEFVAL {active}

::= {  agAntennasListEntry 6 }

-- ****************************************************************

--* 	End of agAntennasList  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPhyAntennaGrp  TABLE

-- ****************************************************************

agPhyAntennaGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyAntennaGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for PhyAntenna.  Implemented

as a Table indexed on Station ID to allow for multiple

instances on an Agent."

::= {  phy 3}

agPhyAntennaGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyAntennaGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyAntennaGrp table."

INDEX {agPhyAntennaGrpTableIndex}

::= {  agPhyAntennaGrp 1 }

TypeagPhyAntennaGrpEntry ::= SEQUENCE {

agPhyAntennaGrpTableIndex	Integer32,

aCurrentTxAntenna		Integer32,

aDiversitySuprt			Integer32,

agPhyAntennaGrpStatus		RowStatus  }

agPhyAntennaGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agPhyAntennaGrp table

according to which Station ID they belong."

::= {  agPhyAntennaGrpEntry 1 }

--  aCurrent_Tx_Antenna

aCurrentTxAntenna OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current antenna being used to transmit.  This value is one of

the attributes appearing in the list defined as the Suprt_Tx_Antennas."

::= {  agPhyAntennaGrpEntry 2 }

--  aDiversity_Suprt

aDiversitySuprt OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute defines this implementation's support for

diversity.  This attribute can have one of three values.  11h

indicates that diversity is available and performed over a

fixed list of antennas define in attribute Diversity_Slct_Rx.

02h indicates that diversity is not supported.  03h indicates

that diversity is supported and that control of diversity is

also available.  Diversity control indicates that the

Diversity_Slct_Rx can be dynamically modified by the LME."

::= {  agPhyAntennaGrpEntry 3 }

agPhyAntennaGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and

deleting instances of the columnar objects in the

agPhyAntennaGrp table."

DEFVAL {active}

::= {  agPhyAntennaGrpEntry 4 }

-- ****************************************************************

--* 	End of agPhyAntennaGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPhyTxPwrGrp  TABLE

-- ****************************************************************

agPhyTxPwrGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyTxPwrGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for agPhyTxPwrGrp.

Implemented as a Table indexed on Station ID to allow for

multiple instances on an Agent."

::= {  phy  4}

agPhyTxPwrGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyTxPwrGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyTxPwrGrp table."

INDEX {agPhyTxPwrGrpTableIndex}

::= {  agPhyTxPwrGrp 1 }

TypeagPhyTxPwrGrpEntry ::= SEQUENCE {

agPhyTxPwrGrpTableIndex	Integer32,

aNbrSuprtPwrLvls		Integer32,

aTxPwrLvl1			Integer32,

aTxPwrLvl2			Integer32,

aTxPwrLvl3			Integer32,

aTxPwrLvl4			Integer32,

aTxPwrLvl5			Integer32,

aTxPwrLvl6			Integer32,

aTxPwrLvl7			Integer32,

aTxPwrLvl8			Integer32,

aCurrentTxPwrLvl		Integer32,

agPhyTxPwrGrpStatus		RowStatus }

agPhyTxPwrGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agPhyTxPwrGrp table according to which

Station ID they belong."

::= {  agPhyTxPwrGrpEntry 1 }

--  aNbr_Suprt_Pwr_Lvls

aNbrSuprtPwrLvls OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the number of power levels supported by the PMD.

This attribute can have a value of 1 to 8."

::= {  agPhyTxPwrGrpEntry 2 }

--  aTx_Pwr_Lvl_1

aTxPwrLvl1 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL1 in mWatts.

This is also the default power level."

::= {   agPhyTxPwrGrpEntry 3 }

--  aTx_Pwr_Lvl_2

aTxPwrLvl2 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL2 in mWatts."

::= {   agPhyTxPwrGrpEntry 4 }

--  aTx_Pwr_Lvl_3

aTxPwrLvl3 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL3 in mWatts."

::= {   agPhyTxPwrGrpEntry 5 }

--  aTx_Pwr_Lvl_4

aTxPwrLvl4 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL4 in mWatts."

::= {   agPhyTxPwrGrpEntry 6 }

--    aTx_Pwr_Lvl_5

aTxPwrLvl5 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL5 in mWatts."

::= {   agPhyTxPwrGrpEntry 7 }

--  aTx_Pwr_Lvl_6

aTxPwrLvl6 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL6 in mWatts."

::= {   agPhyTxPwrGrpEntry 8 }

--    aTx_Pwr_Lvl_7

aTxPwrLvl7 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL7 in mWatts."

::= {   agPhyTxPwrGrpEntry 9 }

--    aTx_Pwr_Lvl_8

aTxPwrLvl8 OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This defines the transmit output power for LEVEL8 in mWatts."

::= {   agPhyTxPwrGrpEntry 10 }

--  aCurrent_Tx_PwrLvl

aCurrentTxPwrLvl OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Tx_Pwr_Lvl_N currently being used to transmit data.  Some

PHYs also use this value to determine the receiver sensitivity

requirements for CCA. "

::= {   agPhyTxPwrGrpEntry 11 }

agPhyTxPwrGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agPhyTxPwrGrp table."

DEFVAL {active}

::= {  agPhyTxPwrGrpEntry 12 }

-- ****************************************************************

--* 	End of agPhyTxPwrGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPhyFHSSGrp  TABLE

-- ****************************************************************

agPhyFHSSGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyFHSSGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for agPhyFHSSGrp.

Implemented as a Table indexed on Station ID to allow for

multiple instances on an Agent."

::= {  phy  5 }

agPhyFHSSGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyFHSSGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyFHSSGrp table."

INDEX {agPhyFHSSGrpTableIndex}

::= {  agPhyFHSSGrp 1 }

TypeagPhyFHSSGrpEntry ::= SEQUENCE {

agPhyFHSSGrpTableIndex	Integer32,

aHopTime			Integer32,

aCurrentChannelNbr		Integer32,

aMaxDwellTime			Integer32,

aCurrentDwellTime		Integer32,

aCurrentSet			Integer32,

aCurrentPattern			Integer32,

aCurrentIndex			Integer32,

agPhyFHSSGrpStatus		RowStatus  }

agPhyFHSSGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances

of the columnar objects in the agPhyFHSSGrp table according to which

Station ID they belong."

::= {  agPhyFHSSGrpEntry 1 }

--  aHop_Time

aHopTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The time in nanoseconds for PMD to change from channel 2 	to 80"

::= {   agPhyFHSSGrpEntry 2 }

--  aCurrent_Channel_Nbr

aCurrentChannelNbr OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This defines the current channel number of the frequency

loaded in the RF synthesizer"

::= {   agPhyFHSSGrpEntry 3 }

--  aMax_Dwell_Time

aMaxDwellTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The maximum time in nanoseconds that the radio can

operate on a single channel"

::= {   agPhyFHSSGrpEntry 4 }

--  aCurrent_Dwell_Time

aCurrentDwellTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The current time in nanoseconds that the radio shall

operate on a single channel set by the MAC"

::= {   agPhyFHSSGrpEntry 5 }

--  aCurrent_Set

aCurrentSet OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute represents the current set of patterns the PHY

LME is using to determine the hop sequence."

::= {   agPhyFHSSGrpEntry 6 }

--  aCurrent_Pattern

aCurrentPattern OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute represents the current pattern the PHY LME is

using to determine the hop sequence."

::= {   agPhyFHSSGrpEntry 7 }

--    aCurrent_Index

aCurrentIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute represents the current index value the PHY

LME is using to determine the Current_Channel_Nbr."

::= {   agPhyFHSSGrpEntry 8 }

agPhyFHSSGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agPhyFHSSGrp table."

DEFVAL {active}

::= {  agPhyFHSSGrpEntry 9 }

-- ****************************************************************

--* 	End of agPhyFHSSG  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPhyDSSSGrp  TABLE

-- ****************************************************************

agPhyDSSSGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyDSSSGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for agPhyDSSSGrp.

Implemented as a Table indexed on Station ID to allow for

multiple instances on an Agent."

::= {  phy  6 }

agPhyDSSSGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyDSSSGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyDSSSGrp table."

INDEX {agPhyDSSSGrpTableIndex}

::= {  agPhyDSSSGrp 1 }

TypeagPhyDSSSGrpEntry ::= SEQUENCE {

agPhyDSSSGrpTableIndex	Integer32,

aCurrentChannel			Integer32,

aCCAModeSuprt		Integer32,

aCurrentCCAMode		Integer32,

aEDThreshold			Integer32,

agPhyDSSSGrpStatus		RowStatus }

agPhyDSSSGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances of the

columnar objects in the agPhyDSSSGrp table according to which

Station ID they belong."

::= {  agPhyDSSSGrpEntry 1 }

--   aCurrent_Channel

aCurrentChannel OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is an indication of the current operating frequency

channel of the DSSS PHY. "

::= {   agPhyDSSSGrpEntry 2 }

--  aCCA_Mode_Suprt

aCCAModeSuprt OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an indication of the DSSS PHY CCA modes which are supported. "

::= {   agPhyDSSSGrpEntry 3 }

--  aCurrent_CCA_Mode

aCurrentCCAMode OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is an indication of the current CCA method in operation. "

::= {   agPhyDSSSGrpEntry 4 }

--  aED_Threshold

aEDThreshold OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is an indication of the current Energy Detect Threshold

being used by the DSSS PHY. "

::= {   agPhyDSSSGrpEntry 5 }

agPhyDSSSGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agPhyDSSSGrp table."

DEFVAL {active}

::= {  agPhyDSSSGrpEntry 6 }

-- ****************************************************************

--* 	End of agPhyDSSSGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPhyStatusGrp  TABLE

-- ****************************************************************

agPhyStatusGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyStatusGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for agPhyStatusGrp.

Implemented as a Table indexed on Station ID to allow for

multiple instances on an Agent."

::= {  phy 7 }

agPhyStatusGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyStatusGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyStatusGrp table."

INDEX {agPhyStatusGrpTableIndex}

::= {  agPhyStatusGrp 1 }

TypeagPhyStatusGrpEntry ::= SEQUENCE {

agPhyStatusGrpTableIndex	Integer32,

aSynthesizerLocked		Integer32,

agPhyStatusGrpStatus		RowStatus  }

agPhyStatusGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances of the

columnar objects in the agPhyStatusGrp table according to

which Station ID they belong."

::= {  agPhyStatusGrpEntry 1 }

-- aSynthesizer_Lockeda

aSynthesizerLocked OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an indication that the PMD's synthesizer is locked

to the current channel specified in the Current_Channel_Nbr.

00h represents unlocked while FFh represents locked. "

::= { agPhyStatusGrpEntry 2 }

agPhyStatusGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agPhyStatusGrp table."

DEFVAL {active}

::= {  agPhyStatusGrpEntry 3 }

-- ****************************************************************

--* 	End of agPhyStatusGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	agPhyPwrSavingGrp  TABLE

-- ****************************************************************

agPhyPwrSavingGrp OBJECT-TYPE

SYNTAX SEQUENCE OF TypeagPhyPwrSavingGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group of attributes for agPhyPwrSavingGrp.

Implemented as a Table indexed on Station ID to allow for

multiple instances on an Agent."

::= {  phy  8}

agPhyPwrSavingGrpEntry OBJECT-TYPE

SYNTAX TypeagPhyPwrSavingGrpEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the agPhyPwrSavingGrp table."

INDEX {agPhyPwrSavingGrpTableIndex}

::= {  agPhyPwrSavingGrp 1 }

TypeagPhyPwrSavingGrpEntry ::= SEQUENCE {

agPhyPwrSavingGrpTableIndex	Integer32,

aCurrentPowerState 		Integer32,

aSleepTurnonTime 		Integer32,

aDozeTurnonTime 		Integer32,

aCCAWatchDogTimerMax 	Integer32,

aCCAWatchDogCountMax 	Integer32,

aCCAWatchDogTimerMin 	Integer32,

aCCAWatchDogCountMin 	Integer32,

agPhyPwrSavingGrpStatus	RowStatus }

agPhyPwrSavingGrpTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The auxillary variable used to identify instances of the

columnar objects in the agPhyPwrSavingGrp table according to

which Station ID they belong."

::= {  agPhyPwrSavingGrpEntry 1 }

--  aCurrent_Power_State

aCurrentPowerState OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is an indication of the current power state of the PHY."

::= {  agPhyPwrSavingGrpEntry 2 }

---    aSleep_Turnon_Time

aSleepTurnonTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an indication of the time in microseconds required by

the PHY to progress from the SLEEP power down state to the

ACTIVE operating state. "

::= {   agPhyPwrSavingGrpEntry 3 }

--  aDoze_Turnon_Time

aDozeTurnonTime OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is an indication of the time in microseconds required by

the PHY to progress from the Doze power down state to the

ACTIVE operating state. "

::= {   agPhyPwrSavingGrpEntry 4 }

aCCAWatchDogTimerMax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION  	" "

::= {   agPhyPwrSavingGrpEntry 5 }

aCCAWatchDogCountMax OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION  	" "

::= {   agPhyPwrSavingGrpEntry 6 }

aCCAWatchDogTimerMin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION 	" "

::= {   agPhyPwrSavingGrpEntry 7 }

aCCAWatchDogCountMin OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-write

STATUS current

DESCRIPTION " "

::= {   agPhyPwrSavingGrpEntry 8 }

agPhyPwrSavingGrpStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the agPhyPwrSavingGrp table."

DEFVAL {active}

::= {  agPhyPwrSavingGrpEntry 9 }

-- ****************************************************************

--* 	End of agPhyPwrSavingGrp  TABLE

-- ****************************************************************

-- ****************************************************************

--* 	Beginning of aStationID   TABLE

-- ****************************************************************

--  Table to link StationID values to a particular Index for  all other MIB tables

--    agStationIDTable	OBJECT IDENTIFIER ::= {ieee802dot11 5}

aStationIDTable OBJECT-TYPE

SYNTAX SEQUENCE OF TypeaStationIDTableEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Station Configuration attributes.  In tablular form to

allow for multiple instances on a Agent"

::= {  agStationIDTable 1 }

aStationIDTableEntry OBJECT-TYPE

SYNTAX TypeaStationIDTableEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the aStationIDTable table."

INDEX {aStationIDTableIndex}

::= {  aStationIDTable 1 }

TypeaStationIDTableEntry ::=

SEQUENCE {

aStationIDTableIndex	Integer32,

aStationIDTableStationID	MacAddress,

aStationIDTableStatus	RowStatus }

aStationIDTableIndex OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The index used to access rows in the aStationIDTable table

it is associated with a particular StationID as a MAC Address. "

::= {  aStationIDTableEntry 1 }

--* 	aStationID

aStationIDTableStationID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"A unique StationID specifying a local Station (radio card).

Agents can contain multiple Stations, and this table

links each stationID with a unique index.  This index is used

to access all the tables of the ieee 802.11 MIB."

::= { aStationIDTableEntry 2 }

aStationIDTableStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The status column used for creating, modifying, and deleting

instances of the columnar objects in the aStationIDTable table."

DEFVAL {active}

::= {  aStationIDTableEntry 3 }

-- ****************************************************************

--* 	End of aStationID   TABLE

-- ****************************************************************

--*

--*

--*   End of 80211 MIB

END