IEEE P802.11
Wireless LANs

Throughput and Cell Radius Comparison of TGa PHY Submissions

Date:	March 1998

Author:                Hitoshi Takanashi, Masahiro Morikura and Richard van Nee+

NTT Wireless Systems Labs., Lucent Technologies Bell Labs. +

##### Abstract

As mentioned in Doc. 98-104 and 98-110 , actual throughput comparison in a presence of interference is one of the most important issues. Although simulations are needed to get accurate results, an easy and possible comparison method is proposed in this document.

A cell radius comparison, that is another important issue, is discussed in the end of this document.

1. Throughput

1-1.	Assumptions

1-1-1.	Propagation loss

A propagation loss that describes an interference condition is assumed as given by

L=A+25 log(d)							(1)

where d is distance between a transmission site and a reception site.

Thus CIR at the edge of a cell is given by

CIR=25 log(D2/D1)		      (2)

where D1 is the distance between a desired signal transmission site and a reception site, D2 is the distance between an undesired signal transmission site and the reception site. This CIR shall be reserved to secure an appropriate quality even if a station is placed at the edge of the cell.

1-1-2.	Traffic Model

Lets assume a flat traffic model and all access points are sending downlink signals in the cells. Hexagonal cells are also assumed as used in the Doc 98-104. A flat user (traffic) distribution is also assumed.

1-1-3.	Interference

All undesired signals (interference) come from neighbor access points that are sending downlink packets to stations in their cells. A perfect CSMA/CA scheme is also assumed to make this comparison easy. When an access point detects intolerable interference, which exceeds CCI immunity, the access point shall share the same frequency. Degradation due to backoff algorithm is discussed in Session 5.

1-2.	Cluster Size

Although a cluster size is an idea for circuit exchange calls, the same concept can be applied to this comparison. Within the cluster, no access points can send a packet on the same frequency that is being used by an access point that causes interference. The cluster size C is given by

C=(interference area by an access point)/(coverage of an access point)

=							(3)

1-3.	Average shared data rate of the downlink

Assuming CIR=CCI, the number of cells in a cluster (cluster size) is obtained with equ. (2) and (3). The total throughput of a band is shared by the cells that are in the cluster. This means that the data rate of a channel (carrier) multiplied by the number of carriers in the band is shared by all cells in the cluster. Therefore, the average shared data rate Sr is given by

(4)

where Nc is the number of carrier in a band, R is the data rate.

1-4.	Numerical Examples

Though still waiting for important evaluation results, numerical comparison can be provided with data as much as we have. The CCIs of 64 bytes packets are used in this numerical comparison attempt because all CCIs of the 64 bytes packets are available in the evaluation templates except PPM.

OFDM

| R | Nc | CCI | D2/D1 | Area of Cluster D22/D12 | C : equ. (3) | Shared rate(Sr) equ. (4) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 5 | 14 | 3.630781 | 41.41425685 | 15.94035489 | 9.410079074 |
| 20 | 5 | 10.5 | 2.630268 | 21.73451095 | 8.365617166 | 11.95369069 |
| 15 | 5 | 8.3 | 2.14783 | 14.49271903 | 5.578250155 | 13.44507649 |
| 10 | 5 | 5.5 | 1.659587 | 8.652664659 | 3.33041218 | 10* |
| 5 | 5 | 2.3 | 1.235947 | 4.798990306 | 1.84713223 | 5* |

* C is less than Nc. Each cell exclusively occupies one carrier.

Single Carrier

| R | Nc | CCI | D2/D1 | Area of Cluster D22/D12 | C : equ. (3) | Shared rate(Sr) equ. (4) |
| --- | --- | --- | --- | --- | --- | --- |
| 50 | 4 | 20 | 6.309573 | 125.0690562 | 48.13910218 | 4.154626716 |
| 42 | 4 | 17 | 4.786301 | 71.96972987 | 27.70116194 | 6.06472755 |
| 25 | 4 | 15 | 3.981072 | 49.79088809 | 19.16452176 | 5.217975238 |
| 21 | 4 | 13 | 3.311311 | 34.44687845 | 13.2586097 | 6.335505902 |

PPM

| R | Nc | CCI | D2/D1 | Area of Cluster D22/D12 | C : equ. (3) | Shared rate(Sr) equ. (4) |
| --- | --- | --- | --- | --- | --- | --- |
| 23.88 | 3 | Not Reported |  |  |  |  |
| 20 | 3 | Not Reported |  |  |  |  |
| 10 | 3 | Not Reported |  |  |  |  |

1-5. Considering MAC procedure

Since MAC sub-layer controls packet sending, aSlotTime, SIFS and some other overhead should be taken into account to predict the actual throughput. When a data stream is divided into fragments and sent on a shared medium, the following sequence is proceeded:

waiting DIFS+backoff - > sending PHY/MPDU - > waiting SIFS - > sending PHY/ACK

The actual throughput (S) is the proportion of a data length in time domain to the packet transmission sequence length in time domain. This is given by

(5)

Assuming an MPDU of 1500 bytes in a fragment of a data stream, predicted throughputs are found in the following Tables. Actual throughput may be less than these results because packet errors are not taken into consideration and CCIs of 64 bytes packet is used instead of 1500 bytes.

OFDM

| R | Data length (1500B) | training | MPDUheader | SIFS | ACK | slottime | backoff | DIFS | Throughput (S) : equ (5) | Actual data rate : S*shared rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 400 | 20.6 | 31.533333 | 13 | 30.2 | 6 | 45 | 25 | 0.734304 | 6.9098610 |
| 20 | 600 | 20.6 | 37 | 13 | 30.2 | 6 | 45 | 25 | 0.799786 | 9.5604031 |
| 15 | 800 | 20.6 | 42.466666 | 13 | 35 | 6 | 45 | 25 | 0.832928 | 11.198786 |
| 10 | 1200 | 20.6 | 53.4 | 13 | 39.8 | 6 | 45 | 25 | 0.871966 | 8.7196628 |
| 5 | 2400 | 20.6 | 86.2 | 13 | 54.2 | 6 | 45 | 25 | 0.914843 | 4.5742166 |
|  | s | s | 34+7 Bytes | s | 21 Bytes+hdr | s | s | s |  | Mbit/s |

Single Carrier

| R | Data length (1500B) | training | MPDUheader | SIFS | ACK | slottime | backoff | DIFS | Throughput (S) : equ (5) | Actual data rate : S*shared rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 240 | 16 | 21.44 | 13.4 | 18.24 | 7.4 | 55.5 | 28.2 | 0.636976 | 2.6463995 |
| 42 | 285.714285 | 16 | 22.476190 | 13.4 | 18.666666 | 7.4 | 55.5 | 28.2 | 0.673922 | 4.0871567 |
| 25 | 480 | 16 | 26.88 | 13.4 | 20.48 | 7.4 | 55.5 | 28.2 | 0.768664 | 4.0108703 |
| 21 | 571.428571 | 16 | 28.952380 | 13.4 | 21.333333 | 7.4 | 55.5 | 28.2 | 0.79496 | 5.0364734 |
|  | s | s | 34 Bytes | s | 14 Bytes+hdr | s | s | s |  | Mbit/s |

PPM

| R | Data length (1500B) | training | MPDUheader | SIFS | ACK | slottime | backoff | DIFS | Throughput (S) : equ (5) | Actual data rate : S*shared rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23.88 | 502.512562 | 20 | 31.390284 | 5 | 24.690117 | 3 | 22.5 | 11 | 0.841598 |  |
| 20 | 600 | 20 | 33.6 | 5 | 25.6 | 3 | 22.5 | 11 | 0.859968 |  |
| 10 | 1200 | 20 | 47.2 | 5 | 31.2 | 3 | 22.5 | 11 | 0.911230 |  |
|  | s | s | 34 Bytes | s | 14 Bytes+hdr | s | s | s |  | Mbit/s |

2. Cell Radius

Coverage of an access point is dominated by its propagation loss, sensitivities, antenna gain, transmission power shadowing and so on. For a comparison, the same the propagation loss and antenna gain are assumed. The cell radius of each proposal for all data rates are shown in the Table below.

OFDM

| Data Rate | 30 Mbit/s | 20 Mbit/s | 15 Mbit/s | 10 Mbit/s | 5 Mbit/s |
| --- | --- | --- | --- | --- | --- |
| TxPower [mW] | 30 | 30 | 30 | 30 | 30 |
| TxPower [dBm] | 14.7712125 | 14.7712125 | 14.77121255 | 14.7712125 | 14.77121255 |
| Gain of ANT [dB] | 12 | 12 | 12 | 12 | 12 |
| Sensitivity [dBm] | -77 | -81 | -83.5 | -86.3 | -89.6 |
| Tolerable Propagation Loss [dB] | 103.771212 | 107.771212 | 110.2712125 | 113.071212 | 116.3712125 |
| Radius (free space:L=47+20log(d)) [m] | 715.591471 | 1134.136051 | 1512.394732 | 2087.68588 | 3052.566655 |
| Radius (L=47+25log(d)) [m] | 186.572874 | 269.679852 | 339.5068193 | 439.388313 | 595.4543906 |
| Fade Margin at 100 m (330 ft.) [dB] | 6.77121254 | 10.7712125 | 13.27121255 | 16.0712125 | 19.37121255 |

Single Carrier

| Data Rate | 50 Mbit/s | 41.9355 Mbit/s | 25 Mbit/s | 20.9677 Mbit/s |
| --- | --- | --- | --- | --- |
| TxPower [mW] | 50 | 50 | 50 | 50 |
| TxPower [dBm] | 16.98970004 | 16.98970004 | 16.98970004 | 16.98970004 |
| Gain of ANT [dB] | 12 | 12 | 12 | 12 |
| Sensitivity [dBm] | -65 | -67 | -75 | -77 |
| Tolerable Propagation Loss [dB] | 93.98970004 | 95.98970004 | 103.9897 | 105.9897 |
| Radius (free space:L=47+20log(d)) [m] | 232.0542521 | 292.1389948 | 733.8199773 | 923.824617 |
| Radius (L=47+25log(d)) [m] | 75.78582833 | 91.11460604 | 190.3653939 | 228.8695426 |
| Fade Margin at 100 m (330 ft.) [dB] | -3.010299957 | -1.010299957 | 6.989700043 | 8.989700043 |

PPM

| Data Rate | 23.88 Mbit/s | 20 Mbit/s | 10 Mbit/s |
| --- | --- | --- | --- |
| TxPower [mW] | 50 | 50 | 50 |
| TxPower [dBm] | 16.98970004 | 16.98970004 | 16.98970004 |
| Gain of ANT [dB] | 12 | 12 | 12 |
| Sensitivity [dBm] | -70 | -72 | -75 |
| Tolerable Propagation Loss [dB] | 98.98970004 | 100.9897 | 103.9897 |
| Radius (free space:L=47+20log(d)) [m] | 412.6572985 | 519.5047594 | 733.8199773 |
| Radius (L=47+25log(d)) [m] | 120.1124434 | 144.4069189 | 190.3653939 |
| Fade Margin at 100 m (330 ft.) [dB] | 1.989700043 | 3.989700043 | 6.989700043 |

Low transmission power and sufficient fade margin contribute for reducing power consumption.