| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Security (3.10) |  |
| Date Submitted | 2011-03-16 |  |
| Source(s) | Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu, Yen Tamkang University (TKU) Hsien-Wei Tseng De Lin Institute of Technology (DLIT) Youn-Tai Lee, Chun-Yen Hsu Institute for Information Industry (III) Shiann-Tsong Sheu National Central University (NCU) Whai-En Chen National Ilan University (NIU) Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada National Institute of Information and Communications Technology (NICT) | yihjan@yahoo.com hwtseng@mail.dlit.edu.tw lyt@nmi.iii.org.tw stsheu@ce.ncu.edu.tw wechen@niu.edu.tw mingtuo@nict.com.sg |
| Re: | Call for contributions for 802.16n AWD |  |
| Abstract | In this contribution we make contribution on Security (3.10) for IEEE 802.16n draft text |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Security Procedures for HR-Netwrok

Yih-Guang Jan, Yang-Han Lee, Jheng-Yao Lin, Chih-Yuan Lo, Liang-Yu Yen

Tamkang University (TKU)

Hsien-Wei Tseng

De Lin Institute of Technology (DLIT)

Youn-Tai Lee, Chun-Yen Hsu

Institute for Information Industry (III)

Shiann-Tsong Sheu

- National Central University (NCU)

- Whai-En Chen

- National Ilan University (NIU)

Ming-Tuo Zhou, Liru Lu (Alina), Xin Zhang, Hoang Vinh Dien, Masayuki Oodo, Hiroshi Harada

- National Institute of Information and Communications Technology (NICT)

- Introduction

This contribution provides information of Security Procedures for HR-Network.

- Proposed IEEE 802.16n AWD Text Modification

The text in BLACK color: the existing text in AWD

The text in BLUE color: the new text added to the Definitions

===================Start of Proposed Text====================

## 3.10 Security

The HR-Network topology shall not degrade the security performance achieved with WirelessMAN-OFDMA or WirelessMAN-Advanced Air Interface in hierarchical network topology.

## 3.10.1 Security procedures for HR-Network

The security architecture of HR-Network consists of the following functional entities: the HR-stations, the HR-MS and the AAA-server (security server).

HR-Network shall support secure communication and session establishment among HR-stations, and between HR-stations and external AAA-servers.

## 3.10.1.1 Network aided mutual authentication of HR-MS and data security

HR-MSs shall be able to establish a security association with each other. A security server may be used to facilitate the establishment of security associations.

## 3.10.1.2 Autonomous (limited) mutual authentication of HR-MS and data security for direct communication

HR-MS shall be able to mutually authenticate themselves without access to a security server.

HR-MS shall be able to establish encrypted communication without access to a security server.

## 3.10.1.3 Security requirements for HR-Network nodes acting as relays

HR-station that functions as a relay shall forward security related messages between other HR-station and a security server, both during security association establishment and ongoing communications.

## 3.10.2 Multicast key Management

HR-Network shall provide the security architecture that provides a group of HR-MSs with authentication, authorization, encryption and integrity protection.

HR-Network shall provide multicast key management for the group of HR-MSs. The key shared within the group should be distributed securely and efficiently. HR-Network should support the group signaling procedure using multicast transmission for multicast key management efficiently.

## 3.10.3 Security Association

A security association (SA) is the set of information required for secure communication between HR-MSs, HR-stations. The security association is applied to the respective flows once an SA is established.

HR-Network supports unicast static SA only.

SA is used to provide keying material for unicast/multicast transport connections and is applied to all data exchanged within the connection.

SA is used to provide keying material for unicast/multicast management connections.

====================End of Proposed Text====================

# References

- IEEE 802.16n-10/0048, “802.16n System Requirements Document including SARM annex”, January 2011.

- IEEE 802.16n-10/0049, “802.16n Table of Contents for Amendment Working Draft”, January 2011.