| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HR-MS Discovers Network Infrastructure with Help from Forwarding HR-MS |  |
| Date Submitted | 2011-07-12 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Zander Zhongding Lei, Wai Leong Yeow, Shoukang Zheng, Choong Hock Mar Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 Eldad Zeira InterDigital | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg Voice: E-mail: eldad.zeira@interdigital.com |
| Re: | In response to the 802.16n (GRIDMAN) AWD Call for Comments |  |
| Abstract | Text proposal to Sections 17.3.7.1.2 of the AWD. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HR-MS Discovers Network Infrastructure with Help from Forwarding HR-MS

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Zander Zhongding Lei, Wai Leong Yeow,

Shoukang Zheng

Institute for Infocomm Research (I2R)

Eldad Zeira

InterDigital

Introduction

This contribution provides the detailed text for Section 17.3.7.1.2, which describe how a forwarding HR-MS can help an out-of-coverage HR-MS to connect to network architecture.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0009r1.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK color: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

Modify section 17.3.7.1.2 as indicated

17.3.7.1.2 HR-MS Discover Network Infrastructure

To enable coverage extension, a serving HR-BS/HR-RS shall schedule some of its directly-associated registered HR-MSs to transmit PA/SA-Ppreamble signals so that an outside-of-coverage HR-MS can detect and start network entry. The directly-associated registered HR-MS shall transmit PA-Ppreamble at the first OFDM symbol beginning of the 2nd frame and SA-Ppreamble at the first OFDM symbol beginning of the 3rd frame (of a super-frame).

Any new non-registered HR-MS scanning for DL preambles for possible network entry shall be able to differentiate between preambles transmitted by normal infrastructure stations (HR-BS/HR-RS) and those transmitted by a coverage-extending HR-MS. For that, the last [TBD] SA-Preamble sequences in each segment are reserved for transmission by coverage-extending HR-MS.

For use case 2, The HR-BS may instruct HR-MS that are associated with it to transmit network configuration information (NCI) access information at pre-defined resources relative to the preambles transmitted by the HR-MS. The NCI, when transmitted,access information defines resources for access by the HR-MS that is not under HR-BS coverage. This corresponds to the coverage extension procedure defined in 17.3.7.1.2.1 and 17.3.7.1.2.2. Access information may be omitted. If NCIaccess information is omitted then access resources are defined by the index and the sub-carrier set index of the SA-Preamble. This corresponds to the coverage extension procedure defined in 17.3.7.1.2.3. All or a group of the directly associated HR-MS may or may not transmit the same access information on the same or different resources.

An unassociated HR-MS that detects the associated HR-MS preamble(s) shall subsequently receive access information to determine the access resource. If access information is omitted then access resources are determined from the SA-Preamble. The unassociated HR-MS transmits a CDMA preamble.

The associated HR-MS that received the CDMA preamble responds with sufficient access information to complete the association procedure.

17.3.7.1.2.1 Coverage Extension with NCI Preceding Initial Ranging – Procedure 1

The procedure starts when serving HR-BS (or serving HR-RS) transmits an HR-CEX-CMD message to instruct one or a group of its directly-associated HR-MS to carry out a coverage-extending process.

Upon receiving the HR-CEX-CMD message, the scheduled HR-MS (or group of HR-MS), from now on simply referred to as forwarding HR-MS, shall start transmitting PA/SA-Preambles, followed by NCI.

When a new HR-MS detects PA/SA-Preamble sequences and NCI transmitted by the forwarding HR-MS and decides to start initial ranging through the forwarding HR-MS, the process can be described as follows:

- The new HR-MS, after acquiring downlink synchronization and uplink transmission parameters shall pick one of the ranging codes (specified in NCI) using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel.

- Upon receiving the ranging code from the new HR-MS, the forwarding HR-MS is supposed to respond with an HR-CEX-RNG-ACK message. If the new HR-MS does not receive an HR-CEX-RNG-ACK message from a forwarding HR-MS, it may ramp-up the transmit power for the next ranging attempt, if necessary.

- There are three possible ranging status responses from the forwarding HR-MS to the new HR-MS in the HR-CEX-RNG-ACK message, i.e.,

  - Continue: The forwarding HR-MS informs the new HR-MS of needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of “continue”.

  - Success: The forwarding HR-MS informs the new HR-MS of status “success”, but may have adjustment suggestions to the new HR-MS if necessary. With status success, the forwarding HR-MS shall inform the new HR-MS of a BW allocation in the uplink for the new HR-MS to send HR-CEX-RNG-REQ message.

  - Abort: The forwarding HR-MS informs the new HR-MS to abort the current initial ranging process.

- Based on the received response of ranging status, the new HR-MS perform the following:

  - Upon receiving a Continue status notification and parameter adjustments in HR-CEX-RNG-ACK message, the new HR-MS shall adjust its parameters accordingly and continue the ranging process by randomly select one ranging code and transmit in the next available ranging opportunity, or as instructed in the HR-CEX-RNG-ACK message.

  - Upon receiving a Success status notification, the new HR-MS shall wait for the forwarding HR-MS to inform it of a BW allocation in the UL to send HR-RNG-REQ message.

  - Upon receiving an Abort status notification, the new HR-MS shall stop the ranging process.

- HR-BS assigns and transfers a TSTID by AAI-RNG-RSP message when ranging status is success. Initial ranging process is over after receiving the HR-RNG-RSP message. The TSTID is used until STID is newly assigned and received at successful registration.

After a successful initial ranging, the forwarding HR-MS shall assist HR-BS and the new HR-MS to exchange control messages to complete basic capability negotiation, HR-MS authorization and key exchange, and registration.

The behavior of the forwarding HR-MS and serving HR-BS/RS during initial ranging of the new HR-MS shall be as follows.

- After transmitting NCI, the forwarding HR-MS shall monitor the specified ranging opportunities for any initial-ranging attempt by new HR-MS. If no such ranging attempt is detected, the coverage extension process terminates.

- When the forwarding HR-MS detects one or more ranging codes transmitted on the allocated ranging opportunities, it shall calculate signal strength and necessary adjustments (time, power, frequency corrections). An HR-CEX-RNG-ACK message shall be transmitted to the new HR-MS.

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit HR-CEX-RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the A-MAP and inform the new HR-MS accordingly.

17.3.7.1.2.2 Coverage Extension with NCI Preceding Initial Ranging – Procedure 2

The procedure starts when serving HR-BS (or serving HR-RS) transmits an HR-CEX-CMD message to instruct one or a group of its directly-associated HR-MS to carry out a coverage-extending process.

Upon receiving the HR-CEX-CMD message, the scheduled HR-MS (or group of HR-MS), from now on simply referred to as forwarding HR-MS, shall start transmitting pre-access PA/SA-Preambles, followed by Initial NCI.

Both pre-access and post-access SA-Preamble and Initial/Supplementing NCI are assigned by the serving HR-BS. The HR-BS may assign same SA-Preamble ID and NCI to multiple forwarding HR-MS. The HR-BS should assign pre-access SA-Preamble and Initial NCI to groups of HR-MS based on their service characteristics. The HR-BS should assign post-access SA-Preamble and Supplementing NCI such that they will be unique within physical proximity.

When a new HR-MS detects PA/SA-Preamble sequences and NCI transmitted by the forwarding HR-MS and decides to start initial ranging through the forwarding HR-MS, the process can be described as follows:

- The new HR-MS, after acquiring downlink synchronization and uplink transmission parameters shall pick one of the ranging codes (specified in NCI) using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel.

- Upon receiving the ranging code from the new HR-MS, the forwarding HR-MS is supposed to respond with  post-access SA-Preamble and Supplementing NCI. If the new HR-MS does not receive, a post access SA-Preamble and Supplementing NCI it may ramp-up the transmit power for the next ranging attempt, if necessary.

- The forwarding HR-MS responds to ranging that has exceeded a threshold determined by the HR-BS.

- If the new HR-MS detects a post-access SA-Preamble and NCI it shall pick one of the ranging codes (specified in NCI) using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel. The initial transmission power for the ranging is determined based on the last ranging prior to detection of post-access SA-Preamble and NCI.

- There are three possible ranging status responses from the forwarding HR-MS to the new HR-MS in the HR-CEX-RNG-ACK message, i.e.,

  - Continue: The forwarding HR-MS informs the new HR-MS of needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of “continue”.

  - Success: The forwarding HR-MS informs the new HR-MS of status “success”, but may have adjustment suggestions to the new HR-MS if necessary. With status success, the forwarding HR-MS shall inform the new HR-MS of a BW allocation in the uplink for the new HR-MS to send HR-CEX-RNG-REQ message.

  - Abort: The forwarding HR-MS informs the new HR-MS to abort the current initial ranging process.

- Based on the received response of ranging status, the new HR-MS perform the following:

  - Upon receiving a Continue status notification and parameter adjustments in HR-CEX-RNG-ACK message, the new HR-MS shall adjust its parameters accordingly and continue the ranging process by randomly select one ranging code and transmit in the next available ranging opportunity, or as instructed in the HR-CEX-RNG-ACK message.

  - Upon receiving a Success status notification, the new HR-MS shall wait for the forwarding HR-MS to inform it of a BW allocation in the UL to send HR-RNG-REQ message.

  - Upon receiving an Abort status notification, the new HR-MS shall stop the ranging process.

- HR-BS assigns and transfers a TSTID by AAI-RNG-RSP message when ranging status is success. Initial ranging process is over after receiving the HR-RNG-RSP message. The TSTID is used until STID is newly assigned and received at successful registration.

After a successful initial ranging, the forwarding HR-MS shall assist HR-BS and the new HR-MS to exchange control messages to complete basic capability negotiation, HR-MS authorization and key exchange, and registration.

The behavior of the forwarding HR-MS and serving HR-BS/RS during initial ranging of the new HR-MS shall be as follows.

- After transmitting NCI, the forwarding HR-MS shall monitor the specified ranging opportunities for any initial-ranging attempt by new HR-MS until told to stop by the HR-BS.

- When the forwarding HR-MS detects one or more ranging codes transmitted on the allocated ranging opportunities, it shall calculate signal strength and necessary adjustments (time, power, frequency corrections). An HR-CEX-RNG-ACK message shall be transmitted to the new HR-MS.

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit HR-CEX-RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the A-MAP and inform the new HR-MS accordingly.

- The HR-BS may request the forwarding HR-MS to report any ranging attempt (whether they have crossed the response threshold or not).

17.3.7.1.2.3 Coverage Extension with Initial Ranging Preceding NCI – Procedure 3

The procedure starts when serving HR-BS (or serving HR-RS) transmits an HR-CEX-CMD message to instruct one or a group of its directly-associated HR-MS to carry out a coverage-extending process.

Upon receiving the HR-CEX-CMD message, the scheduled HR-MS (or group of HR-MS), from now on simply referred to as forwarding HR-MS, shall start transmitting pre-access PA/SA-Preambles.

Both pre-access and post-access SA-Preamble are assigned by the serving HR-BS. The HR-BS may assign same SA-Preamble to multiple forwarding HR-MS. The HR-BS should assign pre-access SA-Preamble to groups of HR-MS based on their service characteristics. The HR-BS should assign post-access SA-Preamble and NCI such that they will be unique within physical proximity.

When a new HR-MS detects PA/SA-Preamble sequences transmitted by the forwarding HR-MS and decides to start initial ranging through the forwarding HR-MS, the process can be described as follows:

- The new HR-MS, after acquiring downlink synchronization and uplink transmission parameters shall pick one of the ranging codes derived from the SA-Preamble ID using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel.

- Upon receiving the ranging code from the new HR-MS, the forwarding HR-MS is supposed to respond with  post-access SA-Preamble and NCI. If the new HR-MS does not receive, a post access SA-Preamble and NCI it may ramp-up the transmit power for the next ranging attempt, if necessary.

- The forwarding HR-MS responds to ranging that has exceeded a threshold determined by the HR-BS.

- If the new HR-MS detects a post-access SA-Preamble and NCI it shall pick one of the ranging codes (specified in NCI) using a uniform random process. The HR-MS shall send the selected ranging code on the first allocated ranging channel. The initial transmission power for the ranging is determined based on the last ranging prior to detection of post-access SA-Preamble and NCI.

- There are three possible ranging status responses from the forwarding HR-MS to the new HR-MS in the HR-CEX-RNG-ACK message, i.e.,

  - Continue: The forwarding HR-MS informs the new HR-MS of needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of “continue”.

  - Success: The forwarding HR-MS informs the new HR-MS of status “success”, but may have adjustment suggestions to the new HR-MS if necessary. With status success, the forwarding HR-MS shall inform the new HR-MS of a BW allocation in the uplink for the new HR-MS to send HR-CEX-RNG-REQ message.

  - Abort: The forwarding HR-MS informs the new HR-MS to abort the current initial ranging process.

- Based on the received response of ranging status, the new HR-MS perform the following:

  - Upon receiving a Continue status notification and parameter adjustments in HR-CEX-RNG-ACK message, the new HR-MS shall adjust its parameters accordingly and continue the ranging process by randomly select one ranging code and transmit in the next available ranging opportunity, or as instructed in the HR-CEX-RNG-ACK message.

  - Upon receiving a Success status notification, the new HR-MS shall wait for the forwarding HR-MS to inform it of a BW allocation in the UL to send HR-RNG-REQ message.

  - Upon receiving an Abort status notification, the new HR-MS shall stop the ranging process.

- HR-BS assigns and transfers a TSTID by AAI-RNG-RSP message when ranging status is success. Initial ranging process is over after receiving the HR-RNG-RSP message. The TSTID is used until STID is newly assigned and received at successful registration.

After a successful initial ranging, the forwarding HR-MS shall assist HR-BS and the new HR-MS to exchange control messages to complete basic capability negotiation, HR-MS authorization and key exchange, and registration.

The behavior of the forwarding HR-MS and serving HR-BS/RS during initial ranging of the new HR-MS shall be as follows.

- After transmitting NCI, the forwarding HR-MS shall monitor the specified ranging opportunities for any initial-ranging attempt by new HR-MS until told to stop by the HR-BS.

- When the forwarding HR-MS detects one or more ranging codes transmitted on the allocated ranging opportunities, it shall calculate signal strength and necessary adjustments (time, power, frequency corrections). An HR-CEX-RNG-ACK message shall be transmitted to the new HR-MS.

- After the ranging status has become Success, HR-BS shall allocate bandwidth in the uplink to allow the new HR-MS to transmit HR-CEX-RNG-REQ message. The forwarding HR-MS shall recognize this allocation IE in the A-MAP and inform the new HR-MS accordingly.

- The HR-BS may request the forwarding HR-MS to report any ranging attempt (whether they have crossed the response threshold or not).

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.