| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text and Control Messages for MAC-layer Support for BS-controlled HR-MS Forwarding to Network (Rev3-based) |  |
| Date Submitted | 2011-10-31 |  |
| Source(s) | Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow, Shoukang Zheng, Joseph Teo Chee Ming Institute for Infocomm Research 1 Fusionopolis Way, #21-01, Connexis (South Tower) Singapore 138632 Eldad Zeira InterDigital | Voice: +65 6408-2271 E-mail: athoang@i2r.a-star.edu.sg |
| Re: | In response to the 802.16n (GRIDMAN) Call for Comments to AWD in 802.16n-11/0024 |  |
| Abstract | This contribution proposed modifications for text and control messages for MAC-layer support for BS-controlled HR-MS forwarding to network. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16n draft Text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text and Control Messages for MAC-layer Support for BS-controlled HR-MS Forwarding to Network (Rev3-based)

Anh Tuan Hoang, Haiguang Wang, Jaya Shankar, Wai Leong Yeow,

Shoukang Zheng, Joseph Teo Chee Ming

Institute for Infocomm Research (I2R)

Eldad Zeira (InterDigital)

Introduction

This contribution proposed modifications for text and control messages for MAC-layer support for BS-controlled HR-MS forwarding to network.

# References

- IEEE 802.16n Amendment Working Draft, IEEE 802.16n-11/0024.

Text Proposal for the 802.16n Amendment Working Document (AWD)

Note:

The text in BLACK or BLUE colors: the existing text in the 802.16n Amendment Draft Standard

The text in RED color: the removal of existing 802.16n Amendment Draft Standard Text

The text in GREEN color: the new text added to the 802.16n Amendment Draft Standard Text

[-----------------------------------------------Start of Text Proposal---------------------------------------------------]

{Remedy 1: Modify Section 6.3.2.3.98.13 as indicated:}

6.3.2.3.98.13 NCI Message

The NCI message is transmitted by the HR-MS so that HR-MS outside the coverage can access the network with the FTN scheme provided in this section.

Table 1 ---- NCI message field and description

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| NCI_Message_Format() { | — | — |
| Management Message Type = [TBD] | 8 | — |
| PHY Synchronization Field | 32 8 | Specified in Table 319, 8 bits for frame duration and 24 bits for frame number. |
| Base StationOperator ID | 48 8 | 8 LSBs of the Operator ID |
| Allocation Start Time | 8 32 | In number of OFDMA symbols, should include the RTG/TTG gap in the calculation. |
| Subchannel offset for ranging | 7 |  |
| No. OFDMA Symbols for ranging | 7 |  |
| No. Subchannels for ranging | 7 |  |
| Ranging Method | 2 | 0b00: Initial ranging over two symbols 0b01: Initial ranging over four symbols 0b10: BR/periodic ranging over one symbol 0b11: BR/periodic ranging over three symbols |

{Remedy 2: Modify Section 16.3.2 as indicated:}

16.3.2 BS-Controlled HR-MS Forwarding to Network

The general operation of BS-controlled HR-MS forwarding to network (FTN) can be described as follows:

- At the beginning of the process, the serving HR-BS/RS instructs one or a group of its associated HR-MS to broadcast discoverable signals so that an out-of-coverage HR-MS can start network entry and get associated to the HR-BS/RS.

- After the out-of-coverage HR-MS has completed network entry and become an attached forwarded HR-MS, the forwarding HR-MS helps forward control and data messages between the forwarded HR-MS and the serving HR-BS/RS.

16.3.2.1 Medium access control

16.3.2.1.1 HR-MS Discover Network Infrastructure

For use case 2, The HR-BS may instruct HR-MS that are associated with it to transmit access information at pre-defined resources relative to the preambles transmitted by the HR-MS. The access information defines resources for access by the HR-MS that is not under HR-BS coverage. Access information may be omitted. If access information is omitted then access resources are defined by the index and the sub-carrier set index of the SA-Preamble. All or a group of the directly associated HR-MS may or may not transmit the same access information on the same or different resources.

An unassociated HR-MS that detects the associated HR-MS preamble(s) shall subsequently receive access information to determine the access resource. If access information is omitted then access resources are determined from the SA-Preamble. The unassociated HR-MS transmits a CDMA preamble.

The associated HR-MS that received the CDMA preamble responds with sufficient access information to complete the association procedure.

16.3.2.1.1 Network entry involving forwarding HR-MS

For an HR-MS that is inside-of-coverage of an HR-BS/RS, the network entry process is as described in 6.3.9.

For an HR-MS that is out-of-coverage of any HR-BS/RS but within reach of a forwarding HR-MS, the process of network entry can be described as follows.

16.3.2.1.2 Discovery for HR-MS Forwarding to Network

To enable coverage extension, a serving HR-BS/HR-RS shall schedule some of its directly-associated HR-MSs to transmit preambles so that an outside-of-coverage HR-MSs who are out of coverage of HR-BS/HR-RS can detect and start network entry. The directly-associated HR-MS shall transmit frame preamble at the first OFDM symbol and followed by the network configuration information (NCI).

The HR-BS may instruct HR-MS that are associated with it to transmit MS-Frame Control Header (M-FCH) and network configuration information (NCI) at pre-defined resources relative to the preamble transmitted by the HR-MS. The M-FCH and NCI, when transmitted, define resources for access by the HR-MS that is not under HR-BS coverage. This corresponds to the coverage extension procedure defined in 16.3.2.1.1.1. If M-FCH and NCI are omitted then access resources are defined by the index and the sub-carrier set index of the transmitted preamble. This corresponds to the coverage extension procedure defined in 16.3.2.1.1.2.

[-----------------------------------------------End of Text Proposal---------------------------------------------------]

# Use of IEEE-Copyrighted Conference and Journal Papers

The IEEE 802.16 Working Group has obtained permission to post IEEE-copyrighted material from IEEE conferences and publications when received in an official contribution to the Working Group. IEEE requires two conditions:

- A full copyright and credit notice must be posted at the top of the paper in the following format:

Copyright ©2002 Institute of Electrical and Electronics Engineers, Inc.  Reprinted, with permission, from [all relevant journal info].

This material is posted here with permission of the IEEE. Internal or personal use of this material is permitted. However, permission to reprint/republish this material for advertising or promotional purposes or for creating new collective works for resale or redistribution must be obtained from the IEEE (contact pubs–permissions@ieee.org).

By choosing to view this document, you agree to all provisions of the copyright laws protecting it.

- If the author is not the submitter of the contribution, the author’s written approval for the reuse of the material must be attached.