| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Revision 1 to Document 5D/TEMP/283 |  |
|  | 12 November 2009 |
|  | English only |
| Working Party 5D |  |
| LIAISON STATEMENT TO external organisations |  |
| IMT PARAMETERS FOR THE BAND 790-862 MHz FOR USE BY JTG 5-6 |  |

Working Party 5D would like to inform external organisations that it has developed an update to the liaison statement sent from the previous meeting (Document 5-6/93).

Working Party 5D at its 6th meeting (14-21 October 2009), has reviewed the IMT parameters to be used in sharing studies in the band 790-862 MHz and agreed that an update to the information would be beneficial.

This updated version of the contribution to JTG 5-6 is attached for easy reference with the new elements highlighted in yellow.

Status:	For information

Contact:	Mr. Colin Langtry			E-mail: colin.langtry@itu.int

| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Document 5D/TEMP/278(Rev.1) | Document 5-6/106-E Document 5B/332-E |
|  | 6 November 2009 |
|  | English only |
| Working Party 5D |  |
| liaison statement to joint task group 5-6 (copy to wp 5B for information) |  |
| imt parameters for the band 790-862 mhz |  |

Following the liaison statement that Working Party 5D sent to the Joint Task Group 5-6 in June 2009 (see Document 5-6/93), Working Party 5D reviewed the document once again during its October 2009 meeting and identified a need to provide an updated version of the tables in Attachment 2. The changes in addition to those already presented in Document 5-6/93 are highlighted in yellow with revision marks and concern the density of user equipments figures. In that respect, this liaison statement replaces and supersedes Document 5-6/93.

Working Party 5D hopes that this proposed update will help the Joint Task Group 5-6 to progress the sharing studies for which it is responsible.

Status:			For action

- Contact: 		J-P Millet				E-mail:	  milletjp@anfr.fr

Attachment 1 (as supplied by JTG 5-6)

IMT parameters for the band 790-862 MHz provided by external organizations

PART 1: Parameters for UMTS (WCDMA), LTE (OFDMA) and WiMAX (OFDMA)

TABLE 1

Parameters for UMTS (WCDMA), LTE (OFDMA) and WiMAX (OFDMA)

| No. | Parameter | Base station | Mobile station | Base station | Mobile station |  | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  | UMTS (WCDMA) |  | LTE (OFDM/SCFDMA) |  |  | Mobile WiMAX (OFDMA) |  |
| 1. | Class of emission | 5M00V7WEC | 5M00V7WEC | For 1.4 MHz, 1M40V7WEW For 3 MHz, 3M00V7WEW For 5 MHz, 5M00V7WEW For 10 MHz, 10M0V7WEW (For 15 MHz, 15M0V7WEW For 20 MHz, 20M0V7WEW |  | For 1.4 MHz, 1M40V7WEW For 3 MHz, 3M00V7WEW For 5 MHz, 5M00V7WEW For 10 MHz, 10M0V7WEW (For 15 MHz, 15M0V7WEW For 20 MHz, 20M0V7WEW | 5 MHz, OFDMA 7 MHz, OFDMA 10 MHz, OFDMA |  |
| 2. | Modulation parameters | QPSK 16-QAM 64-QAM | QPSK ¦16-QAM | QPSK 16-QAM 64-QAM |  | QPSK 16-QAM 64-QAM | QPSK16-QAM 64-QAM | QPSK 16-QAM 64-QAM (optional) |
| 3. | Duplex mode | FDD |  | FDD(/TDD) |  |  | FDD/TDD |  |
| 4. | Spectral mask of signals, including | - | - | - |  | - | The final WiMAX Forum emission masks for equipment in these frequency ranges remain Under development. An example of regulatory unwanted emission requirements can be found in the FCC regulations Sections 27.53 and 90.543 for the 700 MHz band (698-806 MHz). These provide an example of the target levels that will be taken into consideration during development of the final WiMAX Forum emission masks. |  |
| 4.1 | −3 dB radiation bandwidth | - | - | - |  | - |  |  |
| 4.2 | −30 dB radiation bandwidth | - | - | - |  | - |  |  |
| 4.3 | −60 dB radiation bandwidth | - | - | - |  | - |  |  |

| No. | Parameter | Base station | Mobile station |  | Base station |  | Mobile station |  |  | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  | UMTS (WCDMA) |  |  | LTE (OFDM/SCFDMA) |  |  |  |  | Mobile WiMAX (OFDMA) |  |
| 5. | Maximum spectral power density, dB(mW/Hz) | −22.8 | −41.8 |  | −23.5 |  |  | −43.5 |  | 5 MHz: −23.6 7 MHz: −25.1 10 MHz: −26.6 See Note | 5 MHz: −43.5 7 MHz: –45.2 10 MHz: –46.6 See Note 14 |
| 6. | Signal bandwidth (MHz) | 3.84 | 3.84 |  | 1.08, 2.7, 4.5, 9, 13.5 and 18 |  |  | 1.08, 2.7, 4.5, 9, 13.5 and 18 |  | 4.60, 6.57, 9.20 MHz Note | 4.47, 6.57 and 9.20 MHz Note |
| 7. | Transmitter e.i.r.p. (dBm) |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Maximum transmitter e.i.r.p. (dBm) | 55 | 21 |  | 55Error: Reference source not found |  |  | 23Error: Reference source not found |  | 55 | 23 |
| 7.2 | Average transmitter e.i.r.p. (dBm) | Deployment dependant | 1 (rural) −10 (urban) |  | Deployment dependant |  |  | 3 (rural) −8 (urban) |  | Deployment dependant | Not communicated |
| 8. | Typical height of the transmitting antenna (m) | 20 to 30 | 1.5 |  | 20 to 30 |  |  | 1.5 |  | 15 to 32 | 1.5 |
| 9. | Transmitting antenna type (sectorized/omnidirectional) | 3 sectors | omni |  | 3 sectors |  |  | omni |  | sectorized | omnidirectional |
| 10. | Transmitting antenna gain, dBi | 15 | 0 |  | 15 |  |  | 0 |  | 15 | 0 |
| 11. | Feeder loss (dB) | 3 | 0 |  | 3 |  |  | 0 |  | 3 | 0 |
| 12. | Antenna pattern width, degrees |  |  |  |  |  |  |  |  |  |  |
| 12.1 | - in the horizontal plane (at 3 dB) | 65° | NA |  | 65° |  |  | NA |  | 65° | Omni |
| 12.2 | - in the vertical plane | ITU-R F.1336-2 | NA |  | ITU-R F.1336-2 |  |  | NA |  | ITU-R F.1336-2 | NA |
| 12.3 | - antenna downtilt | 3° | NA |  | 3° |  |  | NA |  | 3° | NA |
| 13. | Relative level of side lobes | Not standardized | NA |  | −20 dB |  |  | NA |  | −20 dB See Note | NA |
| 14. | Channel bandwidth (MHz) | 5 | 5 |  | 1.4, 3, 5, 10, 15, 20 |  |  | 1.4, 3, 5, 10, 15, 20 |  | 5, 7, 10 | 5, 7, 10 |
| 15. | Power control range (dB) | >18 | 75 |  | - |  |  | 63 |  | >10 | >45 |
| 16. | density of the equipments (number per km2) | Rural: 4.5 users equipment operating at the same time (average number of simultaneously active users) Urban : 4.5 |  |  |  |  |  |  |  | Deployment parameter | Deployment parameter |
| 17. | density of the equipment (number per km2) operating at co-frequencyError: Reference source not found | Rural: 4.5 users equipment operating at the same time (average number of simultaneously active users) Urban : 4.5Error: Reference source not found |  |  |  |  |  |  |  | Deployment parameter | Deployment parameter |
| 18. | Polarization discrimination (dB) | 3 |  | 0 |  | 3Error: Reference source not found |  |  | 0 | Deployment parameter | NA |
| 19. | Capacity criteria, including capacity per cell | Capacity in a cellular system can be measured in terms of simultaneous voice users per cell, data throughput per cell, etc. The actual capacity is dependent on the assumptions made about system configuration, loading, quality, and fairness, among other things. |  |  |  |  |  |  |  | Deployment parameter | Deployment parameter |
| 20. | Frequency reuse factor | 1 | 1 |  | 1 |  |  | 1 |  | Deployment parameter | Deployment parameter |
| 21. | Receiver thermal noise | NF = 5 dB | NF = 9 dB |  | NF = 5 dB |  |  | NF = 9 dB |  | –109 dBm/MHz | –106 dBm/MHz |
| 22. | Reference sensitivity |  |  |  |  |  |  |  |  | 5 MHz QPSK 1/2: –91.6 dBm 7 MHz QPSK 1/2: –89.9 dBm 10 MHz QPSK 1/2: –88.5 dBm | 5 MHz QPSK 1/2: −91.5 dBm 7 MHz QPSK 1/2: −89.9 dBm 10 MHz QPSK 1/2: −88.5 dBm |
| 23. | Receiver blocking response |  |  |  |  |  |  |  |  |  |  |
| 24 | Coverage radius | Rural: 3.46 km Urban: 2.698 km |  |  | Rural: 3.46 km Urban: 2.698 km |  |  |  |  |  |  |

NOTE 1 – This table contains only parameters relevant to IMT emissions (i.e. when the IMT system is the source of potential interference). Further parameters (e.g. receiver performance) are relevant for the case where the IMT system is the victim of interference.

NOTE 2 – The density of the mobile terminals (number per km2), operating at the same frequency at the same time and in the same geographical area, as well as the average transmit power of terminals are also important parameters when studying potential interference from IMT mobile terminals particularly in the case of stochastic analysis and should therefore be considered in the coexistence studies.

TABLE 2

Complementary information about WiMAX (OFDMA)

| Frequency range MS (MHz) | Frequency range BS (MHz) | Profile name | Channel freq. step (kHz) | Channel bandwidth (MHz) | FFT size | Duplex mode | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 698-862 | 698-862 | 7E | 250 | 5 | 512 | TDD | The bandwidths are applicable to both the MS and BS. MS is dual mode TDD/H-FDD. |
|  |  |  |  | 7 | 1024 |  |  |
|  |  |  |  | 10 | 1024 |  |  |
|  |  |  |  | 2 × 5 | 512 | FDD |  |
|  |  |  |  | 2 × 7 | 1024 |  |  |
|  |  |  |  | 2 × 10 | 1024 |  |  |
| 698-862 | 698-862 | 7A | 250 | 5 | 512 | TDD | The bandwidths are applicable to both the MS and BS. |
|  |  |  |  | 7 | 1024 |  |  |
|  |  |  |  | 10 | 1024 |  |  |

PART 2: Parameters for cdma2000, High Rate Packet Data (HRPD), and Ultra Mobile Broadband (UMB)

The values for requested system parameters are presented in Table 3. Please note that some parameters relating to UMB in this section (Tables 3 and 4) are near completion in 3GPP2 and will be provided in future. Table 4 shows the mobile station and access terminal maximum transmit power as currently specified in the 3GPP2 specifications. Additional information relating to the other band classes in Table 5 will be provided in the future. It is anticipated that the maximum power and emission specifications for Band Classes 18 and 19 will be identical to those for Band Classes 7 and 10.

TABLE 3

cdma2000, High Rate Packet Data (HRPD), and Ultra Mobile Broadband (UMB)

| No. | Parameter |  | Base station | Mobile station | Base station | Mobile station | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  |  | cdma2000 |  | HRPD |  | UMB |  |
| 1. | Class of emission |  |  |  |  |  |  |  |
| 2. | Modulation parameters | Forward link or downlink | Data modulation: BPSK; QPSK, 8-PSK, 16-QAM Spreading modulation: QPSK |  | Data modulation: QPSK, 8-PSK, 16-QAM, and 64-QAM Spreading modulation: QPSK |  | Data modulation: QPSK, 8-PSK, 16-QAM, and 64-QAM |  |
|  |  | Reverse link or uplink | Data modulation: 64-ary Orthogonal Modulation, BPSK; QPSK, 8-PSK Spreading modulation: HPSK |  | Data modulation: BPSK; QPSK, 8-PSK Spreading modulation: HPSK |  | Data modulation: QPSK, 8-PSK, 16-QAM, and 64-QAM Spreading modulation: QPSK (CDMA control segment) |  |
| 3. | Duplex mode |  | FDD |  | FDD |  | FDD/TDD |  |

| No. | Parameter | Base station | Mobile station | Base station | Mobile station | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  | cdma2000 |  | HRPD |  | UMB |  |
| 4. | Spectral mask of signals, including | See the Recommenda-tions ITU-R M.1580-2 and ITU-R M.1581-2 for cdma-MC” | See the Recommenda-tions ITU-R M.1580-2 and ITU-R M.1581-2 for cdma-MC | See the draft revision of Recommenda-tions ITU-R M.1580-2 and ITU-R M.1581-2 for cdma-MC (in Doc. 5D/327) | See the draft revision of Recommenda-tions ITU-R M.1580-2 and ITU-R M.1581-2 for cdma-MC (in Doc. 5D/327) | To be provided | To be provided |
| 4.1 | −3 dB radiation bandwidth |  |  |  |  |  |  |
| 4.2 | −30 dB radiation bandwidth |  |  |  |  |  |  |
| 4.3 | −60 dB radiation bandwidth |  |  |  |  |  |  |
| 5. | Maximum spectral power density, dB(mW/Hz) | −17.9 | −37.9 | −17.9Error: Reference source not found | −37.9Error: Reference source not found | TBD | TBD |
| 6. | Signal bandwidth (MHz) | 1.2288 per carrier. One or three carriers can be used together, with carriers separated by 1.23 MHz for Band Class 0 and 1.25 MHz in other cases | 1.2288 (1x) and 3.6864 (3x) | 1.2288 per carrier. One to fifteen carriers can be used together, with carriers separated by 1.23 MHz for Band Class 0 and 1.25 MHz in other cases. | 1.2288 per carrier. One to fifteen carriers can be used together, separated by 1.23 MHz for Band Class 0 and 1.25 MHz in other cases. | 0.768-19.6608 with step size of 0.1536 | 0.768-19.6608 with step size of 0.1536 |

| No. | Parameter | Base station | Mobile station | Base station | Mobile station | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  | cdma2000 |  | HRPD |  | UMB |  |
| 7. | Maximum Transmitter e.i.r.p. (dBm) | Per Licence typically 58 per carrier at 2 GHz, assuming 43 dBm power amplifier. | See Table 4 | Per Licence typically 58 per carrier at 2 GHz, assuming 43 dBm power amplifier. | See Table 4 | To be provided | To be provided |
| 8. | Typical height of the transmitting antenna (m) | 32 | 1.5 | 32 | 1.5 | 32 | 1.5 |
| 9. | Transmitting antenna type (sectorized/omnidirectional) | 3 sectors | Omni | 3 sectors | Omni | 3 sectors | Omni |
| 10. | Transmitting antenna gain, dBi | 17 | −1 | 17 | −1 | 17 | −1 |
| 11. | Feeder loss (dB) | 2 | 2 | 2 | 2 | 2 | 2 |
| 12. | Antenna pattern width, degrees |  |  |  |  |  |  |
| 12.1 | - in the horizontal plane (at 3 dB) | 70 | NA | 70 | NA | 70 | NA |
| 12.2 | - in the vertical plane | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified |
| 12.3 | - antenna downtilt | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified |

| No/No | Parameter | Base station | Mobile station | Base station | Mobile station | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  | cdma2000 |  | HRPD |  | UMB |  |
| 13. | Relative level of side lobes | Not specified | Not specified | Not specified | Not specified | Not specified | Not specified |
| 14. | Channel bandwidth (MHz) Channel bandwidth(s), some systems being capable of multiple bandwidths | It has a chip rate of N= 1.2288 Mchip/s (currently, N = 1 and 3 are specified) Carrier spacing is 1.23 MHz for band Class 0; 1.25 MHz for all other Band Classes. |  | M carriers of 1.2288 Mchip/s can be aggregated in a single or multi-carrier operation, where M  1, 2, 3, 4, …, 15. Carriers in a multi- carrier operation do not have to be contiguous and their spacing is multiple of 30, 50, 25, 20, or 12.5 kHz depending on the Band Class. |  | Native bandwidth can fit deployment of 1.25 to 20 MHz (0.768 + N  0.1536 MHz, N  0, …, 123). Also, multi-carrier configurations can be used to aggregate two or more carriers of possibly different native bandwidths and operating in non-contiguous spectral allocation. |  |
| 15. | Power control range (dB) | Open loop and Closed loop (800, 400, 200, 50, 25, 12.5 Hz update rate) with power control steps: 0.25  N, N  dB |  | Open loop and Closed loop (600 and 150 Hz update rate) with power control steps: 0.5  N, N  dB |  | Open loop and Closed loop (Once every N data frames (0.911 ms), N  4, 8, 16, 32) with power control steps: 0.25  N, N  dB |  |
| 16. | density of the equipments (number per km2) | See Note | See NoteError: Reference source not found | See NoteError: Reference source not found | See NoteError: Reference source not found | See NoteError: Reference source not found | See NoteError: Reference source not found |
| 17. | density of the equipments (number per km2) operating at co-frequency | See NoteError: Reference source not found | See NoteError: Reference source not found | See NoteError: Reference source not found | See NoteError: Reference source not found | See NoteError: Reference source not found | See NoteError: Reference source not found |
| 18. | Polarization | Not specified; typically vertical | Not specified; typically vertical | Not specified; typically vertical | Not specified; typically vertical | Not specified; typically vertical | Not specified; typically vertical |

| No/No | Parameter |  | Base station | Mobile station | Base station | Mobile station | Base station | Mobile station |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TECHNOLOGIES |  |  | cdma2000 |  | HRPD |  | UMB |  |
| 19. | Capacity criteria, including capacity per cell |  | Capacity in a cellular system can be measured in terms of simultaneous voice users per cell, data throughput per cell, etc. The actual capacity is dependent on the assumptions made about system configuration, loading, quality, and fairness, among other things. The actual capacity of the system is not part of the cdma2000 specifications. |  |  |  |  |  |
| 20. | Frequency reuse factor |  | Frequency reuse of one is supported. |  |  |  |  |  |
| 21. | Receiver thermal noise | Band Class 0, 3, 10 | −105 dBm/ 1.23 MHz (8 dB NF) | −103 dBm/ 1.23 MHz (10 dB NF) | −105 dBm/ 1.23 MHz (8 dB NF) | −103 dBm/ 1.23 MHz (10 dB NF) | To be provided | To be provided |
| 22. | Reference sensitivity | Band Class 0, 3, 10 | −122 dBm/ 1.23 MHz 9 600 bit/s | −104 dBm/ 1.23 MHz (9 600 bit/s) Traffic/cell −15.6 dB | −122 dBm/ 1.23 MHz 9 600 bit/s | −105.5 dBm/ 1.23 MHz (307 kbit/s in 2 slots) | To be provided | To be provided |

TABLE 4

Maximum power for cdma2000 mobile stations and HRPD access terminals
for band classes in the 790-862 MHz range

| Band class | Mobile station class/ access terminal class | Radiating measurement | Lower limit | Upper limit |
| --- | --- | --- | --- | --- |
| 0 and 3 | Class I | e.r.p. | 1 dBW (1.25 W) | 8 dBW (6.3 W) |
|  | Class II | e.r.p. | −3 dBW (0.5 W) | 4 dBW (2.5 W) |
|  | Class III | e.r.p. | −7 dBW (0.2 W) | 0 dBW (1.0 W) |
| 10 | Class I | e.r.p. | −3 dBW (0.5 W) | 4 dBW (2.5 W) |
|  | Class II | e.r.p. | −7 dBW (0.2 W) | 0 dBW (1.0 W) |

Table 5 maps the band classes in Table 4 to the actual frequencies.

Table 5

Band class designations in the 790-862 MHz range

| Band class | Transmit frequency band (MHz) |  |
| --- | --- | --- |
|  | Mobile station | Base station |
| 0 | 815-849 | 860-894 |
| 3 | 889-889 893-901 915-925 | 832-834 838-846 860-870 |
| 10 | 806-824 896-901 | 851-869 935-940 |
| 18 | 787-799 | 757-769 |

Attachment 2

Generic set of parameters for IMT in the band 790-862 MHz to be used for sharing studies called for under AI 1.17

| No | Parameter | Base station | Mobile station |
| --- | --- | --- | --- |
| 1. | Class of emission |  |  |
| 2. | Modulation parameters | QPSK 16-QAM 64-QAM | QPSK 16-QAM 64-QAM |
| 3. | Duplex mode | FDD/TDD |  |
| 4. | Spectral mask of signals, including | - | - |
| 4.1 | −3 dB radiation bandwidth | - | - |
| 4.2 | −30 dB radiation bandwidth | - | - |
| 4.3 | −60 dB radiation bandwidth | - | - |
| 5. | Maximum spectral power density, dB(mW/Hz) | −23 | −42.5 |
| 6. | Signal bandwidth (MHz) | - |  |
| 7. | Transmitter e.i.r.p. (dBm) |  |  |
|  | Maximum Transmitter e.i.r.p. (dBm) | 55, | 21 to 23 |
|  | Average Transmitter e.i.r.p. (dBm) | Deployment dependant | 2 (rural) −9 (urban) |
| 8. | Typical height of the transmitting antenna (m) | 20 to 30 | 1.5 |
| 9. | Transmitting antenna type (sectorized/omnidirectional) | 3 sectors | omni |
| 10. | Transmitting antenna gain, dBi | 15 | 0 |
| 11. | Feeder loss (dB) | 3 | 0 |
| 12. | Antenna pattern model | ITU-R F.1336-2 | omni |
| 12.1 | - aperture in the horizontal plane at 3 dB (in deg.) | 65 | NA |
| 12.2 | - aperture in the vertical plane at 3 dB (in deg.) | 15 | NA |
| 12.3 | - antenna downtilt | 3° | NA |
| 13. | Relative level of side lobes | –20 dB | NA |
| 14. | Channel bandwidth (MHz) | 5 1.25 | 5 1.25 |
| 15. | Power control range (dB) | 20 | 60 |
| 16. | density of the active users, (number per km2) | Rural: 1 active user operating at the same time (average number of simultaneously active users) Suburban : 13 Urban/In-building : 18 |  |
| 17. | density of user equipments (number per km2) simultaneously operating in a 5 MHz bandwidthError: Reference source not found, | Rural = 0.517 (Freq reuse=1) Rural = 0.217 (Freq reuse=3) Suburban = 2.16 (Freqreuse=1) Urban = 3 (Freq reuse=1) Error: Reference source not foundFreq reuse=3 |  |
| 18. | Polarization discrimination (dB) | 3 | 0 |
| 19. | Capacity Criteria, including capacity per cell | Capacity in a cellular system can be measured in terms of voice users per cell, data throughput per cell, etc. The actual capacity is dependent on the assumptions made about system configuration, loading, quality, and fairness, among other things. |  |
| 20. | Frequency Reuse Factor | 1 or 3 |  |
| 21. | Receiver thermal noise (dBm/channel) | −102 | −98Error: Reference source not found |
| 22. | Reference sensitivity | - | - |
| 23. | Receiver blocking response | - | - |
| 24. | Coverage radius | Rural: 8 km Suburban : 2 km Urban: 0.5 km |  |

Annex 1

Uplink emission levels and TPC usage

# UE uplink emission levels

Field surveys indicate that average UE emission levels are typically more than 20 dB below the 3GPP specified maximum levels and, to maintain sufficient signal margins to accommodate fading, obstructions and higher-rate services, rarely approach the specified maximum level even at cell boundaries. For a sample rural cell, average UE emission levels of around +2 dBm have been measured in field surveys of an active IMT system operating in the 800 MHz band in Australia – in comparison to the specified maximum level of +24 dBm.

Implications of uplink TPC

IMT system protocols explicitly ensure that UE emissions are always set to the minimum necessary power level to achieve a target block error ratio (BLER) as measured at the serving base station receiver. If the BLER target is not achieved, the serving base station may issue a sequence of commands to the relevant UE to step-increment its uplink emission level (step size = 0.5~1 dB), until the BLER target is achieved. Notably, because of varying local propagation and traffic loading conditions, there is no algorithm or formula universally applicable to model uplink TPC and so a statistical approach to determining mean uplink emission level is therefore required.

For the cross-border sharing scenarios considered by the JTG 5-6 in accordance with Resolution 749, that refers to ‘cost-effective coverage’ with a particular focus on ‘areas of low population density’, an average UE emission level of +2 dBm (with SD = 6 dB) can be assumed for undertaking sharing studies.

For JTG 5-6 sharing/compatibility studies the following method (“algorithm”) can be used to determine the UE EIRP being transmitted for a given propagation path:

–	Using the JTG 5-6 propagation model the median downlink propagation loss can be calculated for the given propagation path.

–	This same median propagation loss is then used for the uplink path.

–	This median loss is used to determine the UE eirp for the path considered, taking account of the base station receiver sensitivity, etc.

Annex 2

Downlink emission levels and TPC usage

Downlink TPC

Downlink power control is only rarely operational, and applies only to unusual situations where UEs can approach very near to the serving IMT base station. For the sharing studies undertaken by the JTG, only cross-border scenarios are considered where IMT base stations are located some distance from national borders. Moreover, generally only the UEs approaching the cell boundary in the vicinity of national borders can be considered to potentially contribute interference signals. Therefore, the downlink TPC function is not relevant to the task of the JTG.

Annex 3

Derivation of the user density values provided by sector members

Resolution 749 states: “…that the favourable propagation characteristics of the band 470-806/862 MHz are beneficial to provide cost-effective solutions for coverage, including large areas of low population density”.

A number of 4.5 users/km2 is considered as a high-end estimate for the number of users in large areas of low population density. It is noted that in arriving at this estimate a number of simplifications were made, which may lead to an over-estimate of the number of simultaneously transmitting mobile terminals per area. Firstly, the fact that some users might run sessions of multiple types of mobile applications in parallel has not been considered. Furthermore, the calculation is not based on current traffic volume occurring simultaneously in a frequency band; the value of 4.5 users/km2 is based on a forecast of the total amount of mobile traffic in all available frequency bands, radio access techniques and network deployments for the year 2020. In rural areas other frequency bands (e.g. GSM 900 MHz) may be available in addition to the 790-862 MHz band. Some traffic will be carried by those IMT bands which lower the aggregate number of 4.5 users/km2 given above.

The methodology used to determine the number of simultaneously transmitting mobile terminals relies on the calculation, of demographic information, market penetration, session arrival rate per user during the busy hour and the mean session duration for each expected future type of mobile applications (as predicted in UMTS Forum reports 33, 37, 40). Based on these parameters the session arrival and departure process is modelled as a birth-and-death Markov chain, which enables calculating the average number of simultaneously existing sessions per area using well-established mathematical formulas. The values assumed for subscriber density and market penetration were derived from market forecasts for mobile applications in the reports mentioned above that were developed to assess spectrum requirements for WRC-07 Agenda item 1.4. In particular, the market forecasts assumed are representative for the year 2020 and a rather highly populated European country. It is further noted that considering a cell radius of 3.46 km in a rural area (see field No. 24), this value corresponds to more than 170 active users at the same time in a given cell (one base station).

Applying the above mentioned methodology and values, the average number of simultaneously active users per km2 is derived as 4.5.

Annex 4

Derivation of the user density values provided by Working Party 5D

Attachment 1 to this document suggests as a density of equipment 4.5 users/km2. This number is based on the total traffic forecasts in 2020, in all possible IMT frequency bands. If the ERC Report 65 model is used and the fractions of active users per cell in different environments in Table A7 of ERC Report 65 are kept fixed, we can estimate the user density on 790-862 MHz band. As digital dividend is considered as a coverage band, the 10% no coverage area in Table A7 of ERC Report 65 is included here to rural area, changing the rural land from 87% to 97%.

0.97 × 0.3 × P + 0.0298 × 12.5 × P + 0.0002 × 23.7 × P = 4.5 (users/km2)

P 	= scaling factor from 1998 values to 2020 calculations  = 6.7341

The apportionment amongst the different environments should be seen as an example taken from ERC Report 65.

The new user densities for all IMT bands are now:

Rural: 2.02 users/km2

Suburban: 84.18 users/km2

Urban/Central Business District: 159.6 users/km2

The amount of other available spectrum can be taken into account as follows:

Rural case: the traffic and users are using both the GSM900 and the 790-862 MHz band, so, the final user density is (this is a very conservative assumption, as GSM1800 and UMTS 2 GHz coverage is much wider than urban-suburban cases):

30/65 × 2.02 = 0.93 users/km2

Suburban case: The traffic is divided to GSM900, GSM1800, UMTS 2 GHz and the 
790-862 MHz bands. The user density for the 790-862 MHz band is:

30/200 × 84.12 = 12.61 users/km2

Urban/Central Business District: In addition to suburban case, also the 2.6 GHz is in use: The user density in the 790-862 MHz band is:

159.6 × 30/270 = 17.73 users/km2

Then the average user density in the 790-862 MHz band can be calculated:

0.97 × 0.93 + 0.0298 × 12.61 + 0.0002 ×  17.73 = 1.28 users/km2

Rounding these figures, this leads to:

| Active users (users/km2 ) in the band 790-862 MHz | Rural | 1 |
| --- | --- | --- |
|  | Urban/suburban | 13 |
|  | Urban/Central Business District | 18 |

Cross check with the capacity of the interfaces (upper limit on the density of users)

In a radio interface based on OFDMA, the bandwidth is divided into resource blocks (or its equivalent), and one or more resource blocks is assigned to a user at any instant in time. The maximum number of simultaneous users cannot exceed the number of available resource blocks. 
If the number of users exceeds the number of resource blocks, or the service to a user requires a lower data rate than can be provided by a single resource block, the resource blocks are multiplexed in time.

In a 5 MHz LTE FDD channel, there are 25 resource blocks in the uplink. In practice, some users will be using more than one resource block in the uplink; therefore 20 was taken as a practical maximum for the number of co-frequency users in 5 MHz channel.   However, the number of users that can be supported by a base station is not limited by the number of resource blocks - if the number of users is higher, they are supported sequentially in time, and they therefore do not increase the aggregate interference at a particular instant in time.

The transmitting antenna characteristics defined in Attachment 2 equate to a three-sector base station configuration. For a frequency reuse factor of one, each carrier will be used three times in each coverage radius, and for a frequency reuse factor of three, each carrier will be used once.

The  densit of theuser ipments in Row 17 (number per km2) operating at 
co-frequency is therefore the lesser of the values derived from the density of active equipments. (Dmax (equipments)) and the physical limitations of the radio interface (Dmax (radio interface)).

(Dmax (equipments)) = (density of active users) / (number of available 5 MHz channels)

Dmax (radio interface) = (maximum number of resource blocks/5 MHz) * (3 / reuse) / (π r2)

(number of available 5 MHz channels) = 6 for FDD, 12 for TDD

r 	= coverage radius

| FDD | Urban (reuse 1) | Dmax (equipments) | 3 |
| --- | --- | --- | --- |
|  |  | Dmax (radio interface) | 152 |
|  | Suburban (reuse 1) | Dmax (equipments) | 2.166 |
|  |  | Dmax (radio interface) | 9.55 |
|  | Rural (reuse 3) | Dmax (equipments) | 0.17 |
|  |  | Dmax (radio interface) | 0.199 |
|  | Rural (reuse 1) | Dmax (equipments) | 0.17 |
|  |  | Dmax (radio interface) | 0.6 |

Maximum number of resource blocks/5 MHz is assumed to be 40 i.e. multiple user MIMO (assuming two base station antennas) with some terminals simultaneously using resource blocks on different MIMO branches.

Appendix 1

List of relevant documents

| Reference | Title / description | Relevance |
| --- | --- | --- |
| Rec. ITU-R M.687 | International Mobile Telecommunications-2000 (IMT-2000) | [high] |
| Rec. ITU-R M.819 | International Mobile Telecommunications-2000 (IMT-2000) for developing countries | [low] |
| Rec. ITU-R M.1036-3 | Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications-2000 (IMT-2000) in the bands 806-960 MHz, 1 710-2 025 MHz, 2 110-2 200 MHz and 2 500-2 690 MHz | [high] |
| Rec. ITU-R M.1634 | Interference protection of terrestrial mobile service systems using Monte Carlo simulation with application to frequency sharing | [high] |
| Rec. ITU-R M.1635 | General methodology for assessing the potential for interference between IMT-2000 or systems beyond IMT-2000 and other services | [high] |
| Rec. ITU-R M.1767 | Protection of land mobile systems from terrestrial digital video and audio broadcasting systems in the VHF and UHF shared bands allocated on a primary basis | [high/ medium] |
| Rec. ITU-R M.1768 | Methodology for calculation of spectrum requirements for the future development of the terrestrial component of IMT-2000 and systems beyond IMT-2000 | [high] |
| Rec. ITU-R M.1808 | Technical and operational characteristics of conventional and trunked land mobile systems operating in the mobile service allocations below 869 MHz to be used in sharing studies | [high] |
| Rec. ITU-R M.1823 | Technical and operational characteristics of digital cellular land mobile systems for use in sharing studies | [high] |
| Rec. ITU-R M.1824 | System characteristics of television outside broadcast, electronic news gathering and electronic field production in the mobile service for use in sharing studies | [medium] |
| Rec. ITU-R M.1825 | Guidance on technical parameters and methodologies for sharing studies related to systems in the land mobile service | [high] |
| Report ITU-R M.2039 | Characteristics of terrestrial IMT-2000 systems for frequency sharing/interference analyses, ITU, 2004 | [high] |

___________________

_______________

Future possible deployment.

Future possible deployment.

Rec. ITU-R SM.1138 “Determination of necessary bandwidths including examples for their calculation and associated examples for the designation of emissions” does not explicitly cover OFDMA signalling, therefore an alternative simple description is used here.

Future possible deployment and the preliminary parameters can also be used for LTE TDD related sharing studies.

Note that the FDD profile of mobile WiMAX is currently not part of Recommendation ITU-R M.1457. However, there is on-going work to include it.

See 3GPP Documents: TS 25 104 v 8.5.0, see section 6.6.3 and TS 36 141 v 8.5.0, see section 6.6.3.

See 3GPP Documents: TS 25 101 v 8.5.0, see section 6.6.2.1 and TS 34.121-1 v8.4.0, see section 5.9.

See 3GPP Documents: TS 36 104 v 8.5.0, see section 6.6.3 and TS 36 141 v 8.5.0, see section 6.5.2.1.

See 3GPP Documents: TS 36 101 v 8.4.0, see Table 6.6.2.1.1-1 (General E-UTRA spectrum emission mask) and TS 36 521-1 v 8.0.0, see section 6.6.

US Code of Federal Regulations, Title 47, FCC Rules Parts 27 and 90.

Based on a bandwidth of 3.84 MHz.

This value corresponds to a 4.5 MHz bandwidth, noting that other measurement bandwidth are possible: 1.08 MHz, 2.7 MHz, 9.0 MHz, 13.5 MHz and 18 MHz.

These are average spectral power density values based on item 7 over the specified channel bandwidth.

This value is average spectral power density based on item 7 and assumes the mobile station is transmitting on PUSC (Partially Used of SubChannels) with all subchannels.

These values are corresponding to the utilized spectrum within 5, 7 and 10 MHz channel bandwidths in Up Link when PUSC is used.

These values are corresponding to the utilized spectrum within 5, 7 and 10 MHz channel bandwidths in Up Link when PUSC is used.

See Annex 1.

See Annex 2.

This value does not take account of the effect of multiple transmit antennas.

WiMAX numbers for MS related to item 7 are preliminary numbers. WiMAX Forum profiles 7.A and 7.E of Table 2, in general, cover a range of power classes.

See 3GPP Document TS 25.942, section 8.4.3.1 (Antenna installation) or section 10 (Antenna to antenna isolation).

A front-to-back ratio of 25 dB should be assumed.

This value refers to the block size.

See 3GPP Document TS 36104-830, Table 6.3.2.1-1 (E-UTRA BS total power dynamic range).

See Annex 3.

This figure can be used for sharing studies in urban areas, because the increase of population density in urban areas (compared with rural/suburban areas) is expected to be offset by the distribution of offered traffic to parallel networks deployed in the other available frequency bands. The networks expected to be deployed in the 800 MHz band will offer only limited capacity and thus will carry only a small fraction of the total offered traffic in urban areas.

Typically base stations today use cross-polarized antennas (two sets of dipoles slanted at ±45° against the horizontal plane), usually transmitting on one of the two polarisation paths (either +45° or −45° for a given frequency) whilst receiving on both paths (to achieve polarisation diversity). Such signals provide an isolation of 3 dB against both horizontally and vertically polarised signals (e.g. DVB-T signals) due to cross-polarisation discrimination.

See 3GPP Document TS 25.104, section 7.2.

See 3GPP Document TS 25.101, section 7.3.

See 3GPP Document TS 36.104, section 7.2.

See 3GPP Document TS 36.101, section 7.3.

See 3GPP Document TS 25.104, section 7.5.

See 3GPP Document TS 25.101, section 7.6.3.

See 3GPP Document TS 36.104, section 7.6.

See 3GPP Document TS 36.101, section 7.6.3.

Note that the FDD profile of mobile WiMAX is currently not part of the Recommendation ITU-R M.1457. However, there is on-going work to include it.

Note that some of the parameters in this table are typically not included in the cdma2000 specifications. These include: maximum spectral power density, transmitting antenna type, antenna gain, antenna height, antenna pattern, antenna downtilt, feeder loss, and polarization. Some information in these categories found in this table is listed in 3GPP2 report C.R1002 (cdms2000 Evaluation Methodology) and may be considered typical in some deployments.

Note that the TDD profile of UMB is currently not part the Recommendation ITU-R M.1457. However, there is on-going work to include it.

Note that these Recommendations are currently being reviewed by WP 5D.

Assumes 43 dBm maximum transmit power over 1.2288 MHz.

Assumes 23 dBm maximum transmit power over 1.2288 MHz.

This value refers to the block size.

This is a function of frequency, coverage desired, propagation, data rates desired, etc.

See 3GPP Document: TS 36 104 v 8.5.0, see section 6.6.3 and TS 36 141 v 8.5.0, see section 6.5.2.1.

See 3GPP Document: TS 36 101 v 8.4.0, see Table 6.6.2.1.1-1 (General E-UTRA spectrum emission mask) and TS 36 521-1 v 8.0.0, see section 6.6.

Depending on the technology, this parameter can take the following values: 3.84 MHz, 4.5 MHz or 4.6 MHz in a block of 5 MHz; 1.2288 MHz in a block of 1.25 MHz.

See Annex 2.

In particular remote rural areas such as some parts of Russia, the e.i.r.p. value may be higher.

See Annex 1.

Although this ITU-R Recommendation applies to frequency bands above 1 GHz, it is considered that sectorial antennas operating in the 800 MHz band that employ technology comparable to that used in bands on the order of 1 GHz to 3 GHz should exhibit similar off-axis performance.

This value is derived from Recommendation ITU-R F.1336-2 (recommends 3.3) using an antenna gain of 15 dBi and an horizontal aperture of 65°.

This value refers to the block size.

See Annex 4.

These values are expressed in users/km² and should be divided by the number of channels available, as appropriate, when studying the compatibility involving IMT in the band 790-862 MHz, to reflect the distribution of users amongst the available channels.

In particular rural environments such as northern Scandinavia or deserts, the user density may be lower than this. For example, the total population density is 22.6 inhabitants/ km2 for Sweden and 16 for Finland. For areas intended for 800 MHz coverage the estimated population density is estimated to be 
0.1-0.5 inhabitants/km2.

It should be noted that for TDD systems the density of user equipments (numbers per km2) simultaneously operating in a 5 MHz bandwidth would effectively be further reduced.

- the number of 5 MHz blocks is slightly larger for TDD (up to 65 MHz in total) than for FDD 
(60 MHz in total);

- should JTG 5-6 wish to perform a detailed analysis involving TDD, it should be noted that the TDD base station and mobile station in a given 5 MHz bandwidth will only transmit part of the time (time-division duplex), which will reduce the amount of interference from the base station compared to FDD but at the same time add  interference from the mobile station which is not the case for FDD.

As detailed in Annex 4, these calculations of user density only consider carrying traffic in the 
790-862 MHz, 900 MHz, 1.8 GHz , 2.1 GHz and 2.6 GHz bands.  If, in addition to these bands, other frequency bands that were identified for IMT were considered, the user density figures for suburban and urban areas may in reality be lower than the values given in row 17, independent of the duplex method.

Typically base stations today use cross-polarized antennas (two sets of dipoles slanted at ±45° against the horizontal plane), usually transmitting on one of the two polarisation paths (either +45° or –45° for a given frequency) whilst receiving on both paths (to achieve polarisation diversity). Such signals provide an isolation of 3 dB against both horizontally and vertically polarised signals (e.g. DVB-T signals) due to cross-polarisation discrimination.

For systems based on OFDM, in the rural deployment, frequency reuse factor 3 is mostly planned (although 1 is not excluded), and at urban and suburban environments, frequency reuse factor 1 or 3 can be used.

Frequency reuse factor 3 can be implemented by both traffic channels and control channels. And frequency reuse factor 1 can be implemented by traffic channels. In order to avoid inter-cell interference, all of the sub-carriers can be only used in the cell centre with lower power, and parts of the sub-carriers can be used at the cell edge.

For a 5 MHz channel.

See 3GPP Document TS 36.104, section 7.2.

See 3GPP Document TS 36.101, section 7.3.

See 3GPP Document TS 36.104, section 7.6.

See 3GPP Document TS 36.101, section 7.6.3.

http://www.erodocdb.dk/Docs/doc98/official/pdf/REP065.PDF.

The number of simultaneous users will be lower than this for most mixes of mobile broadband services, because each user will be offered the maximum available uplink bit rate.