| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimization of UL grant scheduling for M2M |  |
| Date Submitted | 2011-03-06 |  |
| Source(s) | Joey Chou Shantidev Mohant Intel | Joey.Chou@intel.com |
| Re: | P802.16p |  |
| Abstract | Propose the coordinated paging method to reolve the uplink congestion issue of M2M devices |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimization of UL grant scheduling for M2M

Joeyn Chou,

Shantidev Mohant

Intel

- Introduction

As per the M2M SRD [1], the 802.16p system shall support very large numbers of devices. This poses major challenges to the 802.16-2009 and 802.16m air interfaces, as these huge numbers of devices can jam the ranging channels during network entry. As a result, a lot of proposals have beed discussed on the RG reflector. However, most of the proposals were focused on the assignment of ranging parameters to M2M devices that not only complicate the design of MAC and PHY layers, but can impact the performance of non-M2M devices.

One way to understand how M2M devices can efficiently co-locate with the mobile devices in a single cell is to look at the traffic characteristics of M2M devices. Table 1 lists the traffic characteristics for various M2M usage models, as illustrated in M2M technical report [2].

| Usage Models | Fixed/Nomadic/Mobile | Realtime (RT) / Non-realtime (NRT) | Data types |
| --- | --- | --- | --- |
| Vehicle location tracking | Mobile | NRT – location report | Short message |
| Healthcare | Fixed, Nomadic | RT – emergency call | Short message |
|  |  | NRT – periodic measurement reading | Short message |
| Secured access & surveillance | Fixed | RT – alarm report, video streaming | Short message, streaming |
| Public safety (e.g. rivers, dams water level monitoring) | Fixed | RT – alarm report | Short message |
|  |  | NRT – periodic monitoring |  |
| Point of sale (e.g. payment) | Fixed | RT – sale transaction | Short message |
| Samrt meter | Fixed | RT – outage report | Short message |
|  |  | NRT – periodic meter report | Short message |
| Digital signage | Fixed | RT – viewer identification | Short message |
| Remote sensing (e.g. oil, gas, water, alarm) | Fixed | RT – alarm report | Short message |
|  |  | NRT – periodic meter report | Short message |

Table 1: M2M Traffic Characteristics

The traffic characteristics and service requirements of M2M devices are quite different from AMS. Therefore, the network entry procedure, as defined in section 16.2.15, is not able to support both M2M devices and AMS efficiently.

This contribution proposes that the uplink scheduling of M2M devices should take into account the traffic characteristics and pattern.

- References

- IEEE 802.16p-10/0004r2, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” Jan. 2011.

- IEEE 802.16p-10/0005, “Machine to Machine (M2M) Communications Technical Report”, Jan 2011

- Proposed Text

3.1 Proposed Text #1

----------------------------------------------  Text Start -----------------------------------------------

16.2.11 Bandwidth Request and Allocation Mechanism

16.2.11.1 Bandwidth Request

16.2.11.1.6 Bandwidth Request and Allocation Mechanism

The traffic characteristics and service requirements of M2M devices are quite different from AMS. Therefore, the network entry procedure should take into account the M2M traffic characteristeristics and pattern in order to support both M2M devices and AMS efficiently.

----------------------------------------------  Text End -----------------------------------------------