| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Distribution of ranging access from M2M devices during network reentry |  |
| Date Submitted | 2011-07-20 |  |
| Source(s) | Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim ETRI | Voice: +82-42-860-0626 E-mail: {kimsk0729}@etri.re.kr |
| Re: | Call for comments on IEEE 802.16p AWD |  |
| Abstract | This contribution proposes texts for distribution of ranging access from M2M devices. |  |
| Purpose | To discuss and adopt the proposed text in the 802.16p AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Distribution of ranging access from M2M devices during network reentry

Seokki Kim, Jaesun Cha, Soojung Jung, Chulsik Yoon, Kwangjae Lim

ETRI

# Introduction

In the last meeting, it is agreed that ranging simulation is necessary for comparison of network entry solution. In this contribution, we introduce our ranging simulation results and propose the method which distributes ranging access from large number of M2M devices during network reentry

# Simulation Scenario and Parameters

We have performed the ranging simulation based on ‘Scenario 1’ (fixed devices and network-initiated network entry) defined in [2] and considered individual paging. Simulation parameters for the ranging simulation are listed in Table1.

Table 1 – Simulation Parameters

| Items | Value | Note |
| --- | --- | --- |
| Number of M2M devices | 12000 |  |
| Access Attempt/second | [10, 50, 100, 200, 500, 1200, 2000, 5000] |  |
| Initial backoff window size | [2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048, 4096] | in terms of frames |
| Final backoff window size | 65536 | in terms of frames |
| Arrival distribution | Poisson |  |
| T31 | 60 ms |  |
| Maximum Contention Ranging Retries | 8 |  |
| Time delay for Ranging Success | 30ms | Time from the transmission of a ranging preamble code to reception of AAI-RNG-RSP/RNG-RSP message which is a response to AAI-RNG-REQ/RNG-REQ message. This parameter is applicable only when only one M2M device transmits a ranging preamble code on a ranging channel. |
| UL overhead | 1 ranging channel per frame |  |
| DL overhead | AAI-PAG-ADV: 5byte AAI-RNG-ACK: 14bytes AAI-RNG-RSP: 17bytes | Assuming each message is used once per a ranging attempt |
| Total number of ranging code | 1 |  |
| Ranging code detection capability | 1 code per channel |  |
| Misdetection probability | 0% |  |
| False alarm | 0% |  |
| Reception of same code | No |  |
| Frame duration | 5ms |  |

# Simulation Result

According to performance metric defined in [2], ranging simulation results are represented in Figure 1-8.

We performed the simulation in case of various number of access attempt per second (10, 50, 100, 200, 500, 1200, 2000 and 5000) and initial backoff window size (2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048 and 4096).

When number of access attempt is small (see Figure1 or 2), it seems that there is no problem to perform the ranging access independent of initial backoff window size. Furthermore, access time delay may increase if initial backoff window size is too large. When number of access attempt is large, a lot of devices with small initial backoff window size fail to perform ranging access until maximum contention ranging retries (see Figure 3~8). Also we can find a backoff window size to satisfy a target ranging success rate while minimizing access delay.

As we can see in the figures, it is inescapable that access delay increases as number of devices grows. But we can maintain the high ranging success probability and low contention ratio by extending backoff window size. If devices type is ‘fixed’ and network entry type is ‘network-initiated’ such as ‘scenario 1’, base station can estimate the number of devices which are preparing network reentry and select appropriate backoff window size to reduce access delay and congestion.

Although this simulation is focused on the individual device access, we assume that a network can distribute M2M group accesses by assigning different paging offsets to each M2M groups based on the characteristics of M2M applications.

Because we used only one NS-RCH per a frame and didn’t deal with dedicated ranging channel allocation, overhead is not critical issue in in this simulation. To apply this proposal, we have only to insert additional 8 bits information regarding backoff window size in the paging message.

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 1 – Simulation Result on 10 Access Attempt/second

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 2 – Simulation Result on 50 Access Attempt/second

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 3 – Simulation Result on 100 Access Attempt/second

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 4 – Simulation Result on 200 Access Attempt/second

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 5 – Simulation Result on 500 Access Attempt/second

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 6 – Simulation Result on 1200 Access Attempt/second

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 7 – Simulation Result on 2000 Access Attempt/second

(a) Statistics of Ranging Access Delay                 (b) Statistics of Number of Ranging Preamble Transmission

(c) UL Normalized Overhead                                  (d) DL Normalized Overhead

Figure 8 – Simulation Result on 5000 Access Attempt/second

# Proposal

This contribution proposes some texts which define ranging parameters in a paging message. Network access from large number of M2M devices can be distributed by selecting appropriate value of ‘ranging backoff start’ and ‘ranging backoff end’ in the paging message.

# References

[1] IEEE 802.16p-10/0018r2, “Amendment Working Document,” May. 2011.

[2] C80216p-11-0126, “Evaluation Guideline for Comparison of Network Entry Solutions”, July. 2011.

[3] IEEE 802.16p-10_0005, “Machine to Machine (M2M) Communications Technical Report, Nov. 2010.

# Proposed Text

----------------- Start of the text proposal ---------------------------------------------------------------------------------------

[Remedy 1: Modify Table 706 as follows:]

Table 706—AAI-PAG-ADV Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Paging_Group_IDs bitmap | L | Indicate that the paging information for the corresponding paging group is included in AAI-PAG-ADV message The length of Paging_Group_IDs bitmap is same as Num_PGIDs in PGID-Info message. 0: The paging information for the corresponding PGID is not included 1: The paging information for the corresponding PGID is included | L equals the Num_PGIDs in PGIDInfo message |
| For (i=0;i<M;i++){ |  |  |  |
| For (j=0;j<Num_AMSs;j++){ |  | Num_AMSs indicates the number of paged AMSs in a corresponding paging group 1..32 |  |
| Deregistration Identifier | 18 | Used to indicate Deregistration ID for the AMS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..218– 1 | Present if the S-SFH Network Configuration bit == 0b0 |
| MAC Address Hash | 24 | Used to identify the AMS to be paged | Present if the S-SFH Network Configuration bit == 0b1 |
| Paging Cycle | 4 | Used to indicate Paging cycle for the AMS to be paged 0x00: 4 superframes 0x01: 8 superframes 0x02: 16 superframes 0x03: 32 superframes 0x04: 64 superframes 0x05: 128 superframes 0x06: 256 superframes 0x07: 512 superframes 08-0x15: reserved | Present if the S-SFH Network Configuration bit == 0b0. |
| Action code | 1 | Used to indicate the purpose of the AAI-PAG-ADV message 0b0: perform network reentry 0b1: perform ranging for location update |  |
| } |  |  |  |
| } |  |  |  |
| Extension Flag | 1 | Used to indicate the remaining part of the AAI-PAG-ADV message exists 0b0: This is the last fragment of the AAIPAG- ADV message 0b1: This is not the last fragment of the AAI-PAG-ADV message; the remaining fragments of the message will be transmitted in the subsequent subframes or frames |  |
| Emergency Alert Indication | 1 | Used to indicate the presence of emergency information 0b0: Reserved 0b1: There is emergency information | Optional Present if there is emergency information |
| Ranging backoff start | 4 | Initial backoff window size for ranging contention during network reentry, expressed as a power of 2. Values of n range 0~15 | Present if needed |
| Ranging backoff end | 4 | Final backoff window size for ranging contention during network reentry, expressed as a power of 2. Values of n range 0~15 | Present if needed |
| For (i=0; i<Num_MGID; i++) { |  | Num_MGID indicates the number of MGIDs included in this paging message[0..63] | Shall be included if the ABS sends DL multicast data for M2M after transmission of the AAI-PAG-ADV message. |
| MGID | 15 | M2M Group ID |  |
| Action code | 2 | 0b00: Performing netwok re-entry 0b01: Performing location update 0b10: Receiving multicast traffic 0b11: reserved |  |
| M2M report code | 1 | Indicate the opportunity for the M2M device to send the uplink report 0b0: No action required 0b1: Send uplink report | Present if M2M is supported. |
| If(Action code == 0b10){ |  |  |  |
| Multicast transmission start time (MTST) | TBD | Least significant TBD bits of the frame nuimber in which the ABS starts sending DL multicast data. | Shall be present when the MTST needs to be included in this message. |
| } |  |  |  |
| Else { |  |  |  |
| Ranging backoff start | 4 | Initial backoff window size for ranging contention during network reentry, expressed as a power of 2. Values of n range 0~15 | Present if needed |
| Ranging backoff end | 4 | Final backoff window size for ranging contention during network reentry, expressed as a power of 2. Values of n range 0~15 | Present if needed |
| } |  |  |  |
| … |  |  |  |
| } |  |  |  |

----------------- End of the text proposal ----------------------------------------------------------------------------------------