| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improvement procedure to update of MGID |  |
| Date Submitted | 2011-09-11 |  |
| Source(s) | Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang Intel | E-mail: marta.m.tarradell@intel.com |
| Re: | 802.16p amendment working document (IEEE 802.16p-11/0021) |  |
| Abstract | Proposes a method to improve the way the MGID is re-assigned in a group of M2M devices |  |
| Purpose | Discuss and adopt proposed texts |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Improvement Procedure to Update MGID

Marta Martinez Tarradell, Shantidev Mohanty, Honggang Li, Rui Huang

Intel Corporation

# Introduction

# When the MGID of more than one M2M device in idle mode of a M2M group needs to be updated, the current update procedures using individual location update may result in significant overhead. To eliminate this issue, this contribution proposes methods for efficient update of MGID for idle mode M2M devices.

# Proposal

# Instead of updating the MGID of M2M devices in idle mode, this contribution proposes mechanisms to update the MGID of multiple M2M devices simultaneously.

# Proposed texts

## Proposed Text #1

----------------------------------------------  Proposed Text Start -----------------------------------------------

[Editor’s note] Add the following section after line 21 in page 4 of Section 6.3.1of IEEE 802.16p-11/0021.

Instead of updating the MGID of multiple idle mode M2M devices one by one, the ABS updates the MGID of these devices simultaneously.

----------------------------------------------  Proposed Text End -----------------------------------------------