| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL burst allocation for F-FDD MS in generic H-FDD frame structure |  |
| Date Submitted | 2008-04-20 |  |
| Source(s) | Ji-Yun Seol, Samsung Electronics* | Voice: E-mail: jiyun.seol@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | LB26c |  |
| Abstract | This contribution proposes an signaling method to make UL allocation for F-FDD MS in generic H-FDD frame structure. |  |
| Purpose | Adopt the proposed specification changes in IEEE P802.16Rev2/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL burst allocation for F-FDD MS in generic H-FDD frame structure

Samsung Electronics

# Problem description

# Problem Statements

- In generic structure of FDD/HFDD, MAPs in each Groups have relevance only to the corresponding DL/UL subframes

  - MAP1 (MAP2) in Group1(Group2)  indicates allocations made for DL/UL Group1 (Group2)

  - Not possible to fully utilize the capability of F-FDD MSs that can receive/transmit simultaneously

- Downlink and/or Uplink resources are wasted due to the limitation of H-FDD MSs

  - Transition Gaps (Receive-to-Transmit and Transmit-to-Receive) for H-FDD MSs should be considered

  - Gaps in-between Downlink subframes (DL_gap) and/or Uplink subframes (UL_gap) will take place

Figure 1 Generic FDD OFDMA frame structure supporting H-FDD MS in two groups and subframe relevance for MAPs in each group

Figure 2 Generic FDD OFDMA frame structure supporting H-FDD MS in two groups with associated parameters configuring the frame

- Requisites

- Should inform the zone configuration of the other UL subframe

  - Use 1 reserved bit in UL Zone Switch IE to indicate the Group that it corresponds to

- Should be possible to make a UL allocation to the other UL subframe

  - Use 1 reserved bit in UL Allocation Start Offset Indicators (UL Allocation Start IE, UL HARQ subburst IEs, UL Persistent Allocation IE, etc.) to inform the UL subframe that the UL 2D offset (symbol offset and subchannel offset) corresponds to

- Allow UL allocation over(across) UL subframes in the same frame

  - Use 1 reserved bit in UL Zone Switch IE to enable/disable UL over_subframe allocation

  - Only possible if the last zone of UL Group2 and the first zone of UL Group1 have the same zone configuration

- Allow UL allocation in the gap between UL subframes

  - Use 1 reserved bit in UL Zone Switch IE to explicitly indicate the allocation in-between UL subframes (inter-UL_gap allocation), or Implicitly enable inter-UL_gap allocation if a certain condition is satisfied (e.g., UL_gap  is multiple of UL slot duration)

Figure 3 Proposed subframe relevance for MAPs in each group to support F-FDD MS in generic H-FDD frame structure

- Usage Scenarios

- BS informs the UL zone configuration of the other Group using UL Zone IE with UL_subframe_indicator = 1

  - The allocation for UL control channel regions in the other Group should be made after UL Zone IE with UL subframe indicator=1 and Symbol_offset=0 1 (if overriding the UCD TLVs)

  - For the last zone in the first subframe (or, UL Group2), the over_subframe allocation indicator and/or inter-UL_gap allocation indicator  may be set for the F-FDD MS at BS’ discretion (if certain conditions met, such as the identical zone type for the last zone in UL_G2 and the first zone in UL_G1, inter-UL_gap of multiple of slot durations, etc.)

  - Explicit UL allocation start offset should be indicated either using UL Allocation Start IE or HARQ UL-MAP IE with UL Allocation Start Indication=1 (or Persistent UL Allocation IE) (Other UL subframe indication may set according to the subframe of the Allocation Start Offset belongs to.)

- MS implicit inter-UL_gap allocation

  - Possibly enable inter-UL_gap allocation for the allocation made in the last zone of the first subframe (UL_G2) without BS’ explicit indication, if a certain condition is met (UL_gap = 3n, n=1, 2,3,…)

- Further considerations

- Seed for UL subcarrier randomization (scrambler)

  - For uplink, last 4 bits of “Frame Number” is used for scrambler seed

  - The frame number used for scrambler is the frame that the UL burst is transmitted for TDD, while it is the frame that the UL-MAP is included for FDD/H-FDD (inconsistency)

  - In H-FDD, the BS and F-FDD MS has to use different frame number for the allocation in each UL Groups (If the current frame number is N, ‘N-2’ will be used for UL Group2 and ‘N-1’ for UL Group1)

  - It is suggested to change the standard to use the frame number of the frame that the UL burst is transmitted as in TDD

- PUSC Subchannel Rotation for over_subframe and/or inter-UL_gap allocation

  - Since the allocation is made over two adjacent UL zones in different UL subframes, it is suggested to increment the “Slot Number” up to the slot in-between UL subframes and reset to 0 at the beginning of the zone in the UL subframe for UL Group1 (which comes after the subfame for UL Group2)

(a)

(b)

(c)

(d)

Figure 4 UL burst allocation examples for F-FDD MS: (a) other UL Group allocation (b) other UL Group allocation + inter-UL_gap allocation (c) other UL Group allocation + over_subframe allocation (d) other UL Group allocation + inter-UL_gap allocation + over_subframe allocation

# Proposed Changes

[Change #1: Modifiy Table 380 OFDMA UL Zone IE as follows]

Table 380 OFDMA UL Zone IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| UL_ZONE_IE() { | - | - |
| Extended DUIUC | 4 bits | UL_ZONE = 0x04 |
| Length | 4 bits | Length = 0x03 |
| OFDMA symbol offset | 7 bits | - |
| Permutation | 2 bits | 0b00 = PUSC permutation 0b01 = Optional PUSC permutation 0b10 = Adjacent subcarrier permutation 0b11 = Reserved |
| PUSC UL_IDcellPermBase | 7 bits | - |
| AMC type | 2 bits | Indicates the AMC type in case permutation type = 0b10, otherwise shall be set to 0. AMC type (NxM = N bins by M symbols): 0b00: 1x6 0b01: 2x3 0b10: 3x2 0b11: Reserved |
| Use All SC indicator | 1 bit | 0 = Do not use all subchannels 1 = Use all subchannels |
| Disable subchannel rotation | 1 bit | Only applies to PUSC permutation (see section 8.4.6.2.6) 0 = subchannel rotation enabled 1 = subchannel rotation disabled |
| H-FDD UL subframe indicator | 1 bit | Only applies to FDD/H-FDD 0b0: UL subframe relevant to current UL-MAP (or UL subframe 2 (UL1)) 0b1: The other UL subframe (or UL subframe 1 (UL2)) Shall be set to zero for TDD |
| H-FDD Group over-subframe allocation | 1 bit | Only applies to FDD/H-FDD 0b0: Disable UL over-subframe allocation 0b1: Enable UL over-subframe allocation Shall be set to zero for TDD |
| H-FDD Inter-UL_gap allocation | 1 bit | Only applies to FDD/H-FDD 0b0: Disable inter-UL_gap allocation 0b1: Enable inter-UL_gap allocation Shall be set to zero for TDD |
| Reserved | 41 bit | Shall be set to zero |
| } | - | - |

[Change #2: Modifiy Table 415 UL Allocation Start IE as follows]

Table 415 UL Allocation Start IE format test

| Syntax | Size | Notes |
| --- | --- | --- |
| UL Allocation start IE() { | - | - |
| Extended UIUC | 4 bits | UL_Allocation_start_IE() = 0x0A |
| Length | 4 bits | Length = 0x02 |
| OFDMA symbol offset | 8 bits | This value indicates start symbol offset of all subsequent UL allocations in this UL_MAP message (UL-MAP or SUB-UL-DL-MAP). |
| Subchannel offset | 7 bits | This value indicates start subchannel offset of all subse-quent UL data burst allocations in this message (UL-MAP or SUB-UL-DL-MAP). |
| Reserved | 1 bit | Shall be set to zero. |
| H-FDD UL subframe indicator | 1 bit | Only applies to FDD/H-FDD 0b0: UL subframe relevant to current UL-MAP (or UL subframe 2 (UL1)) 0b1: The other UL subframe (or UL subframe 1 (UL2)) Shall be set to zero for TDD |
| } | - | - |

[Change #3: Modify Table 424 HARQ UL-MAP IE as follows]

Table 424 HARQ UL-MAP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| HARQ UL MAP IE() { | - | - |
| Extended-2 UIUC | 4 bits | HARQ_UL-MAP_IE () = 0x07 |
| Length | 8 bits | Length in bytes |
| RCID_Type | 2 bits | 0b00 = Normal CID 0b01 = RCID11 0b10 = RCID7 0b11 = RCID3 |
| Reserved | 2 bits | - |
| while (data remains) { | - | - |
| Mode | 3 bits | Indicates the mode of this IE 0b000 = Chase HARQ 0b001 = Incremental redundancy HARQ for CTC 0b010 = Incremental redundancy HARQ for convolutional code 0b011 = MIMO Chase HARQ 0b100 = MIMO IR HARQ 0b101 = MIMO IR HARQ for Convolutional Code 0b110 = MIMO STC HARQ 0b111 = Reserved |

| Allocation Start Indication | 1 bit | 0: No allocation start information 1: Allocation start information follows |
| --- | --- | --- |
| If (Allocation Start Indication == 1) { | - | - |
| OFDMA Symbol offset | 8 bits | This value indicates start Symbol offset of subsequent sub-bursts in this HARQ ULMAP IE |
| Subchannel offset | 7 bits | This value indicates start Subchannel offset of subsequent sub-bursts in this HAR-QUL MAP IE |
| Reserved | 1 bit | Shall be set to zero. |
| H-FDD UL subframe indicator | 1 bit | Only applies to FDD/H-FDD 0b0: UL subframe relevant to current UL-MAP (or UL subframe 2 (UL1)) 0b1: The other UL subframe (or UL subframe 1 (UL2)) Shall be set to zero for TDD |
| } |  |  |
| N sub Burst | 4 bits | Indicates the number of bursts in this UL MAP IE is this field value plus 1. |
| For (i =0 ;i < N Sub-burst; i++){ | - | - |
| If (Mode == 000) { | - | - |
| UL HARQ Chase Sub-Burst IE () | - | - |
| } else if (Mode== 001) { | - | - |
| UL HARQ IR CTC Sub-Burst IE () | - | - |
| } else if (Mode== 010) { | - | - |
| UL HARQ IR CC Sub-Burst IE () | - | - |
| } else if (Mode== 011) { | - | - |
| MIMO UL Chase HARQ Sub-Burst IE () | - | - |
| } else if (Mode== 100) { | - | - |
| MIMO UL IR HARQ Sub-Burst IE () | - | - |
| } else if (Mode== 101) { | - | - |
| MIMO UL IR HARQ for CC Sub-Burst IE () | - | - |
| } else if (Mode == 110) { | - | - |
| MIMO UL STC HARQ Sub-Burst IE () | - | - |
| } | - | - |
| } | - | - |
| } | - | - |
| Padding | variable | Padding to byte; shall be set to 0 |
| } | - | - |

# References

[IEEE802.16-Rev2/D4]	IEEE Computer Society and IEEE Microwave Theory and Techniques Society, “DRAFT Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D3 (February 2008). Revision of IEEE Std 802.16-2004 and consolidates material from IEEE Std 802.16e-2005, IEEE Std 802.16-2004/Cor1-2005, IEEE Std 802.16f-2005 and IEEE Std802.16g-2007.