| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ROHC updates |  |
| Date Submitted | 2008-05-14 |  |
| Source(s) | Erik Colban Yair Bourlas NextWave Wireless* Jung Ho Han Geunhwi Lim Samsung Electronics, Co., Ltd. | Voice: E-mail: ecolban@nextwave.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> E-mail: jungho.han@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #26b as announced in IEEE 802.16-08/006 |  |
| Abstract | Text related to ROHC needs to be improved to ensure interoperability between the BS and the SS. |  |
| Purpose | Discuss and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ROHC Updates

Erik Colban

Yair Bourlas

NextWave Wireless

Jung Ho Han

Geunhwi Lim

Samsung Electronics, Co., Ltd.

# Background

802.16Rev2/D4 is not clear on several points related to ROHC. This may create interoperability problems between an SS and a BS.

# Proposed Changes

- The MS and the BS need to negotiate capabilities during REG-REQ/RSP. The code points used when ROHC is supported need to be clarified. We suggest that ROHC not be considered to be a specific convergence sub-layer type, but rather part of the IP convergence sub-layer types. ROHC capability is negotiated during REG-REQ/RSP and use of ROHC is negotiated during service flow establishment. The MS and BS indicate CS types that they support during REG-REQ/RSP, e.g., IPv4, IPv6, Packet IP, etc.

- The SS and BS need to negotiate the use of ROHC during service flow addition (DSA). The method used needs to be clarified. We suggest that this is negotiated by bit 7 of Request/Transmission Policy parameter in DSA-REQ/RSP.

- The ROHC Parameter Payload TLV encoding needs to be specified.

# Proposed Changes in 802.16Rev2/D4

On page 39, delete lines 5 – 8.

On page 39, line 48, Section 5.2.6.1, add new text and figure as follows:

ROHC (refer to RFC 3095) may be used instead of PHS to compress IP headers. The MS and the BS signal enabling of ROHC by setting bit 7 of Request/Transmission Policy (see 11.13.12) to 0. When ROHC is enabled for a service flow, the service flow constitutes what in RFC 3095 is referred to as a ROHC channel. Two service flows cannot share a ROHC channel, and two ROHC channels cannot share the same service flow. All IP packets that are classified onto a service flow for which ROHC has been enabled shall pass through the ROHC compressor on the sender side, and the decompressor on the receiver side.

ROHC compression and decompression operation shall be performed in accordance with RFC 3095, RFC 3759, RFC 3243, RFC 4995, RFC 3843, RFC 4996. To enable ROHC, the following two steps are required.

- Capability negotiation during REG-REQ/RSP message exchange to determine whether ROHC is supported.

- Indication in DSA-REQ/RSP messages to enable ROHC for the service flow.

Figure XX

On page 39, line 58-61, change paragraph as follows:

The CS supports SDUs in two formats that facilitate robust compression of IP and higher layer headers. These This formats are ROHC (RFC 3095) and is ECRTP (RFC 3545) and are is referred to as the IP-header-compression CS PDU formats.

On page 40, lines 26-43, delete section 5.2.7.2. as follows:

5.2.7.2 Compressed-IP-header classification rules

The term ‘ROHC channel’ is defined in RFC3095 and further clarified in RFC3759. The 802.16 standard does not attempt to redefine the definition of ‘ROHC Channel’. A single ROHC channel, which may have multiple ROHC contexts, shall have a one-to-one mapping to a single service Flow (SFID). Since there is a one-one-mapping between a ROHC channel and an SF ID, there is no need to have any additional classifiers associated with that Service Flow. The method of associating a ROHC channel with a Service Flow is left to the implementation. One or more ROHC channels can be established for an SS. For a Service Flow mapped to a ROHC Channel, the ROHC parameters associated with the ROHC Channel shall be negotiated by including the ROHC Parameter Payload TLV (11.13.38) in the DSA-REQ/RSP messages (for a new Service Flow creation) or the DSC-REQ/RSP messages (for an existing Service Flow).

On page 1159, change table 575 as follows:

| Type | Parameter | Type | Parameter |
| --- | --- | --- | --- |
| 1 | ARQ Parameters | 25 | Compressed CID Update Encodings |
| 2 | SS Management Support | 26 | Method for Allocating IP Address for the Secondary Management Connection |
| 3 | IP Management Support | 27 | Handover Supported Field |
| 4 | IP Version | 28 | System Resource Retain Timer |
| 5 | Secondary Management CID | 29 | HO Process Optimization MS Timer |
| 6 | The Number of Uplink TansportCID supported | 30 | MS Handover Retransmission Timer |
| 7 | Classification, PHS Options, SDU Encapsulation Support | 31 | Mobility Features Supported |
| 8 | Maximum Number of Classifiers | 32 | Sleep Mode Recovery Time |
| 9 | PHS Support | 33 | MS-PREV-IP-ADDR |
| 10 | ARQ Support | 34 | SKIP-ADDR-ACQUISTION |
| 11 | DSx Flow Control | 35 | SAID Update Encodings |
| 12 | Reserved | 36 | Total Number of Provisional Service Flow |
| 13 | MCA Flow Control | 37 | Idle Mode Timeout |
| 14 | Multicast Polling Group CID Support | 38 | Reserved |
| 15 | The Number of Downlink Trans- Port CID Supported | 39 | Reserved |
| 16 | Reserved ROHC Support | 40 | ARQ-ACK Type |
| 17 | Reserved | 41 | MS HO Connections Parameters Process- ing Time |
| 18 | Reserved | 42 | MS HO TEK Processing Time |
| 19 | Reserved | 43 | MAC Header and Subheader Support |
| 20 | Maximum MAC Data per Frame Support | 44 | SN Reporting Base |
| 21 | Packing Support | 45 | MS timer T4 |
| 22 | MAC Extended rtPS Support | 46 | Handover Indication Readiness Timer |
| 23 | Maximum Number of Bursts Transmitted Concurrently to the MS | 47 | BS Switching Timer |
| 24 | CID Update Encodings | 48 | Power Saving Class Capability |

On page 1166, insert new section 11.7.8.11 as follows:

11.7.8.11 ROHC support

This parameter is used by the SS or BS to indicate support for ROHC.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 16 | 1 | 0 : ROHC not supported 1 : ROHC supported 2~255 : Reserved | REG-REQ, REG-RSP |

The default value is 0 (ROHC not supported).

On page 1162, lines 5-51, modify table by marking bits 9, 11 as “Reserved”.

On page 1239, lines 13-30, modify table as follows:

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].12 | 1 | Bit 0 : Service flow shall not use broadcast BR opportunities (Uplink only) Bit 1 : Service flow shall not use multicast BR opportunities. (Uplink only) Bit 2 : The service flow shall not piggyback requests with data. (Uplink only) Bit #3 – The service flow shall not fragment data Bit #4 – The service flow shall not suppress payload headers (CS parame- ter) If bit #4 is set to’0’ and both the SS and the BS support PHS (accord- ing to section 11.7.7.3), each SDU for this service flow shall be pre- fixed by a PHSI field, which may be set to 0 (see section 5.2). If bit #4 is set to ‘1’, none of the SDUs for this service flow will have a PHSI field. Bit 5 : The service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs Bit 6 : The service flow shall not include CRC in the MAC PDU. Bit 7 : Reserved, shall be set to zero The service flow shall not compress payload headers using ROHC. If bit #7 is set to’0’ and both the SS and the BS support ROHC (accord- ing to section 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 7 is set to ‘1’, none of the SDUs shall be compressed. | DSA-REQ DSA-RSP DSA-ACK |

On page 1244, lines 9-28, modify table by marking values 10 and 12 as “Reserved”.

On page 1244-1245, lines 41-8, modify table by marking values 108, 110 as “Reserved”.

On page 1262, section 11.19.38, change the section as follows:

11.13.38 ROHC Parameter Payload

This attribute contains the payoad used in the upper ROHC compression layer. The MAC layer does not interpret this attribute. This compound parameter contains the ROHC channel parameters. All parameters pertaining to a specific ROHC channel shall be included in the same ROHC Parameter compound TLV. Refer to RFC3095, section 5.1.1, for the definition of these parameters.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].47 | variable | ROHC Parameter Payload Compound | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP |

On page 1262, line 28, add new section 11.13.38.1 ~ 11.13.38.5 as follows:

11.13.38.1 ROHC Max Context ID

This TLV contains the ROHC parameter MAX_CID.  Both entities may include this TLV. The responder shall not send a value that is larger than the value sent by the requestor. If the requestor omits this parameter, the responder shall include this parameter. If only one entity includes this parameter, then that value is the negotiated value. Otherwise, the negotiated value is the lesser of the two values sent by the entities.

| Type | Length | Value |  |
| --- | --- | --- | --- |
| [145/146].47.1 | 2 | Nonnegative integer. | DSA-REQ, DSA-RSP |

11.13.38.2 Large Context IDs

This TLV contains the ROHC parameter LARGE_CIDS. Both entities may include this TLV. The responder shall not send a value that is larger than the value sent by the requestor. If the requestor omits this parameter, the responder shall include this parameter. If only one entity includes this parameter, then that value is the negotiated value. Otherwise, the negotiated value is the lesser of the two values sent by the entities.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].47.2 | 1 | 0: FALSE (Small Context ID) 1: TRUE (Large Context ID) | DSA-REQ DSA-RSP |

11.13.38.3 ROHC Profiles

This TLV contains the ROHC parameter PROFILES. Both entities may include this TLV. The responder shall not include profiles that are not included by the requestor. If the requestor omits this parameter, the responder shall include this parameter. If only one entity includes this parameter, then that set is the negotiated value. Otherwise, the negotiated set of profiles is the smallest set of two the sets sent by entities.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].47.3 | 2n | A set of nonnegative integers, where each integer indicates a 16 bit profile identifier of a ROHC profile supported by the decompressor. | DSA-REQ DSA-RSP |

11.13.38.4 ROHC Feedback Channel

This TLV contains the ROHC parameter FEEDBACK_FOR. The value of this parameter is an SFID. If provided, this parameter indicates to which service flow the FEEDBACK_FOR channel is mapped. Only the BS may send this TLV containing a non-zero value. The MS indicates a request for a FEEDBACK_FOR channel by omitting this TLV.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].47.4 | 4 | 0x00 : no associated ROHC feedback Otherwise : SFID for ROHC feedback | DSA-REQ DSA-RSP DSC-REQ DSC-RSP |

11.13.38.5 ROHC MRRU

This TLV contains the ROHC parameter MRRU. Both entities may include this TLV. The responder shall not send a value that is larger than the value sent by the requestor. If either entity sends a value = 0, or if both entities omit this TLV, then segmentation shall not apply. If only one entity includes this value, then that value is the negotiated value. Otherwise, the negotiated value is the lesser of the two values sent by the entities.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].47.5 | 2 | 0: no segmentation Otherwise: MRRU | DSA-REQ DSA-RSP |