| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for Load Balancing in Rev2 |  |
| Date Submitted | 2008-10-29 |  |
| Source(s) | Toshiyuki Kuze, Kentarou Sawa Mitsubishi Electric Corp Zhifeng (Jeff) Tao, Jinyun Zhang Mitsubishi Electric Research Lab | Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp Voice: 617-621-{7557, 7595} Fax: 617-621-7550 {tao, jzhang}@merl.com |
| Re: | IEEE802.16Rev2/D7 Sponsor Ballot |  |
| Abstract | IEEE 802.16 Repeater and Standard Clarification in Rev2 |  |
| Purpose | For adoption by IEEE802.16 Maintenance TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification for Load Balancing in Rev2

Toshiyuki Kuze, Kentarou Sawa

Mitsubishi Electric Corp

Zhifeng (Jeff) Tao, Jinyun Zhang

Mitsubishi Electric Research Lab

# Introduction

Load balancing has been discussed and addressed in IEEE C80216maint-08/013 [1].  Basically, a preamble index override TLV is introduced in RNG-RSP message.  This TLV is used in conjunction with UL/DL frequency override TLV to identify the specific target BS.  As an alternative, MOB_BSHO-REQ/RSP can be used to initiate load balancing process.

Several issues are still not clear in the current Rev2 draft [2].  Thus, this contribution provides further clarification for load balancing operation.

# RNG-RSP approach

In order to balance load, serving BS may provide multiple candidate target BSs for MS to choose.  Therefore, the BS can include multiple preamble index override TLV and multiple downlink frequency override TLV in the RNG-RSP message.  And if so, the preamble index override TLV and the associated downlink frequency override TLV shall be transmitted in pair.

# MOB_BSHO-REQ/RSP approach

In the current standard, neighbor BSID can be included in MOB_BSHO-REQ/RSP messages by BS to request HO or load balancing.  MS should be aware of the downlink center frequency associated with the given neighbor BSID.  MS may obtain this information by either reading DCD_settings compound TLV in MOB_NBR-ADV message, or by decoding DCD/UCD when performing scanning.

However, it is not mandatory to include the DCD_settings compound TLV in the MOB_NBR-ADV message.  Meanwhile, scanning usually would cause some delay.  Thus, similar to RNG-RSP message, MOB_BSHO-REQ/RSP message can include downlink center frequency information to expedite the load balancing process.

In addition, the name of “neighbor BSID” field in the current MOB_BSHO-REQ/RSP message is not precise.  For example, it is possible that one BS may have multiple radios, and operate on different channels.  One BSID is associated with one such channel.  MS may be instructed to move from one channel to another, but still remains served by the same BS.  Therefore, it is more appropriate to name the field “target BSID”, instead of “neighbor BSID”.

# Proposed Text

6.3.2.3.6 RNG-RSP (ranging response) message

[Insert the following paragraph after “Preamble Index Override”]

Preamble Index Override

Preamble Indices of new target BS(s) where the MS should redo ranging. If the TLV includes two or more Preamble Indices, the first one in the list is the most preferable and the second is the next preferable. When the TLV is used with Downlink frequency override TLV, the MS should redo ranging on the new DL channel identified by the Preamble Indices.

For load balancing purpose, the BS can include multiple pairs of preamble index override TLV and multiple downlink frequency override TLV in RNG-RSP message.

6.3.2.3.47 MOB_BSHO-REQ (BS HO request) message

[Change Table 150 as follows]

Table 150—MOB_BSHO-REQ message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_BSHO-REQ_Message_format() { | - | - |
| Management Message Type = 56 | 8 | - |
| Mode | 3 | 0b000: HO request 0b001: MDHO/FBSS request: Anchor BS update with CID update 0b010: MDHO/FBSS request: Anchor BS update without CID update 0b011: MDHO/FBSS request: Diversity set update with CID update 0b100: MDHO/FBSS request: Diversity set update without CID update 0b101: MDHO/FBSS request: Diversity set update with CID update for newly added BS 0b110: MDHO/FBSS request: Diversity set update with CID update and CQICH allocation for newly added BS 0b111: Reserved |
| Padding | 4 | Shall be set to zero |
| Frequency TLV indication | 1 | 1: center frequency TLV is included 0: center frequency TLV is not included |
| If (Mode == 0b000) { | - | - |
| …. | …. | … |
| for (j = 0 ; j < N_Recommended; j++) { | - | N_Recommended can be derived from the known length of the message. |
| Neighbor Target BSID | 48 | - |
| Target downlink center frequency | 32 | The downlink center frequency used by the BS that has the neighbor BSID |
| Service level prediction | 8 | - |
| Preamble index/Subchannel Index | 8 |  |
| ... | …. | … |

... ...

Neighbor target BSID

Same as the Basic Station ID parameter in the DL-MAP message of the neighbor target BS

... ...

6.3.2.3.49 MOB_BSHO-RSP (BS HO response) message

[Change Table 152 as follows]

Table 152—MOB_BSHO-RSP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_BSHO-RSP_Message_format() { | - | - |
| Management Message Type = 58 | 8 | - |
| Mode | 3 | 0b000: HO request 0b001: MDHO/FBSS request: Anchor BS update with CID update 0b010: MDHO/FBSS request: Anchor BS update without CID update 0b011: MDHO/FBSS request: Diversity set update with CID update 0b100: MDHO/FBSS request: Diversity set update without CID update 0b101: MDHO/FBSS request: Diversity set update with CID update for newly added BS 0b110: MDHO/FBSS request: Diversity set update with CID update and CQICH allocation for newly added BS 0b111: Reserved |
| Reserved | 4 | Shall be set to zero |
| Frequency TLV indication | 1 | 1: center frequency TLV is included 0: center frequency TLV is not included |
| If (Mode == 0b000) { | - | - |
| …. | …. | … |
| for (j = 0 ; j < N_Recommended; j++) { | - | N_Recommended can be derived from the known length of the message. |
| Neighbor Target BSID | 48 | Neighbor BSs shall be presented in an order so that the first presented is the one most recommended and the last presented is the least recommended. |
| Target downlink center frequency | 32 | The downlink center frequency used by the BS that has the neighbor BSID |
| Preamble index/ Preamble Present and Subchannel Index | 8 | For the OFDMA PHY this parameter defines the PHY-specific preamble for the neighbor BS. For the OFDM PHY the 5 LSB contain the active DL subchannel index for the neighbor BS. The 3 MSB shall be reserved and set to 0b000. |
| ... | …. | … |

……

Neighbor Target BSID

Same as the Base Station ID parameter in the DL-MAP message of neighbor target BS.

……

# Reference

- Jaejeong Shim et al., “Support of load balancing to enhance service availability”, IEEE C80216maint-08/013, Orlando, FL March 2008

- “DRAFT Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”, P802.16Rev2/D7 October 2008