| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to add CDR mode into DL Persistent A-MAP IE ( 15.3.6.5.2.8) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Isamu YOSHII Takaaki KISHIGAMI Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: yoshii.isamu@jp.panasonic.com kishigami.takaaki@jp.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a: P802.16m/D2 |  |
| Abstract | This contribution proposes some modifications on Section 15.3.6.5.2.8 DL PA A-MAP IE |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to add CDR mode into DL Persistent A-MAP IE ( 15.3.6.5.2.8)

Isamu YOSHII, Takaaki KISHIGAMI

Panasonic Corporation

# Introduction

In D2, CDR is adopted as one of MIMO mode(MEF) in DL Basic assignment A-MAP IE. Considering on the consistency, it is useful to add CDR to DL PA A-MAP IE.

# Reference

[1]	IEEE C802.16m-09/1382r4, Description of MIMO Encoding Format for CDR

[2]	IEEE P802.16m/D2, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, October 2009

# Proposed text

- Editorial notes: page 379 in D2

- Current text in IEEE P802.16m/D2

Remove text from IEEE P802.16m/D2

- Add new text to IEEE P802.16m/D2

-------------------------------  Start text proposal  ---------------------------------------------------

Table 812—DL Individual Persistent A-MAP IE

| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |  |
| --- | --- | --- | --- |
| If (Allocation Period==0b00){ |  |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |  |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |  |
| HFA | 5 | Explicit Index for HARQ Feedback Allocation to acknowledge receipt of deallocation A-MAP IE |  |
| Reserved | TBD |  |  |
| } else if (Allocation Period != 0b00){ |  |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |  |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a CDR |  |
| if (MEF == 0b01){ | Parameters for vertical encoding |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |  |
| Reserved | 1 |  |  |
| } else if(MEF == 0b10){ | Parameters for horizontal encoding |  |  |

-------------------------------  End text proposal  ---------------------------------------------------

- Editorial notes: page 383 in D2

- Current text in IEEE P802.16m/D2

Remove text from IEEE P802.16m/D2

- Add new text to IEEE P802.16m/D2

-------------------------------  Start text proposal  ---------------------------------------------------

Table 813—DL Composite Persistent A-MAP IE

| Syntax | Size in bit | Description/Notes |
| --- | --- | --- |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a CDR |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } |  |  |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| if(Nt == 2){ |  |  |

-------------------------------  End text proposal  ---------------------------------------------------