| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IP address refresh indicator and Host configuration parameters (15.2.3.2) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Jin Lee, Ronny Yongho Kim, Kiseon Ryu, Jin Sam Kwak LG Electronics | Email : {jins978,ronnykim, ksryu}@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot#30a |  |
| Abstract | This contribution describes IP address refresh indicator and Host configuration parameters to be used during HO |  |
| Purpose | Update P802.16m/D2 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IP address refresh indicator and Host configuration parameters (15.2.3.2)

Jin Lee, Ronny Kim, Kiseon Ryu, Jin Sam Kwak

LG Electronics

- Text proposal for inclusion in the 802.16m amendment working document

-------------------------------  Text Start  ---------------------------------------------------

[Note to Editor: Please add the following underlined text in the Table 675 of AAI_RNG-RSP on page 39]

| HO Process Optimization | Identifies reentry process management messages that may be omitted during the current HO attempt due to the availability of MS service and operational context information obtained by means that are beyond the scope of this standard, and the MS service and operational status post-HO completion. The AMS shall not enter normal operation with target ABS until completing receiving all network reentry, MAC management message responses as indicated in HO process optimization. | It shall be included when the AMS is attempting to perform network reentry or HO and the target ABS wishes to identify reentry process management messages that may be omitted during the current HO attempt Bit #0: Omit AAI_SBC-REQ/RSP management message during reentry processing Bit #1: Omit PKM Authentication phase except TEK phase during current reentry processing Bit #2: Seamless HO Bit #3: Omit triggering a higher layer protocol to refresh traffic IP address Bit #4 – Bit #7: Reserved |
| --- | --- | --- |
| Host configuration capabilities and its parameters | IPv4-Host-Address IE | The allocated IPv4 Host Address for the AMS. Size: 4 octet |
|  | IPv6-Home-Network-Prefix IE | The allocated IPv6 Home Network Prefix for the AMS. Size: 8 octet |
|  | IPv4-DNS-Server-Address IE | DNS Server Address (IPv4) Size: 4 octet |
|  | IPv6-DNS-Server-Address IE | DNS Server Address (IPv6) Size: 16 octet |
|  | IPv4-P-CSCF-Address IE | P-CSCF Address (IPv4) Size: 4 octet |
|  | IPv6-P-CSCF-Address IE | P-CSCF Address (IPv6) Size: 16 octet |

-------------------------------  Text End  ---------------------------------------------------