| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to E-MBS Control Information Elements |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Eunkyung Kim, Jaesun Cha, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | “P802.16m/D2,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30a: Announcement, IEEE 802.16-09/0057 |  |
| Abstract | E-MBS Control Information Elements on IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Changes to E-MBS Control Information Elements in IEEE 802.16m

Eunkyung Kim, Jaesun Cha, Soojung Jung, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# 1 Instructions

Section 15.3.6.5.3 in IEEE 802.16m Amendment Draft Standard [1] describes the E-MBS Control Information Elements. E-MBS Control Information needs to provide more clearly focusing on the pointer the location of E-MBS Data.

# 2. Text Proposal for the 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Modify the 15.3.6.5.3 of 802.16m/D2 as follows]

15. 3.6.5.3 E-MBS Control Information Elements

The resource mapping for EMBS allows are defined in EMBS-DATA_IE’s as defined in Table 822.

Table 822-E-MBS DATA Information Elements

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| E-MBS-DATA_IE() { |  |  |
| No. of Multicast STIDs+FIDs | [TBD] | Total number of MBS streams in the IE |
| For (i=0; i<No. of Multicast STID+FID’s,; i++){ |  |  |
| MSTID+FID | 16 | Multicast STID + Flow ID of MBS Stream |
| Isize-offset | 5 | Depends on supported modes, 32 modes assumed as baseline Offset used to compute E-MBS MAP size index |
| E-MBS MIMO Mode | [TBD]1 | E-MBS MIMO Encoding Format 0b0: MIMO mode 0 0b1: MIMO mode 1 When MIMO mode-1 is used for E-MBS, the number of streams shall be no more than 2. |
| E-MBS Frame/AAI subframe offset | [TBD] | Includes the location and Allocation size |
| Superframe offset | m+3 | LSB of Superframe number, where m is the value of the MSI Length(m) in the E-MBS MAP. |
| Frame number | 2 | Frame number in the Superframe offset. 0b00: The 1st frame in the super frame 0b01: The 2nd frame in the super frame 0b10: The 3rd frame in the super frame 0b11: The 4th frame in the super frame |
| Subframe number | 3 | Subframe number in the Frame number. 0b000: 1st subframe of the frame …. 0b111: 8th subframe of the frame |
| E-MBS Resource Indexing | [TBD]11 | Includes the location and Allocation size 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index. |
| Allocation Period (p) | [TBD]3 | Frame offset for multiple transmission instances (in number of frames) Transmits every 2p frame from the Frame number till to the next E-MBS DATA IE. |
| } |  |  |
| } |  |  |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” October 2009