| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed clarifications in CMFEH section 15.2.2.2.7 |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics Wookbong Lee, Bin-Chul Ihm LG Electronics | d.mazzarese@samsung.com wbong@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | This contribution proposes clarifications to the usage of the correlation matrix feedback extended header |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed clarifications in CMFEH section 15.2.2.2.7

David Mazzarese, Hokyu Choi
Samsung Electronics Co., Ltd.

Wookbong Lee, Bin-Chul Ihm
LG Electronics

# Introduction

This contribution proposes clarifications to the usage of the correlation matrix feedback extended header. The proposed change leaves the flexibility to feedback to correlation matrix using either the CMFEH or a MAC message, even when the only feedback allocated is for the correlation matrix.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D2, Oct 2009.

# Text proposal for inclusion in the 802.16m amendment

<< Modify section “15.2.2.2.7 Correlation matrix feedback extended header (CMFEH)” on page 26 line 60 >>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

15.2.2.2.7 Correlation Matrix Feedback Extended Header (CMFEH)

This Correlation matrix feedback extended header format is defined in Table 665. This extended header may be used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3.

Table 665 - Correlation matrix feedback extended header format CMFEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CMFEH() { |  |  |
| LAST | 1 | Set to “1” if last EH, else, set to “0” |
| Type | TBD | CMFEH type |
| For (i=1; i ≤ Nt Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in <<<Table 713>>> section 15.3.7.2.5.6 |
| For (j=i+1; j ≤ Nt Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in <<<Table 714>>> section 15.3.7.2.5.6 |
| } |  |  |
| } |  |  |
| Reserved | Variable | For byte alignment |
| } |  |  |

-------------------------------  End of Text Proposal   ---------------------------------------------------