| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Modification on Broadcast Assignment A-MAP IE in P802.16m/D3 (16.3.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Email: lei.huang@sg.panasonic.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b: P802.16m/D3 |  |
| Abstract | The contribution proposes modification to Broadcast Assignment A-MAP IE |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Modification on Broadcast Assignment A-MAP IE in P802.16m/D3 (16.3.6)

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii

Panasonic Corporation

- Introduction

In [1], all broadcast MAC control messages to be transmitted in a DL subframe shall be included into a broadcast burst, and a Broadcast Assignment A-MAP IE may be used for the resource allocation of such a broadcast burst. As shown in [1], the current design of Broadcast Assignment A-MAP IE is still at the early stage. In this contribution, we provide a modification to the content of Broadcast Assignment A-MAP IE for the purpose of improving the design of Broadcast Assignment A-MAP IE.

- Proposed Modification

In [1], a 8-bit signaling field “Assignment A-MAP size” is used for signaling the number of assignment A-MAP IEs in each assignment A-MAP group as shown in Table 805 to Table 808 in order to limit the signaling overhead in the non-user specific A-MAP IE within a reasonable range. As a result, the actual number of assignment A-MAP IEs in each assignment A-MAP group may be less than the number indicated by the signaling field “Assignment A-MAP size”. This implies that an assignment A-MAP group may include unused MLRUs. The unused MLRUs in an assignment A-MAP group would incur extra blind detection to AMS. This would increase false detection probability and power consumption of AMS.

According to the assignment A-MAP group size indication tables as shown in Table 805 to Table 808 in [1], the assignment A-MAP group 1 is always with the most robust MCS. Furthermore, as shown in [2], the exact size of the assignment A-MAP group 1 is always signaled. However, for non-FFR configuration, the size of assignment A-MAP group 2 may be not exactly signaled. For FFR configuration, the sizes of the assignment group 2 and assignment A-MAP group 3 may be not exactly signaled. In other words, for non-FFR configuration, the assignment A-MAP group 2 may include unused MLRUs. And for FFR configuration, the assignment A-MAP group 2 and assignment A-MAP group 3 may include unused MLRUs.

Note that if a Broadcast Assignment A-MAP IE is present in a DL subframe, it has to be received by all AMS. As a result, for non-FFR configuration, Broadcast Assignment A-MAP IE should be located in the assignment A-MAP group 1 since assignment A-MAP group 1 is with the most robust MCS. For FFR configuration, Broadcast Assignment A-MAP IE should be located in either the assignment A-MAP group 1 or the assignment A-MAP group 3.

We propose to use a 2-bit signaling field to signal the number of unused MLRUs in the assignment A-MAP group 2 in Broadcast Assignment A-MAP IE. In FFR case, the number of unused MLRUs in the assignment A-MAP group 3 is also signaled in Broadcast Assignment A-MAP IE using another 2-bit signaling field. The reason for suggesting 2-bit signaling field is that the average number of extra blind detection is smaller than 3 in any case, as shown in Figure 1 to Figure 4 in [2].

In non-FFR case, AMS can always decode the assignment A-MAP group 1 first. If Broadcast Assignment A-MAP IE is present, AMS can avoid extra blind detection in the assignment A-MAP group 2. This would decrease false detection probability and power consumption of AMS.

In FFR case, AMS can decode the assignment A-MAP group 1 and the assignment A-MAP group 3 first. If Broadcast Assignment A-MAP IE is present, AMS can also avoid extra blind detection in the assignment A-MAP group 2 and assignment A-MAP group 3.

- Reference

- [1]	IEEE P802.16m/D3, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, December 2009

- [2]	IEEE C802.16m-09/1940r1, A-A-MAP Group Size Indication in Non-User Specific A-MAP, August 2009

-------------------------------  Start Text Proposal ---------------------------------------------------

- [Proposed text modification (16.3.6.5.2.4.13, page 443)]

# 16.3.6.5.2.4.13 Broadcast assignment A-MAP IE

Table 829 – Broadcast Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast_ Assignment_ A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| MCS (TBD) |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| UMG2 | 2 | Number of unused MLRUs in the assignment A-MAP group 2 0b000: no unused MLRUs 0b001:1 unused MLRUs 0b010: 2 unused MLRUs 0b011: 3 unused MLRUs |
| If (DFPC ≠ 0){ |  |  |
| UMG3 | 2 | Number of unused MLRUs in assignment A-MAP group 3 0b000: no unused MLRUs 0b001:1 unused MLRUs 0b010: 2 unused MLRUs 0b011: 3 unused MLRUs |
| } |  |  |
| Reserved | TBD | Reserved bits |
| } |  |  |

-------------------------------------- End of Text Proposal -------------------------------------------