| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of SCH and SCEH (16.2.2) |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim and Jin Sam Kwak LG Electronics Jaesun Cha ETRI | gwpark@lge.com jscha@etri.re.kr |
| Re: | IEEE 802.16m-09/0073, “IEEE 802.16 Working Group Letter Ballot #30b: Announcement” |  |
| Abstract | The contribution proposes a clarification on multicarrier sleep mode operation in 16m/D3. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D4 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of SCH and SCEH (16.2.2)

Giwon Park, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim and Jin Sam Kwak

LG Electronics

Jaesun Cha

ETRI

- Introduction

The 16m/D3 draft [1] does not properly describe a Sleep Control Header (SCH) and Sleep Control Extended Header (SCEH). This contribution proposes a clarification on SCH and SCEH.

Reference

[1] P802.16m/D3, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, December 2009

- Suggested remedy

-------------------------------  Text Start  ---------------------------------------------------

[Change table 660 in section 16.2.2.1.3.4 Sleep Control Header (SCH):]

16.2.2.1.3.4 Sleep Control Header (SCH)

Table 660 – Sleep Control Header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Sleep Control Header Format () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC Signaling Header Type = 0b0011 |
| SCH sub-type | 23 | 0b00 = Listening Window control 0b01 = Resume Sleep Cycle Indication 0b10 = Sleep Cycle control 0b11 = reserved 0b000 = Listening Window control 0b001 = Resume Sleep Cycle Indication 0b010 = Sleep cycle configuration switch 0b011 = Sleep Cycle control 0b100 = Multi-Carrier Listening Window control 0b101~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e. acknowledgment) to the request . |
| if (SCH sub-type == Listening Window Control) { | - | - |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| if (Listening Window End or Extension ==0) { |  |  |
| Last frame of Extended Listening Window | 7 | The value is only valid with Listening Window End or Extension is set to 0; LSB of frame sequence. Indicate the frame that extended listening window is terminated; |
| } | - | - |
| } else if (SCH sub-type == Resume Sleep Cycle Indication) { | - | - |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { |  |  |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 7 | Number of frames in the future from the frame containing this SCEH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e. range is 1 to 128). |
| } |  |  |
| } |  |  |
| else if (SCEH sub-type == Sleep Cycle configuration change switch) { | - | - |
| New sleep parameter configuration ID (SCID) | 4 | The sleep ID corresponds to the new sleep cycle parameters setting to be switched. |
| Start Frame Offset for new sleep cycle configuration | 3 | Number of frames in the future from the frame containing this SCEH at which the sleep cycle switch will occur. Frame offset is value of this field plus one (i.e. range is 1 to 8). Least Significant 3 bits of Frame Number in which sleep cycle setting is to be applied. |
| } | - | - |
| else if (SCEH sub-type == Sleep Cycle control) { |  |  |
| Next Sleep Cycle Flag (NSCF) | 2 | 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |
| if (NSCF == 0b10) |  |  |
| { |  |  |
| New Initial Sleep Cycle | 4 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. Otherwise, the current Sleep Cycle may be reset to Initial Sleep Cycle or may be updated to min (2 x Previous Sleep Cycle, Final Sleep Cycle). |
| } |  |  |
| } |  |  |
| else if (SCH sub-type == Multi-Carrier Listening Window control) { |  |  |
| Target Carrier Index Bitmap | 4 | If nth bit is set to 1, it indicates that DL data transmission on the secondary carrier of which logical carrier index is equal to (n+1) ends. |
| } |  |  |
| Padding | variable | For byte alignment |
| } | - | - |

[Delete the current table 668 (SCEH) and replace the table 668 to following proposed table:]

16.2.2.2.5 Sleep Control Extended Header (SCEH)

Table 668 – SCEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Sleep Control Extended Header Format () { |  |  |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC Signaling Header Type = 0b0011 |
| SCEH sub-type | 3 | 0b000 = Listening Window control 0b001 = Resume Sleep Cycle Indication 0b010 = Sleep cycle configuration switch 0b011 = Sleep Cycle control 0b100 = Multi-Carrier Listening Window control 0b101~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e. acknowledgment) to the request . |
| if (SCEH sub-type == Listening Window Control) { | - | - |
| Listening Window End or Extension | 1 | 0 = Listening Window End Indication 1 = Listening Window Extension Indication |
| if (Listening Window End or Extension ==0) { |  |  |
| Last frame of Extended Listening Window | 7 | The value is only valid with Listening Window End or Extension is set to 0; LSB of frame sequence. Indicate the frame that extended listening window is terminated; |
| } | - | - |
| } else if (SCEH sub-type == Resume Sleep Cycle Indication) { | - | - |
| Scheduled Sleep Cycle Interruption included | 1 | 0 = no scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication 1 = scheduled Sleep Cycle interruption is included with the Resume Sleep Cycle Indication |
| if (Scheduled Sleep Cycle Interruption included == 1) { |  |  |
| Start Frame Offset for Scheduled Sleep Cycle Interruption | 7 | Number of frames in the future from the frame containing this SCEH at which the scheduled Sleep Cycle interruption will occur. Frame offset is value of this field plus one (i.e. range is 1 to 128). |
| } |  |  |
| } |  |  |
| else if (SCEH sub-type == Sleep Cycle configuration switch) { | - | - |
| SCID | 4 | The sleep ID corresponds to the new sleep cycle setting to be switched. |
| Start Frame Offset for new sleep cycle configuration | 3 | Least Significant 3 bits of Frame Number in which sleep cycle setting is to be applied. |
| } | - | - |
| else if (SCEH sub-type == Sleep Cycle control) { |  |  |
| Next Sleep Cycle Flag (NSCF) | 2 | 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |
| if (NSCF == 0b10) |  |  |
| { |  |  |
| New Initial Sleep Cycle | 4 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. |
| } |  |  |
| } |  |  |
| else if (SCH sub-type == Multi-Carrier Listening Window control) { |  |  |
| Target Carrier Index Bitmap | 4 | If nth bit is set to 1, it indicates that DL data transmission on the secondary carrier of which logical carrier index is equal to (n+1) ends. |
| } |  |  |
| Padding | variable | For byte alignment |
| } | - | - |

-------------------------------  Text End  ---------------------------------------------------