| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the Reserved STID (D3 - 16.2.1.2.1) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes text for reserved STID. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the Reserved STID (D3 - 16.2.1.2.1)

Heejeong Cho, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the reserved STID. There is no description of reserved STID in the IEEE 802.16m DRAFT Amendment [1]. We should reserve STIDs for Broadcast Assignment A-MAP IE (for all AMSs), BR ACK A-MAP IE (for AMSs which perform contention-based bandwidth request) and Assignment A-MAP IE for AAI_RNG-ACK (for AMSs which perform contention-based ranging) in order to reduce AMS unnecessary processing.

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D3, page 19, line 30, make the following changes to subclause 16.2.1.2.1]

16.2.1.2.1 Station Identifier (STID)

The ABS shall assign a 12 bit STID to each AMS during network entry <15.2.15>. The STID shall uniquely identify the AMS in the domain of the ABS. An AMS may be assigned a new STID during network reentry. There are several STIDs defined in Table xxx that have specific meaning. These STIDs shall not be used for any other purposes.

Table xxx—Well-known STIDs

| Value | Description |
| --- | --- |
| 0x000 | Used when AAI_RNG-ACK is sent by ABS in response to the contention-based random access ranging request. |
| 0x001 | Used when BR ACK A-MAP IE is sent by ABS in response to the contention-based random access bandwidth request. |
| 0x002 – 0xFFE | Assigned to each AMS by ABS during network entry. |
| 0xFFF | Used when Broadcast Assignment A-MAP IE is sent by ABS. |

[On the IEEE P802.16m/D3, page 38, line 21, make the following changes to subclause 16.2.3]

Table 673—MAC Control Messages

| Message name | Message description | Security | Connection |
| --- | --- | --- | --- |
| AAI_SCD | System configuration descriptor | N/A | Broadcast |
| AAI_SII-ADV | Service Identity Information Advertisement | N/A | Broadcast |
| AAI_RNG-REQ | Ranging Request | Null: during ranging procedure when there is no SA already established or pre-updated. CMAC: all other cases | Initial Ranging or Unicast |
| AAI_RNG-RSP | Ranging Response | Null: during ranging procedure when there is no primary SA already established or pre-updated. Encrypted/ICV: all other cases in response to the AAI_RNG-REQ message | Initial Ranging or Unicast |
| AAI_RNG-ACK | Aggregated CDMA Ranging Acknowledgement | N/A | Broadcast |

[On the IEEE P802.16m/D3, page 48, line 35, make the following changes to subclause 16.2.3.3]

16.2.3.3 AAI_RNG-ACK

The AAI_RNG-ACK message is sent by the ABS in response to the CDMA ranging request during initial ranging, period ranging and HO ranging in order to provide PHY-level adjustment (e.g. timing offset, power level and frequency offset). Upon reception of ranging codes, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE using the well-known broadcast STID 0x000.

-----------------------------------------------------------End of the Proposal Text ---------------------------------------------------------