| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the E-MBS Configuration Indicators (16.9.3) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “P802.16m/D3,” in response to the IEEE 802.16 Working Group Letter Ballot Recirc #30b: Announcement, IEEE 802.16-09/0073 |  |
| Abstract | E-MBS Configuration Indicators on the IEEE 802.16m Amendment Draft Standard |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on the E-MBS Configuration Indicators (16.9.3)

Eunkyung Kim, Soojung Jung, Jaesun Cha, Sungcheol Chang, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

# Instructions

This contribution provides configuration indicators supporting E-MBS in IEEE 802.16m Amendment Draft Standard [1]. The configuration indicator is transmitted to AMS as a form of AAI_E-MBS-CFG MAC Control Message. For the efficient E-MBS service, it indicates the location of E-MBS MAP for each E-MBS Zone including timing offset (i.e., superframe/frame/subframe offset) and the spanning information of E-MBS MAP.

# Text Proposal for the IEEE 802.16m Amendment Draft Standard

Note:

The text in BLACK color: the existing text in the 802.16m Amendment Draft Standard

The text in RED color: the removal of existing 802.16m Amendment Draft Standard Text

The text in BLUE color: the new text added to the 802.16m Amendment Draft Standard Text

--------------Start of Proposed Text---------------------------------------------------------------------

## [Modify the 16.9.3.1 of 802.16m/D3 as follows.]

16.9.3 E-MBS Protocol Features and Functions

16.9.3.1 E-MBS Configuration Indicators

The information regarding E-MBS configuration is transmitted periodically to the AMSs interested in E-MBS using a MAC managementControl message called the AAI-E-MBS_CFG message. The E-MBS configuration indicators specify the resources reserved for E-MBS traffic in the downlink physical resources and additional information necessary for E-MBS operation.

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isize-offset, E-MBS MAP MIMO Mode and MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. AAI_E-MBS-CFG shall be advertised at a pre-determined frame offset in a superframe.

Table 944—AAI_E-MBS-CFG message Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI-_E-MBS-CFG_Message_Format() { |  |  |
| MAC Control Message Type | 8 | AAI_E-MBS-CFG |
| Num_E-MBS_Zones | 3 | Number of E-MBS Zones with which the ABS is associated. |
| for (i = 0; i< Num_E-MBS_Zones; i++) { |  |  |
| E-MBS_Zone_ID | 7 | The E-MBS_Zone_ID to which this E-MBS MAP applies. |
| MSI Length(m) | 2 | The length of an MSI in units of the number of superframes 0b00: 2 superframes, 40 ms 0b01: 4 superframes, 80 ms 0b10: 8 superframes, 160 ms 0b11: 16 superframes, 320 ms |
| E-MBS time offset (superframe number) | m | m LSB of Superframe number, where m is the value of MSI Length(m). According to the MSI length , m = 1 if MSI Length is 2 superframes m = 2 if MSI Length is 4 superframes m = 3 if MSI Length is 8 superframes m = 4 if MSI Length is 16 superframes |
| E-MBS time offset (frame index) | 2 | Frame number in the superframe 0b00 : 1st frame 0b01: 2nd frame 0b10: 3rd frame 0b11: 4th frame |
| E-MBS time offset (subframe index) | 3 | Subframe offset in the frame 0b000: 1st subframe 0b001: 2nd subframe … 0b111: 8th subframe |
| # of subframes of E-MBS | [TBD] | # of subframes, where E-MBS MAP may span one or more subframes |
| E-MBS MAP Resource Index | [TBD]11 | Resource index includes location and allocation size. |
| E-MBS MAP ISizeOffset | 5 |  |
| } |  |  |
| } |  |  |

## [Add the following row to the Table 673 in the Section 16.2.3 of 802.16m/D3.]

| AAI_E-MBS-CFG | E-MBS Configuration | Null | Broadcast |
| --- | --- | --- | --- |

--------------End of Proposed Text ---------------------------------------------------------------------

# References

- IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” December 2009