| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on AAI_SON-ADV message (16.2.3.6) |  |
| Date Submitted | 2009-12-30 |  |
| Source(s) | Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Whai-En Chen, Shih-Yuan Cheng National Ilan University Kanchei (Ken) Loa, Chun-Yen Hsu, Youn-Tai Lee, Tsung-Yu Tsai, Chiu-Wen Chen Institute for Information Industry Yang-Han Lee, Yih Guang Jan Tamkang University | stsheu@ce.ncu.edu.tw wechen@niu.edu.tw loa@iii.org.tw |
| Re: | P802.16m/D3 |  |
| Abstract | Comments on AAI_SON-ADV message (16.2.3.6) |  |
| Purpose | To be discussed and adopted by TGm into P802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on AAI_SON-ADV message

- Introduction

In section 16.2.3.6, there are several related time parameters. However, it doesn’t clarify how to these parameters. In this contribution, we would clarify the usage of these parameters and add a new parameter.

If the UST and UIT are included in the AAI_SON-ADV message, the available time can be calculated by theses parameters. For example, as shown in figure 1, if the UST is n+2 and the UIT is 10, the end of unavailable time is n+12.

Furthermore, the unavailable end time (UET) can be also used in the AAI_SON-ADV message to indicate the time of the end of the unavailable interval.

- Proposed Text

[Modify the following text in Section 16.2.3.6]

- Action type

    - 1st value: ABS Reconfiguration

    - 2nd value: ABS Restart

    - 3rd value: ABS Scanning

    - 4th value: ABS Reliability

- If(Action type == 1st , 2nd , or 3rd value)

    - Unavailability start time (UST)

    - Unavailable Time Interval (UTI) of the ABS

- Unavailable Start time (UST) : It indicates the start of the unavailable time.

- Unavailable Time Interval (UTI) of the ABS : It indicates the of unavailable time interval.

- Unavailable End time (UET) : It indicates the end of the unavailable time if the UTI is not included in the AAI_SON-ADV message.

- If(Action type == 4th value)

    - Reason

      - 0b00: Power down

      - 0b01: Power reduction

      - 0b10: FA change

      - 0b11: Backhual link down

    - If(Reason == 0b01), dB value of Tx power reduction

    - If (Reason == 0b10), new FA index

    - Expected power down time or resource adjustment time

    - Expected uptime

    - Recommended BSID list