| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of SA-Preamble Selection for Different ABS Types on Section 16.3.6.1.2 |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Chung-Lien Ho, Yan-Xiu Zheng, Chang-Lan Tsai, Yu-Chuan Fang ITRI | Email: zhengyanxiu@itri.org.tw |
| Re: | Contribution in support of a comment on LB31 Section 16.3.6.1.2 SA-Preamnble |  |
| Abstract | The contribution proposes the text of SA-Preamble selection for different ABS types to be included in the 802.16m AWD (P802.16m/D4). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD (P802.16m/D4). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of SA-Preamble Selection for Different ABS Types on Section 16.3.5.4.1

Chung-Lien Ho, Yan-Xiu Zheng, Chang-Lan Tsai, Yu-Chuan Fang

ITRI

- Introduction

The contribution proposes the text of SA-Preamble selection for different ABS types to be included in the 802.16m AWD ([3]). The rank-2 pilot patterns are used to mitigate the pilot collision among different type ABSs. This can be done by appropriately select the SA-Preamble carrier set for different ABS types. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 ([1]); it is compliant to the 802.16m SRD ([2])).

- Rank-2 Pilot Selection for Macro ABS and Non-Macro ABS Based on SA-Preamble Carrier Set

  - Rank-2 Pilot Selection for Macro ABS, Public ABS and CSG Femto ABS

Fig. 1: Segment partition of cell ID

In order to avoid the interference between macro ABS and non-macro ABS, the rank-2 pilot of non-macro ABS should avoid the collision of the rank-2 pilot of macro ABS. The rank-2 pilot selection for a public ABS and that for a CSG femto ABS can be based on the segment information. The segments of the public ABS and CSG femto ABS are interlaced with each other within a sector.

Fig. 1 plots an exemplary segment partition of cell ID. Cell ID can be expressed as following equation:

where n indicates the segment and Idx indicates the value ranges from 0 to 255. If Idx belongs to 0 to 85, it is reserved for macro ABS. If Idx belongs to 86 to z, it belongs to public ABS. If Idx belongs to z + 1 to 255, it belongs to CSG femto ABS. z can be broadcasted by an ABS. If the segment of macro ABS is n, the public ABS can choose the segment as following equation:

i=mod((n+1),3)

The CSG femto can choose the segment following equation:

j=mod((n−1),3)

The cell ID of public ABS can be 256i + Idx, 86≤Idx≤z, and that of CSG femto ABS can be 256j + Idx, .

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m/D4, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD (D4)

------------------------------------------------------------ Text Start ------------------------------------------------------------

Replace all the text of the last paragraph of page 411 in Section 16.3.6.1.2 of D4 by the following:

For the support of macro, public and CSG femto deployment, the public ABS and CSG femto ABS shall configure the segment or subcarrier set for SA-Preamble transmission based on the segment information of the overlay macrocell ABS for minimized interference to macrocell if the public ABS or CSG femto ABS is synchronized to macrocell ABSs. The segment information of the overlay macrocell ABS may be obtained by communications with macrocell ABS through backbone network or active scanning of SA-Preamble transmitted by macrocell ABS. if 0  Idx  85, it is reserved for macro ABS, if 86  Idx  z, it belongs to public ABS, and if z + 1  Idx  255, it belongs to CSG femto ABS, where z is calculated from SA-Preamble sequence soft partitioning information in SFH SP3. The pPublic ABS and CSG femto ABS will use different segments to the co-located overlay macro ABS. CSG femto ABS shallcan be further restricted to one segment as i = mod((n - 1), 3) to reduce interference to public ABS if the overlay macro ABS uses segment n. Public ABS can shall choose a different segment as i = mod((n + 1), 3) to CSG femto ABS so as to minimize interference from both overlay macro ABS and CSG femto ABS. For example: the public ABS can choose the segment as i = mod((n + 1), 3), and CSG femto can choose the segment as i = mod((n - 1), 3) if the co-located macro ABS uses segment n. The Cell ID of public ABS can be 256i + Idx, 86  Idx  z, and that of CSG femto ABS can be 256j + Idx, z + 1  Idx  255. The public ABS also can choose the segment as i = mod((n - 1), 3) to reduce interference between public ABSs.

------------------------------------------------------------- Text End -------------------------------------------------------------