| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modifications to Paging Cycle Values (16.2.3.22) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB31 / Area: Chapter 16.2.3 (MAC Control messages) |  |
| Abstract | This contribution proposes changes to the Section 16.2.3.22 of IEEE P802.16m/D4 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modifications to Paging Cycle Values (16.2.3.22)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

The contribution C80216-09/2265r3 accepted in the Atlanta meeting (#64) is incorrectly implemented in D3. We would like to correct that issue through this contribution.

After correction, the current paging cycle values defined in D4 allows only 32 superframes (640 ms) as the maximum paging cycle length. However, in case of having huge numbers of Idle Mode AMSs in a paging group, the paging group may require a larger value of paging cycle to relax the paging message overheads. Similarly, if a paging group is concerning more about the power saving efficiency than paging latency, it may also require a larger paging cycle value.

Table 1 shows the impact of the paging cycle value on the AAI_PAG-ADV message overheads and paging latency. We can see that when there are 20000 Idle Mode AMSs in a paging group, 32-superframes paging cycle will result in almost 4% system throughput overheads which are used for broadcasting AAI_PAG-ADV messages. If we can apply the value of 128 superframes to the paging cycle, the paging message overheads can be reduced to occupy only 1.05 % system throughput.

Table 1. Paging cycle values vs. AAI_PAG-ADV overheads and paging latency.

| Paging cycle | Number of Idle Mode AMSs in a paging group | Avg. paging latency | AAI_PAG-ADV throughput | System throughput (5MHz BW) | Ratio of AAI_PAG-ADV / system throughput |
| --- | --- | --- | --- | --- | --- |
| 32 superframes | 5000 AMSs | 320 ms | 136.6 kbps | 13000 kbps | 1.05 % |
|  | 10000 AMS | 320 ms | 261.6 kbps | 13000 kbps | 2.01 % |
|  | 20000 AMS | 320 ms | 511.6 kbps | 13000 kbps | 3.93 % |
| 64 superframes | 5000 AMS | 640 ms | 74.1 kbps | 13000 kbps | 0.57 % |
|  | 10000 AMS | 640 ms | 136.6 kbps | 13000 kbps | 1.05 % |
|  | 20000 AMS | 640 ms | 261.6 kbps | 13000 kbps | 2.01 % |
| 128 superframes | 5000 AMS | 1280 ms | 42.85 kbps | 13000 kbps | 0.32 % |
|  | 10000 AMS | 1280 ms | 74.1 kbps | 13000 kbps | 0.57 % |
|  | 20000 AMS | 1280 ms | 136.6 kbps | 13000 kbps | 1.05 % |

- Proposed Text for IEEE P802.16m/D4

================================= Remedy 1 =======================================

[Accept the following modifications marked in blue from line 53 on page 102.]

16.2.3.22 AAI_DREG-RSP message

When the action code is set to 0x05 or 0x07, the following parameters shall be included in AAI_DREG-RSP message.

Paging information

Paging cycle (4 bits)

0x00: 8 superframes

0x01: 16 superframes

0x02: 32 superframes

0x03-0x0f: reserved

Paging offset (4bits)

0x00: 0 superframe

0x01: 8 superframes

0x02: 16 superframes

0x03: 32 superframes

0x04-0x0f: reserved

Paging group ID

Paging controller ID

Idle Mode Retain Information

Deregistration Identifier

References

- IEEE P802.16m/D4. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Feb 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.