| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL MIMO Transmission Format in the 802.16m UL Basic Assignment A-MAP Information Element (16.3.6.5.2.4.2) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Roshni Srinivasan, Alexei Davydov, Hujun Yin, Huaning Niu, Apostolos Papathanassiou Intel Corporation Taeyoung Kim, Hyunkyu Yu, Sangheon Kim Samsung Electronics Wookbong Lee, Jinsoo Choi LG Electronics | roshni.m.srinivasan@intel.com ty33.kim@samsung.com wookbon.lee@lge.com |
| Re: | Comments on IEEE P802.16m/D4 for IEEE 802.16 WG Letter Ballot #31 |  |
| Abstract | The contribution proposes changes to the UL Basic Assignment A-MAP IE in the 802.16m draft standard for more efficient MIMO signaling on the UL. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL MIMO Transmission Format in the 802.16m UL Basic Assignment A-MAP Information Element (16.3.6.5.2.4.2)

Roshni Srinivasan, Alexei Davydov, Hujun Yin, Huaning Niu, Apostolos Papathanassiou

Intel Corporation

Taeyoung Kim, Hyunkyu Yu, Sangheon Kim

Samsung Electronics

Wookbong Lee, Jinsoo Choi

LG Electronics

- Introduction

A 4 bit subset of the 6 bit codebook currently used to signal PMI for UL CSM with beamforming is proposed in [1]. The reduction in the number of bits required to signal PMI facilitates an option of including signaling for UL CSM with beamforming in the UL Basic Assignment A-MAP IE, thereby freeing up one assignment A-MAP IE type for future use. This contribution proposes to merge the UL Basic Assignment A-MAP IE currently defined in the 802.16m Draft standard [2] with the CSM Beamforming A-MAP IE for more efficient MIMO signaling.

Figure 1 describes the parsing of information fields in the basic UL A-MAP IE to determine the MIMO transmission format and associated parameters used for transmission on the UL.

The following considerations have been taken into account in the proposed update:

- The PMI Indicator is used to signal explicit signaling of PMI.

- The CSM flag is used to distinguish between SU-MIMO and CSM operation

- Non-adaptive precoding shall be used for SFBC.

- The precoding flag (PF) need not be signaled always. It is only used when PMI is not signaled and the type of precoding used needs to be identified. For CSM and SU-MIMO with Vertical Encoding, the PF is used as follows

    - When PF is set to 0b0, the MS uses non-adaptive precoding

    - When PF is set to 0b1, the MS uses adaptive precoding. This is done using the precoder of rank Mt of MS’s choice.

- The MEF (SFBC or vertical encoding) needs to be signaled only when neither CSM nor PMI is signaled.

Current text in [1] to be retained is shown in black, current text that has been deleted has been struck through in red and text changes proposed have been underlined in blue.

- References

[1] C802.16m-10/0273, “Uplink 4TX antenna codebook subset for CSM mode (Section 16.3.10 of IEEE 802.16m/D4)”, March 2010.

[2] P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems, Advanced Air Interface”, January 2010.

Figure 1: MIMO Signaling in the UL Basic Assignment A-MAP IE

- Instructions to Editor

- Page 460, starting on Line 17: Update table 818  in [1] on A-MAP IE types by changing ‘UL CSM Beamforming A-MAP IE’ to ‘Reserved’. Add the missing word ‘Allocation’ to the DL/UL Persistent A-MAP IEs

- Page 468, starting on Line 1: Replace Table 822 (UL Basic Assignment A-MAP IE in [1] with the table shown in the proposed text.

- Page 525, starting on Line 20: Delete the entire subclause 15.3.6.5.2.4.14 including Table 840 (UL CSM Beamforming A-MAP IE) in [1]

________________________________Begin proposed text change #1_________________________________

Page 460, starting on Line 17

Table 818 -- A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | DL Subband Assignment A-MAP |
| 0b0011 | UL Subband Assignment A-MAP IE |
| 0b0100 | Feedback Allocation A-MAP IE |
| 0b0101 | UL Sounding Command A-MAP IE |
| 0b0110 | CDMA Allocation A-MAP IE |
| 0b0111 | DL Persistent Allocation A-MAP IE |
| 0b1000 | UL Persistent Allocation A-MAP IE |
| 0b1001 | Group Resource Allocation A-MAP IE |
| 0b1010 | Feedback Polling A-MAP IE |
| 0b1011 | BR-ACK A-MAP IE |
| 0b1100 | Broadcast A-MAP IE |
| 0b1101 | UL CSM Beamforming A-MAP IE Reserved |
| 0b1110 | Reserved |
| 0b1111 | Extended Assignment A-MAP IE |

________________________________End proposed text change #1_________________________________

________________________________Begin proposed text change #2_________________________________

Page 468, starting on Line 1

# 16.3.6.5.2.4.2 UL basic assignment A-MAP IE

Table 822 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 822 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| PMI Indicator | 1 | Flag to indicate if PMI is signaled explicitly 0b0: PMI not signaled explicitly 0b1: PMI signaled explicitly |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (PMI Indicator == 0b0){ |  | PMI not signaled |
| if (CSM == 0b0){ |  | SU-MIMO (Adaptive without PMI or Non- adaptive) |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b1){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 | Reserved bits |
| else{ |  |  |
| Reserved | 7 | Reserved bits |
| } |  |  |
| else{ |  | CSM (without PMI) (Adaptive without PMI or Non- adaptive) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 3 |  |
| } |  |  |
| else { |  | PMI signaled explicitly |
| if (CSM == 0b0){ |  | SU-MIMO with PMI |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| else { |  | CSM with PMI |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 4 | 4 bits PMI |
| } |  |  |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 1 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

Table 822 UL basic assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported. Nt is the number of transmit antennas at the AMS. 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled. 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: SU-MIMO 0b1: CSM |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding Non-adaptive precoding shall be used at the AMS with SFBC. Mt shall be ignored with SFBC. |
| if (MEF == 0b0){ |  |  |
| Reserved | 4 | Reserved bits |
| else if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| Reserved | 4 3 | Reserved bits |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM |
| PF | 1 | Precoding flag for CSM when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive precoding using the precoder of rank Mt of the AMS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4: 0b00, 0b01, 0b10, 0b11 |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 34 | HARQ channel identifier |
| Reserved | 12 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL Basic Assignment A-MAP IE. The CRC is masked by the Station ID.

_________________________________End proposed text change #2_________________________________

________________________________Begin proposed text change #3_________________________________

Page 525, starting on Line 20

# 16.3.6.5.2.4.14 UL CSM beamforming A-MAP IE

Table 840 describes the fields in a UL CSM Beamforming Assignment A-MAP IE.

Table 840 UL CSM Beamforming A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL_CSM_Beamforming_A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL CSM_Beamforming A-MAP IE |
| ISizeOffset | 5 | Offset used to compute burst size index |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt =1, SI =2 0b0010: TNS = 3, Mt =1, SI =1 0b0011: TNS = 3, Mt =1, SI =2 0b0100: TNS = 3, Mt =1, SI =3 0b0101: TNS = 3, Mt =2, SI =1 0b0110: TNS = 3, Mt =2, SI =2 0b0111: TNS = 4, Mt =1, SI =1 0b1000: TNS = 4, Mt =1, SI =2 0b1001: TNS = 4, Mt =1, SI =3 0b1010: TNS = 4, Mt =1, SI =4 0b1011: TNS = 4, Mt =2, SI =1 0b1100: TNS = 4, Mt =2, SI =2 0b1101: TNS = 4, Mt =2, SI =3 0b1110: TNS = 4, Mt =3, SI =1 0b1111: TNS = 4, Mt =3, SI =2 |
| PMI | 6 | 4 bits PMI for Nt = 2, first 2 MSB set to 0 6 bits PMI for Nt = 4 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD If number of DL subframes, Dis less than number of UL subframes, U, Long TTI Indicator= 0b1 |
| HFA | 3 | HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| Reserved | 1 | Reserved bit |
| } | - | - |

*A 16 bit CRC is generated based on the contents of the UL CSM Beamforming A-MAP IE. The CRC is masked by the Station ID.

The Resource Index field in the UL CSM Beamforming A-MAP IE is interpreted as in the DL Basic Assignment

A-MAP IE, with 'DL' specific terminology replaced by 'UL' equivalents.

________________________________End proposed text change #3_________________________________