| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Control Information Encoding for the MIMO Feedback (16.2.2.2.8) |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a on P802.16m/D5 |  |
| Abstract | The contribution proposes to encode the MIMO Feedback as a MAC signaling header (16.2.2.2.8). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Control Information Encoding for the MIMO Feedback (16.2.2.2.8)

Lei Wang

InterDigital Communications, LLC

- Introduction

We propose to encode MIMO Feedback as a MAC signaling header, because:

- it has nothing to do with the MAC PDU structure, as an extended header defined in 16.2.2.2.8; we should try to simplify the MAC PDU processing by not defining extended headers that are irrelevant to the MAC PDU construction and processing.

- it has a fixed size content, 20 bits, which fits very well in the MAC signaling header format.

This contribution proposes a MAC signaling header format for the MIMO feedback.

- Suggested changes in the 802.16m/D5

The following is the proposed change in the 802.16m/D5. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: Delete subsection 16.2.2.2.8, i.e., line 56 on page 64 to line 63 on page 66;

Suggested change #2: change the subsection numbers accordingly for the subsections after section 16.2.2.2.8

Suggested change #3:  in Table 664, change line 11, page 56, as follows:

0b0100     Reserved MIMO feedback extended header       See 16.2.2.2.8

Suggested change #4:  in Table 656, line 21, page 56, replace the last row in Table 656 by the following two rows:

0110			MIMO Feedback

0111 – 1111		Reserved

Suggested change #5: insert a new subsection in line 38 page 54 as follows:

Note that the marked changes are based on the text and Table in the current 16.2.2.2.8 in 16m/D5.

16.2.2.1.3.7  MIMO Feedback MAC Signaling Header

This MIMO feedback extended header format is defined in Table 673. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for one subband for MFM 2, 3, 5 or 6. Variables MaxMt, Codebook_subset, long period q and Measurement Method Indication are indicated in the Feedback Polling A-MAP IE. Nt is the number of transmit antennas at the ABS, indicated in S-SFH SP3. The number of subbands YSB is described in section <<16.3.9.3.1.5>>. The MFEH shall be padded by zeros for byte alignment to 3 bytes.

The decimal value of wideband STC rate is encoded with 2 or 3 bits in binary representation, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of subband and wideband PMI is encoded with 4 or 6 bits in binary representation, but its allowed values depend on Nt and Codebook_subset. The decimal value of subband stream index is always encoded with 2 bits, but its allowed values depend on Measurement Method Indication and MaxMt. The decimal value of logical subband index is always encoded with 5 bits, but its allowed values are limited to the interval [0, YSB-1].

Table 673—MFEH MIMO Feedback Signaling Header Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MIMO Feedback Header () { |  |  |
| FID | 4 | Flow Identifier. Set to 0b0001 |
| Type | 4 | MFEH type MAC signaling header type. Set to 0b0110 for MIMO feedback header. |
| Length | 3 | Indicates the length of the signaling header in bytes. Set to 0b100. |
| Reserved | 1 | Reserved. This field shall be filled by 0 |
| MFM | 3 |  |
| If (MFM == 0){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Zero padding | 10 |  |
| } |  |  |
| If (MFM == 1){ |  |  |
| Wideband CQI | 4 |  |
| Zero padding | 13 |  |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Zero padding | 5 |  |
| } |  |  |
| If (MFM == 3 and Nt == 2 or 4){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 2 |  |
| Subband PMI | 6 |  |
| } |  |  |
| If (MFM == 3 and Nt == 8){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband STC rate | 3 |  |
| Subband PMI | 4 |  |
| Zero padding | 1 |  |
| } |  |  |
| If (MFM == 4){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 |  |
| Wideband PMI | 6 |  |
| Zero padding | 4 |  |
| } |  |  |
| If ( MFM == 5){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband stream index | 2 |  |
| Zero padding | 6 |  |
| } |  |  |
| If ( MFM == 6){ |  |  |
| Subband index | 5 |  |
| Subband CQI | 4 |  |
| Subband PMI | 6 |  |
| Zero padding | 2 |  |
| } |  |  |
| If (MFM == 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 6 |  |
| Zero padding | 7 |  |
| } |  |  |
| } |  |  |

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”