| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Feedback error handling |  |
| Date Submitted | 2010-5-12 |  |
| Source(s) | Bin Chen Yunsong Yang Ronald Mao Huawei Technologies Co.,Ltd. | binchen@huawei.com yunsongyang@huawei.com rmao@huawei.com |
| Re: | Call for LB #31a on “ P802.16m/D5”: |  |
| Abstract | UL CQI/MIMO Feedback error handling to make the resource allocation more efficient and more robust. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL Feedback Error Handling

Yunsong Yang, Ronald Mao, Bin Chen

Huawei Technologies Co., Ltd.

- Introduction

UL CQI/MIMO Feedbacks are carried on UL PFBCH and SFBCH, but these UL fast feedback channels are not protected by CRC or some other schemes, and further more, these channels’ error rate is not low, so both the ABS and AMS does not know whether the feedback information is received correctly. It influences the efficient and robust of the physical resource allocation, since the BS may allocate the resource based on wrong CQI and MIMO feedback information, which will make DL transmitting with wrong resource and MCS, hence reducing the throughput, especially when long period or wideband (average) values are received in error. Therefore, it is desirable to provide means of remedy when such an error occurs.

In this contribution, we introduce a scheme to handle UL CQI/MIMO feedback error with very low cost (overhead).

- Proposed Scheme

There are Best_Subband_M/CQI in the UL feedback information, which can be used as PBI (preferred band information) during resource allocation. However, the PBI feedback are essentially advisory, i.e. the BS can follow it or override it (i.e. scheduling an MS in a sub-band that is not within the preferred band or not follow the CQI as indicated by the MS). When the BS does override it, the MS can not tell if the BS receives the previous feedback information correctly or not.

The solution is: the BS provides frequency sub-channel assignment information with an Override field in the DL grant message; the MS responds as follows:

|  | Override = “True” | Override = “False” |
| --- | --- | --- |
| Sub-channel assignment is within the preferred band or following the DL CQI | MS sends another full information of UL feedback ASAP | MS does nothing |
| Sub-channel assignment is not within the preferred band or following the DL CQI | MS does nothing | MS sends another full information of UL feedback ASAP |

In current D5 text, there are several bits reserved in A-MAP message, one of which can be used as override indicator bit. The MS can send the new full information of UL feedback via UL MAC PDU (with MFEH -- MIMO feedback extended header).

This scheme does not introduce new definition of any message, and no change to the UL feedback scheme and channel. The only cost is a bit reserved in the A-MAP IE (it can be said that there is almost no cost). But with it, the error received UL feedback information could be corrected quickly, hence the resource efficiency can be improved. Further more, since multi-layer MIMO encoder format is most probably used by stable MS or under very good channel condition, at this case the UL feedback is much more reliable than other MIMO mode, and the override indicator will not be introduced to multi-layer encoding format, so that we can use just one of the several reserved bits in the other MIMO encoder format.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D5, Apr. 2010.

- Text proposal for inclusion in the 802.16m AWD

Text Proposal #1 < Modify the text in the section 16.3.6.5.2.4.1 in the page 542 line 63>

-------------------------------  Start of text proposal #1 -------------------------------------------

…

Resource Index: Resource Index with starting LRU index and size of a single allocation spanning contiguous LRUs.

Override Indicator: Override indicator for resource allocation. When it is true, the resource in this A-MAP is allocated with overriding the CQI and MIMO UL feedback information reported by the AMS; while if it is false, the resource in this A-MAP is allocated following the UL feedback information. The AMS may use it to determine whether the reported feedback information was received in error, so that it may react correspondingly. For example, the AMS may determine as below: if the override indicator is true and resource allocated follows the UL feedback, or if the override indicator is false and resource allocated overrides the UL feedback, the UL feedback information was received in error. The AMS may then re-send full UL feedback information via MFEH as soon as possible.

Resource Indexing in the DL/UL Basic Assignment A-MAP IE

-------------------------------  End of text proposal #1 --------------------------------------------

Text Proposal #2 < Modify the text in the section 16.3.6.5.2.4.1 table 848 in the page 545 line 38>

-------------------------------  Start of text proposal #2 --------------------------------------------

Table 848—DL Basic Assignment A-MAP IE

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| …. |  |  |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| Mt | 3 | Number of streams in transmission Nt : Number of transmit antennas at the ABS 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| Override Indicator | 1 | Override indicator for Resource allocation. 0b0: false. The resource is allocated while following the UL feedback received 0b1: true. The resource is allocated while overriding the UL feedback received |
| Reserved | 1 | Reserved bit |
| } else if(MEF == 0b10) { |  | Parameters for multi-layer encoding |
| ….. |  |  |
| }else{ |  |  |
| Override Indicator |  | Override indicator for Resource allocation. 0b0: false. The resource is allocated while following the UL feedback received 0b1: true. The resource is allocated while overriding the UL feedback received |
| Reserved | 4 | Reserved bits |
| } |  |  |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. |
| ….. |  |  |
| Reserved | 1 | Reserved bits |
| } |  |  |

-------------------------------  End of text proposal #2 --------------------------------------------