| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the description of FDD carrier(16.2.8.1) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | E-mail: sjjung@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31a:Announcement |  |
| Abstract | Proposed text to modify the AAI_Global-Config message and description of FDD multicarrier. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the description of FDD carrier (16.2.8.1)

Soojung Jung, Eunkyung Kim, Jaesun Cha, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# 1. Introduction

According to the section 16.2.8, for FDD systems each available downlink or uplink frequency channel is individually referred to as a carrier using a physical carrier index. Therefore, in case of FDD system, FDD related information should also be included in the AAI_Global-Config message. However, information included in AAI_Global-Config message and description of carrier for FDD system are ambiguous and in need of clarification. Therefore the clarification of FDD carrier is proposed. In addition, in the proposed AAI_Global-Config message, the DL/UL indicator of carrier is included.

# 2. Text Proposal

------------------------------------------------------------Start of the Text--------------------------------------------------------

[Add the following text at line 10 on page 297]

For FDD systems each available downlink or uplink frequency channel, and for TDD system each available duplexed frequency channel, is individually referred to as a carrier using a Physical Carrier Index. Physical carrier index is the index assigned by the network for the available carriers and is sorted from lower frequency to higher frequency. For FDD system physical carrier index is assigned to only DL carrier.

[Add the following text at line 16 on page 297]

- Fully configured carrier: A standalone carrier for which all control channels including synchronization, broadcast, multicast and unicast control signaling are configured. Fully configured carrier supports both single carrier AMS and multicarrier AMS. For FDD system a DL carrier shall be paired with a UL carrier.

-------------------------------------------------------------End of the Text--------------------------------------------------------

# References

- IEEE P802.16m/D5, “Advanced Air Interface (Draft),” April. 2010.