| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Texts for Periodic Transmission of S-SFH SPs (16.3.6.2.1.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Hyunkyu Yu, Hyunjeong Kang, and Ji-Yun Seol Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution provides the text to determine the transmission timing of S-SFH. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Texts for Periodic Transmission of S-SFH SPs

(16.3.6.2.1.2)

Hyunkyu Yu, Hyunjeong Kang, and Ji-Yun Seol

Samsung Electronics

- Introduction

In D6, only the transmission periodicity is defined for S-SFH SPs. That is, the transmission location (SFN offset) of each SP is depends on ABS scheduler.

In AMS side, to receive/decode required SPx efficiently, AMS should know the exact timing (when S-SFH SPs are transmitted). However, it is not easy for AMS to calculate this exact timing based on the previous transmission of SP1, SP2, and SP3. Moreover, additional SPx transmission makes the calculation more difficult and ambiguous.

Thus, we recommend specifying the SFNs, where S-SFH SPs are transmitted, according to the transmission periodicity.

Removed text

Added text

- Proposed Text Changes

[Remedy #1: Modify the text in line 48, page 526, section 16.3.6.2.1.2 as follows,]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.2.1.2 Secondary Superframe Header

The Secondary Superframe Header (S-SFH) may be transmitted in every superframe. If the S-SFH is present, the S-SFH shall be mapped to the NS-SFH distributed LRUs following the NP-SFH distributed LRUs.

The information transmitted in S-SFH is divided into three sub-packets.

The sub-packets of S-SFH are transmitted periodically where each sub-packet has a different transmission periodicity as illustrated in Figure 520.

The “SP scheduling periodicity information" field of S-SFH SP3 IE is used to inform indicate the transmission periodicity of the S-SFH SPs (1, 2, 3) defined in Table 836. Table 836 shows the transmission periodicity of different S-SFH SPs for different values of "SP scheduling periodicity information" field.

Table 836 – Transmission Periodicity of S-SFH SPs

| SP scheduling periodicity information | Transmission periodicity of S-SFH SP1 (TSP1) | Transmission periodicity of S-SFH SP2 (TSP2) | Transmission periodicity of S-SFH SP3 (TSP3) |
| --- | --- | --- | --- |
| 0000 | 40 ms | 80 ms | 160 ms |
| 0001 | 40 ms | 80 ms | 320 ms |
| 0010 | 40 ms | 80 ms | 240 ms |
| 0011 | 40 ms | 80 ms | 480 ms |
| 0100 | 40 ms | 120 ms | 240 ms |
| 0101 | 40 ms | 120 ms | 360 ms |
| 0110 | 40 ms | 120 ms | 480 ms |
| 0111 | 40 ms | 160 ms | 320 ms |
| 1000 | 40 ms | 160 ms | 480 ms |
| 1001 ~ 1111 | Reserved | Reserved | Reserved |

The transmission periodicity of SPx (TSPx) is determined with TSP1 < TSP2 < TSP3. The TSP2 is a multiple of TSP1 and the TSP3 is a multiple of TSP2. Also, the S-SFH SP IEs shall be periodically transmitted at the SFN which satisfies following conditions, where n is an integer value.

- The S-SFH SP1 IE is scheduled when SFN mod TSP1 is equal to 0 (i.e., SFN = (TSP1/20×n) mod 212-1).

- The S-SFH SP2 IE is scheduled when SFN mod TSP2 is equal to TSP1 – 1 (i.e., SFN = (TSP2/20×n+TSP1/20-1) mod 212-1).

- The S-SFH SP3 IE is scheduled when SFN mod TSP3 is equal to TSP2 – 1 (i.e., SFN = (TSP3/20×n+TSP2/20-1) mod 212-1).

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy #2: Delete the text in line 8, page 540, section 16.3.6.5.1.2 as follows,]

------------------------------------------------Start proposed text----------------------------------------------------------------

16.3.6.5.1.2 S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into three S-SFH subpacket IEs such as SP1, SP2 and SP3. These SPs are transmitted in different timing and periodicity. The periodicity of SP (TSP) is determined with TSP1 < TSP2 < TSP3.The S-SFH IE format are shown in Table 839.

-------------------------------------------------End proposed text----------------------------------------------------------------