| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up Text for PC-A-MAP IE (16.3.6.5.1.2, 16.3.6.5.2.3) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Hyunkyu Yu, Jeongho Park Samsung Electronics Co., Ltd. | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m. |  |
| Abstract | The contribution clarifies PC-A-MAP IE. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (D7). |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up Text for PC-A-MAP IE

Jeongho Park, Hyunkyu Yu

Samsung Electronics

Introduction

This contribution clarifies the total number of PC-A-MAPs signaled in S-SFH SP1, and the PC-A-MAP resource index.

(1) Total number of PC-A-MAPs signaled in S-SFH SP1

- Because two PC-A-MAP IEs are paired using SFBC, the number of allocable channels should be a multiple of two.

(2) PC-A-MAP resource index

- It needs to be clarified that resource index is calculated based on the AAI subframe/frame.

Proposed Text Changes

[Remedy#1: Change the line 23~29 in page 542 (table 840, section 16.3.6.5.1.2 as]

------------------------------------------------Start proposed text----------------------------------------------------------------

| Power control channel resource size indicator | 2 | Total number of PC-A-MAP IE, NPC-A-MAP 0b00 : 0 (No use of PC-A-MAP) 0b01 : ceil(14 * U/D) 0b10 : ceil(28 * U/D) 0b11 : ceil(44 * U/D) Here U and D are the number of available subframe number per a frame in uplink and downlink respectively. Total number of PC-A-MAPs, NPC-A-MAP 0b00 : 0 (No use of PC-A-MAP) 0b01 : 2∙ceil(7∙U/D) 0b10 : 2∙ceil(14∙U/D) 0b11 : 2∙ceil(22∙U/D) Here U and D are the number of AAI subframes per AAI frame in uplink and downlink, respectively. |
| --- | --- | --- |

-------------------------------------------------End proposed text----------------------------------------------------------------

[Remedy#2: Modify the text in page 554, line 17 (under the Table 846- PC-A-MAP IE format) as follows:]

---------------------------------------------------Start proposed text-------------------------------------------------------------

For FDD/H-FDD/TDD systems, when AMS transmits the qq-th FFBCH at m the m-th uplink subframe in i the i-th frame, the corresponding PC-A-MAP IE to the AMS automatically follows the assignment location defined in Table 847.

NN : total number of AAI subframes in a AAI frame (In TDD systems, N=D+U N=D+U. In FDD/H-FDD, N=D=U N=D=U)

DD : total available number of downlink DL AAI subframes in a AAI frame

UU : total available number of uplink UL AAI subframes in a AAI frame

QQ : total number of FFBCH in a uplink UL AAI subframe given by Q = Nfb*UL_FEEDBACK_SIZE – LHFB/6 Q = Nfb∙UL_FEEDBACK_SIZE – LHFB/6 where Nfb Nfb is 3. When frame structure is supporting the WirelessMAN-OFDMA with FDM-based uplink PUSC zone, Nfb Nfb is 4. And LHFB LHFB is the number of UL HARQ channels defined in S-SFH SP1

qq : the feedback channel index specified in Feedback Allocation A-MAP IE (0≤q≤Q-1)

mm : available UL AAI subframe index (0≤m≤U-1)

TT : necessary number of PC-A-MAPs in a downlink DL AAI subframe corresponding to the number of FFBCH number, which is defined by T = ceil(U*Q/D) T = ceil(U∙Q/D)

ss : the feedback channel index in a AAI frame, i.e., the sequence order of the q q-th feedback channel at m the m-th AAI subframe index, s=Q*m+q s=Q∙m+q

-------------------------------------------------End proposed text----------------------------------------------------------------