| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up for Sleep Cycle Setting parameters (16.2.17.2.4) |  |
| Date Submitted | 2010-08-12 |  |
| Source(s) | Yeongmoon Son, Hyunjeong Kang Samsung Electronics Co., Ltd. | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | IEEE 802 Sponsor Ballot Recirc #1 |  |
| Abstract | The contribution proposes a clean-up for Sleep Cycle Setting parameter in section 16.2.17.2.4 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up for Sleep Cycle Setting parameters

Yeongmoon Son, Hyunjeong Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes to clean up Sleep Cycle Setting in DSx-REQ/RSP message which has TLV format. It should be transformed into the same format as other tables to support ASN.1 PER.

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1: Delete table 792 and 793 on page 398 in section 16.2.17.2.4:]

Table 792—Sleep Cycle Setting

Table 793—Sleep Cycle parameters in Sleep Cycle Setting

[Remedy 2: Modify table 740 on page 199, line 50 in section 16.2.3.46.1:]

| Table 740 — AAI_DSA-REQ |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| O | D.2) Non-common for Coupled Group | Variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |
| O | Sleep Cycle Setting | Variable |  | May be present when Sleep Cycle setting needs to be changed or switched |
| O | A) Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | B) SCID | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | C) Start Frame Number | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | D) TIMF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | E) LWEF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | F) NSCF | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | G) Initial Sleep Cycle | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | H) Final Sleep Cycle | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | I) Listening Window | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | J) New Initial Sleep Cycle | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | K) T_AMS | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | L) Response_Code | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | M) FFBCH_Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | N) SLPID | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | O) REQ_duration | 8 | Refer to table 708 and 709 | Refer to table 708 and 709 |

[Remedy 3: Modify table 741 on page 201, line 56 in section 16.2.3.46.2:]

| Table 741 — AAI_DSA-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| M | D.2) FID | 4 | Flow ID |  |
| O | Sleep Cycle Setting | Variable |  | May be present when Sleep Cycle setting needs to be changed or switched |
| O | A) Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | B SCID | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | C) Start Frame Number | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | D) TIMF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | E) LWEF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | F) NSCF | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | G) Initial Sleep Cycle | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | H) Final Sleep Cycle | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | I) Listening Window | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | J) New Initial Sleep Cycle | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | K) T_AMS | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | L) Response_Code | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | M) FFBCH_Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | N) SLPID | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | O) REQ_duration | 8 | Refer to table 708 and 709 | Refer to table 708 and 709 |

[Remedy 4: Modify table 743 on page 210, line 39 in section 16.2.3.46.4:]

| Table 743 — AAI_DSC-REQ |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| O | Backoff Scaling Factor | 4 | "0010" for Binary Exponential Backoff | For Uplink only |
| O | Sleep Cycle Setting | Variable |  | May be present when Sleep Cycle setting needs to be changed or switched |
| O | A) Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | B) SCID | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | C) Start Frame Number | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | D) TIMF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | E) LWEF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | F) NSCF | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | G) Initial Sleep Cycle | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | H) Final Sleep Cycle | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | I) Listening Window | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | J) New Initial Sleep Cycle | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | K) T_AMS | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | L) Response_Code | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | M) FFBCH_Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | N) SLPID | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | O) REQ_duration | 8 | Refer to table 708 and 709 | Refer to table 708 and 709 |

[Remedy 5: Modify table 744 on page 211, line 30 in section 16.2.3.46.5:]

| Table 744 — AAI_DSC-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| O | Confirmation Code | 4 | FID Change Count from corresponding the AAI_DSC-REQ message | N.A. |
| O | Sleep Cycle Setting | Variable |  | May be present when Sleep Cycle setting needs to be changed or switched |
| O | A) Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | B) SCID | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | C) Start Frame Number | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | D) TIMF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | E) LWEF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | F) NSCF | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | G) Initial Sleep Cycle | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | H) Final Sleep Cycle | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | I) Listening Window | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | J) New Initial Sleep Cycle | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | K) T_AMS | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | L) Response_Code | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | M) FFBCH_Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | N) SLPID | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | O) REQ_duration | 8 | Refer to table 708 and 709 | Refer to table 708 and 709 |

[Remedy 6: Modify table 746 on page 212, line 43 in section 16.2.3.46.7:]

| Table 746 — AAI_DSD-REQ |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| M | B) FID | 4 | Flow identifier |  |
| O | Sleep Cycle Setting | Variable |  | May be present when Sleep Cycle setting needs to be changed or switched |
| O | A) Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | B) SCID | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | C) Start Frame Number | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | D) TIMF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | E) LWEF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | F) NSCF | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | G) Initial Sleep Cycle | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | H) Final Sleep Cycle | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | I) Listening Window | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | J) New Initial Sleep Cycle | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | K) T_AMS | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | L) Response_Code | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | M) FFBCH_Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | N) SLPID | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | O) REQ_duration | 8 | Refer to table 708 and 709 | Refer to table 708 and 709 |

[Remedy 7: Modify table 747 on page 213, line 20 in section 16.2.3.46.8:]

| Table 747 — AAI_DSD-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| O | Confirmation Code | 4 | FID Change Count from corresponding the AAI_DSD-REQ message |  |
| O | Sleep Cycle Setting | Variable |  | May be present when Sleep Cycle setting needs to be changed or switched |
| O | A) Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | B) SCID | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | C) Start Frame Number | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | D) TIMF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | E) LWEF | 1 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | F) NSCF | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | G) Initial Sleep Cycle | 4 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | H) Final Sleep Cycle | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | I) Listening Window | 6 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | J) New Initial Sleep Cycle | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | K) T_AMS | 5 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | L) Response_Code | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | M) FFBCH_Operation | 2 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | N) SLPID | 10 | Refer to table 708 and 709 | Refer to table 708 and 709 |
| O | O) REQ_duration | 8 | Refer to table 708 and 709 | Refer to table 708 and 709 |

[Remedy 8: Modify the text on page 397, line 50 in section 16.2.17.2.4:]

16.2.17.2.4 Sleep Mode parameter update

…....

….The Sleep Cycle change/switching may be performed with the exchange of DSx MAC control message when the AMS is in Sleep Mode. In case that the AMS in Sleep Mode sends an AAI_DSx-REQ with Sleep Cycle Setting (refer to Table 793740,743, and 746), the ABS shall regards Sleep Cycle Setting included in the AAI_DSx-REQ as negotiation of Sleep Cycle parameters in AAI_SLP-REQ. If the ABS decides to accept sleep cycle setting, the ABS shall include the Response Code = 0b01 (i.e., Approval) with the parameters (refer to Table 741,744, and 747) which are different from the AMS's request. Otherwise, the ABS shall either omit the entire Sleep Cycle Setting or include both the Response Code = 0b10 (i.e., Reject) and REQ_duration in AAI_DSx-RSP message, as rejection of the AMS’s request. If the ABS rejects Sleep Cycle setting negotiation while accepting creation/change/deletion of service flow, the ABS or AMS may initiate another Sleep Mode transaction to change/switch Sleep Cycle setting by AAI_SLP-REQ/RSP. On the other hand, if the ABS sends the AMS an AAI_DSx-REQ with Sleep Cycle Setting (refer to Table 741,744, and 747), the AMS shall regard Sleep Cycle Setting in the AAI_DSx-REQ as negotiation of Sleep Cycle parameters in unsolicited AAI_SLP-RSP. Therefore, the AMS shall apply the Sleep Cycle setting sent in the frame specified by Start_Frame_Number in Sleep Cycle setting included in the AAI_DSx-REQ sent by ABS(refer to Table 741,744, and 747).

[Remedy 9: Modify section 16.2.3.46.1 on page 188, line 24:]

        - AAI_DSA-REQ

…

Following parameters may be included in the AAI_DSA-REQ message.

- SCID: To switch sleep cycle setting

- Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure, and are only included in ABS initiated DSA-REQ. They are flow specific and are determined based on the QoS parameters of the service flow in the AAI-DSx messages. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different.

[Remedy 10: Delete the paragraph on page 200, line 9:]

In response to AAI_DSA-REQ message which contains SCID, the SCID may be included in AAI_DSA-RSP message as approval to ABS or AMS’s request. If AAI_DSA-RSP message doesn’t include SCID, the AMS shall regard its request as being failed.

[Remedy 11: Delete SCID in table 740 on page 194, line 39:]

| Table 740 — AAI_DSA-REQ |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| O | SCID | 4 | Sleep cycle ID |  |
| … | … | … | … | … |

[Remedy 12: Modify section 16.2.3.46.4 on page 202, line 42:]

        - AAI_DSC-REQ

…

Following parameters may be included in the AAI_DSC-REQ message.

- SCID: To switch sleep cycle setting

- Predefined BR index parameters: Predefined BR index parameters define the mapping from predefined BR index(es) to BR action and BR size, which is used in 3-step Bandwidth Request procedure, and are only included in ABS initiated DSA-REQ. They are flow specific and are determined based on the QoS parameters of the service flow in the AAI-DSx messages. If BR action is 0b10 (BR), ABS shall assign different BR index to service flows whose UL Grant Scheduling Type is different.

[Remedy 13: Delete SCID in table 743 on page 205, line 43:]

| Table 743 — AAI_DSC-REQ |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| O | SCID | 4 | Sleep cycle ID |  |
| … | … | … | … | … |

[Remedy 14: Delete the paragraph on page 211, line 1:]

In response to AAI_DSC-REQ message which contains SCID, the SCID may be included in AAI_DSC-RSP message as approval to ABS or AMS’s request. If AAI_DSC-RSP message doesn’t include SCID, the AMS shall regard its request as being failed.

[Remedy 15: Delete the paragraph on page 212, line 1:]

The SCID may be included in AAI_DSD-REQ to switch Sleep Cycle setting.

[Remedy 16: Delete the paragraph on page 212, line 63:]

In response to AAI_DSD-REQ message which contains SCID, the SCID may be included in AAI_DSD-RSP message as approval to ABS or AMS’s request. If AAI_DSD-RSP message doesn’t include SCID, the AMS shall regard its request as being failed.

[Remedy 17: Delete SCID in table 783 on page 349, line 27:]

| Table 783 —Service flow/convergence sublayer parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| … | … | … | … | … |
| O | SCID | 4 | Sleep cycle ID |  |
| … | … | … | … | … |

-------------------------------  Text End  ---------------------------------------------------