| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the Relay PHY Parameters (16.6.3) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak LG Electronics Alexander Maltsev, Alexey Khoryaev, Andrey Chervyakov, Mikhail Shilov, Gregory Morozov Intel Corporation Youngbin Chang, Hyunjeong Kang, Jaeweon Cho Samsung Electronics | jiny.chun@lge.com alexander.maltsev@intel,com alexey.khoryaev@intel.com yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Sponsor Ballot Recirc #2 |  |
| Abstract | This contribution provides the clarification of the Relay PHY Parameters. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of the Relay PHY Parameters (16.6.3)

Jinyoung Chun, Jin Sam Kwak

LG Electronics

- Introduction

In the last meeting, most of relay issues are solved. ARS’s TDD HARQ procedure is matched to ABS’s and the existence of additional MIMO midamble is defined in SFH SP2. But still the procedure related to ARS is unclear.

When AMS tries the network entry at first, the AMS doesn’t know whether the BS is ABS or ARS by detecting SA-Preamble. Then the AMS assumes that the BS is ABS and follows ABS operation with ABS frame structure. Then the BS can schedule the AMS as below:

- If the BS is ABS without ARSs, the operation is normal.

- If the BS is ABS with ARSs, the ABS doesn’t allocate the long TTI bursts to the AMS. (implementation issue)

- If the BS is ARS, the ARS doesn’t allocate the long TTI burst to the AMS and doesn’t allocate the burst in the last subframe in DL access zone if DL R-RTI is 1. (implementation issue)

After AMS receives AAI-SCD message, the AMS can interpret the BS type and the next operation is as below:

- If the BS is ABS, AMS know the existence of additional MIMO midamble in SFH SP2 and cell specific mapping in AAI-SCD message. AMS follows ABS’s operation based on the information.

- If the BS is ARS, AMS follows ARS’s operation such as long TTI allocation in access zone. For that, AMS needs to know the ARS’s DL R-TTI and UL Tadv.

In TDD, the HARQ operation is same with ABS and ARS. Therefore even if AMS misunderstands the BS type, there is no problem. But in FDD, the HARQ operation is different with ABS and ARS. It’s very critical problem to the operation of ABS and ARS.

In this contribution, we suggest the indication method of DL R-TTI and UL Tadv. DL R-TTI is for the AMS which recognizes ARS. Therefore we can reuse the field of MIMO midamble in Relay zone. When serving station is ARS, the field indicates whether DL R-TTI of the ARS is zero or one. And if UL timing between ABS and ARS is aligned in Relay mode, UL Tadv always set to zero.

- Text proposal in the P802.16m/D7

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the row in Table 837 in line 23 on the page 589 as below: ]

Table 838 – S-SFH SP2 IE format

| The value of this field depends on type of the station (ABS or ARS) that transmits SP2. The indicator of MIMO midamble in Relay Zone / The indicator of DL R-TTI | 1 | If Serving station is ABS: Additional MIMO midamble indication in AAI DL Relay zone 0b0: Additional MIMO midamble is not transmitted in AAI DL Relay zone 0b1: Additional MIMO midamble is transmitted in AAI DL Relay zone (transmission location is defined in 16.3.7.2.3) If Serving station is TTR ARS: ARS shall set this bit to zero. 0b0: DL R-TTI = 0 0b1: DL R-TTI = Ts |
| --- | --- | --- |

[Modify the text in line 31 on the page 543 as below: ]

SA-Preamble sequences are partitioned and each partition is dedicated to specific base station type like macrocell ABS, Macro Hotzone ABS, Femto ABS, TTR Relay ABS.

[Modify the text in line 49 on the page 544 as below: ]

The AAI-SCD message provides finer partition information. The non-macro public ABS category can be further divided into Macro hotzone, TTR Relay and OSG femto. The CSG femto category can be further divided into CSG-closed and CSG-open.

------------------------------------------------   Text End  -----------------------------------------------------------