| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handling of PHY feedback headers and messages in multicarrier (16.2.8) |  |
| Date Submitted | 2010-09-08 |  |
| Source(s) | Xiangying Yang Intel Corporation | xiangying.yang@intel.com |
| Re: | Sponsor ballot comment on P802.16m/D8 Target topic: “IEEE P802.16m/D8, section 16.2.8”. |  |
| Abstract | The contribution proposes a clarification of PHY level controls in multicarrier |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handling of PHY feedback headers and messages in multicarrier (16.2.8)

Xiangying Yang

Intel Corporation

- Introduction

The contribution proposes the text changes of clarification of PHY level controls in multicarrier to be included in the 802.16m amendment. In the current draft [1], PHY level controls (MIMO/PC/FFR/CINR report) should be clarified for multi-carrier operation. UL channel operation and DL CINR report operation for CA (Carrier Aggregation) with partially configured secondary carrier are already defined. However, PHY level controls using MAC control message or MAC signaling header for CA with fully configured carriers are not clear.

We propose to handle PHY controls per carrier unlike MAC state, mobility and context of an AMS that are managed and controlled by an ABS through the primary carrier. On the other hand, the transmission of these PHY control related message/header should be considered case by case:

  - Uplink Power Status Report header, Correlation Matrix Feedback and MIMO Feedback header

  - AAI_FFR-CMD, AAI_FFR-REP, AAI_SingleBS_MIMO_FBK, AAI_MultiBS_MIMO_FBK, AAI_MULTI_BS_MIMO-REQ, AAI_MULTI_BS_MIMO-RSP, AAI_UL_POWER_ADJ, AAI_UL_PSR_CFG, AAI_ULPC_NI*, AAI_DL-IM *, AAI_SCD*

* Indicates broadcast message

Solution description:

- All the MIMO feedback related MAC header/messages are transmitted in grant provided by Feedback Polling IE as already defined. To minimize spec change, we continue to use this mechanism. Unfortunately Feedback Polling IE is already full and there is NO WAY to indicate cross-carrier information. Therefore we end up with two options

  - Option-1: Everything by default transmitted in primary carrier (preferable)

  - Option-2: Everything transmitted on corresponding carrier if possible (fully configured carrier); otherwise is transmitted on primary carrier (partially configured carrier). Note it would be implicit and no signaling on such selection is needed.

  - Note* either option, it seems to desirable to have carrier index in the feedback for reliability.

- All other PHY control information will be sent on ANY available grant, regardless of which carrier the grant is coming from.

- For all these messages/headers, include 4-bit carrier index to ensure protocol reliability no matter how the usage scenario is handled.

The benefits of doing this are

- Much simpler grant scheduling at ABS and AMS (all grants are treated equally for MAC PDUs)

- Utilize piggyback option to transmit such control as much as possible and therefore reduce contention-based BWREQ as well as incurred latency

- Single framework to support all MC CA deployment scenarios (TDD, paired FDD, unpaired DL only,)

Reference

[1] IEEE P802.16m/D8, Aug. 2010

- Text change

------------------------------------------ Text Start ---------------------------------------------------

Remedy #1

[Modify line 61 of page 332 as following:]

16.2.8 Multicarrier operation

16.2.8.1 Multicarrier Types and Operational Modes

When supporting multicarrier operations, two types of carriers are defined from an AMS point of view:

•Primary Carrier: A primary carrier is a standalone carrier where AMS complete initial network entry or network reentry procedure. When supporting multicarrier operations, an AMS shall only have one primary carrier and may be assigned with multiple secondary carriers. All the unicast MAC control messages relative to multicarrier operations (except MIMO and Multi-BS MIMO related control messages mentioned in 16.2.8.2.8) shall be sent to the AMS through its primary carrier.

•Secondary Carrier: Secondary carriers are additional carriers which may be assigned to the AMS by the ABS.

…

Remedy #2

[Modify the text in line 54 ~ 55 of page 333:]

A secondary carrier for an AMS, if fully configured, may serve as primary carrier for other AMSs. The multiple AMSs with different primary carriers may also share the same physical carrier as their secondary carrier. The following multicarrier operation modes are identified, which may all or independently be supported:

•Multicarrier Aggregation: The multicarrier mode in which the AMS maintains its physical layer connection and monitors the control signaling on the primary carrier while proceeding data transmission on the secondary carrier. The resource allocation to an AMS may span across a primary and multiple secondary carriers. Link adaptation feedback mechanisms are independently handled by each carrier. Link adaptation feedback mechanisms should incorporate measurements relevant to both primary and secondary carriers. In this mode the system may assign secondary carriers to an AMS in the downlink and/or uplink asymmetrically based on AMS capability, system load (i.e., for static/dynamic load balancing), peak data rate, or QoS demand.

Remedy #3

[Modify the section 16.2.8.2.8 on page 338 as following:]

16.2.8.2.8 DL CINR report operation PHY level control and UL Feedback Channel allocation

An ABS may assign UL feedback channels to each active carrier of an AMS as defined in 16.3.5.5.2.4.

In multicarrier aggregation with fully configured carriers, an ABS may assign FastFeedback channels to each active carrier of an AMS. When FastFeedback channel is assigned, the AMS reports CINR for an active carrier over the assigned FastFeedback channel of the corresponding carrier at the feedback region as defined in 16.3.7.3.3.

When only DL of fully configured carrier has been activated, UL feedback channel assigned by the ABS is located at the UL region defined in the SFH on the primary carrier. In this case, the ABS may allocate for an AMS one UL fast feedback channel per a secondary carrier over a primary carrier. If the AMS receives Feedback Allocation A-MAP IE through the DL only activated carrier, the AMS transmits the UL feedback information for the secondary carrier over the assigned UL Fast Feedback channel of the primary carrier.

In multicarrier aggregation with DL-only secondary partially configured carrier, the ABS directs the AMS to report CINRs of active DL-only secondary carriers through FastFeedback channel(s) on the primary carrier at the feedback region The feedback region is indicated using the physical carrier index for the primary carrier, the start DLRU index for feedback channel, the number of DLRUs for feedback channel and the number of HARQ feedback channel per HARQ feedback region in the AAI-SCD message that is transmitted on the active DL-only secondary partially configured carrier as defined in 16.3.8.3.3. The feedback region of the active DL-only secondary partially configured carrier follows the feedback region of the primary carrier. When measurement/report MAC control messages (i.e., AAI-SingleBS_MIMO_FBK, AAI-MultiBS_MIMO_FBK, MIMO Feedback header, Correlation Matrix Feed-back header) are used for DL CINR report operation, the messages are transmitted on the AMS's primary carrier. Feedback Polling A-MAP IE to inform a UL burst region is transmitted on each active DL carrier and the corresponding UL burst is assigned on the AMS's primary carrier. The UL burst carries the messages for DL CINR report for the corresponding active DL carrier.

[option-1]

When measurement/report MAC control messages (i.e., AAI-SingleBS_MIMO_FBK, AAI-MultiBS_MIMO_FBK, MIMO Feedback header, Correlation Matrix Feed-back header) are used for DL CINR report operation, the messages are transmitted on the AMS's primary carrier. Feedback Polling A-MAP IE transmitted on each active DL carrier indicates feedback polling request for the corresponding DL carrier and the UL grant indicated by Feedback Polling IE is assigned on the AMS's primary carrier.

[option-2]

When MIMO related measurement/report MAC control messages and headers, i.e., AAI-SingleBS_MIMO_FBK, AAI-MultiBS_MIMO_FBK, MIMO Feedback header, Correlation Matrix Feed-back header, are used for DL CINR report operation, they are transmitted in the uplink grant indicated by Feedback Polling A-MAP IE. For fully configured carriers, the grant is always on the same active carrier on which the Feedback Polling IE is received. For partially configured carrier, the grant is always on the primary carrier.

Other PHY level control not related to MIMO should also be handled per carrier. But MAC signaling header or MAC control message for non-MIMO PHY level control of each carrier can be transmitted on any available grant on any carrier. A 4-bit carrier index is included to indicate the relevant carrier that this control message or signaling header is associated with. The following MAC signaling header and MAC control message are handled in this way:

- MAC signaling header: Uplink Power Status Report

- MAC control message: AAI_FFR-CMD, AAI_FFR-REP, AAI_UL_POWER_ADJ, AAI_UL_PSR_CFG, AAI_MULTI_BS_MIMO-REQ, AAI_MULTI_BS_MIMO-RSP

In multicarrier operation mode, the pathloss reference (i.e. preamble) for uplink power control shall be the same carrier in TDD mode and the paired downlink carrier in FDD mode, respectively.

Remedy #4

[Modify line 7 of page 338 as following:]

16.2.8.2.6 Bandwidth Request and Resource Allocation

16.2.8.2.6.1 Bandwidth Request

All bandwidth requests that are transmitted on the AMS’s primary carrier follow following the same procedures as defined in 16.2.11. When a bandwidth request is sent on AMS’s secondary carrier using contention-based procedure, this indicates that AMS requests uplink grant specifically for this secondary carrier. Bandwidth request using piggyback may be transmitted in MAC PDUs over the secondary carrier(s) as well as the primary carrier, and does not indicate bandwidth request on any particular carrier.

Remedy #7

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 20 of page 135 as below:]

16.2.3.18 AAI_FFR-CMD (FFR Command) Message

An AAI_FFR-CMD message may be transmitted by the ABS to instruct the AMS to perform measurement over specific frequency partition(s). Table 7012 lists the parameters to be included into the AAI_FFR-CMD message:

Table 702- AAI_FFR-CMD (FFR Command) Message

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_FFR-CMD | N.A. |
| M | frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value "1". The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. | N.A. |
| M | reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value '1', it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value '1'. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| M | frameOffset | 8 | The offset (in units of frames) from the current frame in which AAI_FFR-REP message shall be transmitted on the unsolicited UL resource. |  |
| O | PhysicalCarrierIndex | 4 | The physical carrier index configured by the message |  |

16.2.3.18 AAI_FFR-REP (FFR Report) Message

Once an AMS receives AAI_FFR-CMD message, an AAI_FFR-REP message is sent by the AMS to report the downlink interference and/or SINR statistics of frequency partition(s) specified in AAI_FFR-CMD message. Table 698 shows the parameters for AAI_FFR-REP message:

Table 703- AAI_FFR-REP message format

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | Type of AAI_FFR-REP | N.A. |
| M | frequencyPartitionBitMap | 4 | Each bit indicates the interference statistics report status of corresponding Frequency Partition. 0b0: no report interference statistics 0b1: report interference statistics Frequency Partition Bitmap shall contain at least one bit with value "1". The LSB indicates the lowest available FP and the MSB indicates the highest available FP where the size of an available FP is bigger than zero. | N.A. |
| M | reportType | 4 | Each bit indicates if one type of report is required to be sent by AMS. If one bit has value '1', it indicates the specific report type is required to be sent, otherwise it indicates the specific report type is not required to be sent. At least 1 bit needs to be set to value '1'. LSB#0: Interference-Mean LSB#1: Interference-Variance LSB#2: SINR-Mean LSB#3: SINR-Variance |  |
| O | FFR-REPORT | Variable |  |  |
| O | A) fp0Report | Variable | FFR-FeedbackIE | Present when 0-th LSB of frequencyPartitionBitmap is 0b1 |
| O | B) fp1Report | Variable | FFR-FeedbackIE | Present when 1-th LSB of frequencyPartitionBitmap is 0b1 |
| O | C) fp2Report | Variable | FFR-FeedbackIE | Present when 2-th LSB of frequencyPartitionBitmap is 0b1 |
| O | D) fp3Report | Variable | FFR-FeedbackIE | Present when 3-th LSB of frequencyPartitionBitmap is 0b1 |
| O | FFR-FeedbackIE | Variable |  |  |
|  | A) interferenceMean | 8 | Interference mean. This is noise plus inter-cell interference power level which is averaged over the frequency partition and divided by the number of subcarriers in the frequency partition. -134dBm to -30dBm in units of 1dB. -134dBm is encoded as 0x00, -30dB is encoded as 0x68, 0x69 to 0xFF is reserved. | Present when 0-th LSB of reportType is 0b1 |
|  | B) interferenceVariance | 4 | Interference variance 0dB to 15dB in units of 1dB | Present when 1-st LSB of reportType is 0b1 |
|  | C) sinrMean | 8 | SINR mean -16dB to 53dB in units of 0.5dB -16dB is encoded as 0x00, 53dB is encoded as 0x8A, 0x8B-0xFF are reserved | Present when 2-nd LSB of reportType is 0b1 |
|  | D) sinrVariable | 4 | SINR variance 0dB to 15dB in units of 1dB | Present when 3-rd LSB of reportType is 0b1 |
| O | PhysicalCarrierIndex | 4 | The physical carrier index configured by the message |  |

-----------------------------------------------   Text End  -----------------------------------------------------------

Remedy #8

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 1 of page 165 as below:]

16.2.3.33 AAI_Uplink Power Status Reporting Configuration (AAI_UL_PSR_Config) message

An AAI_UL_PSR_Config message is used to config the AMS uplink power status reporting by unicasting. It is defined as:

Table 717- AAI_UL_RSP-CFG message Field Descriptions

| M/O | Attributes/Array of attributes |  | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | uplinkPowerStatusReport |  | 1 | disable(0), enable(1) | N.A. |
| O | psrReport-ConfigPara-meters | txPower-ReportTh-reshold | 4 | txPowerReportThreshold is a 4 bit unsigned integer value in 0.5 dB steps, the specific value "0b1111" shall be interpreted as "infinite" | Present when uplinkPow-erStatus-Report is enabled |
|  | psrReport-ConfigPara-meters | txPower-Report-Minimum-Interval | 4 | txPowerReportMinimumInterval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |  |
|  | psrReport-ConfigPara-meters | txPower-ReportPe-riodicalIn-terval | 4 | txPowerReportPeriodicalInterval is coded by 4 bit unsigned integer values d representing 2d frames, the specific value d=0b1111 shall be interpreted as "infinite" |  |
|  | psrReport-ConfigPara-meters | Physical-Carrier-Index | 4 | The physical carrier index configured by the message |  |

[Add the text in line 36 of page 52 as below:]

16.2.2.1.3.6 Uplink Power Status Report Header

The Uplink Power Status Report Header should be used to convey AMS ULPC status from AMS to ABS. The control of the reporting is described in section 16.3.9.4.7.

Table 662—Uplink Power Status Report Header Format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| Uplink Power Status Report Header(){ |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 5 | Mac Signaling header type = 0b00101 |
| Length | 3 | Indicates the length of the signaling header: 0b101: 5 bytes |
| ULPC Parameters Updating Indicator | 1 | Indicates whether the AMS has updated its ULPC parameters from AAI_SCD: 0: No ULPC parameters changed comparing to previous reporting. 1: The confirmation of new ULPC parameters from AAI_SCD applied. |
| Change configuration change | 4 | Only valid if ULPC Parameters Updating Indicator == 0b1 The value is the same as 2 LSB of “Change configuration change” in the latest AAI_SCD message. |
| txPowerPsdBase | 8 | txPowerPsdBase (PSD(base)) is coded using 8 bits in 0.5dBm steps ranging from -74dBm(coded 0x00) to 53.5dBm(coded 0xFF). |
| txSirDownlink | 10 | txSirDownlink(SIRDL) is coded using 10 bits in 1/16dB steps ranging from -12dB(coded 0x00) to 51.9375dB(coded 0x3ff) |
| Physical Carrier Index | 4 | Physical Carrier Index for PSR related carrier, this field should be ignored in the case of only single carrier active |
| Reserved | 5 | Shall be filled by 0 for byte alignment |
| } |  |  |

-----------------------------------------------   Text End  -----------------------------------------------------------

Remedy #9

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the text in line 16 of page 54 as below:]

16.2.2.1.3.8 MIMO feedback Header (MFH)

This MIMO feedback header format is defined in Table 664. This header is used by AMS to send only the wideband information for any combinations of MFM 0, 4, 7 with q=0.

Table 664—MFH Format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| MFM(){ |  |  |
| FID | 4 | Flow Identifier. Set to 0001. |
| Type | 5 | Mac Signaling header type = 0b00111 |
| Length | 3 | Indicates the length of the signaling header. |
| MFM bitmap | 3 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling A-MAP IE. LSB #0: MFM 0 LSB #1: MFM 4 LSB #2: MFM 7 |
| If (LSB#0 in MFM_bitmap == 1){ |  | MFM 0 |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 | ‘STC rate - 1’ mapped to 3bit unsigned integer (i.e., STC rate=1 as 0b000 ~ STC rate=8 as 0b111) |
| } |  |  |
| If (LSB#1 in MFM_bitmap == 1){ |  | MFM 4 |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 | ‘STC rate - 1’ mapped to 3bit unsigned integer (i.e., STC rate=1 as 0b000 ~ STC rate=8 as 0b111) |
| Wideband PMI | 6 | wideband preferred matrix index (PMI), size of which is number of PMI bits (‘NB’) used, mapped to NB LSB bits of this field, while the remaining MSB bit(s) set to zero(0) |
| } |  |  |
| If (LSB#2 in MFM_bitmap == 1){ |  | MFM 7 |
| Wideband CQI | 4 |  |
| Wideband PMI | 6variable | wideband preferred matrix index (PMI), size of which is number of PMI bits (‘NB’) used, mapped to NB LSB bits of this field, while the remaining MSB bit(s) set to zero(0) |
| } |  |  |
| Logical Carrier Index | 3 | Logical Carrier Index of the activated carriers for PSR related carrier, this field should be ignored in the case of only single carrier active |
| Reserved | variable | For byte alignment |
| } |  |  |

------------------------------------------------ Text End ---------------------------------------------------