| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Text Additions for Multiprotocol Convergence Sublayer Functionalities in IEEE 802.16m (5.2.6) |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty Intel Phillip Barber Huawei | Voice : +1 408-765-1866 E-mail: shantidev.mohanty@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-1 for P802.16m/D8. |  |
| Abstract | The contribution proposes text modifications for multi-protocol convergence sublayer in IEEE 802.16m (5.2.6). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Text Additions for Multiprotocol Convergence Sublayer Functionalities in IEEE 802.16m (5.2.6)

Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty

Intel

- Introduction

In the 802.16m/D8, multi-protocol convergence sublayer is used to transport different types of protocols over the same MAC service flow. Table 17 is missing some of the protocols that may be used in a IEEE 802.16m system.

This contribution proposes modification to Table 2a to include the missing protocols.

- Suggested changes in the 802.16m/D8

The following is the proposed change in the 802.16m/D8. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: page 17, line 5, insert new paragraph as:

For IP packets for Multiprotocol CS, and for IPinIP packets for Multiprotocol CS, Multiprotocol CS shall use the AAI classification rules as defined in 5.2.5.

For Ethernet packets for Multiprotocol CS, Multiprotocol CS shall use the AAI classification rules as defined in 5.2.4.

Suggested change #2: page 17, Table 2a

Replace Table 2a in page 17 with the following Table xx as shown below

Table xx – Protocol ID for Multiprotocol flow

| Protocol ID | Meaning |
| --- | --- |
| 1 | IP (without RoHC and PHS) |
| 2 | IP with RoHC |
| 3 | IP with PHS |
| 4 | Ethernet |
| 5 | Ethernet with PHS |
| 6-256 | Reserved |

Suggested change #3: page 206, Table 737

Modify Table 737 in page 206 as shown below

| O | G) S1: Request/Transmission policy | 7 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. This bit has no relevance for Multiprotocol CS. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. This bit has no relevance for Multiprotocol CS. |  |
| --- | --- | --- | --- | --- |

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D8, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”