| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI-ARS-CONFIG-CMD Cleanup (section 16.2.3.57) |  |
| Date Submitted | 2010-11-01 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Rakesh Taori Samsung Electronics | yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | IEEE 802.16m Sponsor Ballot Recirc # 3 |  |
| Abstract | This contribution proposes text on AAI-ARS-CONFIG-CMD for Relay |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D10 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI-ARS-CONFIG-CMD Cleanup

Youngbin Chang, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

# Proposed text

[Change 1 : Modify following parameters in table 751:]

Table 751— AAI-ARS-CONFIG-CMD Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| AAI_Relay_zone_AMS_allocation_indicator | 1 | The indicator which signalizes if the ABS may allocate resources to the AMS in the AAI DL Relay zone and AAI UL Relay zone. It has 1 bit to represent the values {0, 1}. If it is set to 1 then the AMSs are allocated in Relay zones | Always present |
| MIMO Midamble indication in AAI DL Relay zone | 1 | 0b0: MIMO midamble is not transmitted in AAI DL Relay zone 0b1: MIMO midamble is transmitted in AAI DL Relay zone | Always present |
| Superframe Number Action | 4 | LSBs of the superframe number when ARS start ARS operation and apply the PHY operational parameters. | Always present |
| If(AAI _Relay_zone_AMS_allocation_indicator == 0){ |  |  |  |
| R_DCASSB0 | 5/4/3 | AAI_DL_Relay_zone_Cell_Specific_Parameter See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_DCASMB0 | 5/4/3 | AAI_DL_Relay_zone_Cell_Specific_Parameter See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_DCASi | 3/2/1 | AAI_DL_Relay_zone_Cell_Specific_Parameter See 16.6.3.3.2 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |  |
| R_UCASSB0 | 5/4/3 | AAI_UL_Relay_zone_Cell_Specific_Parameter See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_UCAS MB0 | 5/4/3 | AAI_UL_Relay_zone_Cell_Specific_Parameter See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| R_UCASi | 3/2/1 | AAI_UL_Relay_zone_Cell_Specific_Parameter See 16.6.3.5.1 Cell-specific resource mapping For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |  |
| } |  |  |  |

[Change 2 : Modify following ASN.1 code from line 46 on page 1076 as follows:]

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Relay Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

RelayZoneCellSpecificParameterAllocation ::= SEQUENCE {

rDCASSB0 INTEGER (SIZE(1..32)),

rDCASMB0 INTEGER (SIZE(1..32)),

rDCASi INTEGER (SIZE(1..8)),

rUCASSB0 BIT STRING (SIZE(3..5)) INTEGER (SIZE(1..32)),

rUCASMB0 BIT STRING (SIZE(3..5)) INTEGER (SIZE(1..32)),

urUCASi BIT STRING (SIZE(1..3)) INTEGER (SIZE(1..8))

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- ARS Configuration Command

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-ARS-CFG-CMD ::= SEQUENCE {

dlRelayZoneDuration INTEGER (1..8),

ulRelayZoneDuration INTEGER (1..8),

relayZoneAMSallocInd BOOLEAN,

mimoMidambleTxInRelayZones BOOLEAN,

superframeNumberAction INTEGER (1..16),

amsRelayZoneAllocation CHOICE {

amsAllocatedInRelayZones NULL,

amsNotAllocatedInRelayZones RelayZoneCellSpecificParameter

allocation SEQUENCE {

dlRelayZone RelayZoneAllocation,

ulRelayZone RelayZoneAllocation

}

}

}