| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of the UL transmission in sleep mode (16.2.17.2.2) |  |
| Date Submitted | 2010-11-05 |  |
| Source(s) | Kiseon Ryu LG Electronics Jaesun Cha ETRI Yuqin Chen ZTE | E-mail: kiseon.ryu@lge.com |
| Re: | Sponsor ballot comment on P802.16m/D9 |  |
| Abstract | This contribution proposes the texts related to sleep mode operation |  |
| Purpose | To be discussed and adopted by SB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of the UL transmission in sleep mode

Kiseon Ryu

- LG Electronics

- Jaesun Cha

- ETRI

- Yuqin Chen

- ZTE

# Introduction

In 16m/D9, UL BW request during sleep mode may interrupt the normal sleep cycle operation, which means that AMS in sleep mode sends UL BW request at any time regardless of sleep cycle. However, there is no description regarding the operation after sending UL BW request. UL data transmission should not affect the sleep cycle operation because sleep cycle is defined as listening window and sleep window for AMS/ABS to receive/transmit DL traffic according to the iterated pattern. If an AMS receives DL traffic while sending UL data, then AMS may update its sleep cycle based on the sleep cycle operation.

# Text proposal for the IEEE 802.16m amendment

--------------Start of Proposed Texts -------------------------------------------------------------------

[Suggested Remedy: Modify the text as follows, on page 470, line 39:]

16.2.17.2.2 Sleep Window operations

During the Sleep Window, the AMS is unavailable to receive any DL data and MAC control signaling from the S-ABS. The AMS may perform power-down or autonomous scan or any other autonomous operations that do not involve the reception of DL transmissions. Handling of MAC control signaling during sleep mode is specified in 16.2.17.2.6. The AMS may interrupt the Sleep Cycle operation to request UL bandwidth in the Sleep Window. After requesting UL bandwidth, the AMS may stay awake until it completes transmission of the UL data for which it has requested UL bandwidth.

--------------End of Proposed Text----------------------------------------------------------------------