| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on DL/UL Frequency Partitioning (16.3.4, 16.3.7) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Taeyoung Kim, Youngbo Cho, Hyunkyu Yu Samsung Electronics | ty33.kim@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Sponsor ballot recirc #4 on P802.16m/D10 |  |
| Abstract | This contribution proposes the texts related to DL/UL frequency partitioning |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on DL/UL Frequency Partitioning (16.3.4, 16.3.7)

Taeyoung Kim, Youngbo Cho, Hyunkyu Yu

Samsung Electronics Co., Ltd.

# Introduction

In D10, the number of minibands for each frequency partition (KMB,FPi) is defined as the equation (190) and (260) for DL and UL. However, those equations do not cover all of DFPC and UFPC. For example, when DFPC=1 (FP0:FP1:FP2:FP3=0:1:1:1), the current equation (190) and (260) define the number of DL and UL minibands for wrong frequency partition. According to equation (190) and (260), the number of minibands for FP0, FP1 and FP2 are defined, but it should be defined for FP1, FP2 and FP3, not FP0. This kind of problem is due to the range of index i (e.g., 0  i < FPCT). So, we propose to change the range of index i in equation (190) and (260). In addition, we propose to clarify the definition on the number of subbands for the ith frequency partition (KSB,FPi) by changing equation (188) and (259).

# Text proposal for inclusion in the 802.16m amendment

<Remedy-1: Modify section “16.3.4.2.3 Frequency partitioning” on line 1 page 570 as follows>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

KSB,FPi=KSB−FPCT−1⋅DFPSCi=0,FPCT=4DFPSCi>0,FPCT=4DFPSCi>0,FPCT=3,DFPC=1KSB−FPCT−1⋅DFPSCi=0,FPCT=3,DFPC≠1DFPSCi=1,2,FPCT=3,DFPC≠1DFPSCi=1,2,FPCT=2KSBi=0,FPCT=1			     	(188)

KSB,FPi=KSB−FPCT−1⋅DFPSCi=0,FPCT=4DFPSCi>0,FPCT=4DFPSCi>0,FPCT=3,DFPC=1KSB−FPCT−1⋅DFPSCi=0,FPCT=3,DFPC≠1DFPSCi=1,2,FPCT=3,DFPC≠1DFPSCi=1,2,FPCT=2KSBi=0,FPCT=10Otherwise

When FPCT = 2, DFPSC shall be KSB/2. 								(189)

The number of minibands for each frequency partition is given by Equation (190).

KMB,FPi = (FPSi – KSB,FPi  N1) / N2       0  i < FPCT4					(190)

-------------------------------  End of Text Proposal  ---------------------------------------------------

<Remedy-2: Modify section “16.3.4.3.1 CRU/DRU allocation” on line 41 page 574 as follows>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

The total number of CRUs in frequency partition FPi, for 0 ≤ i < FPCT4, is denoted by LCRU,FPi, where

-------------------------------  End of Text Proposal  ---------------------------------------------------

<Remedy-3: Modify section “16.3.7.2.3 Frequency partitioning” on line 1 page 780 as follows>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

KSB,FPi=KSB−FPCT−1⋅UFPSCi=0,FPCT=4UFPSCi>0,FPCT=4UFPSCi>0,FPCT=3,UFPC=1KSB−FPCT−1⋅UFPSCi=0,FPCT=3,UFPC≠1UFPSCi=1,2,FPCT=3,UFPC≠1UFPSCi=1,2,FPCT=2KSBi=0,FPCT=1			     	(259)

KSB,FPi=KSB−FPCT−1⋅UFPSCi=0,FPCT=4UFPSCi>0,FPCT=4UFPSCi>0,FPCT=3,UFPC=1KSB−FPCT−1⋅UFPSCi=0,FPCT=3,UFPC≠1UFPSCi=1,2,FPCT=3,UFPC≠1UFPSCi=1,2,FPCT=2KSBi=0,FPCT=10Otherwise

When FPCT = 2, UFPSC shall be KSB/2.

The number of minibands in the ith frequency partition is denoted by KMB,FPi as shown in Equation (260),

KMB,FPi = (FPSi – KSB,FPi  N1) / N2       0  i < FPCT4					(260)

-------------------------------  End of Text Proposal  ---------------------------------------------------

<Remedy-4: Modify section “16.3.7.3.1 CRU/DRU allocation” on line 47 page 784 as follows>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

The total number of CRUs in frequency partition FPi, for 0 ≤ i < FPCT4, is denoted by LCRU,FPi, calculated as shown in Equation (265).

-------------------------------  End of Text Proposal  ---------------------------------------------------