| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Cleanup Text for Multiprotocol Convergence Sublayer Functionalities in IEEE 802.16m (5.2.6) |  |
| Date Submitted | 2010-12-17 |  |
| Source(s) | Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty, Joey Chou Intel | Voice : +1 408-765-1866 E-mail: shantidev.mohanty@intel.com |
| Re: | IEEE Standards Sponsor Ballot Recirculation-4 for P802.16m/D10. |  |
| Abstract | The contribution proposes text modifications for multi-protocol convergence sublayer in IEEE 802.16m (5.2.6). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Cleanup text for Multiprotocol Convergence Sublayer Functionalities in IEEE 802.16m (5.2.6)

Shaocheng Wang, Muthaiah Venkatachalam, Shantidev Mohanty, Joey Chou

Intel

- Introduction

In the 802.16m/D10, multi-protocol convergence sublayer is used to transport different types of protocols over the same MAC service flow.

In the 802.16m #69 meeting (St. Petersburg), some missing protocols that may be used in multi-protocol convergence sublayer had been agreed to be added to 802.16m draft. This contribution proposes some clean-up text to clarify the operations of the Multiprotocol CS.

- Suggested changes in the 802.16m/D10

The following is the proposed change in the 802.16m/D10. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: page 51, line 34, make the following changes:

[Note to editor: black text is the existing text; blue text is the new text to insert; red text is the text to delete]

5.2.6 Support for multiple protocols on the same flow

In order to transport several types of protocols over the same MAC connection, the Multiprotocol flow can be used. The receiver shall identify the protocol to correctly forward the SDU. For instance, if the information carried by the SDU is a RoHC packet, it should be forwarded to the RoHC decompressor. The receiver does this according to the Protocol ID field which is the first byte of a Multiprotocol flow connection as depicted in Figure 18a and Figure 18b.

Multi-protocol CS classification rules operate on the fields of the Multi-protocol Protocol ID, and/or the Packet Classification Rule parameters in Table 782, for example, the header fields of the IEEE 802.3/Ethernet header, VLAN header, IP header, and the transport protocol.

Suggested change #2: page 52, Table 2a

Modify Table 2a in page 52 with additional rows (in blue text) as in the following table

Table 2a – Protocol ID for Multiprotocol flow

| Protocol ID | Meaning |
| --- | --- |
| 1 | IP |
| 2 | IP with RoHC |
| 3 | IP with PHS |
| 4 | Ethernet |
| 5 | Ethernet with PHS |
| 6-256 | Reserved |

Suggested change #3: page 257 Line 8, Table 734

[Note to Editor]: Modify Table 734 in page 257 Line 8 as shown below

| G) S1: Request/Transmission policy | 7 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. This bit has no relevance for Multiprotocol CS. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. This bit has no relevance for Multiprotocol CS. |  |
| --- | --- | --- | --- |

Suggested change #4: page 259 Line 19, (Table 734)

[Note to editor]: Modify Table 734 in page 259 Line 19 as shown below

| Request/Transmission policy | 7 | Bit 0: If this bit is set to 1, the service flow shall not use broadcast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 1: If this bit is set to 1, the service flow shall not use multicast BR opportunities. (UL only) (see 6.3.5 and 6.3.5) Bit 2: If this bit is set to 1, the service flow shall not piggyback requests with data. (UL only) (see 6.3.5 and 6.3.5) Bit 3: If this bit is set to 1, the service flow shall not fragment data. Bit 4: If this bit is set to 1, the service flow shall not suppress payload headers (CS parameter). If bit 4 is set to'0' and both the SS and the BS support PHS (according to 11.7.7.3), each SDU for this service flow shall be prefixed by a PHSI field, which may be set to 0 (see 5.2). If bit 4 is set to '1', none of the SDUs for this service flow shall have a PHSI field. This bit has no relevance for Multiprotocol CS. Bit 5: If this bit is set to 1, the service flow shall not pack multiple SDUs (or fragments) into single MAC PDUs. Bit 6: If this bit is set to 1, the service flow shall not compress payload headers using ROHC. If bit 6 is set to'0' and both the SS and the BS support ROHC (according to 11.7.7.4), each SDU for this service flow shall be compressed using ROHC. If bit 6 is set to '1', none of the SDUs shall be compressed. This bit has no relevance for Multiprotocol CS. | Present if needed |
| --- | --- | --- | --- |

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D9, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”