| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Channel Bandwidth, Frequency Block Assignment and Spectral Efficiency Definitions |  |
| Date Submitted | 2004-01-12 |  |
| Source(s) | Dan Gal 67 Whippany Road, Whippany, NJ 07981 | Voice: 973-428-7734 Fax: 973-386-4555 Email: dgal@lucent.com |
| Re: | MBWA Call for Contributions: Session # 6 – January 12-16, 2003 |  |
| Abstract | This contribution provides a detailed discussion of the concepts listed in the Title and propose text changes to sections 4.1.2 and 4.1.3 of the IEEE 802.20 System Requirements document v10. |  |
| Purpose | Remove the ambiguity in the current text concerning the meaning of the terms “channel BW” versus “Block Assignment” and reflect the issue of in-channel guard-band in the spectrum-efficiency calculation method. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

# Introduction

This contribution proposes an alternative text for sections 4.1.2 and 4.1.3 of the IEEE 802.20 System Requirements Document (SRD v10). 
The purpose of this contribution is to reinstate the channel bandwidth requirement as the fundamental characteristic upon which every proposed radio technology must be based and to require that technology proposals define their RF specifications and performance statements for a specified channel bandwidth, consistent with the definitions provided in this document.

# Discussion

2.1 	Background

In the September 2003 session of the IEEE 802.20 working group, held in Singapore, a controversial proposal was incorporated into the SRD and significantly changed the previous version of section 4.1.3.

Two problems were identified in this proposal: 
(1)   Removal of the channel bandwidth requirements (established by the IEEE 802.20 PAR) leaves the evaluation process with no common ground for comparing the performance of the contending technology proposals.

(2)   Without a clear definition of channel bandwidth, the spectral efficiency calculation method may vary from one proposal to another and be difficult to assess.

The main purpose of this proposal is to show that in the absence of channel bandwidth requirements in the SRD, it would be difficult to evaluate and compare the performance of the contending technologies.

It is also recommended to include in the radio channel bandwidth some extra buffer spectrum (herein referred to as “in-channel guard bands”). This concept is illustrated in figures 2 and 3 (shown as orange bands).

2.2	Tutorial

Terminology

“channel bandwidth” is defined in conjunction with the terms “occupied bandwidth” and “necessary bandwidth” that are defined by ITU Radio Regulations S1.153 (occupied bandwidth) and S1.152 (necessary bandwidth), as follows:

occupied bandwidth: The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage B /2 of the total mean power of a given emission. Unless otherwise specified by the CCIR for the appropriate class of emission, the value of B /2 should be taken as 0.5%.

Note 1: The percentage of the total power outside the occupied bandwidth is represented by B.

Note 2: In some cases, e.g., multichannel frequency-division multiplexing systems, use of the 0.5% limits may lead to certain difficulties in the practical application of the definition of occupied and necessary bandwidth; in such cases, a different percentage may prove useful.“

necessary bandwidth: For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.

channel bandwidth is defined as the spectrum required by one channel and contains the occupied bandwidth plus some amount of buffer spectrum necessary to meet the radio performance specifications in same-technology, adjacent channels deployment.

Note:  Typically, a radio’s channel bandwidth is the same size as a licensed block’s channel spacing, but there are known exceptions to this rule.

The term channel bandwidth refers to a fundamental radio characteristic for which the essential performance characteristics are specified. 

The term channel spacing is a deployment variable which is associated with a 
specific spectrum block assignment.

In common RF engineering practice and in standards development, a given radio technology’s spectral efficiency is defined for its specified channel bandwidth. 

In actual deployment, though, the specified spectral efficiency may be smaller than
the specified when the channel-spacing is greater than the radio channel-bandwidth.

Illustrations

The concepts defined above are clarified in the following illustrations.

Figure 1 shows how a given block assignment, in a regulatory environment that permits changing the channel spacing, accommodates two different technologies, A and B. Note the different channel spacing and block-edge guard-bands. Typically, the channel spacing is chosen such that it is greater or equal to the corresponding radio technology’s channel-bandwidth.

Figure 2 illustrates the concepts of occupied-bandwidth, channel-bandwidth and channel-spacing. In case 1, the channel spacing is equal to the radio’s channel bandwidth and the occupied bandwidth. Case 2 is similar to case 1 except that the channel bandwidth includes in-channel guard bands.

Case 3 is shown in figure 3. Here, the radio’s channel-bandwidth is smaller than the  channel spacing, effectively getting an increased the channel to channel guard bands and providing a greater adjacent-channel protection. The actual spectral efficiency, in this case, is smaller than the specified.

Block Assignment

Radio Technology A

Radio Technology B

Block-edge Guard Band

In-channel Guard Band (figures 2 and 3)

Key:

Figure 1:  Two Radio Technologies, Same Block Assignment, Different Channel-Bandwidth, Channel Spacing and Block-edge Guard-bands

Occupied Bandwidth

Case 1:  Channel Spacing = Channel BW = Occupied BW

Case 2:  Channel Spacing = Channel BW= Occupied BW + Guard-bands

Channel Bandwidth

Occupied Bandwidth

F1

F2

F3

F4

F5

F6

F1

F2

F3

F4

Figure 2: Occupied-Bandwidth vs. Channel-Bandwidth vs. Channel-Spacing

Block Assignment

Channel Bandwidth

Occupied Bandwidth

F1

F3

F2

F4

Channel spacing

Case 3:  Channel Spacing > Channel BW

Figure 3:  An Example of Block Assignment with Channel-Spacing Greater Than Channel-BW

  - Recommendations

- All RF performance characteristics of the 802.20 radio transmitter and receiver should be specified for a given stated channel bandwidth.

- The 802.20 radio channel bandwidth should be greater than its occupied bandwidth and include some in-channel guard bands required to insure the
RF performance specifications in the intended deployment environment.

- The 802.20 standard should support 1.25 MHz and 5 MHz channel bandwidths. Additional, wider channel bandwidths may be proposed.

- Block-edge guard-bands which are necessary to minimize interference with other services (deployed in adjacent bands/channels) may also be required in actual deployments of 802.20 systems. These guard bands should be defined in the 802.20 standard in conjunction with specific regional regulatory requirements.

# Proposed Text for the SRD Sections 4.1.2, 4.1.3

Reference: IEEE 802.20 SRD v10

### [Section] 4.1.2  Support for Different Block Assignments (open)

“channel bandwidth” is defined in conjunction with the terms “occupied bandwidth” and “necessary bandwidth” that are defined by ITU Radio Regulations S1.153 (occupied bandwidth) and S1.152 (necessary bandwidth), as follows:

occupied bandwidth: The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage B /2 of the total mean power of a given emission. Unless otherwise specified in an ITU-R Recommendation for the  appropriate class of emission, the value of B /2 should be taken as 0.5%.

Note 1: The percentage of the total power outside the occupied bandwidth is represented by B.

Note 2: In some cases, e.g., multichannel frequency-division multiplexing systems, use of the 0.5% limits may lead to certain difficulties in the practical application of the definition of occupied and necessary bandwidth; in such cases, a different percentage may prove useful.“

[necessary bandwidth: For a given class of emission, the width of the frequency band which is just sufficient to ensure the transmission of information at the rate and with the quality required under specified conditions.]

{Alternative definition 1:} channel bandwidth is defined as the spectrum required by one channel and contains the occupied bandwidth plus some amount of buffer spectrum necessary to meet the radio performance specifications in same-technology, adjacent channels deployment. The concept is depicted in the following figure.

Occupied Bandwidth

Channel Bandwidth

Note: In this document, the extra buffer spectrum included in a radio channel bandwidth is referred to as “in-channel guard-bands”.

{Alternative definition 2:} channel bandwidth is defined as the occupied bandwidth of a single channel.

{Alternative definition 1:} A block assignment, which may consist of paired or unpaired spectrum, is the block of licensed spectrum assigned to an individual operator. It shall be assumed that the spectrum adjacent to the block assignment is assigned to a different network operator and that at the edges of the block assignment the appropriate out of band emission limits will apply (for example, the limits defined in Part 24.238 for PCS).

{Alternative definition 2:} block assignment is defined as the authorized operating frequency range.

A block assignment is subject to the applicable regulatory emission limits. As an example, the FCC rules for the US PCS band (defined in CFR 47, Part 24 Section 238) requires:

“Sec. 24.238  Emission limitations for Broadband PCS equipment.

The rules in this section govern the spectral characteristics of emissions in the Broadband Personal Communications Service.

(a) Out of band emissions. The power of any emission outside of the

authorized operating frequency ranges must be attenuated below the

transmitting power (P) by a factor of at least 43 + 10 log(P) dB.”

Radio channel-bandwidths specified by the IEEE 802.20 standard should support deployment in the following block assignment sizes:

| FDD Assignments | 2 x 1.25 MHz 2 x 5 MHz 2 x 10 MHz 2x15 MHz 2 x 20 MHz |
| --- | --- |
| TDD Assignments | 2.5 MHz 5 MHz 10 MHz 20 MHz 30 MHz 40 MHz |

Notes:

1. Additional block assignment sizes may be defined in the IEEE 802.20 standard.

2. The individual 802.20 technology proposals may optimize their MAC and PHY designs for specific bandwidth and Duplexing schemes.

3. This section is not intended to specify a particular channel bandwidth. Proposals do not need to fit into all block assignments.

### [section] 4.1.3   System Spectral Efficiency (bps/Hz/sector) (open)

[Sustained spectral efficiency is computed in a loaded multi-cellular network setting. It is defined as the ratio of the expected aggregate throughput (taking out all PHY/MAC overhead) to all users in an interior cell divided by the system bandwidth. The sustained spectral efficiency calculation shall assume that users are distributed uniformly throughout the network and shall include a specification of the minimum expected data rate/user.]

[Downlink > 2 bps/Hz/sector]

[Uplink >1 bps/Hz/sector]

editor’s note: Below is the text that was developed at the November Plenary meeting.

- The system spectral efficiency of the 802.20 air interface shall be quoted for the case of a three sector baseline configuration. It shall be computed in a loaded multi-cellular network setting, which shall be simulated based on the methodology established by the 802.20 evaluation criteria group. It shall consider among other factors a minimum expected data rate/user and/or other fairness criteria, and percentage of throughput due to duplicated information flow. The values shall be quoted on a b/s/Hz/sector basis. The system spectral efficiency of the 802.20 air interface shall be greater than X b/s/Hz/sector.

[Downlink > 2 bps/Hz/sector]

[Uplink >1 bps/Hz/sector]

The system spectral efficiency (expressed as b/s/Hz/sector) is defined as the aggregate throughput per sector, divided by the block size.

The aggregate throughput is defined as the total throughput to all users in the system (user payload only).

Since the base configuration is only required for the purpose of comparing system spectral efficiency, proposals may submit deployment models over and beyond the base configuration.