| Project | IEEE 802.20 Working Group on Mobile Broadband Wireless Access <http://grouper.ieee.org/groups/802/20/> |  |
| --- | --- | --- |
| Title | Proposed Text for Noise Modeling in Evaluation Criteria document |  |
| Date Submitted | 2004-11-16 |  |
| Source(s) | David Huo 67 Whippany Road Whippany, NJ 07981 | Voice: +1 973 428 7787 Fax: +1 973 386 4555 Email: dhuo@lucent.com |
| Re: | Evaluation criteria. |  |
| Abstract | This document proposes a text for noise model according to the proposal made in C802.20-04-81. It is anticipated to be incorporated into the Evaluation Document of ieee802.20. |  |
| Purpose | Discuss and adopt. |  |
| Notice | This document has been prepared to assist the IEEE 802.20 Working Group. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.20. |  |
| Patent Policy | The contributor is familiar with IEEE patent policy, as outlined in Section 6.3 of the IEEE-SA Standards Board Operations Manual <http://standards.ieee.org/guides/opman/sect6.html#6.3> and in Understanding Patent Issues During IEEE Standards Development <http://standards.ieee.org/board/pat/guide.html>. |  |

Chapter xx. Characterization of the Additive Noise of the Receiver.

Additive noises received by the mobile receiver have various sources.  They can be characterized in two classes: exterior noises and interior noises. For the evaluation of the performance of a given system, the exterior noises can be assumed  known  as it has to be make the same for different proposals for the purpose of comparison. The interior noise, which is generated within the receiver, depends on the specifics of the system design, and, as such, requires separate description.  The interior noise can be further decomposed into analog noise and digital noise. While the analog noise comprises shot noise and the thermal noise, the digital noise includes noise caused by the digital processing of the received signal.   The scenario is described in the following figure, where S(k) and N(k) denote the symbol power and noise power, respectively, at discrete time k:

Channel

Analog Noise

Digital

S(k)

Exterior Noise

Shot Noise +

Thermal Noise

DSP

S(k)+N(k)

To capture the interior noise it is important that a common approach be introduced, so that the specific methods used to determine the digital noise can be examined and numbers generated by the methods be compared, when a comparison is applicable. For this purpose, the method of noise figure is used to characterize the interior noises. This method allows for decomposition of the interior noise into further detailed modular components, so that the details of each component can be described and evaluated properly and the noise figure of the entire system can be computed based on the noise figures  of  all components.

By the method of noise figure, a system is decomposed into concatenated sub-systems, each corresponding to a given noise source that is either physically identifiable or mathematically necessary.  The noise  power Ni of each sub-system is given according to the design specific parameters and, when applicable, applied algorithms, where the index   i denotes a sub-system  for i=1,2,3,.... The corresponding noise figure of this sub-system is defined by

Fi=1+SGNi

where S   and G    refers to the transmitted digital symbol power and the power gain of the receiver, respectively.  Then, the noise figure of a system consisting of  n subsystems  can be determined by

F=Fi+i=2n(Fi−1)GN0=1+i=1nNiGN0

Equation 1: Formula for the determination of the noise figure of a system consisting of n sub-systems

using the same parameters of the sub-systems, where N0  refers to the power of the exterior noise.

Within the frame work of noise figure, the proponents shall use the following template to provide details of their noise modeling methods and numbers. More details about the method can be found in C802.20-04-81.doc.

| Physical Meaning (i) | Ni | Fi | Assumptions | Derivations |
| --- | --- | --- | --- | --- |
| e.g. exterior noise 1 |  |  |  |  |
| e.g. exterior noise 2 |  |  |  |  |
| e.g. thermal noise |  |  |  |  |
| e.g. quantization noise |  |  |  |  |
| …. |  |  |  |  |

Table 1: template for the receiver noises

With this template filled with complete information about a specific system, it should be possible for any one skilled on the art to follow the rational of the noise models and determine the noise figure of the system via  Equation 1. , assumed  that the power  gain is given already.

Reference: C802.20-04-81.doc.