IEEE P802.21
Media Independent Handover Service

Draft Technical Requirements

Date:	Aug 17, 2004

Editor:	Vivek Gupta
	Intel Corporation

2111 NE 25th Avenue,

Hillsboro, OR 97124

Phone: 503 712 1754
	e-Mail: vivek.g.gupta@intel.com

Abstract

Draft 802.21 requirements for discussion and modification.

- Overview of Media Independent Handover Service

## Purpose

This document establishes the requirements for 802.21 and will be used in the evaluation of proposed solutions and in evaluation of the standard draft, to insure that the requirements have been met.  Proposed solutions are not limited to the requirements contained herein, but will be evaluated both on how well they meet these requirements, and any additional beneficial capabilities that they provide.

## Assumptions

The Mobile node (MN) is assumed to be multi-mode node with support for more than one interface type between which a handover can take place. The interfaces can be of the following types:

- 802.3 based wire-line type such as Ethernet

- IEEE 802.xx based wireless interfaces such as:

- 802.11

- 802.15

- 802.16

In the future other 802 based wireless interfaces such as 802.20 may also be supported.

- Other wireless interfaces as defined by Cellular standards from groups such as:

- 3GPP

- 3GPP2

- A single 802.11 interface going across ESSs is also in scope.

The Media Independent Handover (MIH) shall be addressed between any of the two interfaces defined above.

The purpose of 802.21 is to enhance user experience of mobile devices by supporting seamless handover between heterogeneous networks. A higher layer mobility protocol such as Mobile IP shall be supported for handover and seamless session continuity. This does not preclude the standard from being used to optimize handovers for other mobility protocols.

Security algorithms or security protocols shall not be defined in the specification. It is expected that security protocols of individual networks will not be compromised as a result of handover.

- Definitions

## Network Detection

The process by which a mobile terminal collects the appropriate information, detects the identity of its currently attached link, and ascertains the validity of its L2 configuration, is called Network Detection (at layer 2).

## Network Selection

The process by which a mobile terminal collects appropriate information and selects a L2 link (out of possibly many) for higher layer communication is called Network Selection.

## Seamless handover

An ideal handover across heterogeneous interfaces is where there is no change in service quality, security, and capability as a terminal moves from a source L2 network to a target L2 network. At the same time there may be handover scenarios where there is some degradation of service capability, security, or quality. A seamless handover is defined as one that is either an ideal handover or is a handover with some degradation in service parameters that are mutually acceptable to both the user at a terminal and the network that is providing the service.

## Service continuity

Transparent maintenance of an active service across wireless or wire-line coverage areas without any manual intervention.

- Functional Requirements

The following list the key functional requirements of the proposed MIH solution.

## Service Continuity

Service continuity shall be maintained within a network interface and between different network interfaces. Different service providers may provide coverage for the different network interfaces involved.

The standard shall support seamless handover (without loss of session continuity) independent of the type of coupling between different networks where the underlying media supports such capabilities.

## Application Class

The standard shall support the following classes of applications and the performance requirements associated with them. These are based on the ATM or UMTS classification system.

- Real time

- Delay sensitive

- Delay insensitive

- Best effort

Applications may also be classified as hard real-time loss tolerant, soft real time loss sensitive, lossless assured delivery and loss tolerant non real-time.

Editor’s Note: To be resolved with Yogesh

## Quality of Service (QoS)

The standard shall provide a means for obtaining QoS information of each network involved in the handover process. There shall also be means to map link-layer QoS requirements between different media access technologies. Admission control in QoS shall be taken into consideration by the proposed solution.

During handovers, if the new network is unable to support the QoS levels of the old network, the handover policy may not preclude the degradation of QoS level of session in new network for maintaining session continuity.

## Network Discovery

The standard shall provide services to the higher layers to help with network detection. The standard shall allow a mobile terminal to optimize detection of a useable attachment to a network above the LLC through reliable MAC and PHY indications (events). These events may also trigger the handover process based on the results of network discovery.

## Information Discovery (Network Selection)

The standard shall provide services to achieve optimum selection of networks prior to the handover process initiation. These services shall include mechanisms to obtain detailed information about different network elements such as link access and utilization, link quality, cost, security mechanisms, provider information and other such information elements which can aid in the handover decision making process. The standard shall enable this information exchange between the mobile terminal and the network attachment point in a standardized manner across different heterogeneous networks. These services shall be available at device attachment or at other times (background scanning) as well.

## Security

The standard shall provide a means to map security requirements according to individual access technologies. Security schemes in individual access technology shall be reused.

Mobile terminals may transition from one network interface to another network interface (different) as a result of handovers. Under such circumstances the handover shall not compromise the security of:

- Old network interface

- New network interface

- Mobile terminal

## Power Management

The standard shall support effective device power management by employing battery efficient network scanning procedures. Wherever applicable the standard shall select network scanning opportunities to coincide with scheduled wake times so as to minimize switching between active and deep sleep states.

## Handovers due to Mobile Terminal Movement

The mobile terminal can move between networks at different speeds. The proposed solution should support handovers with no measurable impact on the quality of any service when handover due to mobile terminal movement between any two networks occurs at speeds below the limits specified by the involved networks.

## Handover Policy

The standard shall be agnostic to handover policies. Handover policies shall not be defined as part of the standard.

- Architectures

This section lists the key architectural elements of the media independent handover solution.

## Layer 2.5

The layer 2.5 resides just below the IP layer and is uniform across bearer types. The L2.5 can use inputs from layer 2 such as trigger events, hints, access to information about different networks and can help in the handover decision making process. Layer 2.5 would also include inputs based on user policy and configuration that shall affect the handover process. For example 802.1x, EAP over WLAN is a case of a Layer 2.5 model wherein the different network elements can be envisaged as client, base station and deep network element (corresponding to supplicant, authenticator, and authentication server in 802.1x).

The standard shall define generic interfaces between layer 2.5 and a higher layer entity such as Mobile IP. These generic interfaces shall provide information about layer 1 (PHY}, layer 2 (MAC) and mobility management to higher layer entity such as Mobile IP. The layer 2.5 can obtain information about the lower layers and the network with the help of Events and the Information Service. There shall exist in L2.5 mechanisms for upper layers to monitor and control the status of different link in the STA. The L2.5 can get “broader” in terms of functionality and can provide many other services seamlessly to higher layers.

## Event Indication (aka Triggers)

### Need for link layer events

For fast handovers the network layer needs information from link layer so that it can re-establish connectivity as quickly as possible. The network layer needs to know from the link layer that handover is imminent or at least it has just happened. Link layer events can help anticipate user (mobile terminal) movement and help prepare the mobile terminal and network in advance. An abstraction of such event notifications from link layer with other relevant parameter information can help reduce handover latencies.

### Requirements for Events

The 802.21 standard shall provide a mechanism for signaling current and any anticipated future changes in condition of different networks in the form of events. These conditions could include changes in the MAC+PHY state or changes in certain network attributes, such as changes in load conditions. The standard shall support events with following capabilities.

- The event delivery shall be based on a push model from event source to destination (consumer). As an example the MAC layer may send a new link available event to the mobility management layer (L2.5).

- Events shall not impose any unbounded state requirements.

- Events shall be provided based on loading and congestion in network environments.

- Events shall not be restricted to supporting any limited set of handover models.

- Events shall be uniquely defined and be distinguishable across all network attachment points.

- Events shall be extensible for supporting additional event types to accommodate other handover algorithms or models.

- Events shall be defined so that they are transportable across all network attachment points.

- Events shall be defined independent of transport and transport protocol. Fragmentation/reassembly of packets carrying event information may occur.

- Events shall be defined independent of security model.

### Requirements for Event Transport

A generic transport shall be provided for delivering remote event notifications. The standard shall support transport with following capabilities.

- Provide bi-directional transfers, i.e. support event notification delivery from mobile terminal to network from terminal originated events, and from network to mobile terminal for network originated events.

- Provide registration/deregistration capabilities

- Provide data integrity

- Be fast in terms of delivery

### Event Grouping

Similar type of events may be grouped into categories as follows.

- PHY layer events

- MAC layer events

- Management events

- L3 events

- Application events

## Information Service

The standard shall provide an Information Service that provides detailed information about various networks that can assist in network discovery and selection. The information service shall be accessible from any network. It shall be possible to access the information service using the MAC + PHY of any network and by using already existing media specific transports, wherever applicable. The information service can provide access to static information.

The standard shall support the following static information elements in the information service.

- Link access parameters

- Security mechanisms

- Neighbor maps

- Location

- Provider and other Access information

- Cost of link

For example Layer 1, Layer 2 and the network shall periodically advertise some specific information. The standard shall provide mechanisms for delivery of this information by the information service.

- Handover Scenarios

The standard shall facilitate any type of cohesion or coupling between different networks as required by higher layer services. Examples of some of these cohesion mechanisms include:

- Tight coupling of 802.xx networks

- Tight coupling of cellular networks

- Tight coupling of 802.xx and cellular networks

- Loose coupling of 802.xx and cellular networks

- Loose coupling of any combination of the above networks.

Mechanisms defined to support service continuity between different networks should effectively cope with all types of coverage scenarios. Some of these scenarios can be as follows:

- Limited coverage for a network in a “sea” of coverage provided by another network or vice versa.

- Selective operation at a geographical boundary, with extensive coverage of a particular network such as IEEE 802 on one side and extensive coverage from another network on the other side.

- Geographically co-located areas of IEEE 802 coverage and another network

- There may or may not be coverage overlap between the networks involved.

The specification may not impose any restrictions or assumptions on how a service provider might deploy or operate a particular network.

Various wire-line and 802.xx wireless networks as well as different cellular networks can lead to several handover scenarios. The primary handover scenarios that shall be supported by 802.21 are listed below. However the standard shall not be restricted to these scenarios only.

## IEEE 802 Family

The standard shall provide service continuity through generic fast handoff mechanisms across different IEEE 802 networks.  At a minimum the standard shall provide solution for the following cases.

- Handover between 802.3 and 802.11 networks

- Handover between 802.3 and 802.16 networks

- Handover between 802.11 and 802.16 networks

- Handover between 802.11 and 802.11 networks, across ESSs.

## IEEE 802 and Non-802 Cellular

The standard shall provide generic fast handoff mechanisms between IEEE 802 family of networks and non-802 Cellular networks (e.g. 3GPP and 3GPP2).At a minimum the standard shall provide solution for the following cases:

- Handover between 802.3 and Cellular networks

- Handover between 802.11 and Cellular networks

- Handover between 802.16 and Cellular networks

The 802.21 standard shall facilitate handover scenarios related to WLAN-cellular inter-working as specified by Scenarios 4 and 5 in 3GPP standard.

- Reference Models

The L2.5 may support several categories of interfaces. Examples of such interfaces include:

- Management Interfaces

- Application Interfaces

- L3 signaling Interfaces

- MAC Interfaces and

- PHY Interfaces.

Each of these interfaces may provide inputs and outputs. Examples of input to Management interfaces include Load condition on bearers or SLA between bearers, while example of output from Management interface includes sensing of potential attachment point.

Similarly input from Application interfaces includes application Class of service and output can include jitter on current bearer.

Examples of input from L3 interfaces could include initiation of binding update while that of output could include that of trigger binding update.

The MAC layer interfaces can provide inputs such as Link Available triggers and can provide output such as Re-association messages.

The Phy layer interfaces can provide inputs on SNR values and output could include indication of drop of link.

## IEEE 802 Family Handover

The L2.5 functionality is shown below based on the interfaces specified in the above section. The L2.5 could include a generic transport for remote triggers and information base as well.

## IEEE 802 and Non-802 Cellular

3GPP and 3GPP2 have ownership over this.

Comment that the base station is like a bridge

Perhaps use terms/examples directly from Scenario 4 & 5

Comment that we might work only with Scenario 5?

RNC makes the decision, but must digest 802 measurements.

3GPP would accept a SAP def’n more than another “layer”

Create API’s to existing layers?

Perhaps we should structure .21 around services?

Use existing measurements as much as possible?

Key to get consensus is communicating above air link e.g. IP

Need strong, persuasive liaisons from multiple companies.

- Appendix

## References

### General

- J Manner, M Kojo, "Mobility Related Terminology ", RFC 3753, June 2004.

### Events (Triggers)

- Alper Yegin, Gang Wu, Fast Handoff for Mobile IP and Link Layer Triggers, handover ECSG IEEE contribution May 2003

- J. Kempf, D. Funato, K. El Malki, Y. Gwon, M. Pettersson, P.Roberts, H. Soliman, A. Takeshita and A. E. Yegin, Supporting Optimized Handover for IP Mobility -Requirements for Underlying Systems, draft-manyfolks-l2-mobilereq-01.txt, November 2001.

- S. Jamadagni, S. R. Pandey and Satish D, Optimized IP Mobility-Requirements for Underlying Systems, draft-satish-l2-mobilereq-00.txt, February 2002

- Alper Yegin et al, Link Layer Hints for Detecting Network Attachments, draft-yegin-dna-l2-hints-00.txt

- Rajeev Koodli, Fast Handovers for Mobile IPv6, draft-ietf-mobileip-fast-mipv6-06

- Qian Zhang, Chuanxiong Guo, Zihua Guo and Wenwu Zhu, Efficient Mobility Management for Vertical Handoff between WWAN and WLAN, IEEE Communications Magazine, November 2003.

- Alper E Yegin, Link Layer Triggers Protocol, draft-yegin-l2-triggers-00.txt

- Pat Calhoun et al, Light weight access point protocol, draft-calhoun-seamoby-lwapp-04

- Rajeev Koodli and Charles Perkins, Fast Handovers and Context Transfers in Mobile Networks, October 2001, ACM Computer Communication Review Vol. 31 No. 5

- Vivek Gupta, David Johnston, Generalized Triggers, IEEE contribution, IEEE 802.21 March 2004

### Layer 2.5

### Information base

## Glossary

3G						Third Generation

3GPP				3G Partnership Project

3GPP2				3G Partnership Project 2

AAA				Authentication, Authorization, and Accounting

AKA	   		Authentication and Key Agreement

AP						Access Point

AT						Access Terminal

ATM           Asynchronous Transfer Mode

BS						Base Station

BSSID   			Basic Service Set IDentifier

CDMA			Code Division Multiple Access

DES					Data Encryption Standard

ESS        		Extended Service Set

EAP					Extensible Authentication Protocol

EIA					Electronic Industries Association

ESP					Encapsulating Security Payload

ETSI					European Telecommunications Standards Institute

GPRS				General Packet Radio Service

GSM				Global System for Mobile Communication

IEEE				Institute of Electrical and Electronics Engineers

IETF					Internet Engineering Task Force

IP						Internet Protocol

ISP					Internet Service Provider

ITU					International Telecommunications Union

L1                Physical Layer (PHY)

L2			         Medium Access Control (MAC)

L2.5             Layer 2.5 (Mobility Management). The layer 2.5 exists below the IP layer.

LAN					Local Area Network

LLC            	Logical Link Control

LSAP          Link Service Access Point

MAC				Medium Access Control

MIH	           Media Independent Handover

MIHO        	Media Independent Handover

MIHS        	Media Independent Handover Services

MIP					Mobile IP

MLME        MAC layer management entity

MN					Mobile Node

MT		   		Mobile Terminal

PHY            Physical Layer

PLME         PHY layer management entity

PPP					Point-to-Point Protocol

QoS				   Quality of Service

RFC					Request for Comment

RLP					Radio Link Protocol

RNC					Radio Network Controller

SA						Security Association

SAP             Service Access Point

SME				   Station Management Entity

SSID				Session Specific IDentifier

TCP					Transmission Control Protocol

UDP					User Datagram Protocol

UMTS				Universal Mobile Telecommunications System

WECA			Wireless Equivalent Compatibility Alliance

WEP					Wired Equivalent Privacy

WISP				Wireless Internet Service Provider

WISPr				Wireless Internet Service Provider roaming

WLAN			Wireless Local Area Network

WSP					Wireless Service Provider

There have been informal requests from IETF for IEEE to formalize the definition of such link layer events, so that layer 3 and above protocols like IP, Mobile IP and others operate better on IEEE 802 links. Please refer to events and trigger related references in Appendix for more information.